Hook up the hardware, and run the CD included with the print server on ONE machine, and install the Print Server Admin software.  But when it asks you to configure the device, don’t bother.

- Go to the server in the back and click the Start menu.

- Select All Programs  Print Server Utilities  Print Server Configuration

- At the IP Cfg tab set the IP address of the print server.  Make sure if there’s a router, that the IP is within that router’s range, and that the router is configured with that static IP.

- Go back to the Server, click the Start menu.

- Select Printers and Faxes.

- In that window, click Add a Printer on the left hand side.

- Click Next, then select “Local printer attached to this computer”.  UNCHECK “Automatically detect and install my Plug and Play printer.”  We don’t want that.  Click next.

- Click on “Create a new port:” and select “Standard TCP/IP Port” from the list.  Click next.

- Click next.  In the “Printer Name or IP Address:” box you will type in the IP address you entered in step 3.  Click Next.

- Click Next at all the rest of the screens until you get to the final screen then click Finish.

- It’ll go through some configurations, then it’ll ask you to choose a Manufacturer and Printer.  Choose Epson for the Manufacturer, and EPSON TM-T88II Partial Cut from the printers list.  Click Next.

- If it asks, go ahead and keep the existing driver.  Click next.

- Under “Printer name:” name the printer.  This name must be the same for ALL stations, and in the Profits software.  Choose NO for the default printer.  Click next.

- Select “Do not share this printer”.  Click next.

- Yes, print a test page.  Click next.

- Click finish.  A test page will now print.  When it does, click OK.

- Repeat steps 4 – 16 for every station.