Sélectionnez une vue "plein écran"                                                                                                                                                                                                                            Select "full screen" view

LISTE DE VÉRIFICATIONS PA27

A- Preparation

1.  Airplane status ..................................................................................   airworthy, papers on board

2. Wheather / Notams ............................................................................   suitable

3. Baggage ...............................................................................................   weighted, stowed and tied

4. Weight and Balance ...........................................................................  within limits

5. Navigation ............................................................................................   planned

6. Charts & Nav equipement .................................................................. on board

7. Performance & Range .......................................................................  computed and safe

B- Preflight check

Inside cabin:

1. Landing gear control ......................................................................... DOWN position

2. Avionics master switch ..................................................................... OFF

3. Master switch ..................................................................................... ON

4. Landing gear lights ..........................................................................  3 green

5. Fuel quantity ......................................................................................  adequate

6. Cowl flaps ...........................................................................................  open

7. Master switch ....................................................................................  OFF

8. Ignition switches ..............................................................................   OFF

9. Mixture controls ..............................................................................   idle-cut-off

10. Trim indicators ................................................................................  neutral

11. Wing flaps ........................................................................................   check operation

12. Control lock ......................................................................................  remove

13. Controls .............................................................................................  free

14. Pitot and static systems ................................................................... drain

15.  Emergency exit latch ......................................................................  locked

16. Fire extinguisher .............................................................................  secured and within range

17. First aid kit .......................................................................................  on board

18. Alternate static source ................................................................... OFF

19. Parking brakes ...............................................................................  set

20. ELT ....................................................................................................  secured and ARMED

Outside cabin:

1. Fuel sump drains ............................................................................ drain, check fuek color

2. Right wing, aileron and flap ......................................................... check, no ice

3. Right landing gear ......................................................................... check, piston 2.6", no leaks, squat switch

4. Strut .................................................................................................. proper inflation

5. Tire ................................................................................................... check

6. Right wing tip ................................................................................. check

7. Right leading edge ......................................................................... check, no ice

8. Fuel cap ............................................................................................ open, check quantity and secure

9. Right engine nacelle ...................................................................... check oil

10. Right propeller ............................................................................... check for nicks and cracks

11. Cowl flaps ........................................................................................ check opened and secure

12. Nose section .................................................................................... check, check air traps

13. Nose strut ........................................................................................ proper inflation 2.7"

14. Nose tire .......................................................................................... check

15. Tow bar ............................................................................................ removed and stowed

16. Landing light .................................................................................. check

17. Windshield ...................................................................................... check and clean

18. Left wing, engine nacelle and landing gear .............................. check as on right side

19. Stall warning vanes........................................................................ check

20. Pitot/static mast ............................................................................. check

21. Antennas ......................................................................................... check

22. Empennage ...................................................................................... check, no ice

23. Dorsal fin air scoop ....................................................................... clear

24. Stabilator ........................................................................................  free

25. Navigation, landing and strobe lights .......................................  check with master switch ON

26. Baggage door ................................................................................   secured and locked

Note: Check pitot heating by hand for normal operation during walk

around with master switch ON, not more than 2 minutes.

C- Before starting engines

1. Cabin door ...................................................................................... close and latch

2. Tachometer reading ..................................................................... noted

3. Passenger briefing ....................................................................... carried (see below)

4. Seats ................................................................................................ adjust

5. Seat belts and harnesses ............................................................. fasten, adjust and test

6. Circuit brakers ............................................................................  IN

7. Master avionics switch ................................................................ OFF

8. Cowl flaps ......................................................................................  OPENED

9. Carburator heat ...........................................................................  OFF

10. Wing flaps ....................................................................................  UP

11. Parking brakes ........................................................................... recheck

Passenger briefing:   Emergency exit

Entrance door

Fire extinguisher

First aid kit

Seat belt and shoulder harness

ELT

Smoking

Questions?

D- Starting engines (left engine first)

1. Fuel selectors ............................................................................... ON

2. Mixtures and pitch ....................................................................... Full forward

3. Throttle .......................................................................................... 1/4 inch opened

4. Primer ............................................................................................ as required ( both engines )

5. Ignition switches .......................................................................... ON ( both engines )

6. Master switch ............................................................................... ON

7. Electric fuel pumps ...................................................................... ON, check fuel pressure (right pump then off, left pump ON)

8. Propeller .......................................................................................  CLEAR

9. Starter ...........................................................................................  Engage

10. Throttle .......................................................................................... Adjust  1000-1200 RPM

11. Oil pressure .................................................................................. Check ( in the green within 30 sec or shutdown)

12. Fuel pump ......................................................................................  OFF, check fuel pressure

13. Mixture  ......................................................................................... Lean a bit ( specially on hot days )

14. Alternator ..................................................................................... ON, check ammeter

15. Repeat for opposite engine from # 7

16. Gyro suction ................................................................................. 4.8" - 5.2" (red inoperative )

17. strobe lights .................................................................................. ON

18. Heather and defroster/ fan ......................................................... as required

E- Pre-taxi checks

1. Master avionic switch ................................................................. ON

2. Call dispatch ................................................................................ 123.35 Mhz

3. Listen ATIS ..................................................................................

4. Altimeter error ............................................................................ noted

5. Transponder ................................................................................. check / stby

6. D.G. ( gyro ) .................................................................................. check slave control and slave

F- Avionics

1. Select cabin speaker or phone................................................... as required

2. Com 1............................................................................................. ACTIVE on twr freq, STBY on dep freq

Com 2 ........................................................................................... ground  freq

3. Nav 1 ............................................................................................. VOT then set for emergency return

Nav 2 ............................................................................................. VOT then set for enroute navigation

ADF ............................................................................................... Test with close station then set

4. Timer............................................................................................. check

5. Marker beacon lights ................................................................ Test and set low, audio switch ON

6. DME .............................................................................................. Set

G- Clearances

1. IFR clearance .............................................................................. on clearance delivery or ground freq, copy , readback

2. Taxi clearance and time check ................................................ on ground freq

3. Set navaids .................................................................................. as per IFR clearance

H- Taxiing

1. Parking brakes ......................................................................... released

2. Fuel selectors ............................................................................ on CROSSFEED

3. Taxi area ..................................................................................... Clear

4. Throttles .................................................................................... Apply slowly

5. Brakes ....................................................................................... Check

6. Steering ....................................................................................  Check

7. Flight instruments ..................................................................  Check

I- Run-up and ground checks

1. Nosewheel ................................................................................ Straight

2. Parking brakes ...................................................................... Set

3. Landing and taxi lights ......................................................... as required

4. Fuel selectors ......................................................................... ON

5. Instrument panel ................................................................... Check from L to R

6. Mixture and prop control ..................................................... Forward

7. Cowl flaps ............................................................................... Open

8. Engine gauges ......................................................................  In the green

9. Throttle ...................................................................................  1500 RPM

10. Carburetor heat ..................................................................... ON then OFF

11. Prop control ........................................................................... Check feathering, 500 RPM max drop

12. Throttle controls ..................................................................  1700 RPM

13. Check governors .................................................................  decrease 100-200 RPM (with prop control)

increase MP slightly, RPM should be steady

14. Magnetos ............................................................................... check, Max drop 175, max difference 50

15. Mixture cut-off ...................................................................... check

16. EGT ......................................................................................... check operation

17. Gyro suction gauge ............................................................. 4.8"- 5.2" Hg

18. Alternators output .............................................................. check with pitot

19. Throttle.................................................................................. Idle check then 1000-1200 RPM

20. Mixtures ................................................................................ Lean slightly

21. Annunciator panel ..............................................................  Press-to-test

22. Warning lights ...................................................................  Press-to-test

J- Before takeoff

1. Electric fuel pumps ............................................................. On, check fuel pressure

2. Alternators ........................................................................... On and check

3. Altimeter .............................................................................. Set

4. Attitude indicator ...............................................................  Set

5. Auto pilot .............................................................................  Verify and OFF

6. Primers ................................................................................  Locked

7. Quadrant friction ................................................................. Adjust

8. Cowl flaps .............................................................................. open

9. Wing flaps ............................................................................. Set ( 0 degrees for normal T/O )

10. Trim tabs/ Electric trim ...................................................... Set T/O range

11. Seat belts and harness ........................................................ Fasten

12. Controls ................................................................................. Free, full travel

13. Door and window ................................................................... Closed and latched

14. Review IFR clearance .......................................................... SID, route, altitudes

15. Takeoff briefing .................................................................... Carried (see below )

Takeoff briefing single pilot :

Normal takeoff rwy___. In the event of a malfunction before Vr 80kias, CLOSE both

throttles immediately and stop straight ahead. If above 80 kias, airborne and sufficient

rwy remains, CLOSE both throttles and land on rwy. If above 80 kias, airborne and

commited to fly ( no more rwy ) , accelerate to 95kts ( blue line ), FEATHER INOPERATIVE

ENGINE. Circuit to rwy____ . DECLARE EMERGENCY, REQUEST PRIORITY LDG.

Takeoff briefing with co-pilot :

Normal takeoff rwy____. Monitor engine instruments on the roll and call Vmc 61kias

and Vr 80 kias. In the event of a malfunction before Vr 80 kias, state it clearly and we

will abort. In the event of a malfunction after Vr 80 kias and commited to fly ( no rwy)

we will continue the climb on the SINGLE ENGINE CLIMB PROCEDURE at a speed

of  95 kias ( blue line ). Circuit to the  L/R to rwy___. You will DECLARE EMERGENCY

AND REQUEST PRIORITY LDG.

K- On the runway (line-up check)

1. Runway ..................................................................................... Clear

2. Compass and D.G. ................................................................... Check , set

3. LOC-ADF-DME ......................................................................  Check

4. Landing light ..........................................................................  As required

5. Strobe light .............................................................................  As required

6. Pitot heat .................................................................................  As required

7. Transponder ........................................................................... ON, Mode C

8. Time up.................................................................................... noted

9. Timer ....................................................................................... ON

L- Takeoff

1. Mixtures, pitch, throttle ....................................................... Full forward

2. Monitor MP and RPM .......................................................... Call NORMAL

3. Call    Vmc .............................................................................. 61 kias

4. Call    Vr ................................................................................. 80 kias

M- After takeoff checks

1. Positive rate-of-climb

2. Accelerate to V2 ( Vx or Vy )

3. Brakes .................................................................................. Apply

4. Gear ...................................................................................... UP

5. Wing flaps ........................................................................... UP on schedule

6. NO turn below 500' AGL

N- Climb checks

1. Climb power ........................................................................ Set 25"/2500 RPM

2. Engine gauges .................................................................... in the green

3. Fuel pumps .......................................................................... OFF at 1000', one at a time, check fuel pressure

4. Landing light ...................................................................... OFF above 2000'

5. Cowl flaps ............................................................................ check opened

6. Engine nacelles .................................................................. check for oil leaks or fire

7. Outside checks .................................................................. for trafic

8. Nav-aids identification ..................................................... VOR and ADF

9. Use NAV 1 as a backup for navigation ......................... identification

10. Lean mixture to 17 GPH/engine above 3000'

O- Cruise checks

1. Power adjusted .................................................................. set per pwr table ( around 23"/ 2200 RPM)

2. Mixtures ............................................................................ Lean to BPM or BEM

3. Cowl flaps .......................................................................... Closed

4. Engine gauges .................................................................. Check in the green

5. Report level

6. Passengers......................................................................... Check

P- IFR approach briefing

A-P-R-A-L-D-I-R

A- ATIS

P- Plate (get the appropriate appch plate)

R- Radio set up for appch and missed appch(appropriate to type of appch)

A- Altitudes

MSA/MEA/MOCA/emergency safe altitude 100nm

L- Landing information

Altimeter setting-ceiling-wind-confirm straight in, circling or full appch

D- D.G. check

Slaved-compass-flags

I- In range checks

Descent checks ( se check list )

Adjust speed in order to give yourself time to prepare the appch

R-Read out of plate

Course and headings-wind crab

Altitudes- MSA, PT. OM, DH or MDA

Timing from FAF to MAP

VDP (Visual descent point )

Missed appch instructions

Q- Descent checks

1. Seat belts and harness....................................................................  Fasten

2. Engine gauges ................................................................................. Check in the green

3. Mixtures ...........................................................................................  Adjust with descent

4. Carburator heat .............................................................................  ON then OFF

5. Cowl flaps .......................................................................................   Closed

6. Fuel selectors ................................................................................  ON

7. NO SMOKING

R- Landing checks

1. Electric fuel pumps  ..................................................................... ON

2. Landing light ................................................................................ ON

3. Gear warning horn ...................................................................... Check

4. Gear speed ( 128 kias ) ............................................................... GEAR DOWN- 3 greens

5. Flaps speed ( 109 kias ) .............................................................. Flaps down 1 notch

6. Brake pressure ........................................................................... Check

S- Landing Final

1. Mixtures and pitch .................................................................... Full forward after power reduction just before flare

2. Wing flaps .................................................................................. As required

3. Speed ........................................................................................... 90 kias (normal), 75 kias ( short field )

4. Recheck gear down ................................................................... 3 greens and left engine nacelle mirror(nose wheel)

T- Go around

1. Mixtures, Pitch , Throttles ...................................................... Full forward

2. Positive Rate-of-climb................................................................ Confirmed(with VSI gauge and/or altimeter)

3. Gear  ...........................................................................................  UP Not above 110 kias for gear retraction

4. Flaps............................................................................................. UP (at 90kias min)

5. Cowl flaps ................................................................................... Open

6. Control ....................................................................................... Hdg, airspeed and altitude

7. Carry-on climb checks ........................................................... see N

U- Post landing checks

1. Flaps ......................................................................................... UP

2. Transponder ........................................................................... STBY

3. Landing light ( day ) .............................................................. OFF

4. Fuel pumps .............................................................................  OFF

5. Nav/Strobe/Pitot heat ........................................................... as required

6. Carburator heat .................................................................... OFF

7. Cowl flaps .............................................................................. OPEN

8. Heather and defroster or fan ............................................. Ground operation

9. Time down ............................................................................. Noted

V- Engine shutdown

1. On the company ramp ........................................................ Call dispatch 123.35

2. Listen to 121.5 for ELT signal

3. Master avionic switch ....................................................... OFF

4. Dead mags check ............................................................... at 1000 RPM

5. Mixtures .............................................................................. idle-cut-off

6. Master switch and strobe lights ....................................... OFF

7. Ignition switches ................................................................. OFF

8. Alternators .......................................................................... OFF

9. Cowl flaps ............................................................................. Closed

10. Parking brakes ................................................................. DO NOT set if  brakes are hot

11. Control lock ....................................................................... in position

POWER SETTINGS AND SPEEDS:

Takeoff:

Full MP / Full RPM , rotate at 80kts.

Climb:        Establish positive rate of climb and retract gear then flaps

25" MP / 2500 RPM , climb at around 130 kias. advance throttle

throughout the climb to maintain 25" MP until full throttle.

Lean mixture to 17 GPH/engine above 3000'.

Cruise:

23" MP / 2200 RPM , mixture leaned around 15 GPH/engine

Enroute descent:

20" MP / 2200 RPM, adjust MP with descent to maintain

20", enrich mixture with descent, full rich at 3000'

and below. Rate of descent should be set at 700 fpm.

Normal descent:

15" MP / 2200 RPM, adjust MP with descent to maintain

15", enrich mixture with descent, full rich at 3000'

and below. Rate of descent set to 1000-1200 fpm.

Holding and IFR approach :

18" MP / 2200 RPM , 160 kias(depending on altitude)

Circuit/appch/landing:

on downwind and abeam the threshold, reduce power to

idle MP / 2200 RPM , at128 kts lower the gear  and then

increase MP to 15", speed will decrease, at 109 kts set flaps

to 1 notch and start turning for base leg and descending.

Set 2 notch of flaps on base and increase MP to 15-20", hold

speed around 95 kts. Make the turn to final, set flaps

to full (except for night landings) and increase MP between

15  to 25 as required,90kts on final, speed should be adjusted to overfly

the treshold at 80 kts (more if turbulence or windshear is present).

at 10-20' abv rwy reduce power to idle, increase pitch(RPM)

to full, touchdown as close as possible to stall speed, pull

stick full back and apply brakes as necessary, retract flaps

as soon as possible, specially during crosswind conditions.

Last updated june 20th 1999

Frédérick Thivierge

Air Québec Central

Président, directeur des opérations

Air Québec Central 1999, Tous droits réservés