Department of Manufacturing Engineering

College of Technological Studies

PAAET, Kuwait

Practical (Workshop) Sheets

Course:	Machining 1 (64/130)

Student Name: 		                      		ID Number:

Group No.:

Semester and Academic Year:   [        -         -        ]

Lecturer Name:                                     Instructor Name:

Exercise 1: General Exercise for Machining 1 Course (64/130) For Turning & Related Operations

Required:

Machine the product shown in the figure according to the following conditions:

- Workpiece Material: Carbon Steel.

- Dimensions: According to material availability.

Primary Objectives:

  - Practicing on turning operations especially centering, facing, grooving, plain turning, external and internal threading, drilling and boring, and external taper (using different methods).

Secondary Objectives:

During machining, the following responses should be signified:

- Safety Regulations are followed.

- Variation in surface finish and dimensional accuracy due to speed and feed change.

- The use of as many universal accessories as possible (Lathe Dog, Taper Attachment, Faceplate, etc.).

D

### F

## E

## C

### G

A

505

755

100

8

455

955

150

80

250

Descriptions:

A	Facing – Plain Turning – External ISO M32×2 mm threading.

C	Plain Turning (Rough and Finish).

D	Recessing using form tool (2×8 mm).

E	Knurling.

F	External taper using tailstock.

G	Drilling, Boring, and internal threading M50×2 mm.

PROCESS SHEET (Exercise 1: Turining)

| Seq. | Operation | Sketch | Cutting Conditions |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Speed m/min | Feed mm/rev | DOC mm |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 11 12 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |

Exercise 2: General Exercise for Machining 1 Course (64/130) For Drilling and Related Operations

Required:

Machine the four holes shown in the figure according to the following conditions:

- Workpiece Material: MILD STEEL.

- Dimensions: According to material availability (preferably 80 * 50 mm).

- Thread (internal smaller diameter M10×2 mm.

- Boring diameter 16 mm.

Primary Objectives:

  - Practicing drilling, boring, and countersinking operations.

  - Practicing tabbing.

Secondary Objectives:

During machining, the following responses should be signified:

- Safety Regulations are followed.

- Variation in surface finish and dimensional accuracy due to speed and feed change.

- The use of as many universal accessories as possible.

PROCESS SHEET (Exercise 2: Drilling, Boring, Reaming and Tabbing)

| Seq. | Operation | Sketch | Cutting Conditions |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Speed m/min | Feed mm/rev | DOC mm |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 11 12 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |

General Notes & Remarks