Differential Thermostat   Lartec

DTL has a simple design, preset threshold with hysteresis, but whitout meter

or overheat cut off.

This could be an advantage as well, as it makes the choice of sensors huge,

because you may use almost any sensor whether it is PTC or NTC.

DTL is fitted with a standard compensation circuit (OP-circuit, operation amplifier)

LM 308 N, which works as a linear comparator of two inputs (In+) and (In-).

When the (In-) is high related to (In+) output is low.

When (In+) is high related to (In-) the output goes high.

Output is backrouted thru C1 to enable hysteresis.

To increase output power, a small darlington powertransistor BD677A is cascaded.

BD677A can hold 4Amp  collector current, but then a heat dissipatcher is needed.

As the inputs of OP-circuit are high impediant, the input impedance consist almost of R1 and R2, which allows fairly distant sensors in most cases.

The value of R1 and R2 shall be 3.3k for low impediant sensors, as the PT 100.

Using the base-collector of two matched pair of transistor BC 109 will be a

useful low cost option.

Their temperature-impedance relation is not linear, but does not affect the

function of the DTL.

As we used a predrilled, prewired laboration circuit board, the assemble

gives little trubble.

These circuit boards are matched for DIL-circuits, and a square of  2" x 1.5"

is sufficiant.

The schematics layout reflect the board layout.

Variable resistor VR1 is used for preset threshold, and the objective is to

secure that the solar collector relative temperature is on a level that  no

energy is lost due to piping or pumping effect.

4 C degree is a common value.

IMPORTANT

DTL is powered by 10 to 15 Voltage direct current.

It is dangerous, and in some cases outlaw  to connect this equippmenrt to

the main power network whitout an approved firewall.

So, you need to use low power thruout the system, unless you are

able to pick up the approved firewall.

There is of cource a lot of 12V dc battery eliminators on the market,

for the DTL-power.

Good luck

Stig