emu71eng.doc

(Short) Emu71 documentation

Presentation:

Emu71 is a software emulator of the HP71B machine.

Features:

- run under DOS (or in a DOS box in Windows).

- can emulate ROM or RAM modules.

- very fast: the emulation engine is written in optimized assembly

language.

- run on 186, 286 (and more), 640Kb only systems (no EMS/XMS needed).

- Version 1 run only in text mode.

- FREEWARE

NOTE : emu71 needs a 80186 or higher. The reason is that emu71 use heavily

the 4 bits shift opcode (eg. shr al,4) to extract the nibbles,

this opcode exist only on 80186 and following x86 processors.

Preparation for use:

emu71 use images of HP71 ROM and modules (Math, Forth, ...).

THESES FILES ARE NOT PROVIDED WITH EMU71

FOR OBVIOUS COPYRIGHT REASONS

You need to transfer these ROMs on PC. The easiest way to do it is to use

the HP9114 drive. An other way is to use a HPIL-RS232 interface,

or the HPIL card on PC:

- You have a HP9114 drive. You can make a dump on a LIF disk, then read it

on the PC with a LIF/DOS transfer utility (one is provided: vlif.exe).

The following program can do the job on the HP71:

10 DIM A$[64]

20 CREATE TEXT "ROM71:TAPE",140000

30 ASSIGN #1 TO "ROM71:TAPE"

40 A=HTD("0")   ! start address of ROM to transfer

50 FOR I=0 TO 64*32-1  ! length of ROM (here for 64Kb)

60 A$=PEEK$(DTH$(A+I*64),64)

70 PRINT #1;A$

80 NEXT I

90 ASSIGN #1 TO *

Now, read this LIF disk on the PC with the following command:

vlif ROM71 >rom71.dmp

Note that ROM71 is the name of the file created by the HP71 in UPPERCASE.

- You don't have a HP9114 or compatible drive, but you can make a link between

your HP71B and a PC with the HP82164A HPIL/RS232 converter or the HP82973A

HPIL interface card in PC (you have to know how to make these interface work).

You can dump the ROMs by a program such as this one:

10 DIM A$[64]

20 A=HTD("0")   ! start address of ROM to transfer

30 FOR I=0 TO 64*32-1  ! length of ROM (here for 64Kb)

40 A$=PEEK$(DTH$(A+I*64),64)

50 OUTPUT :1 ;A$

60 NEXT I

save the data in *.dmp files, eg: ROM71.DMP, MATHROM.DMP, ...

After the end of transfer (disc or link), you must have a ASCII file

with 64 nibbles per line named ROM71.DMP (135168 bytes long).

Use the DMP2BIN command to build the binary files *.bin that can be used by EMU71.

eg:  DMP2BIN ROM71.DMP ROM71.BIN

ROM71.BIN must be 65536 bytes long.

Repeat the same operations for all the ROMs you need.

Note that some module ROMs are protected and can't be read by the standard

PEEK function, so you must have an extra PEEK LEX such as the one in the JPC ROM.

Yoy can use the same way to transfer your own programs saved in independant RAM (IRAM).

Running the emulator:

emu71 [option]

options:

/d : call debugger at startup

/t : system opcodes trace (configuration)

/sx: slow host : 2=medium, 4=slow, 8=very slow

/fx: fast host : 1=medium, 2=fast, 4=very fast

/s option is used to ajust the emulation speed to host processor.

For a 286 8MHz, use /s8,

for a 386SX 25MHz, use /s2

For 486 and more, the emulation speed is higher than the native speed

of HP71 CPU, so don't use /s option.

/f option is used to optimize the emulation speed with fast processors

(486DX4, Pentium).

As a general rule, if the 71 clock (time$) don't follow, slow down

the emulation with the /s or /f options.

Exiting the emulator:

Normal exit by the HP71 OFF command, with automatic saving

of CPU and RAMs (main and independant).

Emergency exit (emulated CPU stuck, or user choice not to save RAMs)

by ctrl-Z, then 'q' (see debugger description below).

In case of difficulty, you can make a master reset (memory lost) by

deleting the sys71.dat file.

Performances:

A 8MHz 286 emulate the HP71 at about half speed.

A 486SX25 emulate the HP71 at about 3x speed.

Files:

emu71.exe   : emulator

vlif.exe    : utility to read LIF disc

dmp2bin.exe : utility to build *.bin files

emu71.ini   : emu71 initialisation file, see description bellow

sys71.dat   : system file (CPU registers and I/O area backup)

rom71.bin   : image of HP71 64Kb ROM (to be build by user)

ram71.bin   : 32Kb main RAM module in port 0

iram1.bin   : 32Kb independant module

Debugger:

A debugger is present in Emu71.

You call it with ctrl-Z, or with the DEBUG command if the

Forth/Assembler ROM is present (emu71 emulate the hardware debugger

at F0000 called by the DBGLEX file of this ROM).

To return to emu71, use the 'r' (run) command.

The 'q' command is used to exit from emu71. In this case,

RAM is NOT saved.

'?' can be used to get the list of all available commands.

This debugger has been written to help the debugging of emu71, it

lacks a lot of commands, you can't change the values of registers or

memory locations (because I didn't need it).

emu71.ini file

This file describe the modules plugged into the emulated machine.

Version 1 is limited to 32Kb main RAM implicitly in port 0,

not to be declared in emu71.ini.

The ROMs et independant RAMs are only limited by the addressing space

of the Saturn processor (512Kb total) and by the available DOS memory.

Each module, of any size, use 32Kb of the Saturn addressing space

(so, the modules are aligned at X0000 addresses).

Modules >32Kb aren't supported!

To add independant RAM, add a line in emu71.ini, rerun emu71,

then do FREE PORT(n).

Format of the lines in emu71.ini :

port(1-5) type(ROM,RAM,HRD) size(Kb) file

Emu71 support up to 10 softwired modules (ROM or RAM), plus one

hardwired module at E0000 (HRD).

Exemple of emu71.ini file with Math (port 1), Forth (port 2),

and 32Kb RAM (port 5) modules :

1 ROM 32 mathrom.bin

2 ROM 16 forthrom.bin

2 HRD 32 hrdforth.bin

5 RAM 32 iram1.bin

(note that the Forth module contains one 16Kb softwired ROM and

one 32Kb hardwired ROM).

Emu71 memory space layout:

00000-1FFFF : 64Kb internal ROM  (hardwired)

20000-2FFFF : I/O and display RAM

30000-3FFFF : 32Kb main RAM

40000-DFFFF : softwired modules (up to ten 32Kb modules)

E0000-EFFFF : 32Kb hardwired ROM (FORTH or HP41/71 Translator)

F0000-FFFFF : hardware debugger

HP71 Keyboard emulation:

HP71 special keys are mapped to the following PC keys:

HP71      PC

ATTN      Escape

I/R       Insert

-CHAR     Delete

<-        <-

->	    ->

v         v                cursor down

^         ^                cursor up

|<-       Home              cursor at beginning of line

->|       End               cursor at end of line

vv       Page v            cursor at bottom

^^       Page ^            cursor at top

BACK      <--               backspace

CMDS      Ctrl-Enter

-LINE     Ctrl-Delete

EDIT	    Ctrl-E

FETCH     Ctrl-F

RUN	    Ctrl-R

Display emulation (Version 1.5):

The display is emulated in text mode.

Only the first 78 characters in the output buffer (96 char.) are displayed.

Utilities:

The JFG file in the iram1.bin module provide some utilities:

SUB MAJDATE: used to setup the time system at each startup.

To be called as STARTUP "CALL MAJDATE".

SUB CLOCK: Display a running clock. Useful to verify the correct emulation

of the clock system. If the clock is running late, slow down the

emulation speed with the /s or /f options of emu71.

SUB SPEED: useful to compare the emulator speed with the real HP71.

Limitations (version 1.5):

- Only 32Kb of RAM main can be used. The CLAIM PORT and FREE PORT

commands work, but main RAM > 32Kb is unsafe (segments crossing problems).

- The main RAM is made of one 32Kb module (four 4Kb modules in the real

HP71B).

- CALC mode doesn't work (no display update).

- BEEP and GDISP commands do nothing (but GDISP$ function works!),

KEYDOWN function always return 0.

- ROM modules are not write protected (ROM image files are not affect).

- HPIL is not supported ("NO LOOP" error).

Known bugs (version 1.x):

- DATE$ function return 'ERR:Insufficient Memory'

- Constants with a decimal part are incorrectly decompiled in programs.

Exemple:

Key in the line : 10 X=1.2345

The decompiled line is displayed as : 10 X=1.2.3.4.5

This bug does not affect execution.

Author

EMU71 has been written with Borland C++ 4.5 and Turbo Assembler 3.1

by J-F Garnier, 47 allee de la Dent de Crolles, 38420 DOMENE (FRANCE).

email: jfgarnier@wanadoo.fr