JPEG Baseline Compression

Cathleen Anne S. Paras

Elc 152 B

Introduction

One of the popular formats used in digital image processing is the Joint Photographic Experts Group, commonly known as JPEG. It is used to compress images. It is a “lossy” standard, meaning the compressed image contains less information than the actual image. JPEG is intended for continuous tone images since the said images could exploit the known limitations of the human eye when it comes to color.

The JPEG standard has four variations for image compression. The variations are the following: sequential DCT mode, sequential lossless mode, progressive DCT mode and hierarchical mode. This report concentrates on the sequential DCT mode or the baseline format, the most popular mode of compression. It only supports lossy coding and analyzes the image in one top-to-bottom pass.

The beginning: color space transformation

Baseline compression begins by transforming the image from its RGB format to the luminance-chrominance color space (YCbCr or YUV).

Color space is a model for color representation in terms of intensity. The components of the color space represent the intensity values. It is usually depicted as a cube with its insides as multitudes of colors and its edges as the most saturated colors. One of the numerous color spaces is the RGB or Red-Green-Blue color space. The said color space is a 24-bit color system where each 8-bit sequence represents the value of red, green, and blue in the color. Each component can range from 0 to 255. In order to create pure green, the red and blue component must be 0 while the green component must be 255 (in coordinate form: 0,255,0). When all components are 0, black is produced. When all components are at maximum, white is seen. Any value where the three are equal yields gray. In this scheme, the RGB color space can produce almost 16 million combinations of colors. An illustration of the RGB color space is shown below

RGB color space from http://www.csbsju.edu/itservices/teaching/cspace/colors.htm

The RGB color space is the color space used by computer monitors to display images.

The YCbCr and the YUV color space, on the other hand are color spaces that arranges color in terms of its intensity. In both color spaces, the Y component represents luminance while the other two represents chrominance. Luminance is the reflection of light, signifying brightness and intensity. Chrominance defines the differences between color.

The human eye is much more sensitive to luminance than chrominance. This is the primary reason that the first step in baseline compression is the conversion of color space. Since the technique involves compression by discarding information, it is easier to know which information to retain if the colors are in terms of luminance and chrominance than if they are in RGB. In fact, most of the discarded information belongs to chrominance in order to maximize the eye’s capacity to readily see the changes in light more than the changes in color.

In order to convert from RGB to YUV or RGB to YCbCr  the following formulas are used:

Y = 0.3 R + 0.6 G + 0.1 B – 128

Cb = -0.15 R – 0.3 G + 0.45 B

Cr = 0.4375 R – 0.375 G – 0.0625 B

Y =  0.299  R + 0.587  G + 0.114  B

U = -0.1687 R - 0.3313 G + 0.5    B + 128

V =  0.5    R - 0.4187 G - 0.0813 B + 128

Notes : R, G, B and Y are unsigned 8-bit numbers while Cb and Cr are 8-bit signed numbers

Downsampling: an optional step

After transforming the RGB to YUV or YCbCr , the next step is to downsamle the obtained data. Downsampling is the first step to discarding data in JPEG. Downsampling is done by averaging the groups of components. The luminance information is left untouched (sampled at full resolution) while the chrominance component is reduced by 2:1 horizontally and 2:1 or 1:1 vertically.

The discarded information can never be recovered. This is why repeated compression of images makes the image worse. Also, since JPEG compression discards chroma data, pictures with a gradual change in tone are recommended for this type of compression. If the image to be compress contained sharp color changes such as cartoons, it is recommended that the date be compressed using the GIF format.

Downsampling is only applicable to colored images. Grayscale images are not downsampled.

Discrete Cosine Transform

After downsampling, the pixel components are then grouped into 8x8 blocks. The blocks are combined to form a Minimum Coded Unit (MCU). An image is processed one MCU at a time. Within the MCU, one 8x8 block undergoes the compression process at a time. The luminance blocks are processed first then the chrominance blocks.

An 8x8 block is first shifted by subtracting 128. This allows the values to range from -127 to 128, with the center at 0.  After shifting the blocks are transformed using Discrete Cosine Transform (DCT). An example of a 8x8 block and its shifted position is shown below

| 118 | 91 | 123 | 125 | 124 | 114 | 117 | 104 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 93 | 88 | 85 | 87 | 100 | 115 | 105 |
| 94 | 101 | 78 | 99 | 112 | 87 | 109 | 115 |
| 104 | 82 | 83 | 78 | 92 | 118 | 124 | 88 |
| 119 | 93 | 99 | 104 | 76 | 111 | 86 | 112 |
| 83 | 119 | 104 | 92 | 111 | 76 | 112 | 100 |
| 97 | 85 | 90 | 100 | 106 | 125 | 123 | 86 |
| 99 | 86 | 85 | 105 | 81 | 99 | 115 | 119 |

An example of an 8x8 block and its shifted form (from www. funducode.com)*

| -10 | -37 | -05 | -03 | -04 | -14 | -11 | -24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -28 | -35 | -40 | -43 | -41 | -28 | -13 | -23 |
| -34 | -27 | -50 | -29 | -16 | -41 | -19 | -13 |
| -24 | -46 | -45 | -50 | -36 | -10 | -04 | -40 |
| -09 | -35 | -29 | -24 | -52 | -17 | -42 | -16 |
| -45 | -09 | -24 | -36 | -17 | -52 | -16 | -28 |
| -31 | -43 | -38 | -28 | -22 | -03 | -05 | -42 |
| -29 | -42 | -43 | -23 | -47 | -29 | -13 | -09 |

The formula for DCT is :

where

for z = 0 or

for z > 0

- 2-D sample value

- 2-D DCT coefficient

The input to the algorithm is the 2D 8x8 samples. Output from the DCT is an 8x8 set of spatial frequencies representing the input section of the image. Using the example above, after DCT, the block would then be:

| -217.625 | 32.686 | -15.582 | 16.972 | -1.376 | 21.802 | -7.132 | 2.729 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15.022 | 5.594 | -3.423 | 3.146 | 7.983 | -1.074 | 3.717 | 10.550 |
| 21.737 | -8.150 | -13.483 | -6.075 | 6.168 | 13.366 | -0.490 | 3.289 |
| 23.404 | 24.748 | -19.724 | -10.992 | -5.045 | -3.321 | 7.425 | 9.585 |
| 12.124 | 4.600 | 4.754 | 4.605 | 6.875 | 20.625 | 30.096 | -12.687 |
| 14.610 | -9.347 | -27.242 | 27.260 | -9.347 | 7.631 | -4.987 | 14.890 |
| 10.343 | 6.876 | -4.990 | -29.439 | 16.523 | -18.653 | -24.266 | 11.388 |
| 13.292 | 6.131 | -5.968 | -2.067 | 20.673 | 1.179 | 14.921 | -16.733 |

(from from www. funducode.com)*

The input block compares the color data but not the intensity level. If, for example, the difference between neighboring pixel is large, the resulting transform produces a large coefficient (and vice versa). After undergoing DCT, 64 coefficients with varying sizes are produces. The resulting output represents the magnitudes of spatial frequencies of increasing order. The one with the lowest order is at the top left position (at 0,0) and it increases in each direction. The (0,0) position is the average value of the input matrix or the DC coefficient. The rest are the AC coefficients.

Visually, performing the DCT is showing a relationship between the input 8x8 block and the basis functions then recording the block’s relative strength to the basis function as an output. An alternative view is asking how much does one basis function contribute to the total discrete landscape shown by the block.(see figure below)

from http://www.planetanalog.com/showArticle.jhtml?articleID=12802332

Quantization

After performing the DCT, the values are then divided by an integer between 1 & 255 with the result being rounded to the nearest integer. This process is called quantization. The divisors to the DCT values are found in a quantization table. The values in the quantization table are chosen in such a way as that the resulting answer is the minimum value required to retain the image (meaning, the human eye is still fooled into believing that what one sees is the original image). The formula for quantization is Quantized Value  = DCT value /  Quantization Table value.

| 8 | 6 |  | 6 |  | 7 |  | 6 |  | 5 |  | 8 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 7 |  | 9 |  | 9 |  | 8 |  | 10 |  | 12 | 20 |
| 13 | 12 |  | 11 |  | 11 |  | 12 |  | 25 |  | 18 | 19 |
| 15 | 20 |  | 29 |  | 26 |  | 31 |  | 30 |  | 29 | 26 |
| 28 | 28 |  | 32 |  | 36 |  | 46 |  | 39 |  | 32 | 34 |
| 44 | 35 |  | 28 |  | 28 |  | 40 |  | 55 |  | 41 | 44 |
| 48 | 49 |  | 52 |  | 52 |  | 52 |  | 31 |  | 39 | 57 |
| 61 | 56 |  | 50 |  | 60 |  | 46 |  | 51 |  | 52 | 50 |
| -27 |  | -5 |  | 2 |  | 2 |  | 0 |  | 4 | 0 | 0 |
| 2 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 | 0 |
| 1 |  | 0 |  | -1 |  | 0 |  | 0 |  | 0 | 0 | 0 |
| 1 |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 | 0 | 0 |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 | 0 |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 | 0 |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 | 0 |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 | 0 |

Quantization table and the resulting Quantized coefficients (from from www. funducode.com)*

Arranging the results in a zig-zag

As one would notice, the quantized values have a greater chance of being zero as the spatial frequencies increases. Therefore, a process is created so that the coefficients of the matrix would be arranged in a one dimensional array of decreasing order. This process of sorting is the zig-zag process. The pattern looks like this:

The resulting matrix is then*:

| -27 | 5 | 2 | 1 | 0 | 2 | 2 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | -1 | 0 | 0 | 4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

After arranging the matrix in the zig-zag order, the DC coefficient and the AC coefficients are processed separately. The resulting DC coefficient is subtracted from the previous DC coefficient processed unless the 8x8 block is the first block to be processed.  The difference is called a “DPCM (Differential Prediction Model) difference.”  The AC coefficients meanwhile undergo run-length encoding. The principle of run-length encoding lies in the belief that when processing image information, it does not make any sense to send a “zero byte” over and over. So instead of individual coding of the zeros, groups of zeros in every “run” are encoded together. The groups are easily seen since zig-zagging the sequence pushed the zero bits together.

For example, the first “run” involves the AC coefficient 5. Run-length encoding this value would result to this interpretation: ‘the AC coefficient is 5 and there are no zero value AC coefficients before it’. Now, the fifth AC coefficient, which is 2, after its “run,” would have this interpretation: ‘the AC coefficient is 2 and there is one zero value AC coefficient before it’.

Entropy Encoding

Entropy encoding is a process wherein specific codes are used to represent the probability of certain symbols. There are two types of code length – fixed and variable. Consider the following table:

| Symbol | Bit Code |
| --- | --- |
| King | 00 |
| Queen | 01 |
| Ace | 10 |
| Joker | 11 |

This is an example of a fixed length code since there are always two bits in every code. This type of code length is useful if there is equal probability. However, real life situations are almost always not equally probable so variable-length code is used.

The non-zero AC coefficients are converted to a variable-length binary numbers that are signed (negative numbers are indicated by a 0 as the MSB). The run-length value is combined with the bit length value of the coefficients. The upper nibble is the run length and the lower nibble is the bit length. Now, if the upper and lower nibbles both read 0, this means that the rest of the AC coefficients are already zero. If the upper nibble is 15 and the lower nibble is zero, this indicates that the run-length is 16 and there is no non-zero AC coefficient.

The DC coefficient together with the AC coefficients are then entropy-encoded. This ends the compression process.

There are two ways to achieve entropy encoding. These are Huffman coding, the more popular form and arithmetic coding.

Huffman Coding

The codes used for Huffman coding are derived from a “tree” structure. First, the probabilities of an event occurring are known. They are placed in column with varying intensity levels and are arranged in a descending order of probabilities. The levels are the branches of the Huffman tree. The levels that contain smaller probabilities are grouped together. A new probability is assigned to the combined levels which is the sum of the probabilities of the branches. The process of combining the least probable branches continues until the sum of the probabilities is 1. The code tree is rearranged so that branches do not intersect with other branches. Codes are then formed by tracing the root of the tree to its last branch. An illustration of the creation of Huffman codes are shown below*.

Huffman code tree with branch crossovers

Huffman code tree rearranged

The ITU-T 81 standard has defined how Huffman tables used for JPEG encoding are derived. According to the paper:

“Huffman tables are specified in terms of a 16-byte list (BITS) giving the number of codes for each code length from1 to 16. This is followed by a list of the 8-bit symbol values (HUFFVAL), each of which is assigned a Huffman code. The symbol values are placed in the list in order of increasing code length. Code lengths greater than 16 bits are not allowed. In addition, the codes shall be generated such that the all-1-bits code word of any length is reserved as a prefix for longer code words.”

The DC coefficients are coded separately from the AC coefficients. It is remembered that the DC coefficient was processed by subtracting the current DC coefficient value to the previous one that results to the DPCM Difference. After the calculation of the difference, the value is compared to a table that would define the signal group (SG) the value belongs. The equivalent Huffman code to the signal group is defined by the JPEG standard.

The generated code using the Signal Group table is not the entire code. Several bits are still appended to the code. The appended bits will identify the sign and the complete of the code. Another table is used to know the bits that are added. Both tables are found on the next page.

| SG |  | DPCM Difference |  |  |  | Additional Bits |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | (binary) |  |  |
| 0 |  |  | 0 |  |  |  | - |  |
| 1 |  | -1 | , | 1 |  | 0 | , | 1 |
| 2 |  | -3,-2 | , | 2,3 |  | 00,01 | , | 10,11 |
| 3 |  | -7,..,-4 | , | 4,..,7 |  | 000,..,011 | , | 100,..,111 |
| 4 |  | -15,..,-8 | , | 8,..,15 |  | 0000,..,0111 | , | 1000,..,1111 |
| : |  |  | : |  |  |  | : |  |
| 16 |  |  |  | 32768 |  |  |  |  |

| SG |  | DPCM Difference |  |  |  | Precision |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DCT-based |
| 0 |  |  | 0 |  |  | 8 |
| 1 |  | -1 | , | 1 |  | 8 |
| 2 |  | -3,-2 | , | 2,3 |  | 8 |
| 3 |  | -7,..,-4 | , | 4,..,7 |  | 8 |
| 4 |  | -15,..,-8 | , | 8,..,15 |  | 8 |
| 5 |  | -31,..,-16 | , | 16,..,31 |  | 8 |
| 6 |  | -63,..,-32 | , | 32,..,63 |  | 8 |
| 7 |  | -127,..,-64 | , | 64,..,127 |  | 8 |
| 8 |  | -255,..,-128 | , | 128,..,255 |  | 8 |
| 9 |  | -511,..,-256 | , | 256,..,511 |  | 8 |
| 10 |  | -1023,..,-512 | , | 512,..,1023 |  | 8 |
| 11 |  | -2047,..,-1024 | , | 1024,..,2047 |  | 8 |

Signal Group Table and the Appended Bits table (from www. funducode.com)*

The codes defined by the JPEG standard for the DC coefficient difference are as follows

Table for luminance DC coefficient Differences

Table for chrominance DC coefficient Differences

To code the AC coefficients, symbols are selected based on the number of zeros preceding the coefficient and the range of the coefficient to be coded. The computation of the range uses the same table that gives the DC coefficient its signal group. After obtaining the number of zeros and the range, the “run-size”symbol is calculated by comparing the two values with the table below. The code is then obtained from the tables that are in the JPEG standard.

|  |  | SG |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
|  | 0 | EOB | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 0A |
|  | 1 | N/A | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 1A |
|  | 2 | N/A | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 2A |
|  | 3 | N/A | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 3A |
|  | 4 | N/A | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 4A |
|  | 5 | N/A | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 5A |
| N | 6 | N/A | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 6A |
| Z | 7 | N/A | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 7A |
|  | 8 | N/A | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 8A |
|  | 9 | N/A | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 9A |
|  | 10 | N/A | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | AA |
|  | 11 | N/A | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | BA |
|  | 12 | N/A | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | CA |
|  | 13 | N/A | D1 | D2 | D3 | D4 | D5 | D6 | D7 | D8 | D9 | DA |
|  | 14 | N/A | E1 | E2 | E3 | E4 | E5 | E6 | E7 | E8 | E9 | EA |
|  | 15 | ZRL | F1 | F2 | F3 | F4 | F5 | F6 | F7 | F8 | F9 | FA |

Table from www. funducode.com*

The symbols EOB and ZRL stands for the exceptions obtained during run-length encoding. That is, for the first case, there are no non-zero values and the second case means that there are 16 zeros in the sequence and there are more non-zero values in the remainder of the sequence. Like its, DC counterpart, additional bits must be appended to complete the code. The table for the additional AC bits is the same as the table for adding bits to the DPCM difference.

The codes defined by the JPEG standard for the AC coefficients for luminance are as follows

Arithmetic Coding

Arithmetic coding is seen as better than Huffman coding since it could represents with fewer bits more probable data sequences without any loss in compression. It is similar to Huffman in using fewer bits to represent more frequent pattern occurrences. It maps the different sequences of values in a region on an imaginary number line between 0 and 1. However, this type of coding is not frequently used since this code is protected by a patent. Although the ITU-T 81 paper has established guidelines on the implementation and use of an arithmetic decoder.

Summary

The JPEG baseline compression process takes advantage of the inability of the human eye to distinguish subtle changes in color. This allows the said method to reduce the file sizes of an image without much sacrifice on image quality. However, since this process is extremely lossy, continuous compression of an already compressed image destroys the said image.

Although the said compression method is still commonly used today, the advent of the lossless JPEG standard is now threatening its place in digital image processing.

Sources

Digital Compression and Coding of Continuous Still Tone Images – Requirements and Guidelines. ITU-T 81 Standard, 1991.

ftp://ftp.uu.net/graphics/jpeg/

http://www.csbsju.edu/itservices/teaching/c_space/colors.htm

http://developer.apple.com/documentation/mac/ACI/ACI-48.html

http://en.wikipedia.org/wiki/YCbCr

http://en.wikipedia.org/wiki/JPEG

http://www.faqs.org/faqs/compression-faq/part2/section-6.html

http://www-ipg.umds.ac.uk/d.hill/teaching/rsc0/internetIntro/jpeg.html

http://murray.newcastle.edu.au/users/students/2000/c9425349/section3.htm

Jain, Anil K. Fundamentals of Digital Image Processing. New Jersey: Prentice-Hall, Inc., c1989.

Pitas, Ioannis. Digital Image Processing Algorithms. London: Prentice-Hall International, 1993.

Wallace, Gregory K. “The JPEG Still Picture Compression Standard(revised edition).” First appeared on Communications of the ACM, April 1991.

www.funducode.com

* search keyword is “JPEG”; the webpage title is JPEG Baseline Compression

* search keyword is “JPEG”; the webpage title is JPEG Baseline Compression

* Due to space conservation, the matrix is arranged on a 8x8 manner even if it is, in reality a one dimensional matrix. The values are to be read from left to right.

* from Digital Image Processing Algorithms by Ioannis Pitas

* search keyword is “JPEG”; the webpage title is JPEG Baseline Compression

from the ITU-T 81 paper

*search keyword is “JPEG”; the webpage title is JPEG Baseline Compression

from the ITU-T 81 paper