SMART Ordering

SP to SP Interface Business Agreement

NMF 504

Issue 1.00

September 1997

ã Network Management Forum 1997.

Members of the Network Management Forum are granted limited copyright waiver, under the terms of the Agreement on Intellectual Property Rights, to reproduce this document.

# NOTICE

The Network Management Forum has made every effort to assure that the contents of this document are accurate; however, due to the inherent complexity in the design and implementation of protocols, no liability is accepted for any errors or omissions or for consequences of any use made of this document.  Under no circumstances will the Network Management Forum be liable for direct or indirect damages or any costs or losses resulting from the use of this specification.  The risk of designing and implementing products in accordance with this specification is done solely by the user of this specification.  This document may involve a claim of patent rights by one or more of the contributors to this document, pursuant to the agreement on Intellectual Property Rights between the Network Management Forum and its members.

Inquiries should be directed to the Network Management Forum office at:

1201 Mt. Kemble Avenue

Morristown, NJ  07960  USA

Tel. No.  +1 201-425-1900

# Disclaimer and Copyright Statements

This document was developed by NMF to help Service Providers communicate their needs for industry agreements to support process automation.  It is intended for use by the members of NMF and other organizations which NMF believes can help achieve its members’ goal of automated service management.  It serves as a starting point for NMF work programs directed at solving specific process automation problems.

NMF accepts no liability for any errors or omissions or for consequences of any use made of this document.  Under no circumstances will NMF be liable for direct or indirect damages or costs or losses resulting from the use of this document.  The risk of purchasing, acquiring, designing and/or implementing products in accordance with the information contained in this document is borne solely by the user of this document.

# Acknowledgments

The SMART ordering work is a genuinely collaborative effort. I would like to thank the following people for contributing their time and expertise to the production of this document.

#### The following shows the people contributed to this document in alphabetical order.

| Last Name | First Name & M. I. | Company | Country |
| --- | --- | --- | --- |
| Alexi | Charles | Tonex | USA |
| Bénain ** | Pierre-Yves | SITA | France |
| Bowman | Bill | Template SW | USA |
| Chen (Dr.) | Graham | CiTR | Australia |
| Choi (Dr.) | Young B. | SERI | Korea |
| Gotoh | Yoshinobu | NTT | Japan |
| Hussain | Rubina | Nortel | Canada |
| Johnson (Dr.) | Claudia | Siemens AG | Germany |
| Kennedy | Nick | BT | UK |
| Lumpkin * | Ali | BT | UK |
| Martin | Declan | Broadcom | Ireland |
| Randini | Marco | Swisscom | Switzterland |
| Sarasua | Toni | Bell Pacific | USA |
| Tang ( Prof.) | Adrian | UMKC | USA |
| Yoo | Jae Hyoung | Korea Telecom | Korea |

- * Previous team leader

- ** Current team leader

# Document Status

- Document History

| Version | Status | Date | Changes |
| --- | --- | --- | --- |
| 1.00 | Requirements Draft | Dec 1996 | None. This version is based on the last version of the White Paper. First cut ready for internal review within the SMART Ordering team meeting. |
| 1.01 | Requirements Draft | Jan 1997 | Update from White Paper Team review in Miami meeting (N°9) First team inputs into Chapters 4,5,6,7 |
| 1.02 | Requirements Draft | March 1997 | Update of Scenarios in Chapter 3.4 Update of Requirements in Chapter 5 Update of Requirements in Chapter 6 |
| 1.03 | Requirements Draft | June 1997 | Integrate Long Beach Comments Integrate OBF Requirements |
| 1.00 | Business Agreements | Sept 1997 | Review complete document (scenarios) and requirements with North American perspective and modify scenarios according to Gap analysis + OBF/ECIC/T1M1 |

- Outstanding issues and risks (e.g., insufficient input from Service Providers to validate the agreements in this document, lack of expertise applied from network level, willingness to implement, how to deal with multiple scenarios,  etc.)

- Contacts for this document

- SMART Ordering Chairman

- Pierre-Yves Bénain SITA R&D department

- 1041 route des Dolines 06560 Sophia Antipolis France

- Tel : +33 (0) 4 92 96 63 39 Fax : +33 (0) 4 92 96 64 92

- Email : Pierre.Benain@ nce.sita.int

- Requirements Document Editor

- Pierre-Yves Bénain SITA

- How can we obtain a copy?

If you are a member, an electronic copy can be retrieved using FTP from the NMF server (nmf.org or 207.113.26.207) using the following path : Members_only/Smart/Ordering/Workdocs/nmf504.doc

If you are not a member please contact the NMF office.

- How can we comment on the document?

Comments have to be formerly addressed to the editor for review with the SMART Ordering team. Please send your comments to the editor using email or fax (see above).

- Use of fonts

Italics: Paragraphs in italic are notes by the authors to the reader to provide additional or background information

# Table of Contents

NOTICE

Disclaimer and Copyright Statements

Acknowledgments

Document Status

Table of Contents

Preface

1. Introduction to This Document

1.1. How this document will be used

1.2. Document structure

1.3. Key terms used in this document

2. Business Objectives

2.1. Introduction to the requirements

2.2. Current environment

2.3. Business Need

2.4. Summary of Business Requirements

2.5. Expected beneficiaries

3. Scope of Interface Requirements

3.1. Process orientation

3.2. Process activities addressed

3.2.1. General

3.2.2. A Generic Ordering Model

3.3. How the interface is expected to be used

3.4. Scenarios to be addressed

3.4.1. Introduction

3.4.2. Generic Provide Order Scenario

3.4.3. Generic Change Service Scenario

3.4.4. Generic Disconnect (Release) Service (Product Instance) Scenario

4. Flow of Information

4.1. Introduction

4.1.1. Description on SP-to-SP Ordering Interface :

4.1.2. Pre-Ordering

4.1.3. Ordering

4.1.4. Tracking

4.2. Workflow Perspective

4.2.1 Pre-Ordering Workflow

4.2.2 Ordering Workflow

4.2.3 Tracking Workflow

4.3.  Event Flow Perspective

4.4.  State Model

4.4.1. Customer Order (MSP)

4.4.2 Enquiry Request

4.4.3 Service Enquiry (SSP)

4.4.4 Offer

4.4.5 Order Request

4.4.6 Subcontracted Service Order (SSP)

4.5 Relationship between actual events and states

5. Specific Requirements of the Ordering Interface

5.1. Core requirements

5.1.1. ORDFR1 : General functional requirements

5.1.2. ORDFR2 : Pre-Ordering functional requirements

5.1.3. ORDFR3 : Ordering functional requirements

5.1.4. ORDFR4 : Administration functional requirements

5.2. Out of scope requirements

6. General Technical Requirements

6.1. Performance of the interface

6.2. Security

6.2.1. Authentication

6.2.2. Access Control

6.2.3. Data Confidentiality

6.2.4. Integrity

6.2.5.Non-Repudiation

6.3. Methods of synchronization upon session failure

6.4. Inventory synchronization / Database reconciliation

6.5. Interface protocols

6.6. Nature of connection

6.7. Other General Requirements

7. Conclusions

7.1. What isn’t new?

7.2. What is new?

7.3. How is the Ordering interface different from the Service Configuration interface ?

APPENDIX A ABBREVIATIONS

APPENDIX B REFERENCES

APPENDIX C STANDARDS

The comparison between SMART and OBF/ECIC Ordering

The T1M1 approach

LIST OF FIGURES

LIST OF TABLES

# Preface

NMF is an international consortium of communications Service Providers and their suppliers, dedicated to promoting industry-wide agreements on how management information should be exchanged between a Service Provider and its Customers, its suppliers and/or other Service Providers.  Such agreements are essential to achieve a seamless, cost-efficient operation where multiple companies are involved in the fulfillment of service requests, the management of services while in use by Customers, the correction of problems that might impair service quality, and the accurate billing of the Customer for services used.

These agreements are becoming an increasing necessity as regulatory forces change the landscape for Service Providers. The addition of new value-add Service Providers such as Internet Service Providers and the development of global alliances mean that the service delivery chain is being extended beyond the traditional, monopoly environment where a single company had end-to-end control over the provision and management of Customer services.  At the same time, some companies are choosing to focus their business on network infrastructure, either providing high-capacity inter-city links to Service Providers, or providing local access to Customer sites.  Finally, as has been the case for many years, Service Providers and network operators rely on equipment suppliers’ element management systems to make it possible to manipulate the network to create Customer services, and to report meaningful information about the status of elements, in order that the network, and ultimately, Customer services, can be maintained at required levels of quality.

NMF members work together to determine how their processes should be linked, and how they should work with Customers’ and suppliers’ systems.  They work together because they believe that to solve this type of problem alone requires extensive time and money (which is better spent developing new offerings) and that proprietary solutions will not be accepted in the marketplace.  They work together because they see that in these areas, there is no competitive advantage to be gained over exchanging data in a proprietary fashion, and that, conversely, they can considerably reduce both the time to development and the risk of making bad decisions.

These requirements pertain to a very specific area in which NMF members have determined that a common, industry-wide approach is warranted.  They invite others to review these agreements, and to adopt the more specific information models and Solution Sets which will be developed to meet these requirements.  They invite suppliers to become familiar with these requirements, and to work with them on the details of how the requirements will be met through the application of standards and other industry-accepted specifications and technology.

# 1  Introduction to This Document

This document defines the requirements for the exchange of management information within a model of service management business functions documented in NMF’s Service Management Business Process Model.

The SMART Ordering work covers Telecommunication Services Order exchange between Service Providers.

As part of the SMART Ordering work, an industry gap analysis was performed and documented, identifying and describing the scope and objectives of the prevailing organizations addressing the service management process of ordering.

The resulting work intended to identify any redundancy and/or gaps of work being performed, or not being performed in industry standards organizations and other industry working groups.  The analysis also identified associated deliverables from work being carried out in the industry to ensure factual representation of the roles, responsibilities and intent of the various organizations carrying out related work.

The following represents a brief summary of the NMF SMART Ordering work as represented in the ‘gap analysis’ and captured in the NMF Business Agreement phase of the ordering work.  The full report , ‘Ordering Gap Analysis’, can be found on the NMF server:  ftp.nmf.org/MembersOnly/smart/ordering/membcont

- NMF SMART Ordering Business Requirements are implementation and service independent and focus primarily on the business issues of the general Ordering process with some general references to technical requirements.

- OBF has addressed specific services in detail.  TCIF Issue 7 has identified industry terms, the associated data elements, mapping structures, and general and service specific requirements.    NMF will leverage much of that work when the ordering team approaches their modeling phase.

- NMF SMART Ordering is the only working group addressing in significant detail the ‘Order tracking’ process. NMF SMART Ordering will formally contribute their work to related industry organizations such as OBF, ECIC, T1M1 and others.

- NMF SMART Ordering has ascertained that all requirements identified in the NMF Business Agreement for ordering are flexible enough to support multiple jurisdictions (North American, European, Asian, etc.).

- NMF SMART Ordering has begun a full analysis on the identified EAO (Electronic Access Ordering) objects and will determine any potential reuse before any object modeling by NMF SMART Ordering begins.  If the existing GDMO model can be reverse engineered into a protocol neutral/independent model using the OMT methodology and tool sanctioned by NMF, a considerable amount of time and rework may be eliminated.

- NMF SMART Ordering has agreed to map service specific North American Requirements (as identified in EAO 95-022r6, TCIF Issue 7) into SMART- Ordering scenarios to validate traceability to other industry work.

## 2	How this document will be used

It is expected that this document will be used in two ways:

- Business managers will be able to determine, using this document, whether their own requirements are fairly and completely represented by the document, such that implementation of an interface that meets the requirements would represent a good investment and a satisfactory solution to existing operational problems.

- For the companies who worked together to identify the requirements (and others as become interested in implementing solutions that meet the requirements) this document will provide sufficient information to develop an information model that meets the requirements.

## 3	Document structure

This document is structured in two main parts with seven major sections.  The first part (Sections 2,3,4) is considered informative and provides information to allow the reader to understand the more detailed requirements definitions to follow.  The second part (Sections 5 and 6) is considered normative and contains the technical requirements relative to the business requirements.  The following reflects what you will find in each section:

Section 1	Introduces the document.

Section 2	States the business objectives that are driving the need for this interface, and the high level operational requirements that must be met.

Section 3	Establishes the scope of the interface agreement, identifying the specific process(es) that are invoked, the parties to the interface, the functions that are the subject of the interface, and the types of information that are to be exchanged.

Section 4	Describes the flow of information across the interface, using multiple diagrams to represent different perspectives.

Section 5	Contains the specific requirements, in detail, and presented in such a way as they can be traced through to implementation.

Section 6	Lists any general technical requirements that may need to be considered in the development of the Information Model or the development of a Solution Set.

Section 7	Offers conclusions and insights from the team members who prepared the requirements document, including any issues they perceive, or their expectations of companies to meet the requirements.

Appendix A	Glossary of terms and acronyms, using precise definitions where they have been agreed upon by NMF.

Appendix B	Detailed information (date, document number, source) regarding any document referenced in the Requirements Document.

Appendix C	A review of work that has been done by other groups (outside the NMF) or that has been done by NMF, and which should be used in the development of the Information Model or Solution Sets that may be derived from this document.

## 4	Key terms used in this document

This section provides a short description of each term used within the requirements document outlined within the different sections to aid understanding.

Contract

A contract is a formalized business agreement between a Customer (in our case the MSP) and a service provider (the SSP).

Customer

The Customer requesting a product or service with a given Service Provider. It is possible that this could be an agent of the Customer rather than the true product or service user. The Customer will be transparent to an SSP as to the SSP the Customer will be the MSP.

Customer Order (CO)

An Order received by an MSP covering the details of the product types requested by a Customer and the actions required (i.e. Provide, Change or Cease a Product Type). This Order could be either a preliminary Order awaiting confirmation from a Customer to proceed, or it could be a confirmed Order where a Customer has requested the work to be undertaken. The preliminary Order is really a form of quotation or enquiry where sufficient details are supplied to a Customer to enable a decision to be made as to whether to proceed with a confirmed Order. It should be noted that a Customer Order could potentially contain many order items, with each order item covering a particular Product Type. The Customer Order will be internal to an MSP.

Customer Request (CR)

This covers the details made by a Customer of their requirements for a product or service capability. This covers the situation where the Customer does not know the exact product type that they require, but only what capabilities that product must meet. The Customer Request will be internal to an MSP.

Enquiry Request (ER)

The request for a Service Enquiry passed from the MSP to the SSP detailing the circumstances of the enquiry, including the Product Type required

Service Provider (SP)

A Service Provider in the context of NMF SMART Ordering is a commercial organisation aiming at delivering a telecommunication service required by a Customer.

Main Service Provider (MSP)

The Service Provider who has received the request from the Customer and needs to evaluate how best to meet the request.

Subcontracted Service Provider (SSP) (or Secondary Service Provider)

The Service Provider who has been subcontracted by the MSP to deliver the original Customer request fully or a component of the request.

Offer

Details an offer made by an SSP to an MSP in response to a Service Enquiry. It will contain details of the Option selected by the SSP to meet the Service Enquiry. If the Service Enquiry cannot be fulfilled by the SSP, then an Offer is not created and the MSP is advised that the Service Enquiry cannot be fulfilled.

Option

Details of an option considered by an SSP to meet a Service Enquiry. A given Service Enquiry could have more than one Option, but one of these will be selected by the SSP and returned with the Offer to the MSP. The Options considered will be internal to an SSP and the preferred Option chosen by an SSP will be included as part of the Offer to an MSP.

Order

Details of a request from a customer to a service provider to undertake work on a product or service offered by the service provider.

Order Request (OR)

The request for an Order passed from the MSP to the SSP detailing the Product Type required. It will either be cross-referenced to the Offer originally made by the SSP or in the case where no offer has been previously received, it will contain sufficient details for an SSP to create a Subcontracted Service Order.

Product Instance (PI)

This is an instance of a product type offered by a service provider that has been provided to a customer.

Product Type (PT)

Identifies a particular type of product or service that forms a recognized product offering to a Service Provider, whether that Service Provider is an MSP or an SSP. The assumption being that a given Product Type identifier is recognizable between two Service Providers working in an MSP - SSP relationship. This seems reasonable as in most cases the scope of this relationship would have already been determined as to what Product Types are covered.

Scenario

Description of a process that uses the SMART Ordering SP-to-SP interface.

Service Enquiry (SE)

Details held by the SSP of an enquiry made by an MSP for a given Product Type or service offering. It is important to note that this is a request for information rather than an Order and as such no commitment has been made by either party to fulfill the enquiry. The possible commitment points for both are as follows:

- SSP- When an Offer has been made, but the commitment is only maintained if a Sub-Contracted     	  Service Order is subsequently placed by the MSP, within the agreed timescales of  the   	  Offer.

- MSP- When a Sub-Contracted Service Order is placed.

In most cases this Service Enquiry will specify a particular Product Type that is identifiable between both Service Providers, but in certain cases where the exact Product Type cannot be identified the details of the service offering required will be passed. In this situation the SSP will determine the optimum Product Type required based on the details supplied from the MSP and this could involve the initiation of a negotiation phase between the Service Providers involved. It should be noted that a Service Enquiry could cover a component of the original MSP Customer Order or cover the overall MSP Customer Order. It is also possible for the same Service Enquiry to be sent to more than one SSP, to enable a full evaluation to be undertaken by the MSP as to what the best option is in fulfilling the original Customer Order.

Service Level Agreement (SLA)

An SLA defined in the context of ordering is a commitment or group of commitments made to a Customer (in our case an MSP) which require routine measurement and reporting of actual performance against targets. The SLA measures considered must all have significance for the Customer. This means that for a particular Customer the QoS parameters mentioned in it's SLA negotiated over the ordering interface must be definable, controllable and measurable by the SSP.

Step

Identification of a component of a scenario that is used to describe the stages or steps in the process being detailed.

Subcontracted Service Order (SSO)

Details held by the SSP of the Order request made by the MSP for a given Product Type.

Disconnect (Release) :

A disconnect (or release) identified in the context of ordering is the deactivation of a particular service ordered by a Customer. The disconnect date is specified in the Order.

Escalation of Order (EoO):

In the context of ordering an Escalation of order can be reactive or proactive :

Reactive Escalation

The MSP has determined that the Order is in jeopardy and requires management be notified.  The MSP expects the SSP to provide the name and telephone number of the management person notified within the SSP's organization.  Reactive escalation received from an MSP can be at any level that the SSP supports, e.g., if the SSP's management levels only support up to a 6th level, then receiving an escalation to 7th level would not be appropriate.

Examples of escalation levels:

- 1st - Work Center Supervisor (Control Office)

- 2nd - Work Center Manager (Control Office)

- 3rd - District Manager

- 4th - Division Manager

- 5th - Assistant Vice President

- 6th - Vice President

- 7th - President

Proactive Escalation -

The SSP has determined that the Order is in jeopardy and requires internal management notification.  As per agreements in the SLA, the appropriate level of management would be notified of the jeopardy condition and their assistance requested, if appropriate.

Order Status Tracking (OST) :

An Order status tracking is the delivery of information an SSP would provide to an MSP to inform the MSP of the progress of a particular Order. The Order status tracking can be Proactive or Reactive.

Proactive Order Status Tracking (POST):

An Order status tracking is proactive when the SSP informs the MSP on the status of the Order progress without any request from the MSP.

Reactive Order Status Tracking (ROST):

An Order status tracking is reactive when the SSP informs the MSP on the Order status after request from the MSP.

# 5	Business Objectives

This section describes the business problem to be solved, and provides a high-level view of the requirements to be satisfied through common information exchange.  It is intended to make clear why investment in such a common exchange is beneficial, and how each stakeholder should expect to benefit.  Finally, this section contains illustrative scenarios of the business relationships that may exist between entities, and the purposes for which the agreement might be used.

## 6	Introduction to the requirements

As the communications industry continues to change with deregulation and liberalization, and as Customers increasingly demand global services, communications Service Providers are less likely to be able to satisfy their Customers’ needs using only their own internal networks and resources.  Therefore, they are obliged to rely upon other providers for access or egress services, or for service segments, that are then combined to deliver an end-to-end service that meets the Customer’s needs.

This new way of doing business brings opportunities (doing business worldwide, for example) as well as risks.  Chief among the risks, when attempting to fulfill a Customer’s end-to-end needs in a timely fashion, is that the process of fulfillment is no longer totally within the Service Provider’s control.  To minimize the risk of failing to meet a Customer commitment, these condition imply the need for one Service Provider to see the status of an Order placed with another Service Provider, at any given time throughout the Ordering process.

A fast and accurate Ordering process will be crucial for Service Providers in an increasingly competitive telecom market. The fragmentation of the market with more actors involved in the provision of services also stresses the need for efficient Pre-Ordering, Order handling and Tracking across company borders. Historically  the Ordering process in most Service Provider organizations was developed for a totally different environment:

- It was not Customer friendly because most Service Providers operated in a regulated, non-competitive environment.

- Customers were prepared to wait for weeks and even months for a new service.

- There were only a few services to order and customization was almost unknown.

- Most of the services were produced in-house minimizing the need for external interactions.

- Service Providers had lots of personnel that could handle activities manually.

- Technology was not mature enough to deal with the complex tasks of the Ordering process.

## 7	Current environment

The current environment is heavily reliant upon manual interactions between Service Providers to determine whether an Order is progressing through the PreOrdering(negotiation stage i.e., whether the other provider has agreed to provide the service), through the ordering stage (Order fulfillment), and to Service Delivery.  As Service Providers do more and more business outside their traditional operating area, or as they are forced through regulatory environment changes to respond to requests for access services from an ever-widening pool of Service Provider. Among the problems of today’s process are the following:

- Lack of consistency of service delivery or quality (e.g., inability to quote a due date and meet it, inability to solve a Customer’s problem within stated intervals, etc.)

- Lack of visibility/control (e.g., inability to see if a subcontracted service provider has done its work so you can meet your Customer due date, unable to provide Customers access to their service status, inability to present an end-to-end service view of what the Customer has purchased)

- Excess complexity (e.g., amount of information, and variety of procedures make manual processes error-prone and expensive).

The current solutions for the Ordering process cannot be used to meet the business needs of Service Providers. The Ordering process has a lot in common with the environment it was developed for:

- It is too slow. Information does not travel fast enough in the organization to meet Customer requirements.

- It requires too much re-keying of data. Information is moved manually between support systems which means time consuming typing of data.

- It is difficult to track the Order to know what the status is. Order tracking is important to be able to answer Customer questions and to ensure that the delivery of the Order is following plans.

- It is too inflexible. The process is adapted to the type of service being ordered. New services usually require organizational as well as support system changes.

- The external interfaces are inefficient. Phone, fax and mail constitute the point of contact between Service Providers and between Service Providers and Customers.

- The accuracy is low. Many errors are made which leads to costly changes and Customer complaints.

- There is too much room for misunderstandings. The terminology is not consistent between different Service Providers leading to many mistakes.

These problems with the current Ordering process are especially serious when several Service Providers are involved in providing an end user service. Therefore, the SMART Ordering team has addressed the problems listed above when developing and recommending new models and solutions for the Ordering process.

## 8	Business Need

The problem described above has emerged from the fact that the traditional Ordering process has become obsolete when applied to the new market environment. When developing new solutions for this process it is not enough to address the current problems. It is also necessary to look into the current trends and try to predict the way the market situation will evolve. Therefore we have listed a couple of extra requirements that are important to take on board to guarantee usefulness of the SMART Ordering recommendations:

- In a competitive environment Service Orders generally come out of a discussion, comparing different alternatives, with the Customer.

- Customers want the option of one-stop-shopping even if the services they are interested in may inherently cross Service Provider borders.

- A pre-Order process is needed that can handle several service design proposals in a short time, to a low cost, even if several service/network providers are involved in the proposals.

- The recommendation must be applicable in a real multi-Service Provider market. Having a situation with several service providers are involved in an offer will not be unusual. Automation and real time capabilities are paramount when the number of parties involved increases.

- Many different types of Service Provider roles must be supported. New roles will be created in the Ordering process, addressing other niches than today (for example service brokers, companies doing resale of capacity, etc.). Roles may also change over time and between different markets.

- The entire Ordering process must be taken into account when describing process and information models, otherwise it will not be possible to get overall efficiency. Addressing the Service Provider to Service Provider interface in isolation will only lead to sub-optimal solutions.

- During the Ordering Process, the best efficiency is obtained when the different parties of  Customer and Service Provider involved, can address information to the right peer party.  Therefore, to ensure the best quality of service, the Customer and Service Provider parties exchanging the information  have to identify and to recognize each other. We assume that  the Customer doesn’t have to make any changes to their legacy Ordering Systems to address the Order to the Service Provider.

## 9	Summary of Business Requirements

Following the business problem and the additional requirements, the SMART Ordering team has identified  some criteria to enable  the success of the solutions being produced for the Ordering process. These criteria are:

- A significant reduction of the total Order life cycle. The life cycle should meet end user needs and expectations and represent a cost effective way to handle Orders.

- Shorter time to billing. The billing process should be turned on as quickly as possible after the offer is accepted by the Customer.

- Better data quality in the Ordering process to reduce the number of errors and the costs in correcting them.

- A generic ordering interface between Service Providers that can be used for all types of products and services.

- Clear procedures and interfaces for tracking Order status across Service Provider Orders.

## 10	Expected beneficiaries

The benefits which the SMART Ordering work will realize, are the  provision of automated and standardized order handling between Service Providers:

- To improve the quality of the overall Ordering Process by ensuring there are less manual interventions (e.g., double keying) and hence less room for error.

- To provide the capability for visibility of an Order progress. This is particularly important for Service Providers who subcontract elements of a Customer Order to multiple  Service Providers.

- To reduce the provisioning time of Service to the end Customer.

- To reduce the costs of the Ordering process.

- To provide an open interface to a Service Providers legacy systems and platforms.

# 11	Scope of Interface Requirements

SMART Ordering covers the ordering interface between a Main Service Provider (MSP) and Subcontracted Service Providers (SSPs). The working definition for the scope is:

SMART Ordering process covers the submission, tracking and confirmation of the delivery of a package ordered from an existing portfolio, across the Service Provider to Service Provider (SP-to-SP) Interface. It supports the enquiry (pre-Ordering) and the firm ordering phases of the ordering process for new service Orders, changes to existing service and disconnection (release) of a service. The scope does not cover the communication model (methods of transfer) or the Customer to Service Provider relationship.

## 12	Process orientation

The following diagram and text make clear which processes are involved in the SMART Ordering interface, and how the Ordering process relates to other processes used by a Service Provider (see ).

- Figure 1 : The Overall Service Management Business Process Model [1]

The Ordering process includes all the functions of accepting a Customer’s Order for service, tracking the progress of the Order, and notifying the Customer when the Order is complete.  Orders can include initial Orders, changes, and disconnection (release) to all or part of a Customer’s service.

## 13	Process activities addressed

### 14 General

The following figure describes the relations between the ordering process and other identified processes related to the SMART process model. It shows in bold type the activities that are of interest in these requirements, and the information (inputs and outputs) that are covered by the Service Provider to Service Provider (SP-to-SP) ordering interface (see ).

Figure 2 : The SMART Ordering Process, Inputs and Outputs [1]

### 15 A Generic Ordering Model

The overall goal of the ordering model is to meet Customer needs for new services as efficiently as possible. This covers a whole range of different cases from a Customer wanting a simple quote on a single service, to going through a complete design and negotiation phase for a complex service offering. The Ordering process can stop with an Offer awaiting consideration by a Customer or with the completion of the delivery of the service ordered. This puts some extra demands on the process model which must be reflected in the high degree of flexibility.

The ordering model is iterative which means there could be a whole chain of SPs involved in providing parts of the offering to the end Customer. However, there is only one Main Service Provider (MSP) directly interacting with the Customer. Figure 3 describes the iterative aspect of the chain of providers involved in the Ordering process.

It is important to notice that the process model only focuses on the activities involved in ordering and mainly on the interactions taking place over the SP-SP interface. This means that details on how the service is realized, how the Customer related information is stored in various data bases and how interfaces to other processes like Performance Reporting, Problem Handling and Invoicing/Collection have not been covered. These areas will need to be addressed later in SMART, perhaps as a separate activity or together with the other SMART teams. In the future there is a need to add other dimensions to this model to show a more complete picture of the ordering area.

Figure 3 : A Chain of Service Providers Involved in the Ordering Process

## 16	How the interface is expected to be used

The SMART Ordering SP-to-SP interface is aiming to cover Pre-Ordering, Ordering and Tracking between two SPs. The Service Provider requesting a service is called the Main Service Provider (MSP) and the one offering the service is called the Subcontracted Service Provider (SSP).

- In the Pre-Ordering phase MSP exchanges, selects and compares service offers received from SSPs.

- In the Ordering phase the MSP submits a confirmed Order (i.e. commitment has been received from the customer to provision the Order) which may or may not be based on an existing Offer previously made by an SSP.

- The Tracking activity allows any MSP to track the status of an Order until the requested Service is delivered.

## 17	Scenarios to be addressed

### 18 Introduction

The aim of this section is to outline the SP-to-SP Order Administration scenarios of the ordering SP- to-SP Interface. It details a generic scenario showing the branching points for a typical product being ordered covering each permissible Order Type :

- Provide Order

- Change Order

- Disconnection (Release) Order

### 19 Generic Provide Order Scenario

This covers the scenario where a new instance of a product is required, typically called a provide Order.

#### 20 Pre-Ordering Phase

This phase is initiated following a request from a customer for a particular requirement to be fulfilled by an MSP. This request could either be an enquiry for service where the customer wants sufficient details to determine whether they want to place an Order or it could be a confirmed Order for a known product.

In the case of an enquiry for service the MSP completes this phase to enable a formal quotation to be returned to the customer. In the case of a confirmed Order, the MSP may want to complete this phase to evaluate the best way to fulfill the Order, but this is unlikely to be the case in the situation where the MSP has a preferred SSP for delivery of the product in question.

A diagrammatic overview of the scenarios covered within this phase is shown below :

Figure 4 :Pre-Ordering Phase Interactions (Order Type = Provide)

A text description of each scenario within the Pre Ordering Phase is detailed below :

##### Scenario 1 - Service Offer made for a Service Enquiry

- MSP receives a Customer request to provide a quotation for a given service. Figure 4 - Flow (1) - Customer > MSP.

- All the details of the Customer request are captured by the MSP as a Customer Order with a status of “Enquiry”. At this stage the MSP completes a credit check of the customer. Internal MSP Process.

- Depending on the level of detail provided in the Customer request, the MSP may need to clarify the requirements with the customer. Figure 4 - Flow (2) - Customer <> MSP.

- The MSP undertakes an evaluation process of the Customer request to determine how best it could be fulfilled and identifies a particular product type that meets the Customer request. Internal MSP Process.

- Once the product type has been identified, the MSP completes an assessment to determine how best to fulfill the provision of it, within the circumstances of the Customer request. As part of this assessment, the MSP determines that a component of the product type or the complete product type would need to be provisioned in conjunction with an SSP. Internal MSP Process.

- MSP establishes which SSPs could be used to provision the product type required. Potentially more than one SSP could be used so that the options to meet the Customer’s request are increased. Internal MSP Process.

- The MSP builds an enquiry request encompassing the details of the work required in the provisioning of the product or component  needed and sends the enquiry request to each of the selected SSPs. Figure 4 - Flow (3) - MSP > SSP.

- Upon receiving the enquiry request the SSP validates that sufficient details have been provided to enable the service enquiry to be completed and that the request is for a service that can be supplied by the SSP. If the validation is successful the enquiry request is accepted. Internal SSP Process.

- If the SSP validation fails the enquiry request is rejected and the failure details are advised to the MSP. Figure 4 - Flow (4) - SSP > MSP.

- On receipt of the rejected enquiry request the MSP would need to evaluate the reason for failure and decide if the enquiry request should be resubmitted. Internal MSP Process.

- If the enquiry request should be resubmitted, then the errors are corrected by the MSP and the enquiry request is sent to the SSP. Figure 4 - Flow (3) - MSP > SSP.

- For a validated enquiry request the SSP stores the details within a service enquiry and evaluates how the request could be fulfilled. Internal SSP Process.

- The SSP needs to consider all the possible options viable in meeting the service enquiry and will need to determine which one is preferable. The term preferable in this case means the option that is best for the SSP in meeting the enquiry request supplied by the MSP. Internal SSP Process.

- Once the preferred option has been selected by the SSP an offer is created and returned to the MSP. Figure 4 - Flow (10) - SSP > MSP.

- When the offer is received from the SSP, this is evaluated by the MSP. Following the evaluation a response is made to the SSP advising that the offer is either rejected or that it should be held until advised further. Figure 4 - Flow (11) - MSP > SSP.

- If the offer received from the SSP is accepted by the MSP, then the MSP needs to consider this     in relation to any other offers received from other SSPs. Internal MSP Process.

- Once the preferred offer has been selected by the MSP, a response is formulated to the Customer detailing the costs and delivery or completion date as a formal quotation. This quotation also contains details of the contractual terms and conditions that will need to be agreed by the customer. These quotation details are stored as a Customer Order with a status of “Quote”. Figure 4 - Flow (12) - MSP > Customer.

- If the Customer accepts the quotation offered by the MSP, then the Customer Order becomes a confirmed Order and the “Ordering Phase” scenario is initiated. This is described in the Ordering Phase described in Section 3.4.2.2 below.

##### Scenario 2 - Service Enquiry Rejected by SSP and No Offer Made

- If it is not possible for an SSP to fulfill a service enquiry then it is closed and the SSP with undertake no further work on this service enquiry. The MSP is advised that no offer will not be made  to the service enquiry. Figure 4 - Flow (10) - SSP > MSP.

- When the SSP advises the MSP that no offer can be made, the MSP needs to determine what other options are available to meet the Customer request. Internal MSP Process.

- If no further options are available then the MSP will advise the Customer that the request could not be fulfilled. Figure 4 - Flow (14) - MSP > Customer.

##### Scenario 3 - Offer Rejected by MSP

- If the offer received from the SSP is not acceptable to the MSP, then the MSP needs to establish what can be done. Internal MSP Process.

- This can either start the process of negotiating the offer with the SSP, which is covered by Scenario 10 below or the MSP can decide not to proceed further with this particular offer. Internal MSP Process.

- If no further options are available then the MSP will advise the Customer that the request could not be fulfilled. Figure 4 - Flow (14) - MSP > Customer.

- The MSP will also advise the SSP that the offer has been rejected. Figure 4 - Flow (11) - MSP > SSP.

##### Scenario 4 - Customer Rejects Quotation Offered by MSP

- If the Customer rejects the quotation offered by the MSP, then the MSP will try to re-negotiate the original service enquiry with the Customer. Figure 4 - Flow (15) - MSP <> Customer.

- If the service enquiry needs to be amended as a result of the negotiation then Scenario 1 above will restart again. Internal MSP Process.

- If the MSP determines that no further action is to be taken, the SSP is advised that the offer has been rejected. Figure 4 - Flow (11) - MSP > SSP.

##### Scenario 5 - Quotation Time Limit Expires

- Once the MSP has made the quotation offer the Customer has a predetermined time limit to accept the quotation. If the  predetermined time limit is not met then the Customer Order is canceled. Internal MSP Process.

##### Scenario 6 - Offer Time Limit Expires

- Once the SSP has made the offer the MSP has a predetermined time limit to accept the offer. If the  predetermined time limit is not met then the offer is canceled. Internal SSP Process.

- The cancellation of the offer is advised to the MSP. Figure 4 - Flow (13) - SSP > MSP.

##### Scenario 7 - Customers Amends Service Request

- If the Customer amends their requirements at some stage during the Pre-Ordering phase, then this will be advised to the MSP. Figure 4 - Flow (7) - Customer > MSP.

- The MSP will cancel the original Service Enquiry (described within Scenario 8 below) and restart the Pre-Ordering process again. Internal MSP Process.

##### Scenario 8 - Customer Cancels Service Request

- If the Customer decides to cancel their original request, then this will be advised to the MSP. Figure 4 - Flow (8) - Customer > MSP.

- The MSP will cancel the Customer Order. Internal MSP Process.

- When a Customer Order is canceled any service enquiries associated will also be canceled and advise of this will be sent to the SSPs involved. Figure 4 - Flow (9) - MSP > SSP.

##### Scenario 9 - Advise of Delay in Response

- If during the process of evaluating a service enquiry the SSP determines that there will be a delay in responding to the MSP within the expected timescales, then the SSP notifies the MSP of the delay and the expected completion date.   Figure 4 - Flow (5) - SSP > MSP.

- On receipt of this notification the MSP determines the impact of the delay. If the MSP considers the delay unacceptable, then the problem can be escalated with the SSP.  This escalation process is described within Scenario 11.

##### Scenario 10 - MSP Negotiates Offer with SSP

This scenario is potentially an iterative process, that could be completed many times during the pre-Ordering phase. It is important to note that each offer returned must be retained, so that at any stage of this cyclic event an earlier offer could be used by both parties as the basis for placing a sub-contracted service Order.

- At any stage following an MSP receiving an offer and placing a sub-contracted service Order, it is possible for there to be a negotiation stage between the MSP and the SSP. The nature of this negotiation is dependent on the acceptability of the offer made by the SSP to the MSP, based on how well it fulfills the original Customer request. Internal MSP Process.

- If the offer needs to be negotiated with the SSP, the MSP builds an enquiry request with the new details required and submits it to the SSP. Figure 4 - Flow (3) - MSP > SSP.

- The steps 8 to 16 within Scenario 1 are now activated and this process will continue to loop until the negotiation phase is completed.

##### Scenario 11 - MSP Escalates Service Enquiry with SSP

- If at any stage while the SSP is working on a Service Enquiry the MSP decides that a problem needs to be escalated, then the MSP will be able to escalate the problem within the SSP’s organisation via a set of pre-determined escalation levels (see description for “Escalation of Order” within Section 1.3). On receipt of this notification the SSP will establish what form of action they wish to take and inform the MSP accordingly. Figure 4 - Flow (6) - MSP <> SSP.

#### 21 Ordering Phase

It should be noted that it is possible to start this phase without completing the Pre-Ordering Phase. This is most likely to be the situation where there is a pre-established agreement between two Service Providers to provide a given product type. When an MSP gets an Order for one of these products within the geographic area covered by the SSP, the MSP would know the terms and conditions of providing that product from the pre-established agreement and therefore the Pre-Ordering phase would normally not be required. In this situation, it would be likely that the first point of contact with the SSP would be the sub-contracted service Order.

A diagrammatic overview of the scenarios covered within this phase is shown below :

Figure 5 : Ordering Phase Interactions (Order Type = Provide)

A text description of each scenario within the Ordering Phase is detailed below :

##### Scenario 1 - Subcontracted Service Order Completed (with Offer)

This scenario covers the situation where a Subcontracted Service Order is created by the SSP based on an earlier offer made following the successful completion of the pre-Ordering phase within Section 3.4.2.1.

- The Customer requests the MSP to initiate delivery of the service required based on the terms and conditions supplied by the quotation. This request contains agreement by the customer to adhere to the terms and conditions of supplying the service. Figure 5 - Flow (1) - Customer > MSP.

- The MSP changes the status of the Customer Order containing the quotation details to “Confirmed”. Internal MSP Process.

- The MSP submits an Order request to the SSP cross referencing the previous offer made by the SSP. Figure 5 - Flow (4) - MSP > SSP.

- Upon receiving the Order request the SSP validates that sufficient details have been provided to enable the Order request to be completed and that the request is for a product or service that can be supplied by the SSP. If the validation is successful the Order request is accepted. Internal SSP Process.

- If the SSP validation fails the Order request is rejected and the failure details are advised to the MSP. Figure 5 - Flow (5) - SSP > MSP.

- On receipt of the rejected Order request the MSP would need to evaluate the reason for failure and decide if the Order request should be resubmitted. Internal MSP Process.

- If the Order request should be resubmitted, then the errors are corrected by the MSP and the Order request is sent to the SSP. Figure 5 - Flow (4) - MSP > SSP.

- If the validation is successful a subcontracted service Order is created by the SSP based on the cross referenced offer. Internal SSP Process.

- The SSP starts work on the activities required to fulfill the subcontracted service Order. Progress on key activities in the provisioning process are advised to the MSP (Pro-active Tracking). Figure 5 - Flow (6) - SSP > MSP.

- When the SSP has completed the delivery of the subcontracted service Order the product is tested by the SSP to confirm that it is working as specified. Internal SSP Process.

- When the SSP has completed the testing of the product, notification of completion of the delivery of the sub-contracted service Order is sent to the MSP. Figure 5 - Flow (17) - SSP > MSP.

- On receipt of the completion of the Order from the SSP, the MSP determines if the Order from the customer has been completed fully. If it has the completion is advised to the customer.   Figure 5 - Flow (18) - MSP > Customer.

Scenario 2 - Subcontracted Service Order Completed (with no Offer)

This scenario covers the situation where a subcontracted service Order is created by the SSP without the completion of the Pre-Ordering phase and where no offer has been previously made.

- MSP receives a Customer request to provide a given product type. Figure 5 - Flow (1) - Customer > MSP.

- All the details of the Customer request are captured by the MSP as a Customer Order with a status of “Enquiry”. At this stage the MSP completes a credit check of the customer. Internal MSP Process.

- Depending on the level of detail provided in the Customer request, the MSP may need to clarify the requirements with the customer. Figure 5 - Flow (2) - Customer <> MSP.

- The MSP undertakes an evaluation process of the Customer request to determine how best it could be fulfilled and identifies a particular product type that meets the Customer request. Internal MSP Process.

- Once the product type has been identified, the MSP completes an assessment to determine how best to fulfill the provision of it, within the circumstances of the Customer request. As part of this assessment, the MSP determines that a component of the product type or the complete product type would need to be provisioned in conjunction with an SSP. Internal MSP Process.

- MSP establishes which SSP should be used to provision the product type required. Potentially more than one SSP could be used and the MSP needs to establish which is the preferred SSP. Internal MSP Process.

- Once the evaluation stage is completed the MSP agrees the terms and conditions of providing the product with the Customer. Figure 5 - Flow (3) - Customer <> MSP.

- If the Customer accepts the terms and conditions, the status of the Customer Order is changed to “Confirmed”. Internal MSP Process.

- The MSP builds an Order request encompassing the details of the work required in the provisioning of the product or component  needed and sends the Order request to the SSP. Figure 5 - Flow (4) - MSP > SSP.

- Upon receiving the Order request the SSP validates that sufficient details have been provided to enable the Order Request to be completed and that the request is for a product or service that can be supplied by the SSP. If the validation is successful the Order request is accepted. Internal SSP Process.

- If the SSP validation fails the Order request is rejected and the failure details are advised to the MSP. Figure 5 - Flow (5) - SSP > MSP.

- On receipt of the rejected Order request the MSP would need to evaluate the reason for failure and decide if the Order request should be resubmitted. Internal MSP Process.

- It is possible that  to correct the validation errors the MSP may need to seek clarification with the customer on the Order request requirements. Figure 5 - Flow (2) - Customer <> MSP.

- If the Order request should be resubmitted, then the errors are corrected by the MSP and the Order request is sent to the SSP. Figure 5 - Flow (4) - MSP > SSP.

- For a validated Order request the SSP stores the details within a subcontracted service Order and evaluates how the request could be fulfilled. Internal SSP Process.

- The SSP needs to consider all the possible options viable in meeting the subcontracted service Order and will need to determine which one is preferable. The term preferable in this case means the option that is best for the SSP in meeting the Order request supplied by the MSP. Once the preferred option has been selected it is used as the basis of providing the subcontracted service Order. Internal SSP Process.

- The SSP starts work on the activities required to provision the subcontracted service Order. Progress on key activities is advised to the MSP (Pro-active Tracking). Figure 5 - Flow (6) - SSP > MSP.

- When the SSP has completed the delivery of the sub-contracted service Order the product is tested by the SSP to confirm that it is working at specified. Internal SSP Process.

- When the SSP has completed the testing of the product, notification of completion of the delivery of the Order request is sent to the MSP. Figure 5 - Flow (17) - SSP > MSP.

- On receipt of the completion of the Order from the SSP, the MSP determines if the Order from the customer has been completed fully. If it has the completion is advised to the customer.   Figure 5 - Flow (18) - MSP > Customer.

Scenario 3 - SSP Agreed Completion Date Failure

- If a key provisioning activity being undertaken by the SSP has failed to complete on the date planned, then the SSP needs to determine if the Order can still be completed within the agreed time scales. Internal SSP Process.

- If this is not possible, then notification should be sent to the MSP detailing the new planned order completion date. Figure 5 - Flow (9) - SSP > MSP.

- Upon receipt of this notification the MSP will determine what course of action they wish to take. Internal MSP Process.

- If the MSP considers the delay unacceptable, then the MSP has the option of escalating the problem to the SSP. This process is covered by Scenario 4.

- If the MSP accepts the delay, then this must be advised to the customer.   Figure 5 - Flow (10) - MSP > Customer.

- If the new date is unacceptable to the Customer it is possible that the Order could be canceled (this is described within Scenario 6 below).

##### Scenario 4 - MSP Escalates Service Order with SSP

- If at any stage while the SSP is working on a subcontracted service Order the MSP decides that a problem needs to be escalated, then the MSP will be able to escalate the problem within the SSP’s organisation via a set of pre-determined escalation levels (see section 1.3 for definition of this term). On receipt of this notification the SSP will establish what form of action they wish to take and inform the MSP accordingly. Figure 5 - Flow (11) - MSP <> SSP.

##### Scenario 5 - Customers Amends Order Request

- If the Customer amends their requirements at some stage during the ordering phase, then this will be advised to the MSP. Figure 5 - Flow (12) - Customer > MSP.

- The MSP will cancel the Customer Order (described within Scenario 6 below) and restart the ordering process again. Internal MSP Process.

##### Scenario 6 - Customer Cancels Order Request

- If the Customer decides to cancel their original request, then notification will be required  to the MSP. It should be noted that a Customer Order can only be canceled prior to order completion. Following order completion any cancellation request received will be rejected. Figure 5 - Flow (13) - Customer > MSP.

- The MSP will cancel the Customer Order. Internal MSP Process.

- When a Customer Order is canceled any associated order requests will also be canceled and notification will be sent to the SSPs involved. Figure 5 - Flow (14) - MSP > SSP.

- Upon receipt of the cancellation request the SSP will determine if any cancellation charges will need to be levied. Internal SSP Process.

- If there are any cancellation charges then these are advised to the MSP. Figure 5 - Flow (15) - SSP > MSP.

- The MSP will need to aggregate all the cancellation charges due as a result of the cancellation. Internal MSP Process.

- Once all the cancellation charges are known, then the customer is billed the outstanding charges. Figure 5 - Flow (16) - MSP > Customer.

##### Scenario 7 - Service Order Reactive Tracking

- At any stage within the ordering phase it is possible for an MSP to query what progress has been made on the order request. The SSP should return a response on the status of the order. Figure 5 - Flow (8) - MSP <> SSP.

- This could be triggered by a request from a customer, but this would not always be the case. Figure 5 - Flow (7) - Customer <> MSP.

### 22 Generic Change Service Scenario

### This covers the scenario when an existing service (product instance) needs to be amended in some way, typically called a change Order.

It should be noted that although a Pre-Ordering Phase is possible, in a lot of circumstances it is considered unlikely to be used.

#### 3.4.3.1. Pre-Ordering Phase

The Scenarios within this phase are the same as for a generic provide Order type as detailed within 3.4.2.1.

#### 3.4.3.2. Ordering Phase

The Scenarios within this phase are the same as for a generic provide Order type as detailed within 3.4.2.2.

### 3.4.4. Generic Disconnect (Release) Service (Product Instance) Scenario

This covers the scenario where an existing service (product instance) needs to be ceased, typically called a cessation Order.

It should be noted that a Pre-Ordering Phase is not possible.

#### 23 Ordering Phase

A diagrammatic overview of the scenarios covered within this phase is shown below :

Figure 7 : Ordering Phase Interactions (Order Type = Cease)

A text description of each scenario within the ordering Phase is detailed below :

##### Scenario 1 - Subcontracted Service Order Completed

- MSP receives a Customer request to cease an existing product instance. Figure 7 - Flow (1) - Customer > MSP.

- All the details of the Customer request are captured by the MSP as a Customer Order with a status of “Confirmed”. Internal MSP Process.

- The MSP submits an Order request to the SSP. Figure 7 - Flow (2) - MSP > SSP.

- Upon receiving the order request the SSP validates that sufficient details have been provided to enable the order request to be completed. If the validation is successful the order request is accepted. Internal SSP Process.

- If the SSP validation fails the order request is rejected and the failure details are advised to the MSP. Figure 7 - Flow (3) - SSP > MSP.

- On receipt of the rejected order request the MSP would need to evaluate the reason for failure and decide if the order request should be resubmitted. Internal MSP Process.

- If the order request should be resubmitted, then the errors are corrected by the MSP and the order request is sent to the SSP. Figure 7 - Flow (2) - MSP > SSP.

- If the validation is successful a subcontracted service Order is created by the SSP. Internal SSP Process.

- The SSP starts work on the activities required to fulfill the subcontracted service order. Internal SSP Process.

- When the SSP has completed the sub-contracted service Order, notification is sent to the MSP. Figure 7 - Flow (7) - SSP > MSP.

- The MSP now advises the customer that the cessation has been completed. Figure 7 - Flow (8) - MSP > Customer.

Scenario 2 - Customers Amends Order Request

- If the Customer amends their requirements at some stage during the Ordering phase, then this will be advised to the MSP. Figure 7 - Flow (4) - Customer > MSP.

- The MSP will cancel the Customer Order (described within Scenario 3 below) and restart the Ordering process again. Internal MSP Process.

##### Scenario 3 - Customer Cancels Order Request

- If the Customer decides to cancel their original request, then notification will  be required  to the MSP. It should be noted that a Customer Order can only be canceled prior to order completion. Following order completion any cancellation request received will be rejected. Figure 7 - Flow (5) - Customer > MSP.

- The MSP will cancel the Customer Order. Internal MSP Process.

- When a Customer Order is canceled notification is sent to the SSP. Figure 5 - Flow (6) - MSP > SSP.

- Upon receipt of the cancellation request the SSP will cancel the subcontracted Service Order.   Internal SSP Process.

# 24	Flow of Information

## 25 Introduction

This section describes the dynamic aspects of the Order exchange between an MSP and an SSP  in terms of an workflow diagram, and an event flow diagram.

As described in the White Paper, the SMART Ordering interface involves a set of processes within the Service Provider organization, but does not recommend any process architecture within the Service Providers organizations.

Therefore the SMART SP-to-SP interface describes only the Order exchange interactions between the Service Providers and only these interactions.

Figure 8 : The SP-to-SP Ordering Interface

Figure 8 represents the bi-directional message exchanges between an MSP and an SSP. The Ordering Interface doesn’t give any visibility to the SP Information  organization except to the resources that are necessary to maintain the Ordering procedures.

### 26 Description on SP-to-SP Ordering Interface :

The interface will be required to support three main events; namely Pre-Ordering, Ordering and Tracking. A description of each of these events is covered below :

Figure 9 : Activities Utilizing an SP-to-SP Ordering Interface

### 27 Pre-Ordering

This is shown on the flow of information diagram within the original White Paper as the interface between the processes “Identify All Services” and “Check Feasibility of Individual Services”.

This interface must support the ability of a MSP to place an enquiry with a SSP as to how a given product type could be provisioned. Essentially, it is a request by the MSP to the SSP to tender for the work required.

It is assumed that this interface will operate within the terms and conditions of a pre-determined agreement governing the provision of the product type in question and that the information passed between both parties will conform to an agreed structure and content.

The MSP will pass the information required to the SSP within the pre-determined format of the enquiry request and on receipt of the request it will be validated by the SSP.

The SSP will send a response based on the outcome of the evaluation of the service enquiry and this will either be in the format of an offer in an agreed format or notification that no offer could be made.

In addition, the interface must support the following :

- Allow an SSP to notify an MSP of a validation failure with a request and allow the request to be resubmitted when corrected by the MSP.

- Support a negotiation phase between the Service Providers so that for each enquiry request a new one could be submitted to allow the opportunity for a new offer to be made.   Each offer should be retained to allow an earlier offer to be used if required.

- Must support the ability for an enquiry request to be canceled once accepted by the SSP.

### 28 Ordering

This interface must support the ability of a MSP to place an Order with a SSP. Essentially it is a request by the MSP to the SSP to complete the provision of a given product type to the requirements of a particular Customer request.

It is assumed that this interface will operate within the terms and conditions of a pre-determined agreement governing the provision of the product type in question and that the information passed between both parties will conform to an agreed structure and content.

The MSP will pass the information required to the SSP within the pre-determined format of the Order request and on receipt of the request it will be validated by the SSP.

The SSP will send a notification to the MSP when the provisioning process is complete.

In addition the interface must support the following :

- Allow an SSP validation failure to be notified to the MSP and allow the original request to be resubmitted when corrected by the MSP.

- If the SSP detects that an agreed completion date for delivery of the product to the MSP cannot be meet, then the ability to notify this failure and the revised completion date to the MSP is required.

- Must support the ability for an Order request to be canceled, but only prior to an order being completed.

### 29 Tracking

This interface must support the ability of a MSP to track the progress of an order placed with a SSP.

It is assumed that this interface will operate within the terms and conditions of a pre-determined agreement  and that the information passed between both parties will conform to an agreed structure and content.

- The MSP will ask for a status report of the current situation for a given item (Reactive Tracking).

- The SSP will send a response detailing the current status of the requested item (Reactive Tracking).

- The SSP will provide progress on key activities within the life of the order provisioning process (Pro-active Tracking).

## 30 Workflow Perspective

The diagram below shows a graphical presentation of the events that trigger some form of interaction across an SP-to-SP interface.

Figure 10 : Order Administration

To aid further understanding, the workflow for each activity shown above has been detailed separately. The details are as follows :

- Section 4.2.1 - Pre-Ordering Activities

- Section 4.2.2 - Ordering Activities

- Section 4.2.3 - Tracking Activities

4.2.1 Pre-Ordering Workflow

Figure 11 : Pre-Ordering Flows and Activities

An explanation of each flow shown within the above diagram follows :

The activity that initiates the flow is used to identify each flow below.

(a) MSP Places Enquiry Request

The MSP determines that an Enquiry Request needs to be raised with an SSP. The Enquiry Request is built with the pre-requisite items and sent to the SSP.

On Receipt of the Enquiry Request the SSP validates it to ensure that all the appropriate information has been provided for the SSP to evaluate the request. If the SSP detects a validation failure then this triggers Flow (b).

(b) SSP Advises MSP of Validation Failure

If the SSP Enquiry Request validation activity fails, then this failure is notified to the MSP with the appropriate details.

On receipt of the validation failure the MSP determines what action should be taken. If the errors can be corrected and the MSP decides to resubmit the Enquiry Request again, then this will be done using Flow (a).

(c)  SSP Advises MSP of Delay in Response

If the SSP establishes that a response on the Enquiry Request to the MSP will be delayed, then the SSP will notify the MSP of the delay and the expected completion date.

On receipt of this notification the MSP evaluates the impact. The MSP will either accept the delay or decide to escalate the Enquiry Request to an appropriate point within the SSP’s organisation. The escalation request is handled by Flow (d).

(d) MSP Escalates Enquiry Request

If an MSP determines the need to escalate an Enquiry Request, then the escalation is forwarded to the appropriate point within the SSPs organisation.

On receipt of this escalation the SSP determines what form of action needs to be taken and advises the MSP accordingly.

(e) MSP Cancels Enquiry Request

If the MSP determines that an Enquiry Request needs to be canceled, then this is advised to the SSP.

On receipt of this cancellation request the SSP undertakes the appropriate action and stops any further work on the Enquiry Request.

(f) SSP Responds to Enquiry Request

When the SSP has completed the evaluation of an Enquiry Request, then one of two conditions can occur. If the SSP can fulfill the Enquiry Request then an Offer is built containing details of the SSP’s proposal to meet the Enquiry Request. If the SSP cannot fulfill the Enquiry Request then it is rejected and no Offer is built. The SSP advises the MSP accordingly.

On receipt of the response from the SSP, the MSP evaluates it to determine what course of action should be taken. The MSP can either accept the response or decide to amend the Enquiry Request details and resubmit them to the SSP. Where an Enquiry Request has been resubmitted, then this is done via Flow (a).

(g) MSP Offer Response

When the MSP has evaluated the Offer from an SSP, the MSP advises the SSP accordingly. This response will either be the rejection or acceptance of the Offer.

On receipt of this response the SSP undertakes the appropriate action.

(h) SSP Advises MSP of Offer Timeout

If the MSP has failed to reply within the time limit specified in the Offer, the Offer is closed and the MSP is advised. On receipt of this notification the MSP undertakes the appropriate action.

### 4.2.2 Ordering Workflow

Figure 12 : Ordering Activities

An explanation of each flow shown within the above diagram follows :

The activity that initiates the flow is used to identify each flow below.

(a) MSP Sends Order Request

The MSP determines that an Order Request needs to be raised with an SSP. The Order Request is built with the pre-requisite items and sent to the SSP.

On Receipt of the Order Request the SSP validates it to ensure that all the appropriate information has been provided for the SSP to meet the request. If the SSP detects a validation failure then this triggers Flow (b).

(b) SSP Advises MSP of Validation Failure

If the SSP Order Request validation activity fails, then this failure is notified to the MSP with the appropriate details.

On receipt of the validation failure the MSP determines what action should be taken. If the errors can be corrected and the MSP decides to resubmit the Order Request again, then this will be done using Flow (a).

(c)  SSP Advises Delay in Order Completion

If the SSP establishes that the order completion will be delayed, then the SSP will notify the MSP of the delay and the expected completion date.

On receipt of this notification the MSP evaluates the impact. The MSP will either accept the delay or decide to escalate the Order Request to an appropriate point within the SSP’s organisation. The escalation request is handled by Flow (d).

(d) MSP Escalates Order Request

If an MSP determines the need to escalate an Order Request, then the escalation is forwarded to the appropriate point within the SSPs organisation.

On receipt of this escalation the SSP determines what form of action needs to be taken and advises the MSP accordingly.

(e) MSP Cancels  Order Request

If the MSP determines that an Order Request needs to be canceled, then this is advised to the SSP.

On receipt of this cancellation request the SSP undertakes the appropriate action and stops any further work on the Order Request. If any cancellation charges need to be raised then these are advised to the MSP via Flow (f).

(f) SSP Advises MSP of Cancellation Charges

If following the cancellation of an order the SSP determines that this has resulted in the need to raise cancellation charges, then these are advised to the MSP.

On receipt of these cancellation charges the MSP records them and takes the appropriate action.

(g) SSP  Advises MSP of Order Completion

When the SSP has completed the order this is notified to the MSP.

On receipt of this notification the MSP records the completion of the order.

4.2.3 Tracking Workflow

Figure 13 : Tracking Activities

An explanation of each flow shown within the above diagram follows :

The activity that initiates the flow is used to identify each flow below.

(a) SSP Provides Order Progress (Pro-Active)

For key activities within the provisioning process of an order the SSP provides progress to the MSP.

On receipt of these progress details the MSP records them as required.

(b) MSP Request for Order  Progress (Re-Active)

If the MSP establishes a need to get the latest progress details from an SSP, then the MSP places a request with the SSP.

On receipt of this request the SSP determines the current situation with the order in question. This triggers the response handled by Flow (c).

- SSP Provides Order Progress (Re-Active)

Following an evaluation by the SSP of the current situation with an order, progress details are advised to the MSP.

On receipt of the progress details from the SSP the MSP records them as appropriate.

## 31  Event Flow Perspective

The Event Flow Perspective details have been covered by the scenarios shown earlier within section 3.4 and as such the details have not been repeated here.

## 32  State Model

The logical status’s required for each of the items referenced earlier are as follows :

### 33 Customer Order (MSP)

All the status’s of this object will be internal to an MSP and will not be visible to an SSP.  Table 1 shows the status’s of Customer Order (MSP).

| STATUS | DESCRIPTION |
| --- | --- |
| Enquiry | Details of Customer request captured, but no enquiry request made to SSP. |
| Awaiting Enquiry Response | Enquiry Request made to SSP and awaiting response. |
| Enquiry Review | Enquiry response received from SSP and awaiting evaluation by MSP. |
| Quote | Quotation made to Customer and awaiting acceptance. |
| Confirmed | Customer accepts quotation offered or places confirmed Order as part of the initial contact. Order request made to SSP and awaiting response on Order completion. |
| Expired | Quotation offer time limit to Customer expires. |
| Closed | Delivery to Customer completed. Billing and maintenance initiated. |
| Canceled | Order canceled. |

Table 1 : Status’s of Customer Order (MSP).

### 4.4.2 Enquiry Request

Table 2 shows the status’s of Enquiry Request.

| STATUS | DESCRIPTION |
| --- | --- |
| Open | Created by MSP and awaiting response on completion from SSP. |
| Rejected | Enquiry request failed SSP validation checks and returned to MSP with failure details. |
| Resubmitted | Enquiry request resubmitted to SSP following a correction of an earlier validation error. |
| Closed Offer Made | Enquiry request completed and Offer made by SSP. |
| Closed No Offer Made | Enquiry request completed, but no Offer made by SSP. |
| Canceled | Enquiry request canceled. |

Table 2 : Status’s of Enquiry Request

### 4.4.3 Service Enquiry (SSP)

All the status’s of this object will be internal to an SSP and will not be visible to an MSP. Table 3 shows the status’s of Service Enquiry (SSP).

| STATUS | DESCRIPTION |
| --- | --- |
| Open | Created by SSP and awaiting completion. |
| Closed Offer Made | Enquiry completed and Offer made by SSP. |
| Closed No Offer Made | Enquiry completed, but no Offer made by SSP. |
| Canceled | Enquiry canceled. |

Table 3 : Status’s of Service Enquiry (SSP)

### 4.4.4 Offer

Table 4 shows the status’s of Offer.

| STATUS | DESCRIPTION |
| --- | --- |
| Open | Created by SSP and awaiting response from MSP. |
| Accepted | Offer accepted by MSP and work in progress to complete delivery as part of a Subcontracted Service Order. |
| Completed | Delivery of Offer complete as part of a Subcontracted Service Order. |
| Closed | Offer closed due to expiration of time limit or rejection by MSP. |

Table 4 : Status’s of Offer

### 4.4.5 Order Request

Table 5 shows the status’s of Order Request.

| STATUS | DESCRIPTION |
| --- | --- |
| Open | Created by MSP and awaiting response on completion from SSP. |
| Rejected | Order request failed SSP validation checks and returned to MSP with failure details. |
| Resubmitted | Order request resubmitted to SSP following a correction of an earlier validation error. |
| Awaiting Acceptance Testing | Notification of Order completion received by SSP. Awaiting acceptance testing by MSP. |
| Acceptance Test Failure | Acceptance test failure notified to SSP. Awaiting completion of corrective action by SSP. |
| Closed | Order request completed by SSP. |
| Canceled | Order request canceled. |

Table 5 : Status’s of Order Request

### 4.4.6 Subcontracted Service Order (SSP)

All the status’s of this object will be internal to an SSP and will not be visible to an MSP. Table 6 shows the status’s of Subcontracted Service Order.

| STATUS | DESCRIPTION |
| --- | --- |
| Open | Created by SSP and awaiting completion. |
| Acceptance Test Failure | Acceptance test failure notified from MSP. Awaiting completion of corrective action by SSP. |
| Closed | Order completed. |
| Canceled | Order canceled. |

Table 6 : Status’s of Subcontracted Service Order (SSP)

## 4.5	Relationship between actual events and states

Not applicable

# 34	Specific Requirements of the Ordering Interface

Mandatory requirements are indicated by use of the word “must”

Desirable requirements are indicated by use of the word “should.”

## 35 Core requirements

### 36 ORDFR1 : General functional requirements

- ORDFR1.1 : The SP-to-SP Ordering Interface must always be available for any of Pre-Order or Order request.

- Rationale : In a competitive environment the SP-to-SP Ordering Interface should be available 24 hours a day and 7 days a week.

- ORDFR1.2 : The SP-to-SP Ordering Interface must reflect the Business Policy of an SSP.

- Rationale : The SP-to-SP Ordering Interface should adhere to the Business  conditions of the SSP, in the Pre-Ordering and Ordering phases. Especially, when canceling an order an MSP must be aware of the eventual Penalties that may be liable.

- ORDFR1.3 : When the SP to SP Ordering Interface is unavailable, the SSP should send a notification to the MSP.

Rationale : In a competitive environment, it is important that an SSP  notifies an MSP when an Ordering Interface is unavailable, so that the MSP can determine when service enquiries and subcontracted service orders can be sent.

### 37 ORDFR2 : Pre-Ordering functional requirements

- ORDFR2.1 : The SP-to-SP Ordering Interface must permit the MSP to submit  enquiries for service to as many SSPs as the MSP  wishes.

Rationale : A competitive environment provides opportunities for  “comparison shopping”. Just as the Customer can make enquiries of several MSPs, the MSP must be able to request services from alternative suppliers.

- ORDFR2.2 : The SP-to-SP Ordering Interface must permit an SSP to reject an enquiry from an MSP and should permit the SSP to supply the reason(s) for the rejection.

Rationale : Unless obligated by law, an SSP may elect to reject an MSP’s service enquiry instead of submitting an offer. The interface must provide a mechanism for the SSP to advise the MSP of its decision to reject. The interface should also provide a mechanism for the SSP to notify the MSP of the rejection reasons. As an example insufficient information or details in the Service Enquiry are  reasons for rejection.

- ORDFR2.3 : The SP-to-SP Ordering Interface must permit the SSP to submit an Offer to the MSP in response to a service enquiry.

Rationale : The SSP must be able to deliver its most competitive offer for a Service Enquiry to an  MSP. The interface must provide the mechanism for offer submission to include service specification, pricing, SLA terms, and completion dates.

- ORDFR2.4 : The SP-to-SP Ordering Interface should permit the SSP to submit an unsolicited Offer for services to the MSP.

Rationale : A number of circumstances can arise where it would be desirable for an SSP to submit a general offer for services to an MSP. These include notifications of an SSP’s special promotions.

- ORDFR2.5 : The SP to SP Ordering Interface should support a negotiation process between the MSP and an SSP following the SSP’s submission of an Offer.

Rationale : Any number of terms in an Offer can be subject to negotiation: price, service availability date, SLA terms, etc. The interface should provide mechanisms for the MSP and SSP to adjust the details of the service enquiry and of the service offer to arrive at an agreed solution.

### 38 ORDFR3 : Ordering functional requirements

- ORDFR3.1 : The SP-to-SP Ordering Interface must permit an MSP to send an Order to an SSP for an Offer previously made in the Pre-Ordering phase.

Rationale : This is the normal situation where after a negotiation between an MSP and an SSP, both parties have agreed on the business and technical terms of the Offer. The MSP addresses an Order to the SSP based on the SSP Offer (including Offer Id, etc.).

ORDFR3.2 : The SP-to-SP Ordering Interface must permit an MSP to send an Order to an SSP without the completion of the Pre-Ordering phase.

Rationale : The MSP could send an Order to an SSP based on a previously ordered service or the MSP could have sufficient information to enable a confirmed order to be placed without making a Service Enquiry.

- ORDFR3.3 : The SP-to-SP Ordering Interface must permit an MSP to send an Order to an SSP based on an unsolicited Offer from the SSP.

Rationale : Should the spontaneous Offer have an interest for the MSP, the MSP can send an Order based on the contractual agreements presented in this spontaneous Offer (see ORDFR2.4).

- ORDFR3.4 : The SP-to-SP Ordering Interface must permit the MSP to cancel an Order previously send to the SSP if the delivery date of the Order cannot be met by the SSP.

Rationale : Delivery date for a Service is a major parameter of the contractual agreements between MSP and SSP.  Failure to meet an agreed delivery date is a major violation of the order and the business agreement between the MSP and the SSP should stipulate the terms for renegotiating or canceling the order.

- ORDFR3.5 : The SP-to-SP Ordering Interface must permit the SSP to cancel an Order if external conditions to the business agreements between MSP and SSP oblige both parties to review their agreements.

- Rationale : In the case of unexpected law conditions, credit check problems, or any other external condition that would compromise the service delivery , the SSP must notify the MSP that he canceled the Order and provide the reason for cancellation.

- ORDFR3.6 : The SP-to-SP Ordering Interface must permit the MSP to query the progress of a Order with an SSP (Reactive Tracking). The interface must permit the SSP to notify the MSP when significant events occur during Order processing (Proactive Tracking). However the MSP should have the option to choose if they are interested in the Order progress or not.

Rationale : The MSP must be able to report Order progress to the Customer as part of overall Customer service. The interface must therefore permit an MSP to query the SSPs on Order status. To simplify the status reporting, the interface must permit the SSP to simply send Order progress notifications to the MSP whenever an event causes the Order status to change (Status/state) if the MSP hasn’t explicitly notified the SSP that he isn’t interested in tracking information.

- ORDFR3.8 : The SP-to-SP Ordering Interface should permit the MSP to receive and interpret unsolicited events coming from the SSP.

Rationale : Unsolicited events sent by the SSP can be any type of information. It is up to the MSP to ignore or acknowledge them.

### 39 ORDFR4 : Administration functional requirements

- ORDFR4.1 : It should be possible for either the MSP or the SSP to maintain an Order History.

- ORDFR4.2 : The MSP should be able to update an Order with additional new information (e.g., contact change, etc.) and pass this information to the SSP.

## 40	Out of scope requirements

- ORDFR5.1 : The SP to SP Ordering interface doesn’t cover temporary disconnections.

- ORDFR5.2 : The SP to SP Ordering interface doesn’t cover maintenance activity such as failed test after Order completion.

# 41	General Technical Requirements

## 42	Performance of the interface

Not applicable

## 43	Security

The Order contains information that may be confidential, so unauthorized access should  be prevented. It also contains sensitive data, such as site access authorization, and time-stamps which may be used to derive financial charges or penalties, so access to certain parts of the Order must be controlled. The following security services should be provided.   Authentication and integrity are mandatory requirements, and the others are optional.

No mention is made here as to how these security services should be provided, as it depends on the protocol used.

### 44 Authentication

- ORDTR01 : Authentication is the process of determining whether a session requester is truly the identity that it claims to be. Service Providers shall be able to identify each other reliably across the Ordering interface.

### 45 Access Control

- ORDTR02 : In general Access Control is the process of establishing a session from start to end, i.e., from login to logoff.  In the context of Ordering, we are concerned only with control of access to resources within the Ordering application itself.  Users have to be identified and authenticated before access is granted.  Certain attributes and values of an Order may be modified or supplied by only one side; e.g., only the Customer can grant authorization for site-access, and only the Service Provider can record the Order completion time.

### 46 Data Confidentiality

- ORDTR03 : Confidentiality is defined in the ISO 7498-2 as “the property that information is not made available or disclosed to unauthorized individuals, entities, or processes.”

### 47 Integrity

- ORDTR04 : Messages exchanged between an MSP and SSP over Service Ordering Interface must not be duplicated, modified or lost.

- ORDTR05 : The SSP must secure all outstanding Ordering messages against any process failure.

### 48Non-Repudiation

- ORDTR06 : Repudiation is defined as “denial by one of the entities involved in a communication of having participated in all or part of the communication”.  Two forms of non-repudiation are defined: non-repudiation with proof of origin and non-repudiation with proof of delivery.  This security service may take one or both of those forms. In non-repudiation with proof origin, the recipient of data is provided with proof of the origin of data.  This will protect against any attempt by the sender to falsely deny sending the data or its contents.  In non-repudiation with proof of delivery, the sender of data is provided with proof of delivery of data.  This will protect against any subsequent attempt by the recipient to falsely deny receiving the data or its contents.  This service is obviously useful in settling any disputes that may arise between Service Providers.

## 49	Methods of synchronization upon session failure

Not applicable

## 50	Inventory synchronization / Database reconciliation

Not applicable

## 51	Interface protocols

- ORDTR07 : The interoperable interface should be defined in a network transport protocol independent manner. It should be possible for the transport to be OSI-based, TCP-based, IPX-based, or whatever.

## 52	Nature of connection

- ORDTR08 : It should be possible for the connection between two Service Providers to be either permanent or transient.  A transient connection should be supported via dial-up.

- ORDTR09 : The connection between two Service Providers should provide for “reliable” transport of data.

- ORDTR10 : It should be possible for one Main Service Provider (MSP) to have connections with multiple Subcontracted Service Providers (SSPs).

## 53	Other General Requirements

- ORDTR11 : Time values (time-stamps) exchanged across the SP-to-SP Ordering Interface should be in a format that accounts for time-zone differences. For example, systems may agree to use GMT.

- ORDTR12 : There is a common content for Ordering on all subjects, composed of mandatory and optional attributes.  Various Order formats may be defined which specify which attributes must be present, and which attributes should be present.  A format must be selected when creating specific Orders.  It must be possible to determine the format of a given Order.

- ORDTR13 : The SSP must be capable of detecting when the MSP becomes unreachable at a time when the SSP expects to move information across the interface.  No information should be lost due to this unreachability.  When the interface subsequently becomes operational again, normal processing should be resumed.

- ORDTR14 : It should be possible for either the MSP or SSP to include comments in free format text in the Service Enquiry and Order exchanges. Comments once included cannot be modified or removed.

# 54	Conclusions

The information contained within the SMART SP-to-SP Ordering Requirements document are mainly extracted from the last version of the White Paper.

## 55 What isn’t new?

The business context described in the White Paper version 4 is the basis of this requirement document. No additional information regarding the business problem have been added.

## 56 What is new?

- The process model described in the White Paper is not part of the requirement document.

- The process description found in the White Paper was extremely useful to understand the information exchange in the context of Ordering. However defining processes is an over-specification of the SP-to-SP Ordering Interface in the sense that it recommends a process architecture within the Service Provider organization.

- The interface specification focuses on the inter-exchanges of information between two Service Providers (the MSP and the SSP). This has been explicitly described in the scenarios (see Section 3.4). Our intention is not to recommend any process architecture or process change within the Service Provider organizations, but to normalize the exchange and type of information in the case of an Order.

- Functional requirements are consistent with the scenarios described in the Chapter 3.

- Technical requirements have been introduced in Chapter 6.

## 57	How is the Ordering interface different from the Service Configuration interface ?

This definition results from a common consensus between the SMART Ordering team and the Service Configuration team.

The SMART SP-to-SP Ordering specification addresses the administrative area of a service request. It is used for exchanges of administrative information and covers :

- The Pre-Ordering phase, including negotiation of a service, of contract, business and technical agreements.

- The Ordering phase including exchange of firm technical and business commitments.

- The Order progress tracking.

The SMART SP-to-SP service configuration specification addresses the configuration of a particular service already Ordered via the SP-to-SP Ordering Interface when the design of the service is flexible enough to allow its configuration by the Customer/User/MSP. A service configuration can never be initiated on a service that hasn’t been ordered before.

The use of the Service Configuration Interface is based on the premise that one SP has Ordered bulk capacity from another Service Provider.  Once the requesting SP is informed their Order has been filled they can use the Service Configuration Interface for real-time management of the purchased capacity.  It is therefore fundamental that the Ordering system notify the service configuration system when an Order has been fulfilled.

The service configuration covers the configuration of an existing service within the limits defined in a contract signed between an MSP and an SSP in the Ordering phase.

Sequentially the Service Configuration phase starts once the Ordering phase is completed (see figure 14).

Figure 14  : The Areas of Ordering and Service Configuration

# APPENDIX A	ABBREVIATIONS

| C-SP | Customer to Service Provider |
| --- | --- |
| CO | Customer Order |
| CR | Customer Request |
| ECIC | Electronic Communication Implementation Committee |
| EoO | Escalation of Order |
| ITU | International Telecommunication Union |
| MSP | Main Service Provider |
| NMF | Network Management Forum |
| OBF | Ordering and Billing Forum |
| OR | Order Request |
| OST | Order Status Tracking |
| PI | Product Instance |
| POST | Proactive Order Status Tracking |
| PT | Product Type |
| QoS | Quality of Service |
| ROST | Reactive Order Status Tracking |
| SE | Service Enquiry |
| SLA | Service Level Agreement |
| SMART | Service Management Automation and Re-engineering Team |
| SO | Service Offer |
| SP | Service Provider |
| SP-to-SP | Service Provider to Service Provider |
| SSO | Subcontracted Service Order |
| SSP | Subcontracted Service Provider/ Secondary Service Provider |
| T1M1 |  |
| TMN | Telecommunications Management Network |

# APPENDIX B	REFERENCES

- A Service Management Business Process Model, Network Management Forum, 1995.

- Statement of User Requirements for Management of Networked Information Systems, Network Management Forum User Advisory Council, October 1992.

- The Lean Communications Provider - Surviving the Shakeout through Service Management Excellence, Elizabeth K. Adams and Keith J. Willetts, McGraw-Hill, 1996.

- Managing a Broadband Environment: “You Can’t Buck the Market”, Keith J. Willets and Elizabeth K. Adams, IEEE Communications Magazine, pp.108-112, December 1996.

- SMART Ordering - SP to SP Interface Requirements, Issue 1.02, The NMF SMART Ordering Team, March 1997.

- SMART Ordering - Analysis and Design Specification, Issue 1.00, The NMF SMART Ordering Team, March 1997.

- SMART Ordering White Paper, Issue 1.1, The NMF SMART Ordering Team, January 1996.

- Service Order Handling, Young B. Choi, Ph.D. Dissertation in Computer Networking and Telecommunications, Computer Science Telecommunications, University of Missouri-Kansas City, USA, December 1995.

- SMART SP-to-SP Order Handling Information Modeling and Interface, Technical Specification, Version 1.0, OSE Laboratory, University of Missouri-Kansas City, January 1996.  (presented at the Colorado Springs, Colorado NMF Meeting)

- SMART SP-to-SP Order Handling Technical Requirement Specification, Version 2.0, OSE Laboratory, University of Missouri-Kansas City, May 1996.  (presented at the Orlando, Florida NMF Meeting)

- SMART SP-to-SP Order Handling Technical Interface Specification, Version 3.0, OSE Laboratory, University of Missouri-Kansas City, October 1996.  (presented at the Barcelona, Spain NMF Meeting)

- A Generic Model for the Service Ordering Interface in the Customer-Service Provider Environment, Adrian Tang, Young Bae Choi, Deokjai Choi, Oh-Young Kang, and Hai Feng Weng, The IEEE Network Operations and Management Symposium (NOMS ’96), Kyoto, Japan, April 15-19, 1996.

- A Generic Service Order Handling Interface for the Cooperative Service Providers in the        Deregulating and Competitive Telecommunications Environment, Young B. Choi and Adrian Tang, The 4th International Conference on Intelligence in Services and Networks (IS&N ’97), Como, Italy, May 27-29, 1997.

- URL=http://www.atis.org/tcif/ecic/.

# APPENDIX C	STANDARDS

- “The Generic Electronic Communications Interface Implementation Requirements for Access Ordering (Working Draft), ECIC/ORD/96-001 DRAFT.

- ANSI T1.214a Defined (OAM&P Generic Network Model for Interfaces between OSs and NEs (Managed Object Class Definitions for Performance Monitoring)).

- Service Layer Related ANSI T1.227-1992 Defined (OAM&P Extension to Generic Network Model for Interfaces between OSs across Jurisdictional Boundaries to support Fault Management - Trouble Administration).

## The comparison between SMART and OBF/ECIC Ordering

There are two major groups of standard bodies and consortia working on the service order handling.  The first group is composed of international organizations, such as, the NMF (Network Management Forum), ITU-T (International Telecommunication Union)-Telecommunication Sector, ETNO (European Telecommunications Network Operators)/EURESCOM (European Institute for Research and Strategic Studies in Telecommunications).  The second group is composed of the North American origins such as the T1M1, ECIC (Electronic Communications Implementation Committee), and OBF (Ordering and Billing Forum).  Both groups are related with the work on the technical specifications on the telecommunications service order handling.

Among the first group of organizations, the NMF is working on the service and network management and especially focusing on the management of the delivery of communications services as well as the large and complex networks that are providing those services to the customers.  Several study groups (SGs) of the ITU-T are related with the NMF’s work: SG 4, 7, 11, and 15.  Among them, the SG 4 (Network Maintenance) is related with the SMART (Service Management Automation and Re-engineering Team) order processing and the ITU-T worked on the X.160 recommendation series for the Customer Network Management (CNM).  The ETNO is a European group working on the TMN implementation since 1994.  The main focus is the identification of a lot of service and operational processes where implementation agreements would be considered important.  The EURESCOM has established PETLABs (Pan European Test Laboratories) to test specifications of the European pilot trial activities.

Among the second group of organizations in the North America working on the service management standards development and implementation, the T1M1.5 group is working on the information modeling and ANSI standards development for domestic OAM&P applications.  The ECIC is focusing on the electronic communications implementation issue definition and resolution by industry users.  It is currently working on the domestic Electronic Binding (EB) implementation issues, especially on the application interfaces for OAM&P information exchange.  The OBF’s main focus is on the business process requirements guidelines of the North American telecommunications industry across several applications: ordering, billing and 800 SMS message handling.  The guidelines are developed by the industry consensus and contributions.

The Comparison between the NMF and ECIC Approaches

Among the current telecommunications service order handling approaches, two major approaches of NMF and ECIC are compared here based on the following criteria: interface scope, protocol, security services, business functions and application information exchange, performance, system management, testing, success criteria, and current status.

## The T1M1 approach

Introduction

Among the group of organizations in the North America working on the service management standards development and implementation, in this paper, we will analyze the T1M1.5 group working on the information modeling and ANSI standards development for domestic OAM&P applications.

T1M1

Besides the international organizations of standard bodies and consortia working on the service order handling, such as, the NMF (Network Management Forum), ITU-T (International Telecommunication Union)-Telecommunication Sector, ETNO (European Telecommunications Network Operators)/EURESCOM (European Institute for Research and Strategic Studies in Telecommunications).  The other group is composed of the North American origins such as the T1M1, ECIC (Electronic Communications Implementation Committee), and OBF (Ordering and Billing Forum).  Both groups are related with the work on the technical specifications on the telecommunications service order handling.

Here, T1M1 is working on the requirements from the OBF which is mainly focusing on the business process requirements guidelines of the North American telecommunications industry across several applications: ordering, billing and 800 SMS message handling.  The guidelines are developed by the industry consensus and contributions.  We consider only the ordering approach among several applications based on the document.

The T1M1 Ordering Approach

The current T1M1 telecommunications service order handling approach is analyzed according to the following criteria: interface scope, protocol, security services, business functions and application information exchange, performance evaluation criteria, system management, testing, success criteria, current document status.

Interface Scope

- The interface the T1M1.5 is working on is the TMN (Telecommunications Management Network) x-interface (M.3010) to support the inquiry functions of the EAO (Electronic Access Ordering) process.  The scope of EAO is to develop an ANSI (American National Standards Institute) standard for electronic access ordering.

- The TMN is a generic model that describes the functions, interfaces, and reference points of a management system.  TMN is organized around OSI concepts, and is also is organized around object-oriented techniques, which makes it capable of supporting CMIP and CMISE.

- Additionally, TMN employs the five management functional areas of OSI.

- The TMN architecture is organized around TMN functions and function blocks.  These function blocks describes the activities that TMN performs in its management responsibilities.  The function blocks are also further organized into functional components.  Any pair of function blocks that exchange information between them are separated by reference points.

- The following describes the function blocks that are directly related to the management operations of TMN.

- Operations Systems Functions (OSF) block: Provides overall management responsibilities for the entire TMN.

- Network Element Function (NEF) block: Communicates with TMN in order to allow the managed network to be managed.  Responsible for representing the managed functions to the TMN; as such these functions are not part of TMN.

- Workstation Function (WSF) block: Acts as the interface to the human user, and as such, has part of it operating on the outside of the TMN.

- Mediation Function (MF) block: Acts as a converter for convergence operator between an OSF and NEF (or a QAF) in the event that these function blocks have some differences that need to be resolved.

- Q Adapter Function (QAF) block: Acts as a converter between TMN functions and non-TMN functions that are somewhat similar to NEF and OSF.  It translates between a TMN reference point and a proprietary reference point.

The following figure (insert figure number here) shows the function blocks and reference points of TMN.

Function Blocks and Reference Points

The TMN uses reference points to delineate the boundaries between function blocks.  These reference points serve to identify the information that is transferred between function blocks.  There are two types of reference points: TMN reference points and non-TMN reference points [5].

TMN reference points:

- The f reference points are located only between the WSF and OSF function blocks and/or the WSF and MF function blocks.

- The x reference points are located only between the OSF function blocks residing in different TMNs.

- The q reference points are used to separate the flow of information between function blocks.  As a general rule, function blocks using the q reference points may not support the full services of the TMN reference model.

Non-TMN reference points:

- The g reference points are located outside a TMN boundary.  This is considered an interface between a WSF and a human user.

- The m reference points are located outside the TMN between the QAF and non-TMN entities.

Workers

- The standard was prepared by the Working Group T1M1.5 of ANSI Technical Subcommittee T1M1 (Internetwork Operations, Administration, Maintenance and Provisioning).

Protocol

- The T1M1 approach is based on the OSI (Open Systems Interconnection) CMISE/CMIP (Common Management Information Service Element/Common Management Information Protocol).

Security Services

- Authentication of inquiry is required.

Business Functions and Application Information Exchange

- Management information exchanged for inquiry function of EAO ins the following:

- Location inquiry

- Service availability inquiry

- CFA (Connecting Facility Assignment) inquiry

Performance Evaluation Criteria

- Performance of the interface is not applicable.

System Management Considerations

- The followings are considered for the system management.

- Fault management (Trouble Administration): ANSI T1.227 (1995)

- Application based security: ANSI T1.254 (1996)

Testing

- No concrete considerations yet.

Success Criteria

- No concrete consideration yet.

Current Standard Status

- Operations Administration, Maintenance, And Provisioning (OAM&P) - Extension To Generic Network Model For Interfaces Across Jurisdictional Boundaries To Support the Inquiry and Ordering Process for Electronic Access Ordering, Draft Standard (ANSI T1.256: Working Draft Standard), T1M1.5, November 27, 1996.

Modeling Tools

- The standard defines an information model using the GDMO (X.722) and ASN.1 (X.680).

Others

- The URL of the T1M1 is http://www.t1.org.

- The OBF (Ordering and Billing Forum) requirements were modeled using the T1 object model.

# LIST OF FIGURES

Figure 1 : The Overall Service Management Model

Figure 2 : The SMART Ordering Process, Inputs and Outputs

Figure 3 : A Chain of Service Providers Involved in the Ordering Process

Figure 4 :Pre-Ordering Phase Interactions (Order Type = Provide)

Figure 5 : Ordering Phase Interactions (Order Type = Provide)

Figure 6 : Pre-Ordering Phase Interactions (Order Type = Change)	19

Figure 7 : Ordering Phase Interactions (Order Type = Cease)	21

Figure 8 : The SP-to-SP Ordering Interface	23

Figure 9 : Activities Utilising an SP-to-SP Ordering Interface

Figure 10 : Order Administration 	26

Figure 11 : Pre-Ordering Event Activities	27

Figure 12 : Ordering Event Activities	29

Figure 13 : Item Tracking Event Activities 	31

Figure 14 : The Areas of Ordering and Service Configuration

# LIST OF TABLES

table 1 : The Overall Service Management Business Process Model [1]

table 2 : The SMART Ordering Process, Inputs and Outputs [1]

table 3 : A Chain of Service Providers Involved in the Ordering Process

table 4 :Pre-Ordering Phase Interactions (Order Type = Provide)

table 5 : Ordering Phase Interactions (Order Type = Provide)