# Peer-To-Peer Service Configuration

# Information Agreement

# TM Forum 604

Evaluation Version 1.0

June 1998

© TeleManagement Forum 1998.

Members of the TeleManagement Forum are granted limited copyright  waiver, under the
terms of the Agreement on Intellectual Property rights, to reproduce this document.

NOTICE

The TeleManagement Forum (“TM Forum”)
has made every effort to assure that the contents of 
this document are accurate; however, due to the inherent 
complexity in the design and implementation of protocols, no 
liability is accepted for any errors or omissions or for 
consequences of any use made of this document.  Under no 
circumstances will the TM Forum be liable for direct or indirect damages

or any costs or losses resulting from the use of this specification.

The risk of designing and implementing products in accordance with

this specification is borne solely by the user of this specification.

This document may involve a claim of patent rights by one or

more TM Forum members, pursuant to the agreement on

Intellectual Property Rights between TM Forum and its members,

and by non-members of TM Forum.

Inquiries should be directed to the 
TM Forum office at:

1201 Mt. Kemble Avenue

Morristown, NJ  07960 USA

Tel. No.  +1 973-425-1900

Fax No. +1 973-425-1515

DOCUMENT LIFE CYCLE

The Peer-to-Peer Service Configuration Information Agreement is being issued as Evaluation Version 1.0.  It can be considered valid until June 30, 1999 at which time the TeleManagement Forum ("TM Forum")expects to update it to reflect comments from implementation experience, as well as to reflect additional member comment and public comment.

The purpose of an Evaluation Version is to encourage input based on experiences of members and the public as they begin to use the document.  Following the Evaluation Period, documents that are seen to deliver value are candidates for formal approval by the TM Forum.  All documents approved by the TM Forum (as well as those previously approved by NMF) undergo a formal review and approval process.

This document will continue under formal change control.  Supporting work will be issued as companions to this document.  A document of this type is a “living document,” capturing and communicating current knowledge and practices.   Further inputs will be made because of detailed work ongoing in the TM Forum and the industry.

Individuals or companies who are not members of the TM Forum are encouraged to provide comments on this document. However, in order for their comments to be considered, a signed waiver must be on file with TM Forum pertaining to intellectual property rights. To obtain this form, please contact the TM Forum.

ACKNOWLEDGMENTS

Editor:

Stephen Fratini

Bellcore

331 Newman Springs Road, Room NVC-2Z351

Red Bank, NJ 07701

USA

Phone: +1 732 758 2152

Email: sfratini@notes.cc.bellcore.com

Document Contributors:

Mike Best (Team Leader, Newbridge), Steve Fratini (Editor, Bellcore), James Green (Fujitsu), Jane Hall (GMD Fokus), Connie Hunt (SBC)

Table of Contents

List of Figures	v

Preface	viii

1	Introduction to this document	1

1.1	How this Document is to be Used and Extended	1

1.2	Document structure	2

1.3	Key terms used in this document	3

1.4	Identity of the requirements definition document	4

2	Summary of Requirements	5

2.1	Introduction to the requirements	5

2.2	Current Environment	5

2.3	Summary of Business Requirements	5

2.4	Summary of Technical Constraints	6

2.5	System Overview	6

2.5.1	How the interface is expected to be used	7

2.5.2	Use Cases	8

3	Modeling Approach	15

3.1	Foundation	15

3.2	Customization of Conventional Approaches	15

4	Analysis Model	17

4.1	Object Model	17

4.1.1	Object Class Descriptions	17

4.1.2	Rationale Behind the Object Class Selection	19

4.1.3	Rules for Object Instance Creation	21

4.1.4	Deferred Attributes and Operations	21

4.1.5	Bandwidth Service Endpoint Modeling	22

4.1.6	Data Dictionary	23

4.2	Dynamic Model	33

4.2.1	Scenario Descriptions and Interaction Diagrams	33

4.2.2	State transition diagrams	41

4.3	Functional Model	45

4.3.1	Operations at the exported interfaces	45

5	Design Considerations	54

5.1	CORBA-GDMO Functional Variations	54

5.2	Deferred Definition of Attributes and Operations	54

5.3	Shared Bulk Capacities	54

6	Requirements Satisfaction Analysis	56

7	Conclusions and Insights	58

Appendix A: Acronym List	a-1

Appendix B: References	b-1

Appendix C: Analysis of Relevant Standards	c-1

Annex A: Summary of OMT Object Modeling	aa-1

List of Figures

Figure 1-1. Plan for PtPSC Modeling Work	2

Figure 2-1. Focus of PtPSC Interface	7

Figure 2-2. Interface Defined in this Document	7

Figure 2-3. Use Case Summary Diagram	9

Figure 4-1. Object Model Diagram	18

Figure 4-2. Routing Options	20

Figure 4-3. One Bandwidth Service Per Bandwidth Service Endpoint	22

Figure 4-4. One Bandwidth Service Endpoint for Many Bandwidth Services	23

Figure 4-5. Scenario for Query Operation	33

Figure 4-6. Scenario for the Modify Operation	34

Figure 4-7. Scenario for the Destroy Operation	35

Figure 4-8. Scenario for the Create Configuration Group Operation	36

Figure 4-9. Scenario for the Create Service Routing Profile Operation	37

Figure 4-10. Scenario for the Create Service Traffic Profile Operation	38

Figure 4-11. Scenario for the Create Scheduled Request Operation	39

Figure 4-12. Scenario for the Establish Bandwidth Service Operation	40

Figure 4-13. Scenario for Notifications	40

Figure 4-14. Scenario for the Release Bandwidth Service Operation	41

Figure 4-15. State Diagram for Bandwidth Service	42

Figure 4-16. State Diagram for Bandwidth Service Endpoint	43

Figure 4-17. State Diagram for Bulk Capacity	44

Figure 4-18. State Diagram for Bulk Capacity Endpoint	45

Preface

TeleManagement Forum ("TM Forum") is an international consortium of communications service providers and their suppliers, dedicated to promoting industry-wide agreements on how management information should be exchanged between a service provider and its customers, its suppliers and/or other service providers or network operators.  Such agreements are essential to achieve a seamless, cost-efficient operation where multiple companies are involved in the fulfillment of service requests, the management of services while in use by customers, the correction of problems that might impair service quality, and the accurate billing of the customer for services used.

These agreements are becoming an increasing necessity as regulatory forces change the landscape for service providers.  The addition of new value-add service providers such as Internet Service Providers and the development of global alliances mean that the service delivery chain is being extended beyond the traditional, monopoly environment where a single company had end-to-end control over the provision and management of customer services.  At the same time, some companies are choosing to focus their business on network infrastructure, either providing high-capacity inter-city links to service providers, or providing local access to customer sites.  Finally, as has been the case for many years, service providers and network operators rely on equipment suppliers’ element management systems to make it possible to manipulate the network to create customer services, and to report meaningful information about the status of elements, in order that the network, and ultimately, customer services, can be maintained at required levels of quality.

TM Forum members work together to determine how their processes should be linked, and how they should work with customers’ and suppliers’ systems.  They work together because they believe that to solve this type of problem alone requires extensive time and money (which is better spent developing new offerings) and that proprietary solutions will not be accepted in the marketplace.  They work together because they see that in these areas, there is no competitive advantage to be gained over exchanging data in a proprietary fashion, and that, conversely, by working together they can considerably reduce both the time to development and the risk of making bad decisions.

This analysis pertains to a very specific area in which TM Forum members have determined that a common, industry-wide approach is warranted.  They invite others to review these agreements, and to adopt the information model reflected in this document to meet these requirements.  They invite suppliers to become familiar with these requirements, and to work with them on the details of how the requirements will be met through the application of standards and other industry-accepted specifications and technology.

# Introduction to this document

This document represents an analysis of the Peer-to-Peer Service Configuration (PtPSC) requirements in NMF 502. The analysis model is technology independent (not specific to any transport technology such as Frame Relay, SDH or ATM) and protocol independent (not specific to any management protocol such as GDMO or CORBA). The object model in this document is based on the ideas of the PtPSC team members as well as ideas inspired by other standards documents (see Appendix C for an analysis of the standards considered by the PtPSC team).

The primary benefit of this document is the definition of a generic object model structure that can be specialized for multiple technologies. The term “structure” refers to the actual object defined, the relationships amongst the defined objects, and the data structure of the attributes and operations within the objects.

## How this Document is to be Used and Extended

This document defines the functionality and information for the exchange of management information to meet the requirements defined in the Peer-to-Peer Service Configuration Business Agreements Document, NMF 502. While it contains sufficient information to be a “stand alone” document, it is better read together with the Business Agreements Document in order to gain a full understanding of the information it contains as an analysis of a solution to the set of requirements.

Version 1 of this document is primarily intended to be used as a basis for the definition of technology/protocol specific solution sets, e.g., an ATM/GDMO solution set, or an SDH/CORBA solution set. The generic object model in this document serves as an intermediate step between the requirements in the Business Agreements document and the detailed object models to be provided in technology/protocol specific Solution Sets. Each Solution Set will contain a technology specific OMT object model, and one or more protocol specific models derived from the technology specific OMT model.

Each technology specific model makes use of the object model structure and the attribute/operation syntaxes defined in this Information Agreements document. The technology specific models may provide refinements of the attributes and operations in the Information Agreements document, and may also add new attributes and operations.

The understanding gained from the definition of technology/protocol specific solution sets will be used to define an extension/refinement of this document, i.e., Version 2. In addition, the output of one or more TM Forum catalysts projects may be used as input to Version 2. It may also be necessary to prepare a second version of the Solution Sets in order to align with Version 2 of the Information Agreements document.

The end goal of the PtPSC team is to define a set of high quality, future-proof, prototype tested interfaces for peer-to-peer service configuration. The plan for the PtPSC modeling work is as shown in Figure 1-1.

Figure 1-1. Plan for PtPSC Modeling Work

## Document structure

This document is structured in seven major sections. Sections 2 and 3 are considered informative and provide information to allow the reader to understand the models produced in section 4.  The following reflects what you will find in each section:

Section 1	Introduces the document

Section 2	Summarizes the business objectives that are driving the need for this interface, and the high level operational requirements that must be met.  It provides an overview of the system that will support the interface in question and other interfaces required to achieve automation of service management or network management functions.

Section 3	Informs the reader about the techniques used by the analysis  team by presenting (mainly through references) the basic modeling language used and the restrictions on the use of the language adopted by the team.

Section 4	Describes the information to be represented by the system and highlights the information flowing across the interface. It also describes the behavior of the system as seen across the interface. Multiple diagrams are used to represent different perspectives.

Section 5	Contains advice from the analysis team to those making the detailed protocol dependent technical specifications. .

Section 6	Analyses the requirements and shows how they have been satisfied or otherwise by the analysis  results. It should draw attention to the steps taken within the models to cater for optionality expressed within the requirements.

Section 7	Offers conclusions and insights from the team members who prepared the analysis  document, including any issues they perceive, or their expectations of companies to implement the models.

Appendix A	Provides a listing of the Acronyms used in the document.

Appendix B	Detailed information (date, document number, source) regarding any document referenced in the Information Agreements document.

Appendix C	Provides a review of work that has been done by other groups (outside TM Forum) or that has been done by TM Forum, and which should be used in the development of Solution Sets that may be derived from this document.

Annex A	Provides a summary of the Object Modeling Technique (OMT) methodology.

## Key terms used in this document

This section provides a short description of the key terms used within this documents

Customer: the entity requesting a product or service from a given Service Provider. It is possible that this could be an agent of the Customer rather than the true product or service user.

Main Service Provider (MSP): the Service Provider who has received a request from a Customer and needs to evaluate how best to meet the request.

Subcontracted (or Secondary) Service Provider (SSP): the Service Provider who has been subcontracted by the MSP to deliver the original Customer request fully or a component of the request.

Order: details of a request from a customer to a service provider to undertake work on a product or service offered by the service provider.

Service Level Agreement (SLA): a commitment or group of commitments made to a Customer (in our case an MSP) which require routine measurement and reporting of actual service delivery against targets. The SLA measures considered must all have significance for the Customer. This means that for a particular Customer the QoS parameters mentioned in its SLA, negotiated over the Ordering interface, must be definable, controllable and measurable by the SSP.

Bandwidth Service: a transport connection setup by an SSP on behalf of an MSP.

Bulk Capacity: a transmission facility used to support one or more bandwidth services.

Peer-to-Peer Interface: an interface between two service providers (or two administrative units within a service provider) related to a specific management activity.

Service Configuration: (in the context of this document) the management activity dealing with the establishment, release and modification of bandwidth services.

Protocol Independent: (in the context of this document) an object model that does not presume a particular “management” protocol. For example, GDMO/CMIP, SNMP, and CORBA are NOT protocol independent models.

Technology Independent: (in the context of this document) an object model that is not specific to a particular transport technology such ATM, SDH or Frame Relay.

## Identity of the requirements definition document

The requirements on which this Information Agreements document is based can be found in the document entitled Peer-to-Peer Service Configuration Business Agreements, NMF 502, Issue 1.0, April 1997.

# Summary of Requirements

This section summarizes the requirements on the Peer-to-Peer Service Configuration interface defined in the document Peer-to-Peer Service Configuration Business Agreements, NMF 502 [1]. Readers needing to have a full appreciation of the requirements should go to the original source document.

## Introduction to the requirements

The global nature of today's emerging telecommunication services is not only multi-national, but is also becoming multi-corporational as companies extend their reach to serve larger geographical areas or to increase the range of services they can offer to their customers.  Although one route to accomplish this extended reach is through direct expansion of networks and development of new service capabilities, another route is via formal or informal relationships (or the formation of consortia) with other providers.  Through these relationships, providers without a physical network capability in a certain area (or with insufficient bandwidth capacity to serve their customers needs) will contract to pay for the use of bandwidth from another provider.  In its simplest form, this contractual arrangement may be considered a sale of ‘wholesale’ services (bandwidth) by one provider to another, and may be effected through a service order.

The new business models for providing services across multiple service providers brings about several challenges to the integration, or more precisely, cooperation or participants in the service delivery chain. This new way of doing business brings opportunities (doing business worldwide, for example) as well as risks.  Chief among the risks, when attempting to fulfill a customer’s end-to-end needs in a timely fashion, is that the process of fulfillment is no longer totally within a single service provider’s control. To minimize the risk of failing to meet a customer commitment, Peer-to-Peer Service Configuration requirements specify the capabilities needed for a service provider to (re)configure, in an automated fashion, bandwidth resources that are physically owned by another service provider.

## Current Environment

The traditional provider to provider configuration process does not meet the requirements of the new market environment.  Single-provider bandwidth-services need to be integrated into a complete, multi-provider end-to-end service.  Internal and proprietary systems are no longer sufficient when a service provider has to negotiate with other service providers.

Among the problems of today’s process are the following:

- Lack of automation (e.g., manual procedures for interacting between service providers makes service delivery time-consuming expensive, complex and error-prone)

- Lack of visibility/control (e.g.,  inability to monitor and react to changing conditions causing service outages and resource shortages)

- Lack of consistency in service delivery and quality (e.g., different paradigms between service providers makes end to end multi-provider services complicated to deploy, maintain etc.)

- Lack of flexibility (e.g., inability to provide new services or expand existing service quickly to take advantage of market changes and opportunities)

## Summary of Business Requirements

A summary of the business requirements and assumptions stated in the Peer-to-Peer Service Configuration Business Agreements document is as follows:

- There is a need for a service provider to establish, modify and release bandwidth-services on-demand within the jurisdiction of another (peer) service provider.

- There is a need for a service provider to have access to an up-to-date inventory of resources it controls within another service provider jurisdiction.  The inventory includes status as well as other relevant information.

- There is a need for a provider to logically group the bandwidth-services it has established within another service provider jurisdiction.

- There is a need for a service provider to be notified as quickly as possible when a service-affecting fault condition exists and when it has been rectified

- Assumption(s):

- Interaction between service providers occurs after a contractual arrangement, i.e., SLA, has been put into place. Such an arrangement could be established through an ordering procedure or other formalized contract. The establishment of this contractual arrangement is out of scope for the purposes of this work, however its existence is a fundamental assumption.

## Summary of Technical Constraints

A summary of the mandatory environmental constraints on the Peer-to-Peer Service Configuration (PtPSC) interface is as follows:

- The Service Configuration interface must operate in an automated time-critical environment. It must therefore operate in real-time.

- The Service Configuration interface will provide only current information on resources.

- The Service Configuration interface shall provide for strong, two-way entity authentication.

- The interface will provide for data integrity. No data shall be lost due to session failure.

- Due to the real-time nature of the interface, use of confirmed messaging, and anticipated high-volume of transactions between Service Providers a dedicated management connection, between the peer systems, must be provided.

- All time values (time stamps) exchanged across the interface should be in a format that accounts for time-zone difference.  For example, systems may agree to use GMT.

## System Overview

The system overview diagram (shown in Figure 2-2) depicts the scope of the system modeled, the system boundary, and elements outside the system that interact with it. The interface under study is labeled as the “Focal Interface” in Figure 2-2. It should be noted that the main focus is on the interface between the Service Configuration processes of the MSP and SSP. A secondary emphasis is on the Service Process Resolution processes (mainly from the perspective of alarm notifications).

Figure 2-2. Focus of PtPSC Interface

### How the interface is expected to be used

The service configuration interface shown in Figure 2-3 is intended to be between the Service Management Layers of the MSP (service requester) and the SSP (service provider). The interface is used by the MSP to request that a bandwidth service (a connection) be established by the SSP over transmission facilities (referred to as bulk capacity instances) previously purchased or leased by the MSP from the SSP. The purchasing/leasing of the bulk capacity is outside the scope of the peer-to-peer service configuration interface.

In addition, the peer-to-peer service configuration interface is used by the SSP to send reports to the MSP concerning status changes to instances of bandwidth service and bulk capacity.

Figure 2-3. Interface Defined in this Document

### Use Cases

The Use Cases described in this sub-section are summarized in the Figure 2-4.

Figure 2-4. Use Case Summary Diagram

#### Establish Bandwidth-service Use Case

Use Case Name:	Establish bandwidth-service

Short Summary:	Allows Service Configuration Process to establish a bandwidth-service.

Actors Involved:  	Service Configuration Process

Pre-conditions:   	Access has been secured.

Begins when:      	Transaction arrives.

Description:      	If a Main Service Provider wishes to establish a bandwidth-service, it must first have been secured as able to perform this interaction.  If the capacity is available, and the details of the request are valid, the bandwidth-service is established.

In the case of a successful establishment, a request confirmation containing a successful indication as well as the unique identifier of the bandwidth-service, is provided to the Main Service Provider.

If the request failed a request confirmation with details about the

reason for the failure is provided to the Main Service Provider.

Ends when:        	Transaction has been processed.

Post-conditions:  	A bandwidth-service has been established.

Exceptions:       		Capacity not available - request cannot be fulfilled.

Request invalid - some aspect of the request is invalid.

#### Establish Service-group Use Case

Use Case Name:    	Establish service-group

Short Summary:    	Allows Service Configuration Process to establish a service-group.

Actors Involved:  	Service Configuration Process

Pre-conditions:   	Access has been secured.

Begins when:      	Transaction arrives.

Description:      	If the details of the request are valid, the service-group is established. In the case of a successful establishment, a request confirmation containing a successful indication as well as the unique identifier of the bandwidth-service, is provided to the Main Service Provider.

Ends when:        	Transaction has been processed.

Post-conditions:  	A service-group has been established.

Exceptions:       	Service-group establishment fails - a confirmed response details the nature of the failure

#### Modify Bandwidth-service Use Case

Use Case Name:    	Modify Bandwidth-service

Short Summary:    	Allows Service Configuration Process to modify a bandwidth-service.

Actors Involved:  	Service Config Process (MSP)

Pre-conditions:   	Access has been secured.  Bandwidth-service exists.

Begins when:      	Transaction arrives.

Description:      	Use case begins when transactions arrives.  If the transaction is valid, the service is modified.  A confirmed response is sent to the Service Config Process (MSP) indicating a successful operation.

Ends when:        	Transaction ends.

Post-conditions:  	Bandwidth-service is modified

Exceptions:       	Bandwidth-service modification fails - a confirmed response details the nature of the failure

#### Modify Service-group Use Case

Use Case Name:    	Modify service-group

Short Summary:    	Allows Service Configuration Process to modify a service-group.

Actors Involved:  	Service Config Process

Pre-conditions:   	Access has been secured.  Service-group exists.

Begins when:      	Transaction arrives.

Description:      	Use case begins when the transaction arrives. If the transaction is valid, the service-group is modified.  A confirmed response is sent to the Service Config Process indicating a successful operation.

Ends when:        	Transaction ends.

Post-conditions:  	Service-group is modified

Exceptions:       	Service-group modification fails - a confirmed response details the nature of the failure

#### Notification Use Case

Use Case Name:    	Fault notification service

Short Summary:    	Provides fault notification

Actors Involved:  	Service Problem Resolution Process

Pre-conditions:   	Access has been secured.

Begins when:      	User registers to receive fault notifications.

Description:      	As part of the registration request the user provides information describing the event service it would like to use.  The system uses this information when deciding whether to emit a fault notification.

If conditions exist which satisfy the fault notification criteria a notification is generated.  This can be a simple fault notification or a context specific fault,  depending on the nature of the service offered.

When conditions are rectified a clearing notification is generated.

Ends when:        	User de-registers fault notifications.

Post-conditions:  	None.

Exceptions:       		System cannot send notifications.

#### Query Bulk-capacity Use Case

Use Case Name:    	Query bulk-capacity

Short Summary:    	Allows Service Configuration Process to query a bulk-capacity.

Actors Involved:  	Service Config Process

Pre-conditions:   	Access has been secured.  Bulk-capacity exists.

Begins when:      	Transaction arrives.

Description:      	Use case begins when the transaction arrives.  If the transaction is valid, the requested information is returned.  A confirmed response is sent to the Service Config Process indicating a successful operation.

Ends when:        	Transaction ends.

Post-conditions:  	Non-service affecting

Exceptions:       	Query fails - a confirmed response describes the nature of the failure  (e.g. info doesn't exist)

#### Query Bulk-capacity endpoint Use Case

Use Case Name: 	Query bulk-capacity endpoint

Short Summary:    	Allows Service Configuration Process to query a bulk-capacity endpoint.

Actors Involved:  	Service Config Process

Pre-conditions:   	Access has been secured.  Bulk-capacity endpoint exists.

Begins when:      	Transaction arrives.

Description:      	Use case begins when the transaction arrives.  If the transaction is valid, the requested information is returned.  A confirmed response is sent to the Service Config Process indicating a successful operation.

Ends when:        	Transaction ends.

Post-conditions:  	Non-service affecting

Exceptions:       	Query fails - a confirmed response describes the nature of the failure  (e.g. info doesn't exist)

#### Query Bandwidth-service Use Case

Use Case Name: 	Query bandwidth-service

Short Summary:    	Allows Service Configuration Process to query a bandwidth-service.

Actors Involved:  	Service Config Process

Pre-conditions:   	Access has been secured.  Bandwidth-service exists.

Begins when:      	Transaction arrives.

Description:      	Use case begins when the transaction arrives.  If the transaction is valid, the requested information is returned.  A confirmed response is sent to the Service Config Process indicating a successful operation.

Ends when:        	Transaction ends.

Post-conditions:  	Non-service affecting

Exceptions:       	Query fails - a confirmed response describes the nature of the failure  (e.g. info doesn't exist)

#### Query Bandwidth-service Endpoint Use Case

Use Case Name: 	Query bandwidth-service endpoint

Short Summary:    	Allows Service Configuration Process to query a bandwidth-service endpoint.

Actors Involved:  	Service Config Process

Pre-conditions:   	Access has been secured.  Bandwidth-service endpoint exists.

Begins when:      	Transaction arrives.

Description:      	Use case begins when the transaction arrives.  If the transaction is valid, the requested information is returned.  A confirmed response is sent to the Service Config Process indicating a successful operation.

Ends when:        	Transaction ends.

Post-conditions:  	Non-service affecting

Exceptions:       	Query fails - a confirmed response describes the nature of the failure  (e.g. info doesn't exist)

#### Query Service-group Use Case

Use Case Name: 	Query Service-group

Short Summary:    	Allows Service Configuration Process to query a service-group.

Actors Involved:  	Service Config Process

Pre-conditions:   	Access has been secured.  Service-group exists.

Begins when:      	Transaction arrives.

Description:      	Use case begins when the transaction arrives.  If the transaction is valid, the requested information is returned.  A confirmed response is sent to the Service Config Process indicating a successful operation.

Ends when:        	Transaction ends.

Post-conditions:  	Non-service affecting

Exceptions:       	Query fails - a confirmed response describes the nature of the failure  (e.g. info does not exist)

#### Release bandwidth-service Use Case

Use Case Name:    	Release Bandwidth-service

Short Summary:    	Allows Service Configuration Process to release a bandwidth-service.

Actors Involved:  	Service Config Process

Pre-conditions:   	Access has been secured.  Bandwidth-service exists.

Begins when:      	Transaction arrives.

Description:      	Use case begins when the transaction arrives.  If the transaction is valid, the service is released.  A confirmed response is sent to the Service Config Process indicating a successful operation.

Ends when:        	Transaction ends.

Post-conditions:  	Bandwidth-Service is destroyed

Exceptions:       		Bandwidth service is in use.

#### Release Service-group Use Case

Use Case Name:    	Release service-group

Short Summary:    	Allows Service Configuration Process to release a service-group.

Actors Involved:  	Service Config Process

Pre-conditions:   	Access has been secured.  Service-group exists.

Begins when:      	Transaction arrives.

Description:      	Use case begins when transaction arrives.  If transaction is valid, the service is released.  A confirmed response is sent to the Service Config Process indicating a successful operation.

Ends when:        	Transaction ends.

Post-conditions:  	Service-group is destroyed

Exceptions:       		None

#### Security access-control Use Case

Use Case Name:	User access control

Short Summary:   	Applies access control policy

Actors Involved:  	Service Config Process or Service Problem Resolution Process

Pre-conditions:   	Session has been established.

Begins when:      	Transaction arrives.

Description:      	If the transaction is valid, it (the activity associated with the transaction) is 
allowed to continue.

Ends when:        	Access control validation is complete.

Post-conditions:  	User transaction is submitted to the system.

Exceptions:       		Access is denied.

#### Security identification/authentication Use Case

Use Case Name:    	User identification and authentication

Short Summary:    	Applies identification and authentication policy

Actors Involved:  	Service Config Process or Service Problem Resolution Process

Pre-conditions:   	Contractual agreement is in force between service providers.

Begins when:      	When session request arrives.

Description:      	If the transaction is valid, a session is established.  A confirmed response is sent indicating a successful operation.

Ends when:        	Session is established.

Post-conditions:  	User is able to access system.

Exceptions:       		Session is denied.

# Modeling Approach

This section has two parts.  The first informs the reader about the foundations of the modeling approach adopted, and the second describes variations from the conventional approach to modeling selected/defined by the modeling team.

## Foundation

The general modeling approach described in the document entitled TM Forum Solution Set Analysis Document Template will be used for the PTP Service Configuration object model. In particular, the object model defined in this document is protocol independent, and is based on the TM Forum Information Modeling teams interpretation of the Object Modeling Technique (OMT).

## Customization of Conventional Approaches

The following requirements apply to the PTP Service Configuration object model. Some of the requirements are followed by a rationale.

- The object model should be technology independent (i.e., not designed for any specific technology such as ATM or Frame Relay).

- Rationale: Each technology specific model, derived from a technology independent model, will have similar structures (at least where the technologies are similar), and thus the likelihood of re-usability is increased. Also, a technology independent model can be applied to new technologies as needed, i.e., a technology independent model is more extensible than a technology specific model.

- The primary method for deriving a technology specific model from the technology independent model should be subclassing. To put it another way, the technology independent model should be defined in such a way that technology specific models can be derived by subclassing. The 4 other methods were considered:

- conditional packages with profiling: This non-runtime approach embodies a methodology specific to GDMO and is not easily supported by the OMT methodology selected by the TM Forum.

- optional attributes: This approach is basically a run-time methodology for gaining some level of generality in an object model - this approach can be used in conjunction with subclassing.

- use of something like the ASN.1 CHOICE construct: This run-time approach is similar to the “optional attributes” approach.

- mandatory packages: This approach entails the use of several mandatory packages, one for each technology to be supported. It is difficult to extend a model that is designed using this approach, since one needs to include all the technologies at the time of initial design.

Rationale for Selecting the Subclassing Approach: The subclassing approach was selected since it was common to all object oriented design methodologies, and appeared to be the most extensible approach (the other four approaches requires one to know what technologies are to be supported at the time the object class is defined). On the other hand, if one first defines a generic set of object classes with no technology specific characteristics, subclasses can be defined at a later point in time for the purpose of supporting a specific technology such as ATM or Frame Relay.

- First a protocol and technology independent model will be defined (this present document), and then one or more technology specific and protocol independent models will be defined, e.g., an OMT model for ATM or Frame Relay. The solution sets will entail technology and protocol specific models, e.g., a GDMO model for Peer-to-Peer Service Configuration of Frame Relay connections. The last step, i.e., mapping from a technology specific and protocol independent model to a technology/protocol specific model is to be automated by an OOA/OOD tool.

- Also considered was the possibility of going from a technology/protocol independent model to a technology independent and protocol specific model, and then defining a technology/protocol specific model. In this case, the automated OMT tool needs to be used at the middle step and not the last step. It was determined that it would be better to put off a decision about the specific protocol as late in the design process as possible, and so the first approach was selected.

- Concerning the ODP information/computation model separation, it was decided to encapsulate both informational and computational aspects within the same object, as opposed to having a mixture of objects some of which are primarily informational and others are primarily computational (e.g., a connection handler object). In the latter approach, the computational objects offer operations that allow clients to act on the information objects.

- Finally, it was decided that the object model will have a service-centric look and feel, as opposed to a network level style, where “service-centric” implies the object model is

- constructed from the point-of-view of the service customer,

- loosely coupled to the underlying resources,

- less detailed with fewer objects than a network view model, and

- focused on the bandwidth service as opposed to the endpoints of the bandwidth service.

# Analysis Model

This section contains the results of the analysis in the form of an Object Model, a Dynamic Model and a Functional Model (basically the OMT methodology).

The object model identifies and describes the ‘things’ of concern to the users of the interface. Such things are defined as objects. Relevant relationships between objects are also defined.

The dynamic model describes the most important interactions between the objects themselves and between the objects and external systems.

The Functional model defines object signatures and operations particularly relevant to the interfaces.

## Object Model

The PtPSC Object Model is the result of object modeling related to the Peer to Peer Service Configuration domain. The object model includes the definition of object classes, object class relationships and an associated data dictionary. This subsection also highlights the interfaces the system exports to users of the Peer to Peer Service Configuration interface.

### Object Class Descriptions

The following class diagram shows the objects and their relationships. These objects are defined more fully in Section 4.1.6, data dictionary

Figure 4-5. Object Model Diagram

Some general guidelines concerning the object modeling approach, taken by the TM Forum, are presented in Annex A of this document.

### Rationale Behind the Object Class Selection

Some of the object classes in the above diagram are obvious, e.g., bandwidth service, bandwidth service endpoint, bulk capacity and bulk capacity were talked about in the Information Agreements document, but the selection of other object classes requires additional explanation.

#### Service Profile Objects

The two profile objects, i.e., Service Routing Profile and Service Traffic Profile, are based on similar objects defined in the ATM Forum M4 Network View CMIP MIB.

Considering the fact that many instances of bandwidth service may have the same traffic characteristics and that some service providers may only offer a fixed set of traffic profiles, it was decided to allow for the creation of a few service traffic profile object instances which can be referenced by many bandwidth service objects. Optionally, if need be, one can include the traffic characteristic information in the service characteristic attribute of the bandwidth service object.

The MSP is afforded two ways of specifying a route for a bandwidth service. In one case the MSP requests that the SSP setup a bandwidth service between two bulk capacity endpoints on the boundary of the SSP’s network. In this case, the SSP selects the route (see Case 1 in Figure 4-6). In the second case, the SSP is afforded the option of selecting a route through the SSP’s territory (see Case 2 in Figure 4-6). The route selection capability is supported in the object model by the service subnetwork and bulk capacity object classes. In particular, the SSP selects a route by either specifying a list of bulk capacity object instances, or by specifying a list of Service Sub-Networks. In Figure 4-6, the bandwidth service in Case 2 follows the path: BC 1, BC 3, BC 5, BC 6. The service routing profile is used to model a particular route through an SSP’s network. Since several bandwidth services may follow the same route, a single instance of service routing profile can be reference by more the one bandwidth service.

As is the case with the service traffic profile, one has the option of including the routing information in the service characteristic attribute of the bandwidth service object.

Figure 4-6. Routing Options

#### Scheduled Request Object

In some cases the MSP may want to request that one or more bandwidth services be established, modified or released at some point in the future. In the case of modification or release, an instance of scheduledRequest can reference multiple instances of bandwidthService. So, for example, a collection of bandwidthServices can be scheduled for release at some set time by

- creating an instance of scheduledRequest,

- indicating in the requestInfo attribute that the scheduled activity is “release”, and

- making the bwsPointerList attribute point to the bandwidthServices that are to be released.

- In the case of scheduled establishment for several bandwidthServices, the following actions take place:

- the desired bandwidthService instances are created with their adminStatus set to “locked”,

- an instance of scheduledRequest is created such that the requestInfo attribute indicates “modify the adminStatus from locked to unlocked”,

- at the indicated time, the modification is made and the bandwidthServices are “turned-up”, i.e., traffic is allowed to flow.

One application of this feature involves support for Pay-Per-View video events where many bandwidth services need to be activated just before the event is to start. Each bandwidth service referencing (pointing to) a particular scheduled request object will be activated at the same time.

### Rules for Object Instance Creation

#### Terminology

In the Data Dictionary (Section 4.1.6), operation “Release” is used to request that an object representing a physical resource, e.g., a Bandwidth Service, release the resource and then delete itself. The operation “Destroy” is used for objects that do not represent physical resources, e.g., traffic profile. Also, the operation “Establish” is used to request the instantiation of an object that represents a physical resource, and “Create” is used to request the instantiation of an object that does not represents a physical resource.

#### Instantiation Rules

It is assumed that the Bulk Capacity, Bulk Capacity Endpoint, Service Network and Service Subnetwork objects are managed (established, modified and released) outside of the Service Configuration process as part of the SLA between by the MSP and SSP.

The bandwidth service endpoint objects are established automatically when the corresponding bandwidth service object is established, or the bandwidth service could make use of existing bandwidth service endpoints as described in Section 4.1.5, or the bandwidth service endpoints could be provisioned (outside of the PtPSC process) based on the type of service or technology being offered. Normally, the bandwidth service is created with the AdminStatus set to “locked”, i.e., with no bits flowing. Subsequently, the SSP will request that the AdminStatus be set to “unlocked”.

Service routing profiles and service traffic profile could be provisioned outside of the PtPSC process or the MSP could request that instances be created, depending on the SLA between the MSP and SSP.

Instances of scheduled request and configuration group are created by the SSP at the request of the MSP.

Capacity and Endpoint are not instantiable objects.

### Deferred Attributes and Operations

Since the object model in this document is not specific to any particular technology such as ATM, SDH/SONET or Frame Relay, many of the attributes and operations could not be defined in any detail. As an example of this issue, consider the service traffic profile object. Each transport technology uses different parameters to specify traffic characteristics. For example, ATM traffic parameters include Peak Cell Rate (PCR), Cell Delay Variation (CDV), Sustained Cell Rate (SCR), QoS and Maximum Burst Size (MBS), and Frame Relay uses Committed Burst Size (Bc), Excess Burst Size (Be), and Committed Information Rate (CIR). The details of the service traffic profile are deferred until technology specific models are defined. So, for example, an ATM specific model will have an atmServiceTrafficProfile based on the technology independent service traffic profile object where the atmServiceTrafficProfile object has traffic attributes specific to ATM.

In general, the PtPSC technology independent model (in this document) only contains details related to all transport technologies. The intent is to define a structure that can be specialized to specific technologies. However, OMT subclassing cannot be used to specialize the objects in the PtPSC model, since OMT allows for neither over-loading (as in C++) nor for deferred/abstract objects (as in the OO language Eiffel and UML). Over-loading or deferred/abstract objects are needed since the attributes and operations in the PtPSC model need to be refined in the technology specific subclasses. So, the plan is to use the objects in the PtPSC technology independent model as templates for the technology specific objects. Another approach is to exclude all attributes and operations that can not be fully specified in the technology independent model, and then add the needed attributes and operations in the technology specific model. This approach allows for the use of subclassing in going from the technology independent to technology specific model, but this approach leads to a very sparse (and not very useful) technology independent model.

In the Data Dictionary section, i.e., Section 4.1.6, the data structures for the attributes and operations are either

- deferred (in which case the corresponding data structure is to be defined in subclasses),

- defined in another standard (in which case a reference is given), or

- defined in line using ASN.1 (note that OMT does not have a data declaration language).

### Bandwidth Service Endpoint Modeling

There are two basic approaches in modeling the relationship between a bandwidth service and its endpoints. In the first case, each bandwidth service endpoint corresponds to exactly one bandwidth service (see Figure 4-7). For each bandwidth service, one object is created for the bandwidth service and one object for each endpoint (3 objects in the point-to-point case, and more than three for multipoint bandwidth services).

Figure 4-7. One Bandwidth Service Per Bandwidth Service Endpoint

The second case allows one bandwidth service endpoint to terminate several bandwidth services (Figure 4-8). Information that would normally be put in the endpoint objects, e.g., the ATM vpi/vci pair, would need to be put in the bandwidth service object for this case.

This approach requires fewer objects to represent a bandwidth service. For the example shown in Figure 4-8, 5 objects are needed to represent the 3 bandwidth services, whereas in Figure 4-7, 9 objects are needed. For 50 point-to-point bandwidth services carried over a single bulk capacity, the first case requires 150 objects but the second case requires only 52.

Figure 4-8. One Bandwidth Service Endpoint for Many Bandwidth Services

As the number of bandwidth services supported by a bulk capacity becomes large, the first case takes about three times as many objects as the second case to represent the bandwidth services. For case where the bandwidth services are virtual connections such as ATM and Frame Relay, the second case is preferred. For cases where the bandwidth services are physical layer connections such as DS3 or STM-1, the first case is preferred. In any event, both cases are supported in this document.

### Data Dictionary

The concepts and terms within the class diagram (i.e., Figure 4-5) are defined in this subsection.

#### Object Class - Endpoint

Endpoint is a non-instantiable superclass from which Bandwidth Service Endpoint and Bulk Capacity Endpoint are derived. Endpoints are used to terminate an instance of Capacity.

Endpoint has attributes which are defined as follows:

ID: the value of this attribute uniquely identifies an instance of Endpoint.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From: "Rec. M.3100 : 1995

GroupID: this optional attribute identifies the Configuration Group with which the Endpoint is associated.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

OperationalStatus: an indication of the state of the Endpoint. OperationalStatus is a read-only attribute.

OperationalState ::= ENUMERATED { disabled(0), enabled(1) } -- From Rec. X.721

AdminStatus: an indication of whether or not the Endpoint is locked or unlocked for use. The AdminStatus is controlled by the manager, and as such is a read-write attribute.

AdministrativeState ::= ENUMERATED { locked(0), unlocked(1), shuttingDown(2) } -- From Rec. X.721

Capacity List: a reference to the instance (or instances) of Capacity that this Endpoint terminates.

CapacityList ::= SEQUENCE OF NameType

The following notification(s) are supported by the Endpoint object class:

Object Establishment/Release Report: this notification is used to report on the establishment or release of an instance of Endpoint

Attribute Value Change Report: this notification is used to report a change to one of the attributes in an instance of Endpoint

State Change Report: this notification is used to report a change to the OperationalStatus and/or AdminStatus

#### Object Class - Capacity

Capacity is a non-instantiable superclass from which Bandwidth Service and Bulk Capacity are derived. Capacity represents the capability of transporting information.

Capacity has attributes which are defined as follows:

ID: The value of this attribute uniquely identifies an instance of Capacity.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From: "Rec. M.3100 : 1995

Group ID: this optional attribute identifies the Configuration Group with which the instance of Capacity is associated.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From: "Rec. M.3100 : 1995

OperationalStatus: an indication of the state of the Capacity. OperationalStatus is a read-only attribute.

OperationalState ::= ENUMERATED { disabled(0), enabled(1) } -- From Rec. X.721

AdminStatus: an indication of whether or not the instance of Capacity is locked or unlocked for use. The AdminStatus is controlled by the manager, and as such is read-write attribute.

AdministrativeState ::= ENUMERATED { locked(0), unlocked(1), shuttingDown(2) } -- From Rec. X.721

Endpoint List: a list of the Endpoints terminating the Capacity.

EndpointList ::= SEQUENCE OF NameType

The following notification(s) are supported by the Endpoint object class:

Object Establishment/Release Report: this notification is used to report on the establishment or release of an instance of Capacity

Attribute Value Change Report: this notification is used to report a change to one of the attributes in an instance of Capacity

State Change Report: this notification is used to report a change to the OperationalStatus and/or AdminStatus

#### Object Class - Bandwidth Service

An instance of the Bandwidth Service class represents an end-to-end connection service. Bandwidth Services may be point-to-point or multipoint in terms of topology.

Bandwidth Service has attributes which are defined as follows:

ID (inherited from Capacity): the value of this attribute uniquely identifies an instance of Bandwidth Service.

Group ID (inherited from Capacity): this optional attribute identifies the Configuration Group with which the Bandwidth Service is associated.

OperationalStatus (inherited from Capacity): an indication of the state of the Bandwidth Service. OperationalStatus is a read-only attribute.

AdminStatus (inherited from Capacity): an indication of whether or not the Bandwidth Service is locked or unlocked for use. The AdminStatus is controlled by the manager, and as such is a read-write attribute.

Endpoint List (inherited from Capacity): a list of the Bandwidth Service Endpoints terminating the Bandwidth Service.

Service Features: this attribute indicates service routing capabilities such as automatic re-route, diversity, and replication (i.e., protection). This attribute will be defined in more detail for technology specific subclasses of bandwidth service.

Service Characteristics: this attribute is used to convey bandwidth/traffic information in cases where service profiles are not used. This attribute will be defined in more detail for technology specific subclasses of bandwidth service.

Routing Profile Pointer: a pointer to the Routing Profile associated with the Bandwidth Service. This pointer may be null if either

- the MSP does not wish to specify a route, or

- if the routing information is included in the Service Features attribute

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

Traffic Profile Pointer: a pointer to the Service Traffic Profile associated with the Bandwidth Service

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

Schedule Pointer: a pointer to the Scheduled Request instance associated with the Bandwidth Service. This pointer is NULL if the requester wishes immediate establishment, modification or release of a Bandwidth Service.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

The following operations are supported by the Bandwidth Service object class:

Query: this operation is used to get information concerning the attributes of a Bandwidth Service

Modify: this operation is used to modify the characteristics of an existing Bandwidth Service.

Release: this is used to request the deletion of a Bandwidth Service

Bandwidth Service inherits the following notifications from Capacity: Object Establishment/Release Report, Attribute Value Change Report, and State Change Report

#### Object Class - Bulk Capacity

An instance of the Bulk Capacity class represents a dedicated transmission capability between two points. As noted, Bulk Capacity establishment is outside the scope of the PtPSC interface. A single Bulk Capacity may be used to support many Bandwidth Services.

Bulk Capacity has attributes which are defined as follows:

ID (inherited from Capacity): the value of this attribute uniquely identifies an instance of Bulk Capacity.

Group ID (inherited from Capacity): this optional attribute identifies the Configuration Group with which the instance of Bulk Capacity is associated.

OperationalStatus (inherited from Capacity): an indication of the state of the Bulk Capacity. OperationalStatus is a read-only attribute.

AdminStatus (inherited from Capacity): an indication of whether or not the Bulk Capacity is locked or unlocked for use. The AdminStatus is controlled by the manager, and as such is a read-write attribute.

Endpoint List (inherited from Capacity): a list of the Endpoints terminating the instance of Bulk Capacity.

Bulk Capacity Features: this attribute describes routing capabilities related to the Bulk Capacity, e.g., whether or not automatic protection is provided. This attribute will be defined in more detail for technology specific subclasses of bandwidth service.

Bulk Capacity Characteristics: this attribute describes the characteristics of an instance of bulk capacity, e.g., the capacity (bits/second) of the Bulk Capacity, the maximum number of bandwidth services supported by the Bulk Capacity. This attribute will be defined in more detail for technology specific subclasses of bandwidth service.

The following operation(s) are supported by the Bulk Capacity object class:

Query: this operation is used to get information concerning the attributes of an instance of Bulk Capacity

Bulk Capacity inherits the following notifications from Capacity: Object Establishment/Release Report, Attribute Value Change Report, and State Change Report

#### Object Class - Bandwidth Service Endpoint

An instance of Bandwidth Service Endpoint represents the termination of a Bandwidth Service.

Bandwidth Service Endpoint has attributes which are defined as follows:

ID (inherited from Endpoint): The value of this attribute uniquely identifies an instance of Bandwidth Service Endpoint. This attribute is inherited from Endpoint.

GroupID (inherited from Endpoint): this optional attribute identifies the Configuration Group with which the Bandwidth Service Endpoint is associated.

OperationalStatus (inherited from Endpoint): an indication of the state of the Bandwidth Service Endpoint. OperationalStatus is a read-only attribute.

AdminStatus (inherited from Endpoint): an indication of whether or not the Bandwidth Service Endpoint is locked or unlocked for use. The AdminStatus is controlled by the manager, and as such is a read-write attribute.

Capacity Pointer (inherited from Endpoint): a reference to the instance of Bandwidth Service that this Bandwidth Service Endpoint terminates.

Bandwidth Service Endpoint Characteristics: this attribute is used to indicate characteristics of the endpoint such as what higher-level protocols are supported. The details of this attribute are to be defined in technology specific subclasses.

The following operation(s) are supported by the Bandwidth Service Endpoint object class:

Query: this operation is used to get information concerning the attributes of an instance of Bandwidth Service Endpoint.

Bandwidth Service Endpoint inherits the following notifications from Endpoint: Object Establishment/Release Report, Attribute Value Change Report, and State Change Report

#### Object Class - Bulk Capacity Endpoint

An instance of Bulk Capacity Endpoint represents the termination of a Bulk Capacity.

It has attributes which are defined as follows:

ID (inherited from Endpoint): The value of this attribute uniquely identifies an instance of Bulk Capacity Endpoint.

GroupID (inherited from Endpoint): this optional attribute identifies the Configuration Group with which the Bulk Capacity Endpoint is associated.

OperationalStatus (inherited from Endpoint): an indication of the state of the Bulk Capacity Endpoint. OperationalStatus is a read-only attribute.

AdminStatus (inherited from Endpoint): an indication of whether or not the Bulk Capacity Endpoint is locked or unlocked for use. The AdminStatus is controlled by the manager, and as such is a read-write attribute.

Capacity Pointer (inherited from Endpoint): a reference to the instance of Bulk Capacity that this Bulk Capacity Endpoint terminates.

Bulk Capacity Endpoint Characteristics: this attribute is used to indicate characteristics of the endpoint. The details of this attribute are to be defined in technology specific subclasses. As an example, this attribute could be used to indicate that an instance of Bulk Capacity provides a service interworking capability, e.g., ATM to Frame Relay. Note that the interworking function is related to the Bulk Capacity Endpoint and not to the Bulk Capacity itself.

The following operation(s) are supported by the Bulk Capacity Endpoint object class:

Query: this operation is used to get information concerning the attributes of an instance of Bulk Capacity Endpoint.

Bulk Capacity Endpoint inherits the following notifications from Endpoint: Object Establishment/Release Report, Attribute Value Change Report, and State Change Report

#### Object Class - Service Network

In the context of the PTP Service Configuration model, an instance of a Service Network is used to represent the SSP’s Network as seen by the MSP. Subclasses of Capacity and Endpoint are related to Service Network via the containment relationship.

It has attributes which are defined as follows:

ID: The value of this attribute uniquely identifies an instance of Service Network.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

The following operation(s) are supported by the Service Network object class:

Establish BWS: this operation is used request the establishment of a Bandwidth Service

Create Scheduled Request: this operation is used to request the creation of an instance of Scheduled Request.

Create Service Traffic Profile: this operation is used to request the creation of an instance of Service Traffic Profile.

Create Service Routing Profile: this operation is used to request the creation of an instance of Service Routing Profile.

Create Configuration Group: this operation is used to request the creation of a Configuration Group.

The following operation(s) are supported by the Service Network object class:

Query: this operation is used to get information concerning the attributes of an instance of Service Network.

#### Object Class - Service Sub-network

An instance of Service Sub-network is used to represent a portion of the SSP’s network. The Service Sub-network is needed when the MSP is afforded route selection capability by the SSP.

It has attributes which are defined as follows:

ID: The value of this attribute uniquely identifies an instance of Service Sub-network. This attribute is inherited from Service Network.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

Availability Status: An indication of the condition of the Service Sub-network. Possible values of AvailabilityStatus are

failed - the Service Sub-network is experiencing significant problems. All existing Bandwidth Services traversing the Service Sub-network are affected.

degraded - the Service Sub-network is not able to accept any new bandwidth service requests. When in the degraded state, existing bandwidth services are not affected.

healthy - there are no known problems affecting the Service Sub-network.

AvailabilityStatus ::= SET OF INTEGER { failed(0), degraded(1), healthy(2) }

-- variation on the X.721 availabilityStatus attribute

Supported Bulk Capacity Endpoint List: A list of the Bulk Capacity Endpoints on the boundary of the Service Sub-network. This information is needed to indicate what Bulk Capacity are terminated on the Service Sub-network.

EndpointList ::= SEQUENCE OF NameType

The following operation(s) are supported by the Service Sub-network object class:

Query: this operation is used to get information concerning the attributes of an instance of Service Sub-network.

The following notification(s) are supported by the Service Sub-network object class:

Object Establishment/Release Report: this notification is used to report on the establishment or release of an instance of Service Sub-network

Attribute Value Change Report: this notification is used to report a change to one of the attributes in an instance of Service Sub-network

State Change Report: this notification is used to report a change to the AvailabilityStatus

#### Object Class - Service Routing Profile

An instance of Service Routing Profile is used to indicate the route to be followed by a Bandwidth Service. Several Bandwidth Services may use the same Service Routing Profile.

It has attributes which are defined as follows:

ID: The value of this attribute uniquely identifies an instance of Service Routing Profile.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

Route Description: This attribute describes a route to be followed by a Bandwidth Service(s). There are two options allowed for routing:

- specify a list of Bulk Capacities (this ensures a unique route), or

- specify a list of Service Sub-Networks (this may not ensure a unique route since there may be more than one Bulk Capacity between two Service Sub-Networks).

In the first case, the route description consists of a list of Bulk Capacity IDs and in the second case, a list of Service Sub-Network IDs.

Note that the Bulk Capacities between the Service Sub-networks have been previously established by a service ordering process.

RouteDescription ::= CHOICE ( bcList BulkCapacities, ssnList ServiceSubnetworks }

BulkCapacities ::= SEQUENCE OF NameType

ServiceSubnetworks ::= SEQUENCE OF NameType

The following operations are supported by the Service Routing Profile object class:

Query: this operation is used to get information concerning the attributes of a Service Routing Profile.

Modify: this operation is used to modify the characteristics of an existing Service Routing Profile.

Destroy: this is used to request that an instance of Service Routing Profile destroy itself.

The following notification(s) are supported by the Service Routing Profile object class:

Object Creation/Deletion Report: this notification is used to report on the creation or deletion of an instance of Service Routing Profile

Attribute Value Change Report: this notification is used to report a change to one of the attributes in an instance of Service Routing Profile

#### Object Class - Service Traffic Profile

An instance of Service Traffic Profile is used to indicate the traffic characteristics of one or more Bandwidth Services.

It has attributes which are defined as follows:

ID: The value of this attribute uniquely identifies an instance of Service Traffic Profile.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

Traffic Characteristics: A description of the traffic characteristics of the Bandwidth Service, e.g., maximum burst size. The details of the attribute are to be defined in technology specific subclasses.

The following operations are supported by the Service Traffic Profile object class:

Query: this operation is used to get information concerning the attributes of a Service Traffic Profile.

Modify: this operation is used to modify the characteristics of an existing Service Traffic Profile.

Destroy: this is used to request that an instance of Service Traffic Profile destroy itself.

The following notification(s) are supported by the Service Traffic Profile object class:

Object Creation/Deletion Report: this notification is used to report on the creation or deletion of an instance of Service Traffic Profile

Attribute Value Change Report: this notification is used to report a change to one of the attributes in an instance of Service Traffic Profile

#### Object Class - Configuration Group

An instance of Configuration Group is used to represent a collection of related Bandwidth Services, a collection of Bandwidth Service Endpoints, a collection of Bulk Capacities or a collection of Bulk Capacity Endpoints.

It has attributes which are defined as follows:

ID: the value of this attribute uniquely identifies an instance of Configuration Group.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

Member List: a list of the members of the Configuration Group.

MemberList ::= SEQUENCE OF NameType

The following operation(s) are supported by the Configuration Group object class:

Query: this operation is used to get information concerning the attributes of an instance of Configuration Group

Modify: this operation is used to remove and/or add instances of Capacity to an existing Configuration Group.

Destroy: this is used to request that an instance of Configuration Group destroy itself. A Configuration Group is destroyed independently of its members, i.e., the referenced instances of Capacity will still exist after a configuration group is destroyed.

The following notification(s) are supported by the Configuration Group object class:

Object Creation/Deletion Report: this notification is used to report on the creation or deletion of an instance of Configuration Group

Attribute Value Change Report: this notification is used to report a change to one of the attributes in an instance of Configuration Group

#### Object Class - Scheduled Request

An instance of Scheduled Request represents a request by the MSP for the establishment, modification or release of a Bandwidth Service at some future point in time. It is also possible to request that a particular activity be repeated at a specified interval. In such cases, one must specify the activity at the beginning (e.g., increase the bandwidth) and end (e.g., decrease the bandwidth) of the interval.

For activities that are to reoccur at unequal intervals (i.e., occasional scheduling), it is necessary to create several instances of Scheduled Request.

It has attributes which are defined as follows:

ID: The value of this attribute uniquely identifies an instance of Scheduled Request.

NameType ::= CHOICE { numericName   INTEGER,

pString   GraphicString } -- From Rec. M.3100 : 1995

BWS Pointer List: the list of instances of Bandwidth Service to which this Scheduled Request applies (used for scheduled modified and releases). Note: for the scheduled establishment of Bandwidth Services, the Bandwidth Service object is not created until the scheduled time, i.e., there is not a BWS Pointer List in the case of scheduled establishments.

BWSList ::= SEQUENCE OF NameType

Request Info: this attribute contains information concerning the specific activity requested, i.e., bandwidth service establishment, modification or release. If, in addition, recurring activity is requested (through the interval and duration attributes), then the Request Info attribute should indicate the activity requested at the beginning of the interval and at the end of the interval. The details of the attribute are to be defined in technology specific subclasses.

Interval: if the scheduled request is to be repeated at regular intervals, this attribute indicates the time duration of the interval. It should be emphasized that the Interval attribute indicates the time when an activity should be initiated, but does not indicate the duration of the activity. “Interval” is an optional attribute.

LenfthOfTime ::= SEQUENCE { months INTEGER, days INTEGER, hours INTEGER, minutes INTEGER, seconds INTEGER }

Interval ::= LengthOfTime

Request Time: the time that the request is scheduled to be performed

GeneralizedTime  -- Request Time is of type GeneralizedTime as defined in ITU-T Rec. X.208

Duration: this attribute is used in conjunction with the interval attribute to indicate when the activity, started at the beginning of the interval, should stop.

Duration ::= LengthOfTime

The following operation(s) are supported by the Scheduled Request object class:

Query: this operation is used to get information concerning the attributes of an instance of Scheduled Request

Destroy: this operation is used to delete an instance of Scheduled Request when the instance has the Interval attribute set. The operation is also used to cancel a Scheduled Request in cases where the requester no longer wants the request to be completed. Otherwise, the Scheduled Request object destroys itself after it completes its mission.

Modify: this operation is used to modify the attributes of the Scheduled Request object.

The following notification(s) are supported by the Scheduled Request Report object class:

Scheduled Request Report: this notification is used to report on the result of the scheduled request

Object Creation/Deletion Report: this notification is used to report on the creation or deletion of an instance of Scheduled Request

Attribute Value Change Report: this notification is used to report a change to one of the attributes in an instance of Scheduled Request

## Dynamic Model

The dynamic model shows the time-dependent behavior of the system and the objects in it. The dynamic model is expressed in scenario descriptions and interaction diagrams (see Section 4.2.1), and state transition diagrams (see Section 4.2.2).

### Scenario Descriptions and Interaction Diagrams

The scenario/interaction diagrams show the behavior of the system in terms of sequences of objects interacting with each other and with the outside world. The scenarios are useful for understanding how objects work together to perform a task in order to reach a goal.

Note:  Some of the following scenarios apply to multiple objects (i.e. Query,  Modify).

#### Query

The following scenario applies to all objects that support a Query operation.

Figure 4-9. Scenario for Query Operation

#### Modify

The following scenario applies to all objects that support a Modify operation.

Figure 4-10. Scenario for the Modify Operation

#### Destroy

The following scenario diagram applies to all objects that support a Destroy operation.

Figure 4-11. Scenario for the Destroy Operation

#### Create Configuration Group

Figure 4-12. Scenario for the Create Configuration Group Operation

#### Create Service Routing Profile

Figure 4-13. Scenario for the Create Service Routing Profile Operation

#### Create Service Traffic Profile

Figure 4-14. Scenario for the Create Service Traffic Profile Operation

#### Create Scheduled Request

Figure 4-15. Scenario for the Create Scheduled Request Operation

#### Establish Bandwidth Service

Figure 4-16. Scenario for the Establish Bandwidth Service Operation

#### Notification

Figure 4-17. Scenario for Notifications

#### Release Bandwidth Service

Figure 4-18. Scenario for the Release Bandwidth Service Operation

### State transition diagrams

State transition diagrams provide information on the states that each class, seen at the PtPSC interface, can occupy and the way that the objects transition from one state to the next. State transition diagrams are included for the following classes.

#### Object Class - Bandwidth Service

Figure 4-19. State Diagram for Bandwidth Service

The bandwidth service is established in either an Unlocked-enabled or a Locked-enabled state. If it is established in the Locked-enabled state it has to be put into the Unlocked-enabled state before it can be used. It may need to be deactivated in order to perform certain modifications (see note below) and its state is changed to Locked-enabled to enable the modifications to be completed.

If an alarm is issued, the state of the bandwidth service transitions to the Unlocked-disabled state. It also transitions to the Unlocked-disabled state if the state of one of the following object instances transitions to the Unlocked-disabled state: a corresponding bandwidth service endpoint; the supporting bulk capacity; one of the corresponding bulk capacity endpoints. Its state can be changed back to the Unlocked-enabled state when the alarm is cleared, or it can be deactivated and transition to the Locked-enabled state. The bandwidth service can also be released from the Unlocked-disabled state before the alarm is cleared. The bandwidth service can therefore be released from any three of the states: Unlocked-enabled, Locked-enabled, Unlocked-disabled.

Notes:

- State transitions related to the Modify operation are not shown, since the transition depends on the technology, i.e., some technologies may require that the state of a BWS be set to “locked” before certain modifications are performed, other technologies may not have such a requirement. This state diagram will need to be enhanced for technology-specific subclasses of bandwidth service.

#### Object Class - Bandwidth Service Endpoint

Figure 4-20. State Diagram for Bandwidth Service Endpoint

The bandwidth service endpoint is created in a Locked-enabled state when the corresponding bandwidth service is created. When the bandwidth service is active (in the Unlocked-enabled state), the bandwidth service endpoint state must be in the Unlocked-enabled state. If an alarm is raised, the bandwidth service endpoint state transitions to the Unlocked-disabled state. When the alarm is cleared the state can be returned to Unlocked-enabled. The bandwidth service may be released before an alarm has been cleared, in which case the bandwidth service endpoint state is changed to the Locked-enabled state. When the bandwidth service is released during normal operation the bandwidth service endpoint state is changed from the Unlocked-enabled state to the Locked-enabled state.

Notes:

- When the state of a bandwidth service endpoint is changed to the Unlocked-disabled state, the state of the corresponding bandwidth service is also changed to the Unlocked-disabled state.

#### Object Class - Bulk Capacity

Figure 4-21. State Diagram for Bulk Capacity

The functional state of the bulk capacity is the Unlocked-enabled state. When an alarm is raised, the state of the bulk capacity transitions to the Unlocked-disabled state. It is returned to the Unlocked-enabled state when the alarm is cleared.

Notes:

- When the state of a bulk capacity instance is changed to the Unlocked-disabled state, all the bandwidth services supported by the bulk capacity instance are also changed to the Unlocked-disabled state.

#### Object Class - Bulk Capacity Endpoint

Figure 4-22. State Diagram for Bulk Capacity Endpoint

The functional state of the bulk capacity endpoint is the Unlocked-enabled state. When an alarm is issued, the state of the bulk capacity endpoint transitions to the Unlocked-disabled state. It is returned to the Unlocked-enabled state when the alarm is cleared.

Notes:

- When an instance of bulk capacity endpoint transitions to the Unlocked-disabled state,

- the corresponding bulk capacity is changed to the Unlocked-disabled state, and

- the supported bandwidth service endpoints are changed to the Unlocked-disabled state.

## Functional Model

The functional model provides specifications of the operations at the exported interfaces. Such interfaces have been presented in Section 2.5.

### Operations at the exported interfaces

The following operation specifications are provided for the interfaces exported to the users at the system boundary. The notion of interface types is used to collect together operations on a logical basis. This is similar to the concept of interface type defined in TINA-C -ODL (Object Definition Language).

#### Operations for Bandwidth Service

The following operations are supported by the bandwidth service:

- query()

- modify()

- release()

Query:

Description:  This operation is used to access the information attributes of a bandwidth service object.  A query can request a full or partial set of attributes as desired by the user.  This operation will always succeed.

Inputs:  BWS ID,  a list of attribute identifiers.

Outputs:  a list of attribute values.

Modify:

Description:  This operation is used to set the information attributes of a bandwidth service object.  A modify operation can set a full or partial collection of attributes to new values.  The following attributes can be modified:

groupId

schedule pointer

status

endpoint list (endpoints can be added or deleted, but at least two endpoints must remain after a deletion)

service features *

service characteristics *

routing profile pointer *

traffic profile pointer *

* For some technologies, the BWS may need to be in the locked state before the attribute can be modified. The technology-specific subclasses of BWS will indicate whether or not the AdminStatus of the BWS needs to be set to “locked” for the modification can occur.

Inputs:  BWS ID,  a list of attribute identifiers and values.

Outputs:  Indication of success or failure.  In the event of a failure further information is provided as to which part of the request failed.

Release:

Description: This operation is invoked to delete the object, it will also free all related resources. A release will only be successful if the BWS is in the Unlocked-enabled state.

Inputs: BWS ID

Outputs: indication of success or failure.

#### Operations for Bulk Capacity

The following operations are supported by the Bulk Capacity: Query();

Query:

Description:  This operation is used to access the information attributes of a Bulk Capacity object.  A query can request a full or partial set of attributes as desired by the user.  This operation will always succeed.

Inputs:  bulk capacity ID,  a list of attribute identifiers.

Outputs:  a list of attribute values.

#### Operations for Bandwidth Service Endpoint

The following operations are supported by the Bandwidth Service Endpoint:

- Query();

- Modify();

- Query:

- Description:  This operation is used to access the information attributes of a Bandwidth Service Endpoint object.  A query can request a full or partial set of attributes as desired by the user.  This operation will always succeed.

- Inputs:  bandwidth service endpoint ID,  a list of attribute identifiers.

- Outputs:  a list of attribute values.

- Modify:

- Description:  This operation is used to set information attributes of a Bandwidth Service Endpoint object.  A modify operation can set a full or partial collection of attributes to  new  values.  The following attributes can be modified:

- bandwidth service endpoint characteristics

Inputs:  bandwidth service endpoint ID,  a list of attribute identifiers and values.

Outputs:  indication of success or failure.  In the event of a failure further information is provided as to which part of the request failed.

#### Operations for Bulk Capacity Endpoint

The following operations are supported by the Bulk Capacity Endpoint: Query();

Query:

Description:  This operation is used to access the information attributes of a Bulk Capacity Endpoint object.  A query can request a full or partial set of attributes as desired by the user.  This operation will always succeed.

Inputs:  bulk capacity endpoint Id,  a list of attribute identifiers.

Outputs:  a list of attribute values.

#### Operations for Service Network

The following operations are supported by the Service Network:

- EstablishBWS ();

- CreateScheduledRequest ();

- CreateRoutingProfile ();

- CreateTrafficeProfile ();

- CreateConfigurationGroup ();

- Establish BWS:

- Description:  This operation is invoked to create a BWS object.  Concerning the optional attributes, if they are not supplied in the request they will be set to a default of NULL in the created object.

- Inputs:

- The following attributes are mandatory:

- endpoint list

- service features (could be set to NULL if routing profile pointer is used)

- service characteristics (could be set to NULL if traffic profile pointer is used)

- admin status

- The following attributes are optional:

- schedule pointer

- routing profile pointer

- traffic profile pointer

- groupID

- Outputs: indication of success or failure.  In the successful case a BWS Id is also returned.

- Create Scheduled Request:

- Description:  This operation is invoked to create a Scheduled Request object.  Concerning the optional attributes,  if they are not supplied in the request they will be set to a default of NULL.

- Inputs:

- The following attributes are mandatory:

- request info

- request time

- BWS pointer list (must be at least one for modify and release,  N/A for establish)

- The following attributes are optional:

- interval

- Outputs: indication of success or failure.  In the successful case a Scheduled Request Id is also returned.

- Create Service Routing Profile:

- Description:  This operation is invoked to create a Routing Profile object.

- Inputs:

- The following attributes are mandatory:

- route description

- The following attributes are optional:

- none

- Outputs: indication of success or failure.  In the successful case a Routing Profile Id is also returned.

- Create Service Traffic Profile:

- Description:  This operation is invoked to create a Traffic Profile object.

- Inputs:

- The following attributes are mandatory:

- traffic characteristics

- The following attributes are optional:

- none

- Outputs: indication of success or failure.  In the successful case a Traffic Profile Id is also returned.

- Create Configuration Group:

- Description:  This operation is invoked to create a Configuration Group object.

- Inputs:

- The following attributes are mandatory:

- none

- The following attributes are optional:

- member list

Outputs: indication of success or failure.  In the successful case a Configuration Group Id is also returned.

#### Operations for Service Subnetwork

The following operations are supported by the Service Subnetwork: Query();

Query:

Description:  This operation is used to access the information attributes of a Service Subnetwork object.  A query can request a full or partial set of attributes as desired by the user.  This operation will always succeed.

Inputs:  service subnetwork ID,  a list of attribute identifiers.

Outputs:  a list of attribute values.

#### Operations for Service Routing Profile

The following operations are supported by the Service Routing Profile:

- Query();

- Modify();

- Destroy();

- Query:

- Description:  This operation is used to access the information attributes of a Service Routing Profile object.  A query can request a full or partial set of attributes as desired by the user.  This operation will always succeed.

- Inputs:  service routing profile ID,  a list of attribute identifiers.

- Outputs:  a list of attribute values.

- Modify:

- Description:  This operation is used to set information attributes of a Service Routing Profile object.  A modify operation can set a full or partial collection of attributes to new  values. The following attributes can be modified:

- route description

Inputs:  service routing profile Id,  a list of attribute identifiers and values.

Outputs:  indication of success or failure.  In the event of a failure further information is provided as to which part of the request failed.

Destroy:

Description:  This operation is invoked to delete the object.

Inputs: service routing profile ID

Outputs: indication of success or failure.

#### Operations for Service Traffic Profile

The following operations are supported by the Service Traffic Profile:

- Query();

- Modify();

- Destroy();

Query:

Description:  This operation is used to access the information attributes of a Service Traffic Profile object.  A query can request a full or partial set of attributes as desired by the user. This operation will always succeed.

Inputs: service traffic profile ID,  a list of attribute identifiers.

Outputs: a list of attribute values.

Modify:

Description:  This operation is used to set information attributes of a Service Traffic Profile object.  A modify operation can set a full or partial collection of attributes to new  values.  The following attributes can be modified:

traffic characteristics

Inputs:  service traffic profile ID,  a list of attribute identifiers and values.

Outputs:  indication of success or failure.  In the event of a failure further information is provided as to which part of the request failed.

Destroy:

Description:  This operation is invoked to delete the object.

Inputs: service traffic profile ID

Outputs: indication of success or failure.

#### Operations for Configuration Group

The following operations are supported by the Configuration Group:

- Query();

- Modify();

- Destroy();

- Query:

- Description:  This operation is used to access the information attributes of a Configuration Group object.  A query can request a full or partial set of attributes as desired by the user.  This operation will always succeed.

- Inputs:  configuration group ID,  a list of attribute identifiers.

- Outputs:  a list of attribute values.

- Modify:

- Description:  This operation is used to set information attributes of a Configuration Group object.  A modify operation can set a full or partial collection of attributes to new values.  The following attributes can be modified:

- member list

Inputs:  configuration group ID,  a list of attribute identifiers and values.

Outputs:  indication of success or failure.  In the event of a failure further information is provided as to which part of the request failed.

Destroy:

Description:  This operation is invoked to delete the object.

Inputs: configuration group  ID

Outputs:  indication of success or failure.

#### Operations for Scheduled Request

The following operations are supported by the Scheduled Request:

- Query();

- Modify();

- Destroy();

- Query:

- Description:  This operation is used to access the information attributes of a scheduled request object.  A query can request a full or partial set of attributes as desired by the user.  This operation will always succeed.

- Inputs:  scheduled request ID,  a list of attribute identifiers.

- Outputs:  a list of attribute values.

- Modify:

- Description:  This operation is used to set information attributes of a Scheduled Request object.  A modify operation can set a full or partial collection of attributes to new values.  The following attributes can be modified:

- ID

- request info

- interval

- request time

Inputs:  scheduled request ID,  a list of attribute identifiers and values.

Outputs:  indication of success or failure.  In the event of a failure further information is provided as to which part of the request failed.

Destroy:

Description:  This operation is invoked to delete the object. If there is no interval set, an instance of the scheduledRequest object class could destroy itself after it has performed its mission.

Inputs: scheduled request  ID

Outputs:  indication of success or failure.

# Design Considerations

In designing the object model in this document the PtPSC team discovered several important design issues, including issues related to CORBA-GDMO mapping, deferred attribute and operation definitions and the SLA assumption.

## CORBA-GDMO Functional Variations

The protocol independent object model will need to be augmented when mapping to CORBA, since CORBA does not support some of the basic capabilities required for the PtPSC interface, e.g., notifications, scoping and filtering, and Sets and Gets. One approach is to define three objects that would provide the needed capabilities, i.e., a notification manager (to handle event reports), a log manager (to handle the logging of event reports) and a request manager (for Sets, Gets, and scoping and filtering).

## Deferred Definition of Attributes and Operations

Some of the attributes and operations of some objects can not be completely defined until a particular technology (such as ATM or SONET/SDH) is considered. For example, the details of the Service Traffic Profile are dependent of the technology being modeled.

The plan is to define solution sets for several specific technologies, e.g., ATM and SONET/SDH. The solution set documents will provide more complete detail for the attributes and operations.

## Shared Bulk Capacities

A basis assumption is that the bulk capacities (for use by an MSP) are established before the PtPSC can be used. It is also assumed that each bulk capacity is reserved for only one SSP.

A possible extension to the current model would allow for bulk capacities to be shared by more that one MSP. In this approach, each SSP would advertise its public bulk capacities for use by MSPs. (Note: The MSP would still need to that have a business agreement with the SSP in order to use the bulk capacity). An MSP could establish a PVC over the bulk capacities of several SSPs without actually leasing the entire bulk capacities. This approach is somewhat similar to SVCs.

# Requirements Satisfaction Analysis

This section examines the requirements defined in Appendix A of the Peer-to-Peer Service Configuration: Business Agreements Requirements Specification [1], and states which requirements have been fully satisfied and which have not. In the case of the latter, a rationale for this shortcoming is given. This section also addresses how the optionality within the requirements is accommodated.

Table 6-1 provides a detailed summary of requirements satisfaction.  Individual requirements are referenced by the identifier assigned by the Requirements Specification.

Table 6-1. Requirement Satisfaction Table

| - Requirement | - Satisfaction Analysis |
| --- | --- |
| - CONFR2.5 | - Satisfied by: - The Establish BWS method of the Service Network object class (see 4.1.5.7). - The Create Service Routing Profile method of the Service Network object class (see 4.1.5.7). - The Create Scheduled Request method of the Service Network object class (see 4.1.5.7). |
| - CONFR3.2 | - Satisfied by the Modify method of the Bandwidth Service object class (see 4.1.5.3). |
| - CONFR4.2 | - Satisfied by the Release method of the Bandwidth Service’s object class (see 4.1.5.3). |
| - CONFR5.3 | - Satisfied by: - Attributes of the Bandwidth-Service object class (see 4.1.5.3). - Service Routing Profile object class (see 4.1.5.9). - Service Sub-network object class (see 4.1.5.8) |
| - CONFR5.4 | - Satisfied by: - Attributes of the Bandwidth-Service endpoint object class (see 4.1.5.5). - The Query method of the Bandwidth-service endpoint object class (see 4.1.5.5). |
| - CONFR5.5 | - Satisfied by: - Attributes of the Bulk-capacity object class (see 4.1.5.4). - The Query method of the Bulk-capacity object class (see 4.1.5.6). |
| - CONFR5.6 | - Satisfied by: - Attributes of the Bulk-capacity endpoint object class (see 4.1.5.4). - The Query method of the Bulk-capacity endpoint object class (see 4.1.5.6). |
| - CONFR5.7 | - Satisfied by: - Attributes of the Configuration Group object class (see 4.1.5.11). - The Query method of the Configuration Group object class (see 4.1.5.6). |
| - CONFR6.3 | - Satisfied by the Create Configuration Group method of the Service Network object class (see 4.1.5.7). |
| - CONFR7.2 | - Satisfied by the Modify method of the Configuration Group object class (see 4.1.5.11). |
| - CONFR8.2 | - Satisfied by the Release method of the Configuration Group object class (see 4.1.5.11). |
| - CONFR9.2 | - Satisfied by: - Notification scenario (see 4.2.1.9). - State Diagram for Bandwidth-service (see 4.2.3.1). - State Diagram for Bandwidth-service endpoint (see 4.2.3.2). - State Diagram for Bulk-capacity (see 4.2.3.3). - State Diagram for Bulk-capacity endpoint (see 4.2.3.4). |
| - CONFR9.4 | - Satisfied by: - Notification scenario (see 4.2.1.9). - State Diagram for Bandwidth-service (see 4.2.3.1). - State Diagram for Bandwidth-service endpoint (see 4.2.3.2). - State Diagram for Bulk-capacity (see 4.2.3.3). - State Diagram for Bulk-capacity endpoint (see 4.2.3.4). |
| - CONFR9.5 | - Satisfied by: - Notification scenario (see 4.2.1.9). - State Diagram for Bandwidth-service (see 4.2.3.1). - State Diagram for Bandwidth-service endpoint (see 4.2.3.2). - State Diagram for Bulk-capacity (see 4.2.3.3). - State Diagram for Bulk-capacity endpoint (see 4.2.3.4). |
| - CONTR1.1 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR1.2 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR1.3 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR2.1 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR2.2 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR2.3 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR2.4 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR2.5 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR2.6 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR3.1 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR5.1 | - This requirement is fully satisfied by this document. The interface is defined using the OMT methodology which is independent of management technology and protocol. |
| - CONTR6.1 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |
| - CONTR7.1 | - This requirement is out of scope. It will be satisfied in a subsequent technology-dependent specification. |

# Conclusions and Insights

This document contains the technical analysis and design of an interface that meets the requirements defined in the peer-to-peer configuration business agreements document [1]. The analysis work has adopted OMT as the basic methodology and presents three orthogonal views of the peer-to-peer configuration management interface: an object model, a dynamic model and a functional model.

A generic interface has been specified that is both technology independent and protocol independent. This enables technology specific and protocol specific peer-to-peer service configuration solution sets to be developed by specializing the base classes defined in this document. The mapping to the protocol specific models will be automated by the OOA/OOD tool adopted by the TM Forum.

The intention of the TM Forum peer-to-peer service configuration team is to develop a technology specific information model based on the generic model in this document. The first technology selected will probably be ATM. On the basis of the technology specific information model the team will develop protocol specific models for the given technology, such as CMIP/GDMO or CORBA/IDL, according to the requirements of the industry.

Appendix A: Acronym List

ASN.1 	-   Abstract Syntax Notation One

API	-   Application Programmer’s Interface

ATM	-   Asynchronous Transfer Mode

Bc	-   Committed Burst Size

Be	-   Excess Burst Size

BWS 	-   Bandwidth-service

CDV	-   Cell Delay Variation

CDVT	-   Cell Delay Variation Tolerance

CIR	-   Committed Information Rate

CLEC 	-   Competitive Local Exchange Carrier

CMIP 	-   Common Management Information Protocol

CMISE 	-   Common Management Information Service Element

CORBA	-   Common Object Request Broker Architecture

CPE	-   Customer Premises Equipment

DCE	-   Distributed Computing Environment

DLCI	-   Data Link Connection Identifier

EL 	-   Element Layer

EML 	-   Element Management Layer

FRS	-   Frame Relay Service

IDL	-   Interface Definition Language

ILEC	-   Incumbent Local Exchange Carrier

MBS	-   Maximum Burst Size

MIB	-   Management Information Base

MSP	-   Main Service Provider

NE 	-   Network Element

NML 	-   Network Management Layer

NMS 	-   Network Management System

ODL	-   Object Definition Language

OMG	-   Object Management Group

OMT	-   Object Modeling Technique

OOA	-   Object Oriented Analysis

OOD	-   Object Oriented Design

OS 	-   Operations System

PCR	-   Peak Cell Rate

PtPSC	-   Peer-to-Peer Service Configuration

PVC 	-   Permanent Virtual Connection

QoS 	-   Quality of Service

SDH 	-   Synchronous Digital Hierarchy

SLA	-   Service Level Agreement

SMART	-   Service Management, Automation and Re-engineering Team

SONET	-   Synchronous Optical NETwork

SSN	-   Service SubNetwork

SSP	-   Secondary (or Subcontracted) Service Provider

SVC 	-   Switched Virtual Connection

TINA-C	-   Telecommunications Information Networking Architecture - Consortium

UML	-   Unified Modeling Language

UNI 	-   User Network Interface

VC 	-   Virtual Channel

VP 	-   Virtual Path

Appendix B: References

- NMF 502, Peer-to-Peer Service Configuration: Business Agreements Requirements Specification, Issue 1.0, April 1997.

- NMF 503, SMART Ordering: SP to SP Interface Requirements, Issue 1.03, June 1997.

Appendix C: Analysis of Relevant Standards

The team considered the following standards as input to the definition PtPSC Information Agreements document.

- ATM Forum document af-nm-0073.000, M4 Network View CMIP MIB Specification, Version 1.0, January 1997.

- ITU-T Recommendation M.3208.1, TMN Management Services for Dedicated and Reconfigurable Circuits Network: Leased Circuit Services, Draft from June 1997.

- ITU-T Recommendation G.805, Generic functional architecture of transport networks, November 1995.

- ITU-T Recommendation G.853.1, Common elements of the information viewpoint for the management of a transport network, November 1996.

- NMF 038, Bandwidth Management Ensemble, Issue 1.0, October 1995.

- Draft European Telecommunication Standard pr I-ETS 300 653, Telecommunications Management Network (TMN); Generic managed object class library for the network level view, June 1995.

- Draft European Telecommunication Standard pr ETS 300 820-1, Asynchronous Transfer Mode (ATM); Configuration management information model for the X-type interface between Operation Systems (OSs) of a Virtual Path (VP)/Virtual Channel (VC) cross connected network; Part 1: Configuration management aspects, December 1996.

- T1 draft document T1M1.5/97-113, Operations, Administration, Maintenance, and Provisioning (OAM&P) - Extension to Generic Information Model for Interfaces Across Jurisdictional Boundaries to Support Service Level Connection Management, July 1997.

In reviewing the above documents, the PtPSC team had several general goals/requirements in mind:

- the model should be a technology independent model that could be specialized (via subclassing) to support various technologies such as ATM, SDH and Frame Relay,

- the model should be at the service layer,

- the focus should be on bandwidth services (i.e., connections) as opposed to the endpoints of bandwidth services,

- the model should be efficient, having as few objects as possible.

Of the models mentioned above, only NMF 038 [5] and the T1 draft [8] are service-level models. The T1 model was seen as being very focused on a particular technology, i.e., SONET, but NMF 038 was determined to have the right level of detail. The concern with NMF 038 was that it used mandatory packages to represent various technologies and the PtPSC thought that various technologies should be defined via subclasses of technology independent objects. So, the team used general concepts from NMF 038 but did not used the object model.

The ITU-T G.850 series, ATM Forum model and the ETSI model (based on Eurescom’s Xcoop model) were seen as network level models that required the instantiation of a large number of endpoint related objects (such as CTPs, and TTPs) and as such only parts of these models were used in the PtPSC model. The ATM Forum and ETSI models are specific to ATM, and the PtPSC model is technology independent.

The ETSI model assumes a different business environment than the PtPSC model. In the ETSI model, Public Network Operators (PNOs) within a cooperative notify all PNOs within the cooperative (even non-neighboring PNOs) when they add or modify interconnection capacity between their network and other neighboring PNOs. Once advertised, the additional capacity becomes available to the other PNOs (at a price, of course). Any PNO may request that an ATM connection (or subnetwork connection) be established using the advertised interconnection capacity. So, information about the interconnection capacity between the PNOs is sent to all PNOs within the cooperative, and the interconnection capacity may be shared by multiple PNOs. In the PtPSC model, interconnection capacity is leased or purchased as part of an SLA, and the capacity is dedicated to the leasing/purchasing party. Information about availability is determined via a pre-ordering process and is not sent as a CMIP notification (as is the case in the ETSI model).

The PtPSC team did use the partitioning concept from G.805 (i.e., service subnetworks). However, the team decided not to use the layering concept from G.805, i.e., each instance of the PtPSC interface is related to only one transmission layer (e.g., the ATM VP layer). The representation of multiple layers in an interface was decided to be a network-level capability. The service view simply shows bandwidth services to the user, but does not show how the bandwidth service is support by the lower transmission layers.

M.3208.1 is a requirements document and did not have a supporting object model at the time the PtPSC team reviewed the work. The PtPSC team did, however, submit a draft of their object model to the ITU-T team working on the information model in support of M.3208.1.

The concept of routing profile and traffic profile, from the ATM Forum M4 Network View, was used in the PtPSC model, since these concepts allowed for more efficient modeling of information. For example, one could define a commonly used traffic profile and the have a large number of connection objects (atmTrail, atmSubnetworkConnection or atmLinkConnection) point to the single instance of traffic profile, thereby saving the replication of traffic information in each connection object.

Annex A: Summary of OMT Object Modeling

The first step in analyzing the requirements is to construct an object model. The following steps should be performed in this construction:

A-1.	Identify Objects and classes

Candidate object classes found in the written description of the problem (or the requirements) should be listed without being too selective. All object classes that come to mind should be written down. Classes often correspond to nouns.

Figure A.1: Object Classes Identification Process

Unnecessary and incorrect classes must be discarded and the following steps provide ideas on how this may be approached:

- Redundant Classes - If two classes express the same information the most descriptive name should be kept.

- Irrelevant Classes - delete (this is a matter of judgment)

- Vague Classes - Classes should be specific. Tentative classes may have ill-defined boundaries or be too broad in scope.

- Attributes - Names that primarily describe individual objects should be restated as attributes.

- Operations - If a name describes an operation that is applied to objects and not manipulated in its own right, then it is not a class.

- Roles - the name of an object class should reflect its intrinsic nature and not a role that it plays in an association.

- Implementation constructs - Any constructs extraneous to the real world and real resources should be eliminated. For example, arrays, linked lists, process and algorithm are implementation constructs and should be discarded.

- A-2.	Prepare a Data Dictionary

- This is a precise textual description of each class describing the scope of the class within the problem. It includes assumptions or restrictions on its use. Associations, attributes and operations should also be described in the data dictionary when they have been identified (guidelines for identifying associations and attributes are in the following sections).

- A-3.	Identifying Associations between classes

- Any dependency between two or more classes is an association. They often correspond to verbs in the problem description or requirements statement. This is the first step in identifying associations. Unnecessary and incorrect associations must be discarded using the following criteria:

- Associations between eliminated classes;

- Irrelevant or implementation associations;

- Actions: An association should describe a structural property of the application domain, not a transient event;

- Ternary associations: Associations between three or more classes should decomposed into binary associations or phrased as qualified associations;

- Derived associations: Omit associations that can be defined in terms of other associations.

- The following guidelines can assist in specifying the semantics of associations:

- Misnamed associations: It should not be stated how or why an association came about, but it should be stated what it is.

- Role names: Role names should be added where appropriate. A role name describes the role that a class in the association plays from the point of view of the other class.

- Multiplicity: Multiplicity specifies how many instances of one class may relate to a single instance of an associated class. Multiplicity is often described as being "one" or "many." Multiplicity should be specified as far as possible but this often changes during analysis.

- Missing associations: Missing associations that are discovered should be added.

- A-4.	Identify attributes of objects and links.

- Attributes often correspond to nouns followed by possessive statements (i.e., the address of the customer). Attributes are less likely to be fully described in the problem statement. Give attributes meaningful names. Derived attributes (i.e., the values of which may be derived from other attributes) should be omitted or clearly labeled. Derived attributes should not be expressed as operations, even though they may be eventually implemented as such.

- Link attributes should be identified. This is a property of the link between two objects, rather than being a property of an individual object.

- Unnecessary attributes and incorrect attributes should be eliminated based on the following criteria:

- Objects - If the independent existence of an entity is important and not just it's value then it is an object and not an attribute.

- Qualifiers - If the value of an attribute depends on a particular context, then consider restating the attribute as a qualifier. A qualifier is a special attribute that reduces the effective multiplicity of an association. One-to-many and many-to-many associations may be qualified. The qualifier distinguishes among the set of objects at the end of an association. For example, a directory has many files. A file may only belong to a single directory. Within the context of a directory, the file name specifies a unique file. 'Directory' and 'File' can be modeled as object classes and 'file name' is a qualifier. A Directory plus  a file name reduces the effective multiplicity of this association from many-to-one to one-to-one.

- Names -  A name is an object attribute when it does not depend on context, especially when it need not be unique. For example, names of persons, unlike names of companies may be duplicated and are, therefore object attributes.

- Identifiers - Do not list object identifiers in object models, as object identifiers are implicit in object models.6  Only attributes which are listed in the application domain should be listed.

- Link Attributes - An attribute is a property of the objects in a class. Similarly, a link attribute is a property of the links in an association.  If a property depends on the presence of a link, then the property is an attribute of the link and not of a related object.

- Internal values - If an attribute describes the internal state of an object that is invisible outside the object, then eliminate it from the analysis.

- Discordant attributes - An attribute that seems completely different from and unrelated to all other attributes may indicate a class that should be split into two distinct classes. A class should be simple and coherent.

- A-5.	Refine Information Model with Inheritance

- Bottom Up: Search for classes with similar attributes, associations or operations. For each generalization, define a super class to share common features. Some generalizations will suggest themselves based on existing taxonomy in the real world; use existing concepts wherever possible.

- Top Down: Top-down specialization’s are often apparent from the application domain.

- A-6.	Verify access paths exist for likely queries

- This involves the tracing of access paths through the object model diagram to see if queries yield sensible results. This process is not really equivalent to a scenario walkthrough or a use case (which can be used to test the behavior and efficiency of a specification) but  tests that queries on the object model yield the expected results. Where a unique value is expected, there should be a path yielding a unique result. For multiplicity "many" there should be a way to pick out unique values where needed. If there are useful questions that cannot be answered, this indicates that information is missing.

- A-7.	Iterate Object Modeling

- An object model is rarely correct after a single pass. Repeated iteration of the process may reveal missing object classes or classes that are not necessary. Signs of missing objects may include:

- asymmetries in associations and generalizations;

- disparate attributes and operations on a class;

- difficulty in generalizing cleanly; this may indicate that one class is playing more than one role and splitting the class may a better solution;

- an operation with no good target class; the class should be added;

- duplicate associations with the same name and purpose; a superclass should be created to unite them;

- a role may substantially shape the semantics of a class; creating a new class may be involved. This often means converting an association into a class. For example, a person can be employed by several different companies with different conditions of employment at each. Employee is then a class denoting a person working for a particular company, in addition to class Person and Company.

- Signs of unnecessary classes include:

- lack of attributes, operations, and associations on a class; the designer should ascertain if this class is really needed.

- Signs of missing associations include:

- missing access paths for operations. New associations should be added so that queries can be answered.

- Signs of unnecessary associations include:

- redundant information in the associations. Associations that do not add new information should be removed or marked as derived.

- Signs of incorrect placement of associations:

- role names that are too broad or too narrow for their classes. The association should be moved up or down in the class hierarchy.

- Signs of incorrect placement of attributes:

- needing to access an object by one of its attribute values. A qualified association should be considered in this case.

The term “functional state” means that the bulk capacity is working correctly and able to transmit information.

A qualified association relates two object classes and a qualifier. The qualifier is a special attribute that reduces the effective multiplicity of an association. One-to-many and many-to-many associations may be qualified. The qualifier distinguishes among the set of objects at the end of an association. A qualified association can also be considered a form of ternary association. For example a directory has many files. A file may only belong to a single directory. Within the context of a directory, the file name specifies a unique file. Directory and File are object classes and file name is the qualifier, A directory plus a file name yields a file. A file corresponds to a directory and a file name. Qualification reduces the effective multiplicity of this association from one-to-many to one-to-one. A directory has many files each with a unique name.

A link is a physical or conceptual connection between object instances. Mathematically, a link is defined as a tuple, that is, an ordered list of object instances. A link is an instance of an association. For example, 'Has-Capital' is an association. This is a one-to-one association between the MOCs 'Country' and 'City.' All links in this association connect an instance of 'Country' with an instance of 'City.' Associations are often implemented in programming languages as pointers from one object to another. This is acceptable, but associations should not be modelled this way. Modelling a link as a pointer disguises the fact that the link is not a part of either object by itself, but depends on both of them together.