Peer-to-Peer Service Configuration
Business Agreement

NMF 502

Issue 1.0

April 1997

ã NMF 1997.

Members of NMF are granted limited copyright waiver, under the terms of the
Agreement on Intellectual Property Rights, to reproduce this document.

NOTICE

The Network Management Forum (“NMF”) has made every effort to assure that the contents of this document are accurate; however, due to the inherent complexity in the design and implementation of protocols, no liability is accepted for any errors or omissions or for consequences of any use made of this document.  Under no circumstances will the NMF be liable for direct or indirect damages or any costs or losses resulting from the use of this specification.  The risk of designing and implementing products in accordance with  this specification is borne solely by the user of this specification.  This document may involve a claim of patent rights by one or more NMF members, pursuant to the agreement on Intellectual Property Rights between NMF and its members, and by non-members of NMF.

Inquiries should be directed to the NMF office at:

1201 Mt. Kemble Avenue

Morristown, NJ  07960 USA

Tel. No.  +1 201-425-1900

E-Mail:  info@nmf.org

http://www.nmf.org

Document History

Draft 0.1,  Aug. 11/1996 - Initial Draft

Draft 0.2,  Sept.  8/1996 - Post-Heidelberg

Draft 0.3,  Sept.  30/1996 - Pre-Barcelona

Draft 0.4,  Oct.  22/1996 - Pre-Barcelona

Draft 0.5,  Oct.  29/1996 - Post-Barcelona

Draft 0.6,  Jan.  24/1997 - Pre-Miami

Draft 0.7,  Feb.  13/1997 - Post-Miami

Draft 0.8,  Mar.  11/1997 - Pre-Long Beach

Draft 0.9,  Apr.   22/1997 - Final Draft

Outstanding issues and risks

TBD

Contacts for This Document

Team Leader:	Bob Ferry

US WEST COMMUNICATIONS

700 West Mineral

MN H18.01

Littleton, CO 80120

USA

Phone:		+1(303) 707-7160

Fax:		+1(303) 707-9497

e-mail:		rferrry@uswest.com

Editor:		Michael Best

NEWBRIDGE NETWORKS

600 March Road

P.O Box 13600

Kanata,  Ontario K2K-2E6

CANADA

Phone:		+1(613) 591-3600

Fax:		+1(613) 599-3669

e-mail:		mbest@newbridge.com

Contributors

Michael Best,  Newbridge Networks

Bob Ferry,  US West Communications

Steve Fratini,  Bellcore

Jane Hall,  GMD Fokus

Connie Hunt,  South Western Bell

Rubina Hussein,  Nortel

Jeremy Lin,  Fujitsu

Chris Chris Liou,  Cisco

Svend Pederson,  TeleDenmark

How to obtain a copy

An electronic copy of this document in MS-Word V6.0 format can be obtained using ftp from the NMF server.  Alternatively, contact the NMF as indicated above.  The file is located at:

199.183.141.190:specifications/service/busagreement/nmf502.doc

Comments

Comments should be sent to the editor.

NMF

PEER-TO-PEER SERVICE CONFIGURATION
REQUIREMENTS SPECIFICATION

CONTENTS

1.0	INTRODUCTION TO THIS DOCUMENT	6

1.1	How This Document Will Be Used	6

1.2	Document Structure	6

1.3	Key Terms Used In This Document	7

2.0	BUSINESS OBJECTIVES	8

2.1	Introduction to the Requirements	8

2.2	Current Environment	8

2.3	Business Need	9

2.4	Summary of Business Requirements	11

2.5	Expected Beneficiaries	11

3.0	SCOPE OF INTERFACE REQUIREMENTS	12

3.1	Process Orientation	12

3.2	Process Functions Addressed	13

3.3	How the Interface Is Expected to be Used	15

3.4	An Illustrative Notation for Representing the Service Provider to Service
		Provider Relationship	17

3.5	Scenarios to be Addressed	19

4.0	FLOW OF INFORMATION	24

4.1	Workflow Perspective	24

4.1.1  Bandwidth-Service Establishment	24

4.1.2  Bandwidth-Service Modification	24

4.1.3  Bandwidth-Service Release	25

4.1.4  Bandwidth-Service Grouping	25

4.1.5  Fault/Event Reporting and Service Problem Resolution	26

4.1.6  Bandwidth-Service Inventory	28

4.2	Event Flow Perspective	30

4.3	State Model	32

5.0	SPECIFIC REQUIREMENTS OF THE SERVICE CONFIGURATION INTERFACE	33

5.1	Establish Inventory	33

5.2	Bandwidth-Service Establishment	33

5.3	Bandwidth-Service Modification	34

5.4	Bandwidth-Service Release	34

5.5	Query/Access Inventory	34

5.6	Group Establishment	34

5.7	Modify Group	34

5.8	Release Group	35

5.9	Service Affecting Fault Conditions	35

6.0	GENERAL TECHNICAL REQUIREMENTS	37

6.1	Performance of the Interface	37

6.2	Security	37

6.2.1  Entitity Authentication	37

6.2.2  Data origin authentication	37

6.2.3  Access control	38

6.2.4  Data confidentiality	38

6.2.5  Data integrity	38

6.2.6  Non-repudiation	38

6.3	Methods of synchronization upon session failure	39

6.4	Inventory synchronization / Database Reconciliation	39

6.5	Interface protocols	39

6.6	Nature of connection	39

7.0	CONCLUSIONS	40

Appendix A  Summary of Requirements	41

Appendix B  Glossary of Terms, Acronyms and Abbreviations	42

Appendix C  References	43

Appendix D  Standards	44

FIGURES

FIGURE 1:	MANAGEMENT ARCHITECTURE AND MOTIVATIONS	4

FIGURE 2:	SERVICE MANAGEMENT INTERACTION	5

FIGURE 3:	SERVICE PROVIDERS AND END-CUSTOMER RELATIONSHIPS	5

FIGURE 4:	SERVICE MANAGEMENT BUSINESS PROCESS MODEL	7

FIGURE 5:	THE SERVICE CONFIGURATION PROCESS

FIGURE 6:	SERVICE PROBLEM RESOLUTION PROCESS

FIGURE 7:	PROCESS INTERACTION AT THE INTERFACE

FIGURE 8:	CUSTOMER-PROVIDER RELATIONSHIP MODEL FOR PEER-TO-PEER	
	SERVICE CONFIGURATION

FIGURE 9:	CASCADING PEER-TO-PEER SERVICE CONFIGURATION

FIGURE 10:	HIERARCHICAL PEER-TO-PEER SERVICE CONFIGURATION

FIGURE 11:	BULK RESOURCES IN SUPPORT OF SUBSEQUENT PVC ESTABLISHMENT

FIGURE 12:	PVC ESTABLISHMENT

FIGURE 13:	ALTERNATE ROUTE FOR PVC ESTABLISHMENT

FIGURE 14:	VPC ESTABLISHMENT AND MODIFICATION

FIGURE 15:	RECONFIGURABLE VIRTUAL PRIVATE NETWORK (VPN)

FIGURE 16:	SWITCH – SERVICE PROVIDER

FIGURE 17:	BANDWIDTH SERVICE ESTABLISHMENT

FIGURE 18:	BANDWIDTH SERVICE MODIFICATION

FIGURE 19:	BANDWIDTH SERVICE RELEASE

FIGURE 20:	IMPLICIT BANDWIDTH SERVICE GROUPING

FIGURE 21:	EXPLICIT BANDWIDTH SERVICE GROUPING

FIGURE 22:	FAULT REPORT FROM CUSTOMER

FIGURE 23:	FAULT REPORT FROM ANOTHER SERVICE PROVIDER

FIGURE 24:	PROBLEM RESOLUTION VIA RECONFIGURATION

FIGURE 25:	INVENTORY

FIGURE 26:	BANDWIDTH-SERVICE EVENT FLOWS

FIGURE 27:	BANDWIDTH-SERVICE GROUPING EVENT FLOWS

# Preface

NMF is an international consortium of communications service providers and their suppliers, dedicated to promoting industry-wide agreements on how management information should be exchanged between a service provider and its customers, its suppliers and/or other service providers.  Such agreements are essential to achieve a seamless, cost-efficient operation where multiple companies are involved in the fulfillment of service requests, the management of services while in use by customers, the correction of problems that might impair service quality, and the accurate billing of the customer for services used.

These agreements are becoming an increasing necessity as regulatory forces change the landscape for service providers.  The addition of new value-add service providers such as Internet Service Providers and the development of global alliances mean that the service delivery chain is being extended beyond the traditional, monopoly environment where a single company had end-to-end control over the provision and management of customer services.  At the same time, some companies are choosing to focus their business on network infrastructure, either providing high-capacity intercity links to service providers, or providing local access to customer sites.  Finally, as has been the case for many years, service providers and network operators rely on equipment suppliers’ element management systems to make it possible to manipulate the network to create customer services, and to report meaningful information about the status of elements, in order that the network, and ultimately, customer services, can be maintained at required levels of quality.

NMF members work together to determine how their processes should be linked, and how they should work with customers’ and suppliers’ systems.  They work together because they believe that to solve this type of problem alone requires extensive time and money (which is better spent developing new offerings) and that proprietary solutions will not be accepted in the marketplace.  They work together because they see that in these areas, there is no competitive advantage to be gained over exchanging data in a proprietary fashion, and that, conversely, they can considerably reduce both the time to development and the risk of making bad decisions.

These requirements pertain to the specific exchange of information to support in-service reconfiguration of services purchased by one service provider from another.  This can be thought of as a Customer Network Management (CNM) capability for the wholesale market.  It is an area  in which NMF members have determined that a common, industry-wide approach is warranted.  They invite others to review these agreements, and to adopt the more specific information models and Solution Sets which will be developed to meet these requirements.  They invite suppliers to become familiar with these requirements, and to work with them on the details of how the requirements will be met through the application of standards and other industry-accepted specifications and technology.

# 1.0	Introduction to this document

This document defines the requirements for the exchange of management information for the purpose of configuring or reconfiguring a service that has been purchased by one service provider from another,  within a model of service management business functions documented in NMF’s Service Management Business Process Model.

## 1.1	How this document will be used

It is expected that this document will be used in two ways:

- Business managers will be able to determine, using this document, whether their own requirements are fairly and completely represented by the document, such that implementation of an interface that meets the requirements would represent a good investment and a satisfactory solution to existing operational problems.

2.	For the companies who worked together to identify the requirements (and others as become interested in implementing solutions that meet the requirements) this document will provide sufficient information to develop an information model that meets the requirements.

## 1.2	Document structure

This document is structured in two main parts with seven major sections.  The first part (Sections 2,3,4) is considered informative and provides information to allow the reader to understand the more detailed requirements definitions to follow.  The second part (Sections 5 and 6) is considered normative and contains the technical requirements relative to the business requirements.  The following reflects what you will find in each section:

Section 1	Introduces the document

Section 2	States the business objectives that are driving the need for this interface, and the high level operational requirements that must be met.

Section 3	Establishes the scope of the interface agreement, identifying the specific process(es) that are invoked, the parties to the interface, the functions that are the subject of the interface, and the types of information that are to be exchanged.

Section 4	Describes the flow of information across the interface, using multiple diagrams to represent different perspectives.

Section 5	Contains the specific requirements, in detail, and presented in such a way as they can be traced through to implementation.

Section 6	Lists any general technical requirements that may need to be considered in the development of the Information Model or the development of a Solution Set.

Section 7	Offers conclusions and insights from the team members who prepared the requirements document, including any issues they perceive, or their expectations of companies to meet the requirements.

Appendix A	Glossary of terms and acronyms, using precise definitions where they have been agreed upon by NMF

Appendix B	Detailed information (date, document number, source) regarding any document referenced in the Requirements Document

Appendix C	A review of work that has been done by other groups (outside the NMF) or that has been done by NMF, and which should be used in the development of the Information Model or Solution Sets that may be derived from this document.

## 1.3	Key terms used in this document

TBD

# 2.0	Business objectives

This section describes the business problem to be solved, and provides a high-level view of the requirements to be satisfied through common information exchange.  It is intended to make clear why investment in such a common exchange is beneficial, and how each stakeholder should expect to benefit.  Finally, this section contains illustrative scenarios of the business relationships that may exist between entities, and the purposes for which the agreement might be used.

## 2.1	Introduction to the requirements

The global nature of today's emerging telecommunication services is not only multi-national, but is also becoming multi-corporational as companies extend their reach to serve larger geographical areas or to increase the range of services they can offer to their customers.  Although one route to accomplish this extended reach is through direct expansion of networks and development of new service capabilities, another route is via formal or informal relationships (or the formation of consortia) with other providers.  Through these relationships,  providers without a physical network capability in a certain area (or with insufficient bandwidth capacity to serve their customers needs) will contract to pay for the use of bandwidth from another provider.  In its simplest form, this contractual arrangement may be considered a sale of ‘wholesale’ services (bandwidth) by one provider to another, and may be effected through a service order.

The new business models for providing  services across multiple service providers  brings about several challenges to the integration, or more precisely, cooperation or participants in the service delivery chain.  This new way of doing business brings opportunities (doing business worldwide, for example) as well as risks.  Chief among the risks, when attempting to fulfill a customer’s end-to-end needs in a timely fashion, is that the process of fulfillment is no longer totally within  a single service provider’s control. To minimize the risk of failing to meet a customer commitment, these requirements specify the need for one service provider to (re) configure, in an automated fashion, bandwidth resources that are physically owned by another service provider.  These requirements limit the scope of command to I only a restricted, pre-purchased set of bandwidth resources.

## 2.2	Current environment

The traditional provider to provider configuration process does not meet the requirements of the new market environment.  Single-provider bandwidth-services need to be integrated into a complete, multi-provider end-to-end service.  Internal and proprietary systems are no longer sufficient when a service provider has to negotiate with other service providers.

Among the problems of today’s process are the following:

- Lack of automation (e.g., manual procedures for interacting between service providers makes service delivery time-consuming expensive, complex and error-prone)

- Lack of visibility/control (e.g.,  inability to monitor and react to changing conditions causing service outages and resource shortages)

- Lack of consistency in service delivery and quality (e.g., different paradigms between service providers makes end to end multi-provider services complicated to deploy, maintain etc..)

- Lack of flexibility (e.g.,  inability to provide new services or expand existing service quickly to take advantage of market changes and opportunities)

## 2.3	Business Need

In a customer-centric market service providers will need to meet the requirements of sophisticated, often multinational, corporate customers if they are to stay in business.  The globalization of business requires networking capacity to be available worldwide, and demands for improved availability and quality of international communications services by customers are increasingly being expressed.  Services must provide value to a customer, and they must be able to support not only current customer needs but also be able to adapt to changing customer needs.  Service providers have to ensure that customer requirements for bandwidth can be met flexibly and in a timely fashion.  A fast and efficient configuration process will therefore be essential for service providers in a liberalized, global, competitive telecommunications service market.

Figure 1:  Management architecture and motivations

Customers are playing an increasingly significant role in the marketplace.  A customer will purchase end-to-end services on a one-stop shopping basis regardless of the number of service providers involved in actually providing any one service. If the one-stop service provider (see Fig. 1) is unable to meet the customer's requirements it must enter into cooperative arrangements with other service providers in order to provide an end-to-end service to the customer. In this context multiple service providers interact in a peer environment in order to provide end-to-end connections which may span several service provider jurisdictions (see Fig. 2).  Service providers must be able to provide the required quality of service, even where they involve multiple service providers with each service provider managing the end-to-end segment in its own domain.

Changing business requirements also mean that the service provider must be able to provide a flexible service and arrange cooperative agreements quickly and efficiently. A critical success factor for such service providers is how they manage these relationships with other service providers so that customer demands are met without customers having to be aware of these relationships (see Fig. 3) Service provider-to-service provider management of a service must be efficient, flexible, reliable and fast.

Figure 2:  Service management interaction

The traditional configuration process does not meet the requirements of the new market environment. There must be a standardized and consistent approach to configuration functionality across service provider boundaries. Configuration of bandwidth must be on-demand, in real-time and processed through an automated interface between service providers on either side of the inter-domain boundary.

Figure 3:  Service providers and end-customer relationships

The interface between any two service providers should not be unique for just those two service providers, or every service provider will need several management interfaces, one for each of the service providers it is cooperating with.  By adopting a standard automated interface the service providers can offer a more effective service more efficiently.  If they do not do this, they will not be so competitive and the business will go elsewhere.

## 2.4	Summary of Business Requirements

- There is a need for a service provider to establish, modify and release bandwidth-services on-demand within the jurisdiction of another (peer) service provider.

- There is a need for a service provider to have access to an up to date inventory of resources it controls within another service provider jurisdiction.  The inventory includes status as well as other relevant information.

- There is a need for a provider to logically group bandwidth-services it has established within another service provider jurisdiction.

- There is a need for a service provider to be notified as quickly as possible when a service-affecting fault condition exists and when it has been rectified

Assumptions:

- Interaction between service providers occurs after a contractual arrangement has been put into place.  Such an arrangement could be established through an ordering procedure or other formalized contract.  The establishment of this contractual arrangement is out of scope for the purposes of this work, however its existence is a fundamental assumption.

## 2.5	Expected beneficiaries

- Service providers will benefit from a common provider-provider interface for configuring bandwidth-services.  It will result in:

-	Improved Customer Service:  Expedited offerings of service are the primary drivers of this work, as automation can greatly improve the "throughput" of service providers offering services across multiple domains or jurisdictions.

-	Reduced operating costs:  These costs include operator costs for equipment configuration, human costs for handling requests, data costs for transferring information back and forth in a non-automated way, and other costs associated with people and processes related to establishing bandwidth-services across several service providers.

-	Scaleable, distributed service offerings that motivate future cooperative ventures.  The new business model of cooperative service offerings via multiple service providers demonstrates a means of scaling and distributing services other than voice.  Success of this new type of business model may propagate to other services and serve as a proof-of-point that will spawn new ventures and services involving multiple service providers.

- Management system vendors and software providers will benefit from an agreement on provider-to-provider bandwidth configuration.  They will be able to develop configuration management systems and applications that can be used by multiple service providers.

# 3.0	Scope of interface requirements

## 3.1	Process orientation

The Service Management Business Process Model (SMBPM)[1] (see Fig. 4)  focuses on processes within a single service provider.  The interaction between service providers is not well defined.  Good examples of this can be seen in the process description diagrams where external process interaction flows to ‘other provider’ without describing the target process in the other provider (see Fig. 5 and Fig. 6).

Figure 4:  Service Management Business Process Model

There are two processes of interest to the interface (see bolded parts of Fig. 4), the Service Configuration Process and the Service Problem Resolution Process.

The Peer to Peer Service Configuration interface is meant for use between service providers.  Because of the lack of detail in the SMBPM two inter-provider process assumptions were made:

- The first assumption was that a service provider’s Service Configuration Processes (SCP) can interact directly with the Service Configuration Process of another service provider.  These interactions includes service reads, adds, deletes and modifications.

- The second assumption was that a Service Problem Resolution Process can interact directly with the Service Problem Resolution of another service provider.  These interactions involve the sending of real-time fault information.  The interface diagram in Fig. 7 incorporates these two  assumptions.

## 3.2	Process functions addressed

Figure 5:  The Service Configuration Process

The Service Configuration Process (SCP) provides functions for configuring services (see Fig. 5).  These can be either inside the jurisdiction of a service provider, or within the jurisdiction of another provider.  In the first case configuration is done through an internal procedure, for the purposes of this work this case is not considered.  In the second case there is interaction between the Service Configuration Processes of the two providers involved, the interface will include this interaction.

In a typical scenario one provider  acts  as the Main Service Provider, the other  as the Secondary Service Provider,  where the Main Service Provider is the customer of the Secondary Service Provider.  The SCP of the  Main Service Provider can send query, add, delete and modify requests to the SCP of the  Secondary Service Provider.  Requests are valid only for services which are owned by the  Main Service Provider.

Figure 6:  Service Problem Resolution Process

The Service Problem Resolution (SPR) process provides functions for resolving service problems locally (through internal systems) as well as services within another service provider jurisdiction (see Fig. 6).  In the first case problem resolution is accomplished through an internal procedure, for the purposes of this work this case is not considered.  In the second case there is interaction between the SPRs of the two providers involved, the proposed interface will address this interaction.

In a typical scenario one provider  acts  as the Main Service Provider, the other   as the Secondary Service Provider,  where the Main Service Provider in the customer of the Secondary Service Provider.  The SPR of the  Main Service Provider receives event notifications from the  Secondary Service Provider’s SPR.  Notifications are sent to indicate the existence of service affecting conditions.  They are only sent against services which are owned by the  Main Service Provider.

## 3.3	How the interface is expected to be used

Figure 7:  Process interaction at the interface

Service providers A and B interact over the interface shown in Fig. 7.  The scope of the interface is limited to interaction between Service Providers.  There are internal process interactions occurring however these are not in scope, they are shown in the diagram only for illustrative purposes.

In a typical scenario one service provider acts as the Main Service Provider , the other as the Secondary Service Provider,  where the Main Service Provider in the customer of the Secondary Service Provider. .  The  Main Service Provider’s  SCP can query, modify, create or delete services by sending requests through the interface to the  Secondary Service Provider’s SCP.  The Secondary Service Provider’s SPR will notify the  Main Service Provider’s SPR when service affecting faults occur.  These interactions apply only to services which are owned by the  Main Service Provider .

The Main Service Provider’s  SPR upon receiving a fault report from the Secondary Service Provider’s SPR may choose corrective action such as requesting the Main Service Provider’s SCP do a are-configuration.  This internal interaction is not in scope.  If the Secondary Service Provider’s  SPR determines a fault condition exists for a service owned by the Main Service Provider it will issue fault notification.  The Secondary Service Provider’s  SPR derives this information from internal network fault information it receives from the Network Monitoring/Problem Resolution Process (NMPRP).  This internal interaction is not in scope.

## 3.4	Relationships with other SMART projects

This project proposes on a Peer to Peer Service Configuration interface between service providers.  This represents one part of the overall Service Management Business Process Model.  Several other groups within the NMF are actively working on other parts of the model.

One of the goals of the SMART program is to facilitate flow-through and interworking of different service management processes.  The Peer to Peer Service Configuration project has strong relationships with other SMART projects.  For the purposes of this project these interactions are out of scope, however, they may be facilitated later in the project cycle through the use of common objects and information.  The following is a brief description of these relationships.

### 3.4.1  Ordering:

The use of the Peer to Peer Service Configuration interface is based on the premise that one service provider has ordered bulk capacity from another service provider.  Once the requesting service provider is informed their order has been filled they can use the Peer to Peer Service Configuration interface for real-time management of the purchased capacity.  It is, therefore, fundamental that the ordering system notify the service configuration system when an order has been filled.

### 3.4.2  Performance:

The Service Configuration interface is notified of service affecting faults.  This information is important for the performance monitoring process(es) since fault conditions represent service degradation or outage.  This information is also useful for monitoring SLA compliance.

### 3.4.3  Billing:

The Service Configuration interface is used to establish, modify and release bandwidth-services.  Each of these interactions could have an effect on customer billing.  Service affecting faults are also reported through the interface.  This information is important for the billing process(es) since it could trigger a billing adjustment or rebate.

### 3.4.4  Trouble Ticketing:

A Trouble Ticket is used to determine the cause,  and provide rectification for service outage.  The Peer to Peer Service Configuration interface can be used to reconfigure services.  In order to rectify customer outage a service provider could use this mechanism to establish alternative service delivery.

## 3.5  An Illustrative Notation for Representing the Service Provider to Service Provider Relationship

A better understanding of the interfaces addressed within the scope of this document can be achieved using diagrams to represent the relationship between service providers.  The interface between service providers supporting peer to peer service service configuration  can be represented by a  Main Service Provider (service provider A) to Secondary Service Provider (service provider b) relationship,  where the Main Service Provider is the customer of the Secondary Service Provider.

With this relationship in mind, we introduce the following notation and diagram to represent the  Main Service Provider - Secondary Service Provider relationship for service providers A and B.

Figure 8:   Main Service Provider - Secondary Service Provider relationship model for Peer-to-peer Service Configuration

In Fig. 8 we show two non-intersecting domains of service providers A and B, represented by the boxes.  The containment hierarchy of boxes reflects the Main Service Provider - Secondary Service Provider  relationship between A and B - service provider A plays the role of Main Service Provider, invoking configuration services offered by service provider B via the interface represented by triangle i.  The arrow denotes invocation of interface i offered by service provider B. Note that the containment  illustration does not imply network domain A is a “superset” of or contains network domain B - it is a convenient notation used to show the relationship with respect to interface usage.

The empty circles on the border of domain B represent physical network points in service provider B’s domain that are accessible by service provider A, and which border domain A.  These endpoints, b1 and b2, are what service provider A can connect to for establishing service between points of presence a1 and a2 in domain A, represented by the solid circles.  The dashed line connecting network points b1 and b2 represents an instantiation of the configuration service which service provider A has created within service provider B’s domain, linking the network points b1 and b2.  In summary, service provider A configures a service through service provider B’s domain between network points b1 and b2, utilizing the configuration interface i offered by service provider B.

This illustrative notation can be used to describe a referral or cascading model, in which a service provider that is playing the Secondary Service Provider  role may in turn be a Main Service Provider  to another service provider.  This is illustrated in Fig. 9 below.  Service provider B, in configuring service between b1 and b2 for service provider A, utilizes service configuration interface j to provision service between network points c1 and c2 in service provider C’s domain.  Service provider A is the Main Service Provider to service provider B which is a Secondary Service Provider.  In turn service provider B is the Main Service Provider  to its Secondary Service Provider, service provider C.

Figure 9:  Cascading Peer-to-peer Service Configuration

The peer-to-peer service configuration interfaces can also be used in a hierarchical model, whereby a service provider plays the role of Main Service Provider  to more than one Secondary Service Provider.  This is illustrated in Fig. 10 where service provider A is a Main Service Provider  to Secondary Service Providers  B and C, utilizing interfaces i and j, consecutively, to provision service between b1 and b2 in service provider B’s domain, and between c1 and c2 in service provider C’s domain.  The network point represented by the circle labeled b2 and c1 logically represents a network point that is common to both domains B and C, e.g. a pair of network points connected together.

Fig. 10 Hierarchical Peer-to-Peer Service Configuration

The cascading and hierarchical scenarios can be mixed, and can involve several other service providers in a recursive manner.  With new telecommunication services emerging and deregulation occurring in several parts of the world, more and more service providers are looking to extend new and existing services into and/or through other providers’ domains - the peer-to-peer service configuration interface supports the automated configuration of such services .

## 3.6	Scenarios to be addressed

This section presents several examples related to the configuration of services over resources that have been previously ordered and provisioned. The intent is demonstrate business drivers for the Peer-to-Peer Service Configuration work. After each example, a list of related business drivers are given.

### 3.6.1  Scenario #1:  End to End,  multi-domain Connection setup

In the situation shown below, Service Provider (SP) X’s territory is interrupted by SP Y’s territory. SP Y can be viewed as a service provider with a “friendly” relationship with SP X, or SP Y could be a service provider strictly dealing in the leasing of long-haul high-bandwidth facilities. In either case, SP Y is willing to lease bulk transmission capacity to SP X.

In order to provide end-to-end service to its customers, SP X leases (orders) SDH links EJ, JF, JG and JH from SP Y, as shown in the Figure 11. Point J represents an ATM cross connect capability within SP Y’s territory but is visible to SP X. The points E, F, G, and H belong to SP Y and provide ATM cross connect capability. The points A, B, C, D and M, shown in the figure, represent the end-points of a particular customer of SP X. It should be noted that this part of the scenario, i.e., service ordering, is outside the scope of the Peer-to-Peer Service Configuration work.

Figure 11:  Bulk Resources in Support of SuBWSequent PVC Establishment

After the SDH links are configured, SP X may, for example, get a request from one of its customers to create an ATM PVC from A to B, and a multipoint PVC from C to D and M. The PVCs will need to use some of the resources previously leased from SP Y. In order to establish the two PVCs, SP Y needs to perform several cross-connects at Point J.

In order for SP Y to understand SP X’s request to perform cross-connects at Point J, the management systems of both service providers need to share a common understanding of the resources involved (i.e., they need to use a common information model) and, of course, they need to support the same management protocol.  The Peer-to-Peer Service Configuration work intends to support both needs.

Figure 12 shows the routes traversed by the 2 PVCs. Each PVC traverses two or more UNIs (represented as thick lines in the figure), and several NNIs in each service provider’s territory. The clouds shown in SP X’s territory indicate that the specific network resources employed in SP X’s network are not important to the discussion of this example.

Figure 12: PVC Establishment

Alternatively, SP X could request the two PVCs be established as shown in Figure 13. In this case the multipoint PVC branches at point G rather than J. (In this case, G provides ATM cross connect capability and is owned by SP Y.) This configuration would achieve the same end as the previous configuration.

Figure 13:  Alternate Route for PVC Establishment

The business drivers illustrated in this example are as follows:

- ATM PVC establishment over bulk transmission resources leases by one service provider from another, (i.e., a management service for an ATM PVC establishment, modification and release provided over the bulk transmission facilities of one service provider and bought by another service provider), and

2.	Although not mentioned in the previous discussion, the establishment of Frame Relay PVCs follows a similar pattern.

### 3.6.2  Scenario #2:  End to End,  multi-domain virtual path connection

This example is similar to Example 1 in that SP X leases bulk transmission facilities from SP Y. Example 2 differs from Example 1 because the end-points of interest represent tandem switches rather than customer end-points.  In Figure 14, the lines A to B, C to D, and K to M represent high-speed ATM Virtual Path Connections (VPCs) between tandem switches.  One can envision that these VPCs carry a large number of multiplexed phone calls.  The, yet to be defined, protocol supporting the Peer-to-Peer Service Configuration model could be used by SP X to request that SP Y setup a portion of a VPC, or adjust the bandwidth of part of an existing VPC.  Bandwidth adjustment may be needed if, for some reason, the traffic load between A and B becomes greater relative to the load between C and D.

As another example, the VPCs shown in Figure 14, could be used to carry ATM SVC traffic.

Figure 14: VPC Establishment and Modification

The business drivers illustrated in this example show how the Peer-to-Peer Service Configuration work could be used to support the setup and modification of a VPC whose route traverses the leased facilities of another service provider.  The VPCs could carry, for example, phone calls (possibly long distance) or ATM SVCs.

### 3.6.3  Scenario #3:  Virtual Private Network (VPN)

In Example 3, a customer of SP X, known as Customer Z, purchases a reconfigurable VPN.  In order to support the VPN, SP X leases DS3 links from SP Y, i.e., links EJ, JF, JG and JH.  Using the DS3 facilities leased from SP Y, SP X can provide various connectivity arrangements between the endpoints owned by Customer Z.  For example, SP X can establish end-to-end Frame Relay connections between B and H (PVC #1 in Figure 15), C and G (PVC #2 in Figure 15), and D and F (PVC #3 in Figure 15).  As in the previous examples, SP X requires Cross-Connect capabilities from SP Y at point J.

Figure 15:  Reconfigurable Virtual Private Network (VPN)

With a small modification, Example 3 can also be viewed as a Value Added Network (VAN) provider example.  Consider a Service Provider W who owns no network resources but rather leases resources from SP X and SP Y.  In this case, Customer Z is the customer of SP W who in turn is a customer of SP X and Y. Here SP W uses the resources of SP X and Y to provide the three connections shown in the previous figure.

In both variations of this example, the leasing of the DS3 facilities is outside the scope of the Peer-to-Peer Service Configuration work, but the suBWSequent setup and configuration of logical resources, e.g., Frame Relay PVCs, over the DS3 transmission facilities is with the scope.

The business drivers illustrated in this example are

- The support of a reconfigurable VPN over previously leased bulk transmission resources such as DS3 or SDH transmission facilities and

2.	The support of virtual resource setup, e.g., PVCs, for a Value Added Network provider

### 3.6.4  Scenario #4:  Local Digital Switch Unbundling

The following drawing depicts unbundling of a Local Digital Switch (LDS).  Part (or all) of the switch could be reserved for a Competitive Local Exchange Carrier (CLEC) by leasing line and trunk ports.  The scenario of unbundling in the access area with either GR-303[2] or TR-008[3] will be handled via the Ordering interface between Service Providers and those processes.

Figure 16: Switch – Service Provider

The resources represented by the lines and trunks in Fig. 16 indicate a reserved/leased capacity for a CLEC from the Incumbent Local Exchange Carrier (ILEC).  Also, with other switch types (e.g., ATM, Frame Relay, toll, tandem) the line side could be considered trunks.  In this configuration scenario the software features (e.g., switch translations, AIN triggers are considered modifications from the original order.  These are considered in the event flows and are similar to the Ordering event flows for the software translations.  One of the considerations in the negotiation/contract phase of the process should be the limit of usage or bandwidth for engineering purposes, so ensure that expected service levels will be met.

# 4.0	Flow of information

This section describes the dynamic aspects of the provider-provider service configuration process in terms of a workflow diagram, an event flow diagram, and a state model.

## 4.1	Workflow Perspective

Note:  For workflow diagrams the following abbreviations are used:

- SPA : Service Provider ‘A’ (Internal Network)

- SPB : Service Provider ‘B’ (External Network)

- CIMP : Customer Interface Management Process

- SCP : Service Configuration Process

- SPRP : Service Problem Resolution Process

- PHP : Problem Handling Process

- NMPRP : Network Monitoring Problem Resolution Process

- NSAP : Network System Administration Process

- BIIP : Bandwidth Information Inventory Process

- BWS : Bandwidth service

The term like “SCP(SPA)” indicate Service Configuration Process of Service Provider ‘A’.  The shaded box in the diagrams indicates the process is in scope.

### 4.1.1  Bandwidth-service establishment

(see Fig. 17 for a graphical workflow representation)

1.	(out of scope) SPA receives bandwidth service(BWS) establishment request from a customer via the Customer Interface Management process.

2.	(in scope) SPA's service config process notices requested BWS requires resources outside its jurisdiction.

3.	(in scope) SPA's service config process requests that SPB's serv config process setup part of the BWS.

4.	(out of scope) SPA's service config process requests the NSAP(SPA) to setup SPA's part of the BWS requested.

5.	(successful case) The service config process SCP(SPA) receive the positive acknowledgment from SCP(SPB) & NSAP(SPA) that both local and external resources have been successfully set.

6.	(out of scope) The SCP(SPA) responds to CIMP(SPA) about successful BWS establishment.

7.	(unsuccessful case) SPB's service config process (or SPA's internal sys) indicates that the requested BWS cannot be setup,  may try to re-route.

8.	(out of scope) If no re-route is possible, SPA's service config process sends a negative indication to the customer interface process.

9.	(out of scope) The Cust Int Mgmt process responds to the customer.

Editors Note:  In the following three diagrams - Figures 17-19,  replace the capture text in the bottom left box of the figure from NSAP (SPA) to NSAP (SPB).

Figure 17:  Bandwidth Service Establishment

### 4.1.2  Bandwidth-service modification

(see Fig. 18 for a graphical workflow representation)

1.	(out of scope) Service Provider A receives a request to modify an established BWS from a customer via the Customer Interface Management process.

2.	(out of scope)SPA's service config process notices requested BWS modification affects resources outside its jurisdiction.

3.	(in scope) If modification affects the resource outside the jurisdiction of SPA, SPA's service config process requests SPB's service config process to modify the BWS.

4.	(out of scope) If local modification is to be executed, the SPA’s service config process requests its network system admin process (NSAP).

5.	(out of scope) Both service config process returns a positive response.

6.	(in scope) SPA's service config process sends either a positive or a negative indication to the Cust Int Mgmt process, depending on the response received.

7.	(out of scope) The Cust Int Mgmt process responds to the customer.

Figure 18:  Bandwidth Service Modification

### 4.1.3  Bandwidth-service release

(see Fig. 19 for a graphical workflow representation)

1.	(out of scope) SPA receives a request to release an established BWS from a customer via the Customer Interface Management process.

2.	(out of scope) SPA's service config process notices requested BWS release resources exists outside its jurisdiction.

3.	(in scope) SPA's service config process requests SPB's serv config process to release its part of the BWS.

4.	(out of scope) SPA's service config process requests NSAP(SPA) to release SPA's part of the BWS.

5.	(in scope) Both the service configuration process of SPA & SPB release the resource requested.

6.	(out of scope) SPA's service config process indicates that the requested BWS has been released to CIMP (SPA).

7.	(out of scope) The Cust Int Mgmt process responds to the customer.

Figure 19:  Bandwidth Service Release

### 4.1.4  Bandwidth-service grouping

[In the following discussion, it should be noted that the actual mechanism of the VPN is out of scope, since VPN management is a customer facing interaction.]

(see Fig. 20 for a graphical workflow representation)

The Peer-to-Peer Service Configuration interface needs to provide some sort of bandwidth-service grouping functionality to support implementation of a VPN (there could be other applications, but nothing specific has been identified).  In particular, the VPN service provider needs some mechanism to associate bandwidth-services (or components of bandwidth-services), in another service provider’s territory, with a particular VPN.  This association can be done either implicitly or explicitly.

A short process flow is given for each case. Service Provider A (SPA) plays the role of a Main Service Provider  that leases facilities from a Secondary Service Provider  (SPB).

In the implicit grouping case, the following steps take place:

(see Fig. 20 for a graphical workflow representation)

1.	(out of scope) SPA leases bulk transmission facilities from SPB.

2.	(out of scope) A customer of SPA (named Customer Z) purchases a bandwidth service group. SPA determines that it will need to use some of the resources leases from SPB in order to support this bandwidth service group.

3.	(in scope) SPA requests SPB to setup connections over previously leased facilities.  In the case where there is more than one possible route between locations (e.g., managed elements) that are to be connected, SPA specifies the location name and the name of the end-point at the location, e.g., there are multiple routes between Location 1 and Location 2 in Figure 1-1.

4.	(in scope) SPB responds to SPA’s request. SPA takes note of the resources allocated to Customer Z’s bandwidth service group but SPB is not aware of it.

Figure 20: Implicit Bandwidth Service Grouping

In the explicit group case, the following steps take place:

(see Fig. 21 for a graphical workflow representation)

1.	(out of scope) SPA leases bulk transmission facilities from SPB.

2.	(out of scope) A customer of SPA (named Customer Z) purchases a bandwidth service group from SPA.  SPA determines that it will need to use some of the resources leases from SPB in order to support the this bandwidth service group.

3.	(in scope) SPA requests SPB to setup connections over previously leased facilities, and to associate a Group Id with the allocated resources.  In this case, SPB is aware that a collection of connections are associated together but is not specifically aware that how this BWS group association is used by SPA.

4.	(in scope) The SCP(SPB) send the acknowledgment to SCP(SPA).

Figure 21:  Explicit Bandwidth Service Grouping

The Group Id is not essential, since SPA is able to group the leased resources independent of any help from SPB.  However, the Group Id could be used by SPB (the Leaser) to provide high-level services to SPA (the Leasee), e.g.,

- SPB could provide SPA the ability to release all resources allocated to a particular Group Id (bulk transmission resources and/or bandwidth-services established over the transmission resources), via a single request,

- SPB could provide SPA the ability to make the same modification to all resources belonging to a particular group, via a single request.

The Group Id may also be useful in turning alarms “on” and/or “off” for groups of resources.

### 4.1.5  Fault/Event reporting and service Problem resolution

1.	Fault/Event reporting process flows can affect the service configuration process.  The following cases are described in detail.

1.  Fault report from the customer

2.  Fault report from another SP

3.  Problem resolution via reconfiguration if fault reported

4.1.5.1  Fault reports from the customer:

(see Fig. 22 for a graphical workflow representation)

1.	(out of scope) Customer detects the bandwidth segment failure and report it to Customer Interface Management Process(CIMP) of SPA.

2.	(out of scope) CIMP(SPA) issue a fault report to problem handling process PHP(SPA) in order to register this fault in SPA system.

3.	(out of scope) PHP(SPA) submit the detail fault description to service problem resolution process SPRP(SPA) for localization.

4.	(in scope) SPRP(SPA) determine which end to end connection has failed. In order to localize the fault it send the request to network monitoring problem resolution process(NMPRP) of SPA.

5.	(in scope) If the fault exist in external resources of SPB, it submit a fault report to SPRP(SPB).

6.	(out of scope) SPRP(SPB) localize the fault through NMPRP (SPB).

7.	(out of scope) As per the responses received by SPRP(SPA) from NMPRP(SPA) and SPRP(SPB) it inform PHP(SPA) in order to respond to customer.

Figure 22:  Fault report from customer

4.1.5.2  Fault report from another SPB

(see Fig. 23 for a graphical workflow representation)

1.	(out of scope) The network monitoring/Problem resolution(NMPRP) of SPB detects a bandwidth segment failure.  The fault is reported to service problem resolution process(SPRP) of SPB.

2.	(out of scope) The SPR-SPB reports this fault to Problem handling process (PHP-SPB) through a fault report.

3.	(out of scope) The PHP-SPB reports this fault to Customer information mgmt. system (CIMP-SPB) in order to take care of the customer queries.

4.	(in scope) The SPRP(SPB) send the fault report to SPRP (SPA).

5.	(out of scope) The SPR-SPA determines that the bandwidth segment failure implies that a particular end to end connection has failed. It send the fault description to PHP (SPA).

6.	(out of scope) The PHP (SPA) inform CIMP (SPA) in order to take care of customer.

Figure 23:  Fault report from another Service Provider

4.1.5.3  Problem resolution via reconfiguration if fault reported

(see Fig. 24 for a graphical workflow representation)

The SPRP (SPA) is informed about the fault either from its own network or from external network.

1.	(out of scope) The SPRP (SPA) send the fault report to PHP (SPA).

2.	(out of scope) The SPRP (SPA) send BWS reconfigure request to SCP (SPA) in order to resolve the faulted bandwidth service.

3.	(out of scope) The SCP (SPA) determine whether the requested bandwidth service reconfigure request need local or external resource. If it needs local resource then it inform NSAP (SPA) to allocate new resource.

4.	(out of scope) The SCP (SPA) receive the acknowledgment from NSAP (SPA) and inform the bandwidth information inventory process (BIIP) of SPA about the faulted and new allocated bandwidth service.

5.	(in scope) If the requested bandwidth service resides outside the jurisdiction of SCP (SPA), it submit the request to SCP (SPB) to allocate new bandwidth segment and send acknowledgment.

6.	(out of scope) The SCP (SPA) inform SPRP (SPA) about the reconfiguration.

7.	(out of scope) The SPRP (SPA) inform PHP (SPA) about the reconfiguration so that the corresponding fault report in PHP (SPA) can be updated.

8.	(out of scope) The PHP (SPA) inform CIMP (SPA) about the problem resolution. The CIMP (SPA) close the fault report due to reconfiguration and notify the customer.

Figure 24:  Problem resolution via reconfiguration

### 4.1.6 Bandwidth-service inventory

The following assumptions apply to the bandwidth-service inventory process flow:

- Initially, the inventory consists of the resources ordered, or contracted between service providers.  Initialization of the service configuration process establishes the inventory of services within its jurisdiction.

- The service configuration process of Service Provider A (SPA) must be able to synchronize itself with the service configuration process of Service Provider B (SPB).

- The view of a service provider’s resources in another service provider’s is restricted to the resources that it leases (put in place by an order of other contractual agreement) and the logical resources setup over the leased facilities, e.g., a bandwidth segment.

- Changes caused by (re)configurations will be reflected in the inventory information.  These changes are made visible to other service providers, if they own the said resources.

- Internal changes of a resource’s status will affect the inventory.  Such changes will be reflected in the inventory information and made viewable by the service configuration information.

- A service provider will report status changes to other service providers for all leased resources (physical or logical).

(see Fig. 25 for a graphical workflow representation)

1.	(in scope) The SCP (SPA) queries SCP (SPB) about inventory in order to determine inconsistency.  If inconsistency is found then a resolution mechanism is needed which is out of scope.

Figure 25:  Inventory

## 4.2	Event Flow Perspective

This section provides graphical representations of event flows.  It describes the data transfer and linkages across key boundaries and interfaces in the environment considered.  (It should be noted that the event-flow highlights the same information as the workflow, but presents a different representation showing key events and the resulting information flow.)

Figure 26:  Bandwidth-service Event flows

Figure 27:  Bandwidth-service grouping Event flows

## 4.3	State Model

It is useful to make a distinction between process states and resource states.  For the problem at hand, an instance of a bandwidth service, e.g., a PVC, will have a resource state.  In addition, the process associated with the setup of the resource will also have a state.  The two states are different.  This section addresses process states; resource states will be discussed later in the project cycle as part the information modeling work.

The Inventory Change Notification and Service-affecting Fault Notification events do not need to have any state information associated with them, since the process of sending a notification is a one-step stateless procedure.

Other events such as Bandwidth-service and group-service establishment, modification or release are very simple, their process state will not be stored in the interface.  The possible process states are:  ACTIVE, COMPLETED, DENIED.

# 5.0  Specific requirements of the Service 
Configuration interface

Requirements are captured using the following naming convention:

CON = Configuration

FR = Functional Requirement

TR = Technical Requirement

## 5.1	Establish Inventory

5.1.1 CORE  The Service Provider acting as a Main Service Provider  (service provider A)  orders bulk capacity from a Secondary Service Provider (service provider B).  After the order has been filled and the capacity deemed ready for use service provider A will update their inventory based on the order. (CONFR1.1)

5.1.2 CORE The following information must be provided as part of the inventory establishment(CONFR1.2)

- Bulk-capacity identifier(s)

- Bulk-capacity endpoint identifier

- Bulk-capacity characteristics

- Bandwidth-service endpoint identifier(s)

## 5.2	Bandwidth-service establishment

5.2.1 CORE  A confirmed request shall be used to establish a bandwidth-service.  The request confirmation will contain, in the case of a successful request, a simple indication.  In the case of a request failure the confirmation must contain information indicating the cause of the failure. (CONFR2.1)

5.2.2  CORE  A bandwidth-service identification will be assigned by the system receiving the request.  It will be unique and unchanged for the life of the service.  This identification is returned as part of the establishment response.  (CONFR2.2)

5.2.3 OPTIONAL A bandwidth-service may support scheduled activity.  This shall include times and intervals when a service will turn up/turn down or change bandwidth. (CONFR2.3)

5.2.4 OPTIONAL A bandwidth-service may support routing specification.  This shall provide for the specifying of  internal logical routing point(s) between endpoints. (CONFR2.4)

5.2.5  CORE  The following information must be provided as part of the service establishment. (CONFR2.5)

- Service status (administrative,  operational  other)

- Endpoints (1..n)

- Service Features: 	automatic reroute

- diversity

- replication (protection)

- Service Type

- Group Id (may be NULL)

- Service characteristics (i.e. committed bandwidth)

- Internal logical routing points  OPTIONAL

- Scheduling Information (start,  stop and interval times)  OPTIONAL

## 5.3	Bandwidth-service modification

5.3.1 CORE  A confirmed request shall be used to modify a bandwidth-service.  The request confirmation will contain, in the case of a successful request, a simple indication.  In the case of a request failure the confirmation must contain information indicating the cause of the failure. (CONFR3.1)

5.3.2  CORE  The following information must be provided as part of the bandwidth-service modification. (CONFR3.2)

- Bandwidth-service identifier

- List of items to be modified

## 5.4	Bandwidth-service release

5.4.1 CORE  A confirmed request shall be used to release a bandwidth-service.  The request confirmation will contain, in the case of a successful request, a simple indication.  In the case of a request failure the confirmation must contain information indicating the cause of the failure. (CONFR4.1)

5.4.2  CORE  The following information must be provided as part of the bandwidth-service release. (CONFR4.2)

- Bandwidth-service identifier

## 5.5	Query/access inventory

Note:  In the resource information descriptions given below the unique identifier for each resource is underlined.

5.5.1 CORE  A confirmed request shall be used to query the interface.  The request confirmation will contain,  in the case of a successful request, a simple indication.  In the case of a request failure the confirmation must contain information indicating the cause of the failure. (CONFR5.1)

5.5.2 CORE  A query can access one or more resources visible at the interface. (CONFR5.2)

5.5.3 CORE  A bandwidth-service resource must have the following information.  The unique identifier for the resource is underlined. (CONFR5.3)

- Service status (up, down, other)

- Endpoints (1..n)

(note:  endpoints should be logical points - don’t use any physical

endpoint types)

- Service Features: 	automatic reroute

diversity

replication (protection)

- Service Type

- Group Id (may be NULL)

- Service characteristics (i.e., committed bandwidth)

- Bandwidth-service identifier

- Internal logical routing points  OPTIONAL

- Scheduling Information (start, stop and interval times)  OPTIONAL

5.5.4 CORE  A bandwidth-service endpoint resource must have the following information.  The unique identifier for the resource is underlined. (CONFR5.4)

- Bandwidth-service endpoint identifier

- Endpoint characteristics

- Endpoint status

- Group Id (may be NULL)

5.5.5 CORE  A bulk-capacity resource, either logical (VPN) or physical, must have the following information.  The unique identifier for the resource is underlined. (CONFR5.5)

- Bulk-capacity identifier

- Status

- Bulk-capacity endpoints (1..n)

- Bulk-capacity characteristics

- Group-Id (may be NULL)

5.5.6 CORE  A bulk-capacity endpoint resource must have the following information.  The unique identifier for the resource is underlined. (CONFR5.6)

- Bulk-capacity endpoint identifier

- Status

- Bulk-capacity endpoint characteristics

- Group Id (may be NULL)

5.5.7 CORE  A service-group resource must have the following information.  The unique identifier for the resource is underlined. (CONFR5.7)

- List of Bandwidth-service identifiers

- Group Id (may not be NULL)

## 5.6	Group establishment

5.6.1 CORE  A confirmed request shall be used to establish a service group.  The request confirmation will contain, in the case of a successful request, a simple indication.  In the case of a request failure the confirmation must contain information indicating the cause of the failure. (CONFR6.1)

5.6.2  CORE  A group identification will be assigned by system receiving the establishment request,  this will remain unchanged for the life of the group.  This identification will be returned as part of the establishment response.(CONFR6.2)

5.6.3  CORE  The following information must be provided as part of the service-group establishment. (CONFR6.3)

- List of Bandwidth-service identifiers

## 5.7	Modify group

5.7.1 CORE  A confirmed request shall be used to modify a group.  The request confirmation will contain, in the case of a successful request, a simple indication.  In the case of a request failure the confirmation must contain information indicating the cause of the failure. (CONFR7.1)

5.7.2 CORE  The following information must be provided as part of the service group modification. (CONFR7.2)

- Group ID

- Bandwidth-service identifier

- Type of operation: (add/delete)

## 5.8	Release group

5.8.1 CORE  A confirmed request shall be used to release a group.  The request confirmation will contain, in the case of a successful request, a simple indication.  In the case of a request failure the confirmation must contain information indicating the cause of the failure. (CONFR8.1)

5.8.2 CORE  The following information must be provided as part of the service group modification. (CONFR8.2)

- Group ID

## 5.9	Service affecting fault conditions

5.9.1 CORE  A confirmed notification shall be sent to inform a Mains Service Provider of a service affecting fault existing against a resource that it is controlling. (CONFR9.1)

5.9.2 CORE  The following information must be provided as part of the service-affecting fault notification (CONFR9.2)

- Resource identifier

- Fault identifier

- Time fault occurred

5.9.3 OPTIONAL  The Service Level Agreement defines what should occur for other than out-of-service faults.  Major, minor events might/might-not trigger a reconfiguration.  This could be determined by the end customer.  This is context specific, if a service is critical in nature than different levels of degradation may be considered service affecting. (CONFR9.3)

5.9.4 OPTIONAL  In the case of context specific fault conditions the following information must be provided. (CONFR9.4)

- Resource identifier

- Fault severity

- Cause of fault

- Fault information

- Time fault occurred

5.9.5 CORE  The following information must be provided as part of a fault clearing notification (CONFR9.5)

- Resource identifier

- Time of clear

# 6.0	General Technical Requirements

In addition to the business requirements within the Service Configuration context, there are a number of technical requirements that must be met to provide a robust and practical solution.

## 6.1	Performance of the interface

6.1.1 CORE  The Service Configuration interface must operate in an automated time-critical environment.  It must therefore operate in a  real-time manner.  (CONTR1.1)

6.1.2  CORE  The Service Configuration interface will provide only current information on resources.  (CONTR1.2)

6.1.3  OPTIONAL  The Service Configuration interface may provide 24x7 availability. (CONTR1.3)

## 6.2	Security

Service providers have an internal responsibility to secure their own systems and operations network.  In the case of external communications, however, additional security measures are required and must be agreed upon and implemented b both communicating parties.

Depending upon the security threats and risks for the Service Configuration Interface, different security mechanisms with different strengths may be used to protect all or selective information.  The specific mechanisms will be defined in SP-SP implementation agreements, based on protocol capabilities.

### 6.2.1	Entity Authentication

Entity authentication provides assurance of the identity of an entity.  An entity can be a user,  a system, or a logical entity.  The entity authentication process can be accomplished through a number of mechanisms. Asymmetric cryptographic techniques (public/private key combinations) are recommended because they provide strong authentication and less key management effort.

CORE  The interface shall provide for strong, two way entity authentication.  (CONTR2.1)

### 6.2.2	Access control

Access control protects against unauthorized used of a system resource.  It may be desirable for a service provider to have various levels of access permissions, however, in most cases entity authentication provides sufficient credentials for access control.  However it may be desirable for a service provider to have various levels of access privileges (e.g.  by type of operation,  time of day,  etc.).

OPTIONAL The interface may provide for access control. (CONTR2.3)

### 6.2.4	Data confidentiality

Data confidentiality protects data from unauthorized disclosure.  This security feature would be used if there was a need to protect valuable or sensitive information (e.g., calling card information).

OPTIONAL The interface may provide for full or selective data confidentiality. (CONTR2.4)

### 6.2.5	Data integrity

Data integrity is the protection of data from unauthorized modification, duplication or deletion.

CORE The interface may provide for data integrity. (CONTR2.5)

### 6.2.6	Non-repudiation

Repudiation is defined as "denial by one of the entities involved in a communication of having participated in all or part of the communication".  Two forms of non-repudiation are defined: non-repudiation with proof of origin and non-repudiation with proof of delivery.  This security service may take one or both of those forms. In non-repudiation with proof origin, the recipient of data is provided with proof of the origin of data.  This will protect against any attempt by the sender to falsely deny sending the data or its contents.  In non-repudiation with proof of delivery, the sender of data is provided with proof of delivery of data.  This will protect against any subsequent attempt by the recipient to falsely deny receiving the data or its contents.  This service is obviously useful in settling any disputes that may arise between Service Providers.

OPTIONAL  The interface may provide for non-repudiation of a communication. (CONTR2.6)

## 6.3	Methods of synchronization upon session failure

### 6.3.1 Data integrity

CORE  No data shall be lost due to session failure.  (CONTR3.1)

## 6.4	Inventory synchronization / Database Reconciliation

There are no requirements in this area.

## 6.5	Interface protocols

### 6.5.1 Protocol independence

CORE  The interoperable interface should be defined independent of any specific management protocol.  The decision of which management protocol to use for a specific specification will be made at a later time. (CONTR5.1)

## 6.6	Nature of connection

### 6.6.1 Type of connection

CORE  Due to the real-time nature, use of confirmed messaging and high volume of transactions between Service Providers a dedicated connection must be provided.  (CONTR6.1)

## 6.7	Time stamps

### 6.7.1 Time stamp correlation

CORE  All time values (time stamps) exchanged across the interface should be in a format that accounts for time-zone difference.  For example, systems may agree to use GMT.  (CONTR7.1)

# 7.0	Conclusions

This document is meant to be a cohesive statement of industry requirements.  A business need has been identified, described and qualified in a precise manner.  It has been developed deliberately in a management and networking technology independent manner.

The requirements team expects further work to be done in this area, specifically technical analysis and design/specification.  The intention of these efforts being to deliver implementation specifications for industry use.

# Appendix A  Summary of requirements

The following table captures the data/information to be transferred across the interface.  It will dictate the main attributes to be represented in the information model.  This appendix will be used as the 'checklist' against which the Information Model designers will ensure they have accounted for all requirements.

| Requirement | Information |
| --- | --- |
| Bandwidth-service establishment (CONFR2.5) | • Service status (administrative, operational other) • Endpoints (1..n) • Service Features: automatic reroute, diversity, replication (protection) • Service Type • Group Id (may be NULL) • Service characteristics (i.e. committed bandwidth) • Internal logical routing points OPTIONAL • Scheduling Information (start, stop and interval times) OPTIONAL |
| Bandwidth-service modification (CONFR3.2) | • Bandwidth-service identifier • List of items to be modified |
| Banwidth-service release (CONFR4.2) | • Bandwidth-service identifier |
| Banwidth-service query (CONFR5.3) | • Service status (up, down, other) • Endpoints (1..n) (note: endpoints should be logical points - don’t use any physical endpoint types) • Service Features: automatic reroute, diversity, replication, (protection) • Service Type • Group Id (may be NULL) • Service characteristics (i.e. committed bandwidth) • Bandwidth-service identifier • Internal logical routing points OPTIONAL • Scheduling Information (start, stop and interval times) OPTIONAL |
| Bandwidth-service endpoint query (CONFR5.4) | • Bandwidth-service endpoint identifier • Endpoint characteristics • Endpoint status • Group Id (may be NULL) |
| Bulk-capacity query (CONFR5.5) | • Bulk-capacity identifier • Status • Bulk-capacity endpoints (1..n) • Bulk-capacity characteristics • Group-Id (may be NULL) |
| Bulk-capacity endpoint query (CONFR5.6) | • Bulk-capacity endpoint identifier • Status • Bulk-capacity endpoint characteristics • Group Id (may be NULL) |
| Service-group query (CONFR5.7) | • List of Bandwidth-service identifiers • Group Id (may not be NULL) |
| Service-group establishment (CONFR6.3) | • List of Bandwidth-service identifiers |
| Service-group modification (CONFR7.2) | • Group ID • Bandwidth-service identifier • Type of operation: (add/delete) |
| Service-group releasem(CONFR8.2) | • Group ID |
| Fault notification (CONFR9.2) | • Resource identifier • Fault identifier • Time fault occurred |
| Context specific fault notification(CONFR9.4) | • Resource identifier • Fault severity • Cause of fault • Fault information • Time fault occurred |
| Fault clearing notification (CONFR9.5) | • Resource identifier • Time of clear |

# Appendix B  Glossary of terms, acronyms and abbreviations

ACSE

AIN

ATM

Bandwidth-service

Bandwidth-service endpoint

BIIP

BWS

Bulk-capacity

Bulk-capacity endpoint

CIMP

CLEC

CMIP

DS3

ILEC

LDS

Main Service Provider

NMF

NMPRP

NNI

NSAP

OSI

PHP

PVC

SCP

Secondary Service Provider

SDH

Service Provider

SLA

SMART

SMBPM

SONET

SP

SPRP

SVC
UNI

VPC

VPN

# Appendix C  References

1.	Service Management Business Process Model [TBD]

2.	GR-303, Integrated Digital Loop Carrier System Generic Requirements, Objectives and Interface, Bellcore, Dec. 1996

3.	TR-TSY-000008, Digital Interface Between The SLC(R)96 Digital Loop Carrier System and a Local Digital Switch, Bellcore, Sept. 1993

# Appendix D  Standards

The following standards are candidates for use in developing an information model to support the requirements.

NMF SS201,  Multi-Network Bandwidth Management Solution Set

ATM Forum M4 Network View

ITU-T M.3208

ITU-T M.3100,  Generic Network Information Model

ITU-T X.721,  Definition of Management Information

ITU-T G.805

Eurescom Xco-op