# Sensor Glove and Robotic Hand

# Advanced Digital Systems Lab

# Spring 2000

# Aaron Trask & Mariusz Zaczek

# 1.0 Introduction

The sensor glove allows an operator to move a computer-generated hand or a robotic hand like a puppeteer.  Each joint of the human hand (only the fingers) is encoded and then converted to an angle command.  The human hand has twenty degrees of freedom not including the wrist.   Each degree of freedom is encoded with a potentiometer.  The voltage across each potentiometer is converted to a binary number with an A/D converter.  This number is then converted to the corresponding angle for each joint.

2.0 Mechanics

The human hand has twenty degrees of freedom (see Figure 2.1).  Each finger has two degrees of freedom where they meet the palm and then one degree of freedom for each of the remaining two joint per finger.  Figure 2.2 explains the terminology adopted for this project.  The glove is constructed to encode each joint without restricting the operator’s movement.

### Figure 2.1:  Twenty Degrees of Freedom of the Human Hand

Figure 2.2:  Adopted Terminology

The first glove design consists of an exoskeleton surrounding a golf glove to protect the users skin.  The exoskeleton is composed of aluminum plate and steel shafts, which allow free motion of the users hand (see Figure 2.3).  A reference plate for all of the fingers is mounted on the back of the glove.  On the reference plate, five joystick potentiometers are mounted for each compound knuckle joint (not shown in Figure 2.3).  From these joystick potentiometers, a two-link joint is attached.  The other end is then bracketed to the top section of the finger.  Attached to the bracket is a potentiometer with a two-link joint.  The other end of this two-link joint is bracketed to the middle section of the finger.  This is repeated for the last remaining section.  This design allows the mechanics to be mounted within the width of a finger so that there is no obstruction of movement.  Each joint other than the compound knuckle must have a two-link joint in order to allow free movement since the axes of rotation are not coincident.  The compound knuckle has the yaw axis coincident but the pitch axis requires a two-link joint since it is not coincident.

Figure 2.3:  3-D Modeling of the Sensor Glove Design

The current glove design consists of a spandex glove with a plastic exoskeleton.  The plastic exoskeleton was made from thermoplastic, which can be easily formed when heated and hardens when cooled.  The potentiometers are attached in a similar fashion as the old design.

# 3.0 Circuit

3.1 Original Circuit for use with Parallel Port

In order to control the 3D hand a connection to the sensor glove is required. This connection has been made via the standard parallel port of a PC. The potentiometers of the glove are analog devices but the computer is digital as a result a conversion of one is necessary. This can be done via an A/D (analog to digital) converter. The converter used to read the potentiometers of the index finger and thumb is the ADC0808 8-bit, 8-channel converter. Only eight channels are required since only 8 joints will be controlled. The A/D converter requires a clock with frequency between 640 & 1280 kHz to be able to perform all conversion. As we only had on hand a 2 MHz crystal oscillator (clock) a J-K flip-flop was used to halve this frequency and thus fit the required range. If a 1MHz clock is available then the flip-flop portion of the circuit can be avoided. The output of the 2MHz clock was tied to both the J and K inputs as well as the flip-flop's own clock input. The resulting output produced a 1 MHz signal.

Figure 3.1:  Control Circuit of Sensor Glove

Once the clock is connected to the A/D converter the potentiometers are connected to the IN# pins (where #=0,1..7). The A/D converter reads the voltage of a given potentiometer based on the values of the A, B, and C pins of the converter (the pins correspond to the 2, 3, and 4 pins of the parallel port). These pins represent the front connection of a multiplexer which takes this 3-bit signal to determine which of the eight potentiometers to read (2^3 = 8). The output of the A/D converter is through the eight pins labeled (MSB) 2^-1, 2^-2 etc. These outputs are then connected to the parallel port. Certain bit values are first inverted by using the hex inverter. This is required since certain pins inside the parallel port of the computer itself are hardware side inverted.

To successfully convert the potentiometer analog input to a digital output the A/D converter requires a sequence of initializations in order to read and convert each value. The timing diagram of the ADC0808 converter shows the required sequence quite clearly. For each potentiometer conversion the following must be done: First the START and ALE pins of the converter are set low to clear them. Next, the ALE pin is set high, shortly afterwards the START pin is also set high. Next the ALE pin is brought low and then the START pin is brought low. At this instant the conversion routine begins and after about 100 microseconds the conversion is complete.  Finally, the output enable, OE, is set high and the A/D converter can read the data. Once done, the OE pin is again set low and the process can begin anew.

# 3.2 Motorola 68332 Circuit

# In the current design two Motorola 68332 microcontrollers are being used to read the input from the sensor glove. The first microcontroller will read 12 inputs corresponding to 3 fingers while the second microcontroller will read the remaining 8 inputs, 2 fingers. The split was necessary because the robotic hand will require twenty servo controlled via pulse width modulated (PWM) signals and each microcontroller has only 16 channels on which to output PWM. The circuit for the first three fingers and the circuit for the next 2 fingers is nearly identical except that the second will read in fewer potentiometer inputs. The two circuits are the same as the circuit used in the previous semesters control system and thus will not be repeated. One major change is that the A/D converter used by the “3-finger” circuit will be replaced by the ADC0816. This new A/D converter has sixteen analog inputs. The final major change is that the parallel port is no longer used to read the A/D converter and no output to the screen is produced. Instead the

MC68332 takes the input and outputs the PWM signals to the corresponding servos, which will control the robotic fingers. Attached in Appendix B is the new code used on the MC68332 microcontroller.

# 4.0 Code Basics

The code attached in appendix B is downloaded to the MC68332 microcontroller via an attached serial port of the board. The AS32.exe program is first used to compile the program and the TELIX terminal program is used to connect to the MC68332.

Once activated, the code interfaces with the A/D converter and selects the analog inputs that are converted to an 8-bit digital signal, which is read by the MC68332. The code then computes the current position of the potentiometer and determines the PWM signal in order to set the corresponding servo to its correct position. The next potentiometer is then read and the process is done for every potentiometer attached and then repeated.

## 5.0 Conclusion and Future Work

The new sensor glove is not completed and the robotic hand is still under construction.  Both are far along in the process of a prototype.  The code and equations will be modified to allow a more precise control and corresponding simulated movement.

APPENDIX A: OpenGL Code for 3D display

/* **************************************************************

*                                                            *

* (c) Copyright 1999 Mariusz Zaczek                          *

* ALL RIGHTS RESERVED                                        *

*                                                            *

* Permission to use, copy, modify, and distribute this       *

* software for any purpose and without fee is hereby         *

* granted, provided that the above copyright notice          *

* appear in all copies and that both the copyright notice    *

* and this permission notice appear in supporting            *

* documentation, and that the name of Mariusz Zaczek. not    *

* be used in advertising or publicity pertaining to          *

* distribution of the software without specific, written     *

* prior permission. (zaczek@uiuc.edu)                        *

*                                                            *

*    THE MATERIAL EMBODIED ON THIS SOFTWARE IS PROVIDED      *

*    TO YOU "AS-IS" AND WITHOUT WARRANTY OF ANY KIND,        *

*    EXPRESSED, IMPLIED OR OTHERWISE, INCLUDING WITHOUT      *

*    LIMITATION, ANY WARRANTY OF MERCHANTABILITY OR          *

*    FITNESS FOR A PARTICULAR PURPOSE.  IN NO EVENT          *

*    SHALL MARIUSZ ZACZEK BE LIABLE TO YOU OR ANYONE         *

*    ELSE FOR ANY DIRECT, SPECIAL, INCIDENTAL, INDIRECT      *

*    OR CONSEQUENTIAL DAMAGES OF ANY KIND, OR ANY DAMAGES    *

*    WHATSOEVER, INCLUDING WITHOUT LIMITATION, LOSS OF       *

*    PROFIT, LOSS OF USE, SAVINGS OR REVENUE, OR THE         *

*    CLAIMS OF THIRD PARTIES, WHETHER OR NOT MARIUSZ         *

*    ZACZEK HAS BEEN ADVISED OF THE POSSIBILITY OF           *

*    SUCH LOSS, HOWEVER CAUSED AND ON ANY THEORY OF          *

*    LIABILITY, ARISING OUT OF OR IN CONNECTION WITH THE     *

*    POSSESSION, USE OR PERFORMANCE OF THIS SOFTWARE.        *

*                                                            *

*    US Government Users Restricted Rights                   *

*    Use, duplication, or disclosure by the Government is    *

*    subject to restrictions set forth in                    *

*    FAR 52.227.19(c)(2) or subparagraph (c)(1)(ii) of the   *

*    Rights in Technical Data and Computer Software          *

*    clause at DFARS 252.227-7013 and/or in similar or       *

*    successor clauses in the FAR or the DOD or NASA FAR     *

*    Supplement. Unpublished-- rights reserved under the     *

*    copyright laws of the United States.                    *

*                                                            *

************************************************************** */

/*

* hand.c

*

*  This program uses the Parallel Port of a PC running Win95

*  which reads the potentionmeter voltages of the glove. These

*  voltages are transformed into angle values and a 3D hand is

*  draw using OPENGL and GLUT on the screen with appropriate

*  angles of the fingers.

*

*/

#include <GL/glut.h>

#include <stdlib.h>

#include <math.h>

#include <stdio.h>

#include <dos.h>

#include <conio.h>

#include <string.h>

/*

* >debug

* -d 0040:0008 L8

*

*  0040:0000          BC 03 78 03 78 02 C0 9F

*

*  This means that LPT1 = 03BC

*/

/* 0x0378  - Win95 Computer in ADSL

0x03BC  - the NT computer in ADSL */

#define DATA    0x0378          /* Output pins */

#define STATUS  DATA+1          /* Input pins...interrupt */

#define CONTROL DATA+2

#define TRUE  1

#define FALSE 0         /* Never actually used... :) */

#define PI 3.1415925

int     data;     /* Data FROM A/D converter */

int     status;   /* Bits which will be set */

int     control;  /* Bits which will be set */

/*

F2

F3  __   F1

F4  __ (  ) __

__ (  )|  |(  )

(  )|  ||  ||  |

T    |__||__||__||__|

___   |  ||  ||  ||  |

(   \  |__||__||__||__|

\   \ |              |

\   \|   Right      |

\   \      Hand    |

\   \             /

\               /

*/

int     T_tip=0,       T_mid=0,       T_joint=0;

int     T_tip_ZERO=0,  T_mid_ZERO=0,  T_joint_ZERO=0;

int    F4_tip=0,      F4_mid=0,      F4_joint=0;

int    F4_tip_ZERO=0, F4_mid_ZERO=0, F4_joint_ZERO=0;

/* Define the rest of the fingers */

GLenum doubleBuffer;

static int shoulder = 0, elbow = 0;

int position = 0, updown = 0;

static int  Afirst  = 0,  Bfirst  = 0,  Cfirst  = 0;

static int  Asecond = 0,  Bsecond = 0,  Csecond = 0;

static int  Athird  = 0,  Bthird  = 0,  Cthird  = 0;

static int  Afourth = 0,  Bfourth = 0,  Cfourth = 0;

static int  wrist   = 0;

static int  Athumb  = 0,  Bthumb  = 0,  Cthumb  = 0;

int flip_bits(int value)  /* ???? */

{

int new_val = 0;

if ( value & 128 ) new_val = new_val | 0x01;

if ( value & 64 )  new_val = new_val | 0x02;

if ( value & 32 )  new_val = new_val | 0x04;

if ( value & 16 )  new_val = new_val | 0x08;

if ( value & 8 )   new_val = new_val | 0x10;

if ( value & 4 )   new_val = new_val | 0x20;

if ( value & 2 )   new_val = new_val | 0x40;

if ( value & 1 )   new_val = new_val | 0x80;

return new_val;

}

void display_data(int dat)

{

if ( dat & 128 )

printf("\n    1");

else

printf("\n    0");

if ( dat & 64 )

printf("1");

else

printf("0");

if ( dat & 32 )

printf("1");

else

printf("0");

if ( dat & 16 )

printf("1");

else

printf("0");

if ( dat & 8 )

printf("1");

else

printf("0");

if ( dat & 4 )

printf("1");

else

printf("0");

if ( dat & 2 )

printf("1");

else

printf("0");

if ( dat & 1 )

printf("1");

else

printf("0");

}

/* ++++++++++++++++++++++++++++++

+                            +

+  getkey                    +

+                            +

+    - function to determine +

+      number of key pressed.+

+    - parameters: none.     +

+                            +

++++++++++++++++++++++++++++++ */

int getkey(void)

{

int i;

switch( i = (int)getch() )

{

case 0xe0:

case 0:

return 256 + (int)getch();

default:

return i;

}

}

int angle(int ZERO, int CURRENT)

{

float newangle=0.0;

/* 0.711 = 180/128 = 360/256  */

newangle = ( CURRENT - ZERO ) * 0.711;

return (int)newangle;

}

/* ++++++++++++++++++++++++++++++

+                            +

+  posit                     +

+                            +

+    - uses status & control +

+      data to combine them  +

+      and get full 8bit val.+

+    - parameters: none.     +

+                            +

++++++++++++++++++++++++++++++ */

int posit(void)

{

/* Clear variable */

int newpos = 0;

int temp = 0;

/* Combine bits */

/* Take bits from status */

if ( status & 128 )          /* Pin 11 */

newpos = newpos | 0x80;

if ( status & 64 )           /* Pin 10 */

newpos = newpos | 0x40;

if ( status & 32 )           /* Pin 12 */

newpos = newpos | 0x20;

if ( status & 16 )           /* Pin 13 */

newpos = newpos | 0x10;

if ( status & 8 )            /* Pin 15 */

newpos = newpos | 0x08;

/* Take bits from control */

if ( control & 2 )           /* Pin 14 */

newpos = newpos | 0x04;

if ( control & 4 )           /* Pin 16 */

newpos = newpos | 0x02;

if ( control & 1 )           /* Pin 1 */

newpos = newpos | 0x01;

return newpos;

}

void give_get( int pot, int stat)

{

data = data & (~0x20);    /* Clear bit 5 */

data = data & (~0x10);    /* Clear bit 4 */

data = data & (~0x08);    /* Clear bit 3 */

/*    1) Set the channel select switches (3 bits) */

_outp(DATA,data);

_sleep(4);

// display_data(data);

/* 2) Set the ALE (address latch enable) switch to HI (wait 50 ns)*/

data = data & (~0x10);    /* Clear bit 4 */

data = data | 0x08;       /* SET bit 3 */

_outp(DATA,data);

_sleep(1);                 /* 50 nanoseconds ....NOT CORRECT */

// display_data(data);

/*    3) Set the START switch to HI (wait 50 ns) */

data = data | 0x10;       /* SET bit 4 */

data = data | 0x08;       /* keep bit 3 HI */

_outp(DATA,data);

_sleep(1);                 /* 50 nanoseconds ....NOT CORRECT */

//display_data(data);

/*    4) Set the ALE switch back to LO (wait 50 ns) */

data = data | 0x10;       /* keep bit 4 HI */

data = data & (~0x08);    /* Clear bit 3 */

_outp(DATA,data);

_sleep(1);                 /* 50 nanoseconds ....NOT CORRECT */

// display_data(data);

/*    5) Set the START switch to LO (wait 200 microseconds) */

data = data & (~0x10);    /* Clear bit 4 */

data = data & (~0x08);    /* Clear bit 3 */

_outp(DATA,data);

_sleep(10);              /* wait 200 microseconds....NOT CORRECT */

data = data | 0x20;    /* Set bit 5 - Output Enable*/

_outp(DATA,data);

_sleep(4);

// display_data(data);

status = 0;

control = 0;

/*    6) ....begin reading data for 1 pot. */

status = _inp(STATUS);    /* Convert input to data */

control = _inp(CONTROL);

/* NEED A DELAY */

_sleep(2);

data = data & (~0x20);    /* Clear bit 5 */

_outp(DATA,data);

/*

*  Bit value:   7 6 5 4 3 2 1 0

*               x x x x x x x x

*

*  Note:  !## corresponds to INVERSE of ##

*

******( DATA )********************************************

*  Only 5 bits will be written to the data port.

*  They will control the selection of pot on the A/D

*  converter.

*

*  Bits 2 1 0 will be the only needed bits with

*  corresponding pin numbers of: 4, 3 & 2

*

*  Bits 4 3 will be used to control the START and ALE

*  functions of the A/D converter. In order to get a

*  reading off the A/D converter the following sequence of

*  events must occur:

*

*    1) Set the channel select switches (3 bits)

*    2) Set the ALE (address latch enable) switch to HI (wait 50 ns)

*    3) Set the START switch to HI (wait 50 ns)

*    4) Set the ALE switch back to LO (wait 50 ns)

*    5) Set the START switch to LO (wait 100 microseconds)

*    6) ....begin reading data for 1 pot.

*

******( STATUS )******************************************

*  From STATUS we read the following bits:

*               7 6 5 4 3

*    which corresponds to pins:( 11, !10, 12, 13, !15 )

*      ( 10 & 11 flipped )

*

*             ( Yellow, Orange, Green, Red, Purple )

*

******( CONTROL )*****************************************

*  From CONTROL we read the following bits:

*                         2 1 0

*    which corresponds to pins:( !14, 16, !17 )

*

*                           ( Yellow, Orange, Green )

*

*/

if ( stat == 0 ) /* Initialize all ZERO values */

{

if ( pot == 0 )

T_tip_ZERO =   posit();

else if ( pot == 1 )

T_mid_ZERO =    posit();

else if ( pot == 2 )

T_joint_ZERO =  posit();

else if ( pot == 3 )

F4_tip_ZERO =   posit();

else if ( pot == 4 )

F4_mid_ZERO =   posit();

else if ( pot == 5 )

F4_joint_ZERO = posit();

else

printf("\n (give_get-1):Incorrect Potentionmeter was specified.”);

}

else if ( stat == 1 )     /* Already initialized...read normally */

{

if ( pot == 0 )

T_tip =    angle( T_tip_ZERO,    posit() );

else if ( pot == 1 )

T_mid =    angle( T_mid_ZERO,    posit() );

else if ( pot == 2 )

T_joint =  angle( T_joint_ZERO,  posit() );

else if ( pot == 3 )

F4_tip =   angle( F4_tip_ZERO,   posit() );

else if ( pot == 4 )

F4_mid =   angle( F4_mid_ZERO,   posit() );

else if ( pot == 5 )

F4_joint = angle( F4_joint_ZERO, posit() );

else

printf("\n (give_get-2): Incorrect Potentionmeter was specified.”);

}

else

printf("\n (give_get-3): Incorrect status selection. ");

}

void read_pots(int stat)

{

/* Pot 0:  xxxxx000 */

data = data & (~0x04);

data = data & (~0x02);

data = data & (~0x01);

give_get(0,stat);

/* Pot 1:  xxxxx001 */

data = data & (~0x04);

data = data & (~0x02);

data = data | 0x01;

give_get(1,stat);

/* Pot 2:  xxxxx010 */

data = data & (~0x04);

data = data |   0x02;

data = data & (~0x01);

give_get(2,stat);

/* Pot 3:  xxxxx011 */

data = data & (~0x04);

data = data |   0x02;

data = data |   0x01;

give_get(3,stat);

/* Pot 4:  xxxxx100 */

data = data |   0x04;

data = data & (~0x02);

data = data & (~0x01);

give_get(4,stat);

/* Pot 5:  xxxxx101 */

data = data |   0x04;

data = data & (~0x02);

data = data |   0x01;

give_get(5,stat);

/* Pot 6:  xxxxx110   - NOT USED YET*/

/*

data = data |   0x04;

data = data |   0x02;

data = data & (~0x01);

*/

/* Pot 7:  xxxxx111   - NOT USED YET*/

/*

data = data |   0x04;

data = data |   0x02;

data = data |   0x01;

*/

}

int initialize_hand(void)

{

int choice = 0, i = 0;

/* Output introduction and instructions. */

printf("\n***************************************"

"\n*                                     *"

"\n*   Robotic Hand                      *"

"\n*                                     *"

"\n*  by Mariusz Zaczek & Aaron Trask    *"

"\n*                                     *"

"\n* (ADSL) Advanced Digital Systems Lab *"

"\n*             Fall 1999               *"

"\n*                                     *"

"\n***************************************" );

printf("\n Please lay your hand on a flat surface"

"\n and hit the ENTER key to initialize   "

"\n all angles to zero.\n");

while(TRUE)

{

choice = getkey();

if ( choice == 13 )     /*     ENTER - Lock location     */

{

/* Read in values of all potentionmeters */

read_pots(0);

/* Done reading all pots...begin motion */

printf("\n POSITIONS LOCKED - Begin Motion \n");

return 1;

}

}

}

void idle(void)

{

read_pots(1);

glutPostRedisplay();

}

void init(void)

{

float light_position[]   = {0.0, 3.0, 3.0, 0.0};

float local_view[] = {0.0};

GLfloat ambient[]    = { 0.0, 0.0, 0.0, 1.0 };

GLfloat diffuse[]    = { 0.7, 0.0, 0.0, 1.0 };

GLfloat specular[]   = { 1.0, 1.0, 1.0, 1.0 };

glEnable(GL_DEPTH_TEST);

glDepthFunc(GL_LESS);

glLightfv(GL_LIGHT0, GL_POSITION, light_position);

glLightModelfv(GL_LIGHT_MODEL_LOCAL_VIEWER, local_view);

glFrontFace(GL_CW);

glEnable(GL_LIGHTING);

glEnable(GL_LIGHT0);

glEnable(GL_AUTO_NORMAL);

glEnable(GL_NORMALIZE);

glMaterialfv(GL_FRONT, GL_AMBIENT, ambient);

glMaterialfv(GL_FRONT, GL_DIFFUSE, diffuse);

glMaterialfv(GL_FRONT, GL_SPECULAR, specular);

glMaterialf(GL_FRONT, GL_SHININESS, 128.0);

glClearColor(0.5, 0.5, 0.5, 1.0);

glColor3f(1.0, 1.0, 1.0);

}

void display(void)

{

GLUquadricObj *cone;

glClear(GL_COLOR_BUFFER_BIT|GL_DEPTH_BUFFER_BIT);

glLoadIdentity();

/* Set pot values to the corresponding values */

Cthumb = -T_tip;

Bthumb = -T_mid;         /* Thumb */

Athumb = -T_joint;

Cfourth = -F4_tip;

Bfourth = -F4_mid;        /* First finger...Index finger */

Afourth = -F4_joint;

gluLookAt(120.0, 120.0, -120.0,

0.0,   18.0,   15.0,

1.0,    0.0,    0.0);

glRotatef(position,1.0,0.0,0.0);

glColor3f(0.0,0.0,0.0);

cone = gluNewQuadric();

glPushMatrix();

{

glTranslatef(0.0, 0.0, 0.0);

glRotatef( (GLfloat) wrist, 0.0, 1.0, 0.0);

glPushMatrix();

{

glTranslatef(0.0,0.0,-15.0);

glScalef(0.25,1.0,1.0);

gluCylinder(cone, 21.0, 19.0, 30.0, 20, 20);

}

glPopMatrix();

/* First Finger */

glTranslatef(0.0,-13.5,15.0);

glPushMatrix();

{

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Afirst, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 4.0, 20, 20);

glTranslatef(0.0,0.0,4.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Bfirst, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 9.0, 20, 20);

glTranslatef(0.0,0.0,9.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Cfirst, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 8.0, 20, 20);

glTranslatef(0.0,0.0,8.0);

glutSolidSphere(3.0,10,10);

}

glPopMatrix();

/* Second Finger */

glTranslatef(0.0,9.0,0.0);

glPushMatrix();

{

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Asecond, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 12.0, 20, 20);

glTranslatef(0.0,0.0,12.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Bsecond, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 12.0, 20, 20);

glTranslatef(0.0,0.0,12.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Csecond, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 8.0, 20, 20);

glTranslatef(0.0,0.0,8.0);

glutSolidSphere(3.0,10,10);

}

glPopMatrix();

/* Third Finger */

glTranslatef(0.0,9.0,0.0);

glPushMatrix();

{

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Athird, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 14.0, 20, 20);

glTranslatef(0.0,0.0,14.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Bthird, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 12.0, 20, 20);

glTranslatef(0.0,0.0,12.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Cthird, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 9.0, 20, 20);

glTranslatef(0.0,0.0,9.0);

glutSolidSphere(3.0,10,10);

}

glPopMatrix();

/* Fourth Finger */

glTranslatef(0.0,9.0,0.0);

glPushMatrix();

{

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Afourth, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 12.0, 20, 20);

glTranslatef(0.0,0.0,12.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Bfourth, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 9.0, 20, 20);

glTranslatef(0.0,0.0,9.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Cfourth, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 10.0, 20, 20);

glTranslatef(0.0,0.0,10.0);

glutSolidSphere(3.0,10,10);

}

glPopMatrix();

/* THUMB */

glTranslatef(0.0,4.5,-15.0);

glPushMatrix();

glRotatef(180, 1.0, 0.0, 0.0);

glPushMatrix();

{

glRotatef(90.0, 1.0,0.0,0.0);

glRotatef( (GLfloat) Athumb, 0.0, 1.0, 0.0);

glutSolidSphere(3.0,10,10);

gluCylinder(cone, 3.0, 3.0, 5.0, 20, 20);

glTranslatef(0.0,0.0,5.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Bthumb, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 9.0, 20, 20);

glTranslatef(0.0,0.0,9.0);

glutSolidSphere(3.0,10,10);

glRotatef( (GLfloat) Cthumb, 0.0, 1.0, 0.0);

gluCylinder(cone, 3.0, 3.0, 6.0, 20, 20);

glTranslatef(0.0,0.0,6.0);

glutSolidSphere(3.0,10,10);

}

glPopMatrix();

glPopMatrix();

}

glPopMatrix();

glutSwapBuffers();

}

void reshape (int w, int h)

{

glViewport (0, 0, (GLsizei) w, (GLsizei) h);

glMatrixMode (GL_PROJECTION);

glLoadIdentity ();

glFrustum(-14.0, 14.0, -14.0, 14.0, 30.0, 400.0);

glMatrixMode(GL_MODELVIEW);

glLoadIdentity();

glTranslatef (0.0, 0.0, -50.0);

}

void keyboard (unsigned char key, int x, int y)

{

switch (key) {

case 'a':

Afirst = (Afirst + 5) % 360;           break;

case 's':

Bfirst = (Bfirst + 5) % 360;           break;

case 'd':

Cfirst = (Cfirst + 5) % 360;           break;

case 'f':

Asecond = (Asecond + 5) % 360;         break;

case 'g':

Bsecond = (Bsecond + 5) % 360;         break;

case 'h':

Csecond = (Csecond + 5) % 360;         break;

case 'z':

Athird = (Athird + 5) % 360;           break;

case 'x':

Bthird = (Bthird + 5) % 360;           break;

case 'c':

Cthird = (Cthird + 5) % 360;           break;

case 'v':

Afourth = (Afourth + 5) % 360;         break;

case 'b':

Bfourth = (Bfourth + 5) % 360;         break;

case 'n':

Cfourth = (Cfourth + 5) % 360;         break;

case 'w':

wrist = (wrist + 10) % 360;            break;

case 't':

Athumb = (Athumb - 10) % 360;          break;

case 'y':

Bthumb = (Bthumb - 10) % 360;          break;

case 'u':

Cthumb = (Cthumb - 10) % 360;          break;

case 'p':

position = (position + 10) % 360;      break;

case 'o':

updown += 1;                           break;

case 'i':

updown -= 1;                           break;

case 'q':

case 'Q':

exit(0);

break;

case 27:

exit(0);

break;

default:

break;

}

glutPostRedisplay();

}

int main(int argc, char** argv)

{

GLenum type;

glutInit(&argc, argv);

initialize_hand();

type = GLUT_RGB | GLUT_DEPTH;

type |= (doubleBuffer) ? GLUT_DOUBLE : GLUT_SINGLE;

glutInitDisplayMode(type);

glutInitWindowSize (500, 500);

glutInitWindowPosition (100, 100);

glutCreateWindow (argv[0]);

init();

glutDisplayFunc(display);

glutReshapeFunc(reshape);

glutKeyboardFunc(keyboard);

glutIdleFunc(idle);

glutMainLoop();

return 0;

}

APPENDIX B: MC68332 Code

***********************************************

*                                             *

*  finger_3.s                                 *

*                                             *

* -Author  : Mariusz Zaczek (zaczek@uiuc.edu) *

* -Date    : 04/24/2000                       *

*                                             *

***********************************************

opt	nolist

***********************************************

*                                             *

*                                             *

***********************************************

*

*  EQUATES

*

FUNCNUM_3       equ     $9999       * 12) PWM, 11) PWM, 10) PWM,  9) PWM

FUNCNUM_2       equ     $9999       *  7) PWM,  6) PWM,  5) PWM,  4) PWM

FUNCNUM_1       equ     $9999       *  3) PWM,  2) PWM,  1) PWM,  0) PWM

max_X           equ     $e8             * 222/256 +5 VDC

min_X           equ     $24             *  46  "   "

max_Y           equ     $e1             * 215  "   "

min_Y           equ     $1d             *  39  "   "

Servo_Period    equ     $28f6       * 20 ms

Servo_Max       equ     $037b       * 1.7 ms

Servo_Min       equ     $016f       * 0.7 ms

Servo_Mid       equ     $029d       * Mid value ... 1.2 ms

Servo_Diff      equ     $020c       * Diff between max & min = 1.0ms

* Finger #1 - Joint 0a

CHANNEL_0       equ     $FFFF00      * CH 0 - PWM (control reg)

HIGH_TIME_0     equ     $FFFF04      * ( high time )

PERIOD_0        equ     $FFFF06      * ( period )

* Finger #1 - Joint 0b

CHANNEL_1       equ     $FFFF10      * CH 1 - PWM (control reg)

HIGH_TIME_1     equ     $FFFF14      * ( high time )

PERIOD_1        equ     $FFFF16      * ( period )

* Finger #1 - Joint 1

CHANNEL_2       equ     $FFFF20      * CH 2 - PWM (control reg)

HIGH_TIME_2     equ     $FFFF24      * ( high time )

PERIOD_2        equ     $FFFF26      * ( period )

* Finger #1 - Joint 2

CHANNEL_3       equ     $FFFF30      * CH 3 - PWM (control reg)

HIGH_TIME_3     equ     $FFFF34      * ( high time )

PERIOD_3        equ     $FFFF36      * ( period )

* Finger #2 - Joint 0a

CHANNEL_4       equ     $FFFF40      * CH 4 - PWM (control reg)

HIGH_TIME_4     equ     $FFFF44      * ( high time )

PERIOD_4        equ     $FFFF46      * ( period )

* Finger #2 - Joint 0b

CHANNEL_5       equ     $FFFF50      * CH 5 - PWM (control reg)

HIGH_TIME_5     equ     $FFFF54      * ( high time )

PERIOD_5        equ     $FFFF56      * ( period )

* Finger #2 - Joint 1

CHANNEL_6       equ     $FFFF60      * CH 6 - PWM (control reg)

HIGH_TIME_6     equ     $FFFF64      * ( high time )

PERIOD_6        equ     $FFFF66      * ( period )

* Finger #2 - Joint 2

CHANNEL_7       equ     $FFFF70      * CH 7 - PWM (control reg)

HIGH_TIME_7     equ     $FFFF74      * ( high time )

PERIOD_7        equ     $FFFF76      * ( period )

* Finger #3 - Joint 0a

CHANNEL_8       equ     $FFFF80      * CH 8 - PWM (control reg)

HIGH_TIME_8     equ     $FFFF84      * ( high time )

PERIOD_8        equ     $FFFF86      * ( period )

* Finger #3 - Joint 0b

CHANNEL_9       equ     $FFFF90      * CH 9 - PWM (control reg)

HIGH_TIME_9     equ     $FFFF94      * ( high time )

PERIOD_9        equ     $FFFF96      * ( period )

* Finger #3 - Joint 1

CHANNEL_10       equ     $FFFFA0     * CH 10 - PWM (control reg)

HIGH_TIME_10     equ     $FFFFA4     * ( high time )

PERIOD_10        equ     $FFFFA6     * ( period )

* Finger #3 - Joint 2

CHANNEL_11       equ     $FFFFB0     * CH 11 - PWM (control reg)

HIGH_TIME_11     equ     $FFFFB4     * ( high time )

PERIOD_11        equ     $FFFFB6     * ( period )

include equ332.asm

org	$9000

*        org	$e0000

* only when using own EEPROM chips

*        dc.l    $2ffc           ; Power on/Reset stack pointer

*        dc.l    Start           ; Power on/Reset program counter

*

*  Start

*

*    - This is where it all begins ...

*

Start

move.w	#$2700,SR

clr.b	SYPCR

* only when using own EEPROM chips

*       bra.l	csInit		* Initialize chip selects

csInitReturn

move.b	#$7F,SYNCR

* only when using own EEPROM chips

*       lea	bos,sp		* Point the stack pointer to bottom

jsr     TPUInit         * Initialize TPU

jsr	PortEInit	* Initialize Port E (Input)

jsr	PortFInit	* Initialize Port F (Input)

jsr	PortQSInit	* Initialize Port QS (Output) 7pins

clr.w	D1		* Start Pot counter at 0

*

*  Main_Loop

*

*     - Main program body.

*

Main_Loop

jsr 	CaptureData		* Get data from pots and i/o

jsr	CalculateData		* Calc Data and update servo

bra     Main_Loop		* Loop back ...

*

*  CaptureData

*

*    - Reads Port E for potentiometer input and updates PWM signals

*

CaptureData

* Start sequence for reading potentiometers

*   bits 3,2,1,0 will be for pot select (0000, 0001, 0010, etc.)

*   bit  4 will be for ALE

*   bit  5 will be for START

*   bit  6 will be for Output Enable

jsr	Delay1u

jsr	Delay1u

jsr	Delay1u

jsr	Delay1u

*

* Read next potentiometer ...

*

* Increase pot counter by 1

Counter

cmp.b	#%001000,D1

bge	ResetCounter

add.b	#1,D1

jmp	CounterEnd

ResetCounter

clr.w	D1

CounterEnd

* Check if pot counter is over the number of pots limit

*	If yes then start at 0

*	If no then continue on

andi.w  #%00001111,D1

move.b	D1,PORTQS	        * Clear Port QS, set Pot num.

jsr	Delay1u

andi.w  #%00011111,D1

move.b  D1,PORTQS	        * Set ALE bit

jsr	Delay1u

jsr	Delay1u

andi.w  #%00111111,D1

move.b	D1,PORTQS	        * Set START bit

jsr	Delay1u

jsr	Delay1u

andi.w  #%00101111,D1

move.b	D1,PORTQS	        * Clear ALE bit

jsr	Delay1u

jsr	Delay1u

andi.w  #%00001111,D1

move.b  D1,PORTQS	        * Clear START bit

jsr	Delay125u		* Delay 125microseconds

andi.w  #%01001111,D1

move.b	D1,PORTQS	        * Enable Output

jsr	Delay1u

jsr	Delay1u

move.b	PORTE0,D2		* Get A/D converter output

jsr	Delay1u

jsr	Delay1u

rts				* End ( CaptureData )

CalculateData

clr.l   D5              * Clear reg 5

move.w  #max_X,D5       * Set D5 to max X value

sub.w   #min_X,D5       * Calculate range between max & min

sub.w   #min_X,D2       * Subtract min value to get numerator

mulu.w  #$ff,D2

divu.w  D5,D2           * Scale value to 0-256

jsr	SetServo

rts				* End ( CalculateData )

*

*  SetServo

*

*     - Set the Servo to the new position as specified by Pot

*

SetServo

mulu.w  #Servo_Diff,D2

divu.w  #$00ff,D2

add.w   #Servo_Min,D2

* Test which servo to move ...

move.w	D1,D0

andi.w  #%00001111,D0

cmp.w   #%00000000,D0

beq	High_0

cmp.w	#%00000001,D0

beq	High_1

cmp.w   #%00000010,D0

beq	High_2

cmp.w	#%00000011,D0

beq	High_3

cmp.w   #%00000100,D0

beq	High_4

cmp.w	#%00000101,D0

beq	High_5

cmp.w   #%00000110,D0

beq	High_6

cmp.w	#%00000111,D0

beq	High_7

cmp.w	#%00001000,D0

beq	High_8

cmp.w   #%00001001,D0

beq	High_9

cmp.w	#%00001010,D0

beq	High_10

cmp.w	#%00001011,D0

beq	High_11

jmp	ServoEnd

High_0

move.w  D2,HIGH_TIME_0

jmp	SetServoEnd

High_1

move.w  D2,HIGH_TIME_1

jmp	SetServoEnd

High_2

move.w  D2,HIGH_TIME_2

jmp	SetServoEnd

High_3

move.w  D2,HIGH_TIME_3

jmp	SetServoEnd

High_4

move.w  D2,HIGH_TIME_4

jmp	SetServoEnd

High_5

move.w  D2,HIGH_TIME_5

jmp	SetServoEnd

High_6

move.w  D2,HIGH_TIME_6

jmp	SetServoEnd

High_7

move.w  D2,HIGH_TIME_7

jmp	SetServoEnd

High_8

move.w  D2,HIGH_TIME_8

jmp	SetServoEnd

High_9

move.w  D2,HIGH_TIME_9

jmp	SetServoEnd

High_10

move.w  D2,HIGH_TIME_10

jmp	SetServoEnd

High_11

move.w  D2,HIGH_TIME_11

SetServoEnd

rts				* End ( SetServo )

*

*  PortEInit

*

*     - Intializes all pins of Port E to input pins.

*

PortEInit

move.b  #$00,PORTE0     * Clear the data register

move.b  #$00,PEPAR      * Configure Port E as I/O

move.b  #$00,DDRE       * Configure Port E as input

rts			* End ( PortEInit )

*

*  PortQSInit

*

*    - Initialize Port QS to output (only 7 pins available)

*

PortQSInit

move.b	#$00,PORTQS	* Clear data register

move.b	#$80,PQSPAR	* 0-6 I/O, bit 7 TxD

move.b	#$ff,DDRQS	* Set all pins as Digital Output

rts			* End ( PortQSInit )

*

* TPUInit

*

*  - Initialize the TPU for channels 0-11 to be PWM

*

TPUInit

* Set channel 0-11 to be PWM

move.w  #FUNCNUM_1,(CFSR3).L

move.w  #FUNCNUM_2,(CFSR2).L

move.w  #FUNCNUM_3,(CFSR1).L

* Emu mode off, sup on, sysclk/32  ---> 1 TCR1 = 238ns*8

move.w  #$0080,(TPUMCR).L

* Set Host seq. reg. to NO PARITY

move.w  #$0000,(HSQR1).L

move.w	#$0000,(HSQR0).L

* Call _SetPWM_ function to ...

*  ... set the PWM periods and control Regs

jsr     SetPWM

* ($00a0 = %10100000)

move.w  #$aaaa,(HSRR1).L

move.w  #$00aa,(HSRR0).L

* ($00f0 = %11110000)

move.w  #$ffff,(CPR1).L

move.w  #$00ff,(CPR0).L

rts                              * End ( TPUInit )

*

*  SetPWM

*

*    - Sets the original control reg., periods and High times

*

SetPWM

clr.l   D0

clr.l   D1                      * Clears all pertinent registers

clr.l   D2

clr.l   D3

clr.l   D4

clr.l   D5

clr.l   D6

clr.l   D7

C Initialize Channels

move.w  #$0092,(CHANNEL_0).L         * PWM (channel 0)

move.w  #$0092,(CHANNEL_1).L         * PWM (channel 1)

move.w  #$0092,(CHANNEL_2).L         * PWM (channel 2)

move.w  #$0092,(CHANNEL_3).L         * PWM (channel 3)

move.w  #$0092,(CHANNEL_4).L         * PWM (channel 4)

move.w  #$0092,(CHANNEL_5).L         * PWM (channel 5)

move.w  #$0092,(CHANNEL_6).L         * PWM (channel 6)

move.w  #$0092,(CHANNEL_7).L         * PWM (channel 7)

move.w  #$0092,(CHANNEL_8).L         * PWM (channel 8)

move.w  #$0092,(CHANNEL_9).L         * PWM (channel 9)

move.w  #$0092,(CHANNEL_10).L        * PWM (channel 10)

move.w  #$0092,(CHANNEL_11).L        * PWM (channel 11)

C Set Current position of servo

move.w  #Servo_Mid,(HIGH_TIME_0).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_1).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_2).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_3).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_4).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_5).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_6).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_7).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_8).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_9).L   * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_10).L  * 50% High Time

move.w  #Servo_Mid,(HIGH_TIME_11).L  * 50% High Time

C Define period of servo

move.w  #Servo_Period,(PERIOD_0).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_1).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_2).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_3).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_4).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_5).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_6).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_7).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_8).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_9).L   * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_10).L  * PERIOD = 20 ms

move.w  #Servo_Period,(PERIOD_11).L  * PERIOD = 20 ms

rts                             * End ( SetPWM )

*

*  csInit - code to initialize the chips (from jake's buggy code)

*

csInit

move.w  #$0E04,CSBARBT  * Initialize EPROM chip select

move.w  #$68b0,CSORBT

move.w  #$0003,CSBAR0   * Initialize RAM write (MSB)

move.w  #$503e,CSOR0

move.w  #$0003,CSBAR1   * Initialize RAM write (LSB)

move.w  #$303e,CSOR1

move.w  #$0003,CSBAR2   * Initialize RAM read (MSB and LSB)

move.w  #$683e,CSOR2

bra.l   csInitReturn    * Jump back to the startup sequence

*

* Delay 125 microseconds

*

Delay125u

clr.l	D5

Delay125uMid

cmp.l	#5000,D5

bge	Delay125uEnd

nop

add.l	#1,D5

jmp	Delay125uMid

Delay125uEnd

rts

*

* Delay 1 microseconds

*

Delay1u

clr.l	D5

Delay1uMid

cmp.l	#400,D5

bge	Delay1uEnd

nop

add.l	#1,D5

jmp	Delay1uMid

Delay1uEnd

rts

* Stack Section

org     $a000

stack           ds.l    32

bos