-----BEGIN PGP SIGNED MESSAGE-----

EuroCry -- A EuroCrypt Smartcard Emulator for DOS

=================================================

1996-04-04  Marcus Better <Marcus.Better@abc.se>

Overview

========

This program emulates a smartcard used to decode satellite TV broadcasts

encrypted with the EuroCrypt-M system. It is used together with a D2MAC

satellite receiver/decoder and a simple "season-type" interface which

connects the decoder to a serial port on the PC.

The EuroCry program is distributed as C source code only, and compiles with

Borland C 3.1. Other compilers may require small modifications. Rather than

being a full-featured program, EuroCry is intended to be an example that can

be studied by anyone who wants to learn about the EuroCrypt system. For this

reason, there are no actual DES keys included in this program. There are many

similar programs around with nice user interfaces and other features, but

without source code.

EuroCry is loosely based on the Season7 program written by Markus Kuhn

<mskuhn@cip.informatik.uni-erlangen.de>, and incorporates the Async serial

communications library by David Kessner <dkessner@vigra.com>.

EuroCry may be distributed freely. It may not be used commercially. If you

update or modify the program or base any other programs on it, you are

expected to make your work freely available to others.

The source code

===============

The program consists of the following files:

eurocry.c     The main program

decode.c      The decryption module

eurocry.h     Header file

decode.h      Header file

des.h         Header file containing tables for the DES algorithm

async.obj     The Async library

async.h       Header file for the Async library

There are a few #defines at the beginning of eurocry.c which may need to be

changed. These determine the serial port used and the baudrate of the decoder.

Also, the channel data in eurocry.h may be updated for other channels or keys.

The program can be compiled using the small memory model.

The algorithm

=============

I do not intend to give a detailed description of the protocol used between

the decoder and the PC, because the information I have is too limited. The

encryption algorithm used is based on the DES, the main difference being that

the initial and final permutations have been removed.

Disclaimer

==========

Use this software at your own risk. The author assumes no responsibility for

damages which may result from the use of this program.

Marcus Better

<Marcus.Better@abc.se>

-----BEGIN PGP SIGNATURE-----

Version: 2.6.3i

Charset: cp850

iQCVAgUBMW+d3/P+4fZCeHwBAQFmXwP+KnDVCLLQLNRK1mTUyZW7psjeeuQkfHay

Uun4AP8EruY5PC4M18cw4Fh0uTfKIgK73c8fXn2H9OOFPafNd5FoKC27NpWqLYpZ

iqz1ZDBPX3DTQDf9vJmq+QBRrW6Os3uIt5sA12r1UftVEcWy1eMeN18z5e0M9aKz

fkMAmf4TyLI=

=MuD2

-----END PGP SIGNATURE-----