*****************************************

*                                       *

*           Documentation for           *

*                                       *

*               WinIO.DLL               *

*                                       *

*              Version 1.4              *

*                                       *

*****************************************

WinIO V1.4   (c) Copyright 94/95  Vincent Himpe

All rights reserved.    Released as Public Domain /Shareware

What is WinIO.

______________

WinIO is a DLL or Dynamic Linkable Library for MS-Windows Version 3.0 and later.

It allows you to perform very low level i/o functions which windows locks out

under normal circumstances.

Most programming languages for Windows will not let you read or write physical

ports of your machine directly.(Pascal/Visual Basic)

This DLL gives any programming language the capability of performing these IO

actions.

Why ?

_____

WinIO was originally created because somebody needed it (I did).

I wanted to control an electronic circuit connected to the printerport.

Therefore i needed a way to send some data to the port and read back data from

the port.This can easily be done from within DOS but not from within Windows.

After experimenting with several tricks (eg run a dos program from within

windows that saves data to file and then read this file from within windows) i

decided there was only one neat solution : a DLL.

Description of the functions.

_____________________________

INP  (ADRESS)      reads a byte (8 bit) from port ADRESS

OUT  ADRESS,DATA   writes a byte DATA to port ADRESS

INPW (ADRESS)      reads a word (16 bit) from port ADRESS

OUTW ADRESS,DATA   write a word (16 bit) to port ADRESS

IDENTIOVER         gives you the version of the DLL (curent : 1.4)

INP  Reads a byte

example :   A = INP (889)

A = INP (&H379)

both lines read a byte from the same adress.

The first line uses decimal notation.

The second uses hexadecimal notation.

The resulting value will be put in variable A in decimal form.

Note : reading port &h 379 gives you the status of the printer on

LPT1.

OUT   Writes A Byte

example      OUT &H378,65

this sends a 65 to port &h378 which is the printer port LPT1.

65 will represent a capital a ( A ) on the printer.

Just as with INP it does not matter what form the numbers are in.

They can be DECIMAL or hexadecimal or even octal or binary provided

the language you are using can handle these systems.

INPW (input Wide) reads a word

example:    A = INPW(&H379)

This will read the word on port 889.Since the printerport only uses

the lower 8 bit (low Byte) only the these 8 bits will be of meaning.

The high Byte or upper 8bit will be meaningless and could contain

anything.Therefore you can expect INP (&h379) and INPW (&h379)

to give completely different readings.

OUTW (output wide) writes a word

example:     OUTW &H378,65

this will send also character 65 (capital a) to the printer, but it

could as well screw up your computer.(it will not damage the computer

but it could be needed to reboot).

This is due to the fact that you are actually sending 0065.

Where 65 goes to the 8 bit in use by the printerport.and the 00

goes to the upper 8 bits or the high byte.In this case we were sending

it to the printerport so nothing should happen,

but you never know if some manufacturer suddenly uses these bytes for

other purposes.(I know of one card that uses these techniques.My Video

Card !.The High byte of COM2 which , is never used, serves as the

top adress lines to my video memory. (4Mbyte on board).Usually this is

not used since all communication goes trough the video processor.But

if you want to grab the videomemory it works faster trough I/O ports)

Therefore use these WIDE commands only on your own cards or on cards of

which you know how they work.

Keep in mind that , when using these routines , you are not allowed to use :

numbers below 0

numbers above 65535 for the target adress .

numbers above 255 for the data in the OUT routine

numbers above 65535 for the data in OUTW routine

Actually the DLL will accept them but will 'rollover'.What i mean is :

If you send 65537 as an adress the DLL will actually write to Adress 1

65537 in Binary looks like this :          10000000000000001

since only the last 16 bits are used    ��������������������

you will write to adress 0000000000000001 which equals to 1

As demonstration there is a little windows program included that monitors the

2 printerports continuously and displays what is happening on the ports.

The program is written in Visual Basic 3.0 but could be run under 2.0 or even

1.0. I included the sourcecode so you can modify it as you want.

Note ! the program needs VBRUN300.DLL to run !

Using the DLL from within Visual Basic

Select FILE and then ADD FILE

The system will ask you for a file to add

enter there WINIO.BAS.

This file includes the directives for VB on how to use the DLL and where it

can be found.

this is what is contained in WINIO.BAS

Declare Function identiover Lib "WinIO.DLL" () As Integer

Declare Sub out Lib "WinIO.DLL" (ByVal addr%, ByVal byte%)

Declare Function inp Lib "WinIO.DLL" (ByVal addr%) as Integer

Declare Sub outw Lib "WinIO.DLL" (ByVal addr%, ByVal byte%)

Declare Function inpw Lib "WinIO.DLL" (ByVal addr%) As Integer

Best is that you edit it and insert the location of the DLL into this file.

I put it in my windows main directory (d:\win31) in a subdirectory winio

so i have to change the file like this:

������������������������

Declare Function identiover Lib "D:\WIN31\WINIO\WinIO.DLL" () As Integer

Declare ....                     ������������������������

.

.

the first is an identify routine which returns the version of the DLL

To test the communication between the dll and your application you could write

the following program.

-  Draw a label and a command on an empty form.

-  Load the WINIO.BAS as new file ( described somewhere above )

-  double click the command button , a window opens showing the code attached

to the button

- make the screen look like this

sub command1.click

a=identiover

label1.caption = a

end sub

- close the window and run the program

- when you click the command button the label should show 14.

- Now you can write your own programs that do I/O

Should you have questions or hints you can always contact me

via E-Mail :

internet   :  vincent.himpe@ping.be

Copyright notice and users licence.:

WinIO.DLL  (c)1994-1995 Vincent Himpe All rights reserved.

You are allowed to distribute the DLL provided that you do not modify it in

any way.If you distribute it you must make sure that the

distribution media contains following files.

- WinIO.DLL   : The DLL itself

- WinIO.DOC   : This File in unmodified form

- WinIO.BAS   : Include file for Visual Basic

- PMON.EXE    : Printmonitor sample program

- PMON.BAS    : Printmonitor Source in Visual Basic

- PMON.FRM    : Printmonitor form

- PMON.MAK    : Printmonitor project file for VB

- PMONABT.FRM : About form for PMON

- HIT.EXE     : Hardware I/o Terminal Version 1.0

- HIT.BAS     : Sourcecode for HiT

- HIT.FRM     : HiT Main form

- HIT.MAK     : Project file for HiT

- ABOUTHIT.FRM: About from for HiT

- HIT.DOC     : Textfile describing HiT

You are allowed to use the DLL for private purposes only.

You are NOT allowed to sell the DLL , neither alone or as part of a program

you wrote.If you write a commercial program that will use this DLL then

you owe me money !.

As a non-commercial user you can use the DLL for free.

However i would like to ask you to send me the registration form or an

e-mail message.( you do not have to send any money , to know that my DLL

is beeing used somewhere out there is enough to keep me satisfied )

If you write software that uses the DLL, and you want to sell this software

you MUST register and obtain a distribution licence.

Distribution:

This may be distributed without further notice.Any media

(floppy,tape,modem,cd) is allowed. This package(WINIO.ZIP) may be

included in commercial collections like the Simtel CD-roms or Cica

CD-roms without written agreement.

Registration Form for WinIO.DLL

Name   :___________________________________________

Adress :________________________________________________ Nr: ____________

City   :________________________________________________ Zip :___________

State  :___________________________ Country :____________________________

( ) Uses the WinIO DLL Version 1.4 for non-commercial purposes only

Orders :

( ) Distribution licence               rated at    50$

Comments :

......................................................................

......................................................................

......................................................................

......................................................................

.....................................................................d

Send To :

By Snail mail :

Vincent Himpe

A.Detayelaan 12

8792 Desselgem

Belgium

Europe

By E-Mail :

vincent.himpe@ping.be

*************************************** WinIo Version 1.4 ******************************

*                                                                                      *

* Ms Windows 3.0 and 3.1 DLL to perform low level port I/O                             *

* Written by  : Vincent Himpe                                                          *

* Last update : 12/14/1994                                                             *

* DLL Generated using Turbo Pascal V1.0 For Windows                                    *

* Low Level routines written in Inline 80286 Assembler                                 *

* To be distributed as Shareware                                                       *

*                                                                                      *

****************************************************************************************

History

Version 1.0      : basic I/O  INP and OUT only

Version 1.1      : Never made it

Version 1.2      : the 2 stands for Double (16 bit)

INPW and OUTW are added

also the IDENTioVER function is added

Version 1.3      : IDENTioREG added (registratioon keyword

modified WIDE operations (type BYTE and WORD used instead of

integers

Version 1.4      : Calling convention changed

adresses are passed as long integers

data is returned as integer (8bit) or longinteger(16bit)