HH       HH   TTTTTTTTTTTT

HH       HH   TTTTTTTTTTTT

HH       HH        TT

HH       HH   ii   TT

HH       HH        TT

HHHHHHHHHHH  iii   TT

HHHHHHHHHHH   ii   TT

HH       HH   ii   TT

HH       HH   ii   TT

HH       HH   ii   TT

HH       HH   ii   TT

HH       HH iiiiii TT

Hardware Interface Terminal

Version 1.0

(c)1996 Vincent Himpe

All rights reserved

Released to Public domain

Disclaimer:

Use at own risk.

I take no responsability

whatsoever for the results

of this program.

Welcome to HiT. A CoolTool !.

This tool allowes programmers and hardware developers to monitor card

activity on /O ports from within windows.

The prgram may be started a number of times.Doing so lots of different

ports can be read and written (both 8 and 16 bit ports).

The program makes use of the WinIO.DLL.

This DLL gives programming language like turbo pascal and Visual Basic the

ability to do low level I/O again.

It restores the missing INP and OUT command to the Visual basic

Read WINIO.DOC for more information.

Since this program uses WinIO.DLL it comes shipped with the WinIO.DLL .

This means that you can use the DLL and distribute it provided you don't

charge money for it (except for the media on which you distribute it)

You are not allowed to sell the HiT program nor the WinIO.DLL neither as is

or as part of a commercial package.HiT is and will always be Public Domain.

As an individual or an educational institution you are allowed to use it

for free.

When you want to use the WinIO.DLL in commercialised programs then you must

obtain a distribution licence .(See WinIO.DOC for further details.)

However , if you use the program i would like you to send me an e-mail with

your name and adress. Knowing that it is beeing used somewhere out there

is rewarding enough for me.

Okay. Let's see... Well start it !.

The window is split in 2 portions. The Adress and data section.

You can select a number of standard adresses with the list box on the left

side.

If you select user there then you can type in your own adress on the

textbox below the list box.

The button 'Immediate' does the following:

When activated the data you enter in the data field is beeing transitted on

the spot.

When disactivated you must press the GO button to transmit the data.

On the menu there are 4 function

Quit : quits the program

Decimal/Hexadecimal : toggles the readout mode

8 Bit/16 bit : toggles bus width.

About : Ego screen :-)

Click on the lower row of selection boxes to code the bits.

The upper row of selection boxes shows the current data on the adress

selected.

.

That's it.

Enjoy

Vincent Himpe

Alfred De Taeyelaan 12

8792 Desselgem

Belgium

e-mail : vincent.Himpe@ping.be

http://www.ping.be/~ping0751