MODBIN

USER'S GUIDE

V4.50.58

Documentation Revision 1.5

(C) Copyright 1996 Award Software, International Inc.

All Rights Reserved

All trademarks are the property of their respective owners

Europe	U.S.A	Asia

Award Software, Inc.	Award Software, Inc.	Award Software, Inc.

Elsenheimerstr. 50	777 E. Middlefield Road	9F, No. 17, Sec. 1

80687 Munchen, Germany	Mountain View, CA 94043	Cheng Te Road

Taipei, Taiwan

Tel: 49-89-575750	Tel: 415-968-4433	Tel: 886-2-555-0880

Fax: 49-89-575998	Fax: 415-968-0274	Fax: 886-2-555-4420

Notice

The information in this guide is subject to change without notice.

Award Software ,International  Inc. shall not be liable for technical or editorial omissions made herein; nor for incidental or consequential damages resulting from the furnishing, performance, or use of this material.

This guide contains information protected by copyright.  No part of this guide may be photocopied or reproduced in any form without prior written consent from Award Software, International Inc.

Introduction

MODBIN is an executable program that offers many customization features for Award BIOS for IBM Personal Computer and compatible systems.  MODBIN is a menu driven program that will take you menu by menu through the customization process.  In addition to the PCI Configuration Display function (new function), the current Modbin Utility version 4.50.58 can also support 2MB BIOS ROM size.

Remark: Users are unable to make any changes under the condition that users have already requested us before to set the PCI Display function to Disable always in the previous BIOSes.

Before You Begin

The MODBIN diskette contains MODBIN.EXE, which is an executable file.  To use MODBIN you must have available MODBIN.EXE and a binary (.BIN) file or a mainboard on which binary code version is later than 4.5x and is contained in a ROM.

Menu Driven MODBIN

To enter MODBIN, type "MODBIN" at the DOS prompt.  The Main Menu will be displayed.  Figure 1 shows an example of the Main Menu.

Figure 1  Main Menu

| MODBIN (c) Award Software 1993-1996 V . 4.50 . 58 |  |  |
| --- | --- | --- |
| Update File |  |  |
| Load File |  | Directory : |
| Change BIOS Message |  | File : |
| Edit Drive Table |  |  |
| Load Drive Table |  |  |
| Save Drive Table |  |  |
| Change BIOS Options |  |  |
| Installed Option |  |  |
| Chipset Regs. Default |  |  |
| Chipset Setup Default |  |  |
| Auto Table (c) |  |  |
| Speed Pin Option |  |  |
| Reg. for Cyrix CPUs |  |  |
| Reg. for IBM CPUs |  |  |
| EISA option |  |  |
| Standard CMOS Default |  |  |
| BIOS timing table |  |  |
| POST Display Select |  |  |
| Additional Information |  |  |
| Power management |  | Press [ALT-L] to download binary code from F000:0000H |
| ¯-Move cursor ENTER-Update ESC-Abort TAB-Change item CTRL-A..PgUp PgDn |  |  |

Control Keys

Control keys are displayed at the bottom of the screen.

Use <> and <¯> cursor arrows or type the highlighted or underlined letter to move the highlighted bar around the MODBIN functions.

Press <Enter> to accept the selection.

Press <Esc> to abort the selection.  Press <Esc> again to exit MODBIN.

Press <TAB> to move around the items.

<PgUp> or <PgDn> will move to the previous or next page.

Press <F2> to show Award BIOS CMOS setup to let you see if the items modified by MODBIN have also been changed in CMOS setup.  Note that if you make any CMOS changes directly in setup at this time, the changes will not be saved.

MODBIN functions

MODBIN functions are displayed at the left side of the Main Menu.

Update File

Choosing this option will write all the modified data out to the output file and then exit the program.  The output file can be a new file or an existing file.  To update file, select the Update File option and press <Enter>.  Use the <> and <¯> keys to choose from the file names listed by MODBIN, or use <TAB> key to move the highlighted bar to "Directory" and "File", type the output directory and file name, then press <Enter>.  If you are at the Main Menu screen, and you wish to leave without updating the .BIN file, press the <Esc> key.  The MODBIN program will ask you if you want to abort the edit.  Type "Y" and it will put you back to the DOS command line.  If you are not sure or decide not to exit, type "N".

Load File

This option will load a desired .BIN file before you use other MODBIN functions for customization.  When you enter MODBIN, the highlighted bar will highlight Load File option.  Press <Enter>, and MODBIN will automatically search for the .BIN files in the disk drive where MODBIN exists and list them at the right side of Main Menu screen.  Use the <> and <¯> arrow keys to go to the desired .BIN file and press <Enter>.  If the desired .BIN file is not in the disk drive MODBIN automatically searches.  Use <TAB> key to move the highlighted bar to "Directory" and "File".  Type the correct drive and file name, then press <Enter>.  If the desired .BIN file has been successfully loaded, a brief description of the file and BIOS message will be displayed.  Figure 2 shows an example screen of a loaded .BIN file.

If you wish to load binary code from ROM directly, press <Alt> and L at the

same time.

Selecting this option will display the following options menu:

Figure 2  Loaded .BIN File

| 19 /09 /96 - i430HX - 2A59FE39C - 00 MODBIN (c) Award Software 1993-1996 V . 4.50 . 58 |  |
| --- | --- |
| Update File |  |
| Load File |  |
| Change BIOS Message |  |
| Edit Drive Table | File name : C : \ 2A59FE39.BIN |
| Load Drive Table |  |
| Save Drive Table |  |
| Change BIOS Options |  |
| Installed Option |  |
| Chipset Regs. Default |  |
| Chipset Setup Default |  |
| Auto Table (c) |  |
| Speed Pin Option |  |
| Reg. for Cyrix CPUs |  |
| Reg. for IBM CPUs |  |
| EISA option |  |
| Standard CMOS Default |  |
| BIOS timing table |  |
| POST Display Select |  |
| Additional Information |  |
| Power management |  |
| F2-Show setup ¯-Move cursor ENTER-Accept ESC-Abort & Exit |  |

If the .BIN file is loaded successfully, then a message for BIOS information such as "19/09/96-i430HX-2A59FE39C-00" will be displayed at the top of the screen indicated as follows,

19/09/96 - BIOS make date

I430HX - Chipset name

2A59FE39C-00 - Award source code part number

When you call the Award office for technical service, please advise us of the BIOS information message.

Change BIOS Message

This option is used to change the BIOS message that is displayed when the system is powered on.  In MODBIN, the default message is displayed near the bottom of the Main Menu screen, above the Control Keys.

To change the message, select the Change BIOS Message field and press <Enter>.  The original message will be highlighted with the cursor stopped at the last character of the original message.  Type the message you require to display and then press <Enter>.  The message should not exceed 79 characters; if it does, the last characters will be ignored.  If you make an error, use the <¬> key to go back up and then retype the correct entry.

Edit Drive Table

The Edit Drive Table option allows you to change the default hard disk drive table.  To create your own drive type, select the Edit Drive Table option and press <Enter>.  The Drive Table screen, as Figure 3 shows, will be displayed at the right side of the screen:

Figure 3  Edit Drive Table

| 19 / 09 / 96 - i430HX - 2A59FE39C -00 MODBIN (c) Award Software 1993-1996 V . 4.50 . 58 |  |
| --- | --- |
| Update File |  |
| Load File | Type Cyldrs Heads Precomp Control Landzone Sectors |
| Change BIOS Message | 1 306 4 128 0 305 17 |
| Edit Drive Table | 2 615 4 300 0 615 17 |
| Load Drive Table | 3 615 6 300 0 615 17 |
| Save Drive Table | 4 940 8 512 0 940 17 |
| Change BIOS Options | 5 940 6 512 0 940 17 |
| Installed Option | 6 615 4 65535 0 615 17 |
| Chipset Regs. Default | 7 462 8 256 0 511 17 |
| Chipset Setup Default | 8 733 5 65535 0 733 17 |
| Auto Table (c) | 9 900 15 65535 0 901 17 |
| Speed Pin Option | 10 820 3 65535 0 820 17 |
| Reg. for Cyrix CPUs | 11 855 5 65535 0 855 17 |
| Reg. for IBM CPUs | 12 855 7 65535 0 855 17 |
| EISA option | 13 306 8 128 0 319 17 |
| Standard CMOS Default | 14 733 7 65535 0 733 17 |
| BIOS timing table | 16 612 4 0 0 663 17 |
| POST Display Select | 17 977 5 300 0 977 17 |
| Additional Information | 18 777 7 65535 0 977 17 |
| Power management |  |
| F2-Show setup ¯-Move cursor ENTER-Accept ESC-Abort & Exit |  |

Use the <> and <¯> arrow keys or <PgUp> and <PgDn> to move through the selections (1-14, 16-47) and select the drive table number that you want to change.  Refer to your BIOS CMOS Setup Guide for a complete list of the default drive types available.  If your drive type specification is not in this list, this option allows you to modify an existing drive type to conform to the specifications of your particular drive.

NOTE: If the BIOS used is Version 4.5x, drive table number is only up to 46.

Load Drive Table

This option is most useful for the OEMs which allows you to read in a text file to modify the current drive table.  The format of the file should be as follows:

Type Cylinders Heads Precomp Control Landzone Sectors

Each of the items should be separated by space and each line should be terminated with a carriage return.

NOTE: In order to enter the value "None" for the Precomp, enter -1 in the drive table file.

To load the file, select the Load Drive Table option and press <Enter>.  The screen will display the prompt,

Drive Table File Name:

Type the file name, including path and extension, and press <Enter>.

This option can be used together with Save Drive Table.

Save Drive Table

Choosing the option will write all the modified data in Drive Table out to the output text file to create a new drive table.  Select Save Drive Table option and press <Enter>.  Use <TAB> key to move the highlighted bar to "Directory" and "File", type the correct drive and file name, then press <Enter>.

This option is useful for the OEM manufacturers and is used together with Load Drive Table.

Change BIOS Options

The Change BIOS Options is used to change many of the BIOS features, including the floppy speed condition, security conditions and the Num-Lock status.

Num-Lock on 101 Keyboard

This item is only for BIOS Version 4.5x or before, it toggles the bit that determines if the Num-Lock will be on or off when computer boots up and a 101-key keyboard is detected.

Floppy Speed Switching

If Floppy Speed Switching is set to enabled, and when a floppy drive cannot handle high-speed access, this option will switch the speed to LOW during the access and then back to system default speed when it is done.

Security Default Password

Changes the default password from AWARD_SW to your own default password.  The password is limited to 8 characters.

NOTE: Even though you have changed the security default password and updated the .BIN file, MODBIN will not display your own default password when you enter the .BIN file the next time.  However, the default password was actually modified.

Security Number of Retry

Changes the retry number for typing the correct password after power on the system.  If the attempts to type the correct password have exceeded the Security Number of Retry, system will be automatically reset.

PIE Wait for FDD

Select the wait type for motor spin up when accessing floppy disk drive.   Default value of this option is disable.

Power On Delay Count

The delay time between Power-on to POST, from 0 CPU instruction to maximum 65335 CPU instructions.

Setup Default Color Value

From 00 to 15.  14 is color black and white.  The default color value will only be loaded when CMOS failed.

POST Default Color Value

From 00 to 15.  14 is color black and white.  The default color value will only be loaded when CMOS failed.

POST Color Option

2 selections, POST default and setup setting.  The color selected in this option will be loaded when CMOS is O.K.

POST default - POST color same as POST default

Setup setting - POST color same as setup setting

Update Revision Number :

For customization PROJECT, manufacturers can define the value by themselves.

PCI configuration display:

This option can let users set the PCI configuration display to Enable or

Disable.  Default value of this option is enable.  The showing display will be

shown after “system Configuration Table” as follows:

PCI device listing ……

| Bus No. Device No. Function No. Vender ID Device ID Device Class IRQ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 7 | 1 | 8086 | 7010 | IDE Controller | 14 |
| 0 | 10 | 0 | 10B9 | 3307 | Multimedia device | 11 |

(the above figures depend on the status of users’ cards and hardware)

If you do not need the PCI device shown on the screen, please set the function to disable.

To Set the function to disable, follow the below procedures:

- type “modbin”

- select “Change BIOS Options”

- move the cursor to “PCI Configuration Display:”

- set the function to “Disable” by pressing “_” or “+” key

- power on the computer again, you will find the PCI Configuration Display function is not shown on the screen before system booting.

In addition to the PCI Configuration Display function, the current ModBin Utility version 4.50.58 can also support 2MB BIOS ROM size.

Remarks: Users are unable to make any changes under the condition that users

have already requested us before to set the PCI Display function to

Disable always in the previous BIOSes.

Installed Option

Selecting this option will display the following Installed Options Menu, which shows all of the options that the BIOS will check for.

DRAM parity check

Enable or disable DRAM parity check.  Note that this item is only BIOS Version 4.5x or before.

Keylock checking

Enable or disable keylock check.

Onboard E000h video ROM scan

Enable or disable BIOS scanning for video at onboard E000h.

Onboard C000h video ROM scan

Enable or disable BIOS scanning for video at onboard C000h.

Manufacture loop checking

Installed to perform POST loop when keyboard controller manufacturing pin is active.

PS2 mouse support

Installed to perform PS2 mouse support.

Novell HDD work around

Installed to avoid hard disk failure for Novell server when the disk type is set to Type User.

NOTE: Available only when system shadow is enabled.

Chipset Reg. Default

This option allows you to edit the default value location of the chipset registers.  To edit the default, select Chipset Reg. Default option and press <Enter>.  Use arrow keys to move around "Data" field.  To change the value, directly type the number you want.

Chipset Setup Default

If the BIOS used is Version 4.5x, this option allows you to edit the setup items showed by Award BIOS CMOS Setup Utility.

You may select from total 5 pages of Award BIOS CMOS setup.  Furthermore, you may select the value of each setup function and the setup item or decide whether the setup function or the setup item being displayed on screen.  By this, the end-user can only see or change the CMOS setup from the items you selected.

For detail description of each setup function and item listed below, please refer to the user's guide of  Award BIOS CMOS Setup Utility Version 4.51PG.

| Control keys | Name | Description |
| --- | --- | --- |
| F1 | Help | Pop out the help window |
| F2 | Item name | Status of each option for setup items. If you do not wish the option to be selected by end-user, you can disable the option. For BIOS default and setup default only. |
| F3 | Posi modi | Auto arrange the position of selected setup items |
| ¯ ¬ ® |  | move the cursor up or down, left or right |
| PgUp/PgDn |  | move to previous page or next page |
| +- | Change value | change value of the setup item |
| Enter | Update | Update the value of the setup item |
| Esc | Abort | Abort the changes |

In addition, you may rename the selected setup item by directly typing the name you wish.

5 pages from the CMOS Setup Utility may be modified by MODBIN, they are,

| MODBIN | CMOS Setup Utility |
| --- | --- |
| Page 0 | Main Menu (CMOS SETUP UTILITY) |
| Page 1 | Standard CMOS Setup Menu |
| Page 2 | BIOS Features Setup Menu |
| Page 3 | Chipset Features Setup Menu |
| Page 4 | Power Management Setup Menu |
| Page 5 | PCI Configuration Setup Menu |

In each of the 5 pages you may modify, there will be 6 status reports listed at the right hand side.  They are,

| Status report | Description |
| --- | --- |
| Item status | If it is normal, the setup item will be displayed and can be changed by CMOS Setup Utility. If it is show only, the setup item will be displayed but can not be changed by CMOS Setup Utility. If is disabled, the setup item will not be displayed nor be changed by CMOS Setup Utility. |
| BIOS default | The value of BIOS default |
| Setup default | The value of setup default |
| X | The X axis position of the setup item which displayed in CMOS Setup Utility. LT indicates the left side of the screen, and RT indicates the right side of the screen. |
| Y | The Y axis position of the setup item which displayed in CMOS Setup Utility. The Y axis is indicated by row number. |

The setup items in Page 0 are,

STANDARD CMOS SETUP

BIOS FEATURES SETUP

CHIPSET FEATURES SETUP

POWER MANAGEMENT SETUP

PCI CONFIGURATION SETUP

LOAD BIOS DEFAULT

LOAD SETUP DEFAULT

PASSWORD SETTING

IDE HDD AUTO DETECTION

HDD LOW LEVEL FORMAT

SAVE & EXIT SETUP

EXIT WITHOUT SAVE

The setup items in Page 1 are,

Daylight saving

Primary Master

Primary Slave

Secondary Master

Secondary Slave

Drive A type

Drive B type

Video

Halt on

The setup items in Page 2 are,

Virus Warning

CPU Internal Cache

External Cache

Quick Power On Self Test

Boot Sequence

Swap Floppy Drive

Boot Up Floppy Seek

Boot Up NumLock Status

Boot Up System Speed

Gate A20 Option

Memory Parity Check

Typematic Rate Setting

Typematic Rate (Chars/Sec)

Typematic Delay (Msec)

Security Option

Video BIOS Shadow

C8000 - CFFFF Shadow

D0000 - D7FFF Shadow

D8000 - DFFFF Shadow

The setup items in Page 3 (will vary from chipset)

Auto-Configuration

ISA Bus Clock

LBD# Sample Point

Cache Write Cycle

Cache Burst Read Cycle

L2 Cache/DRAM Cycle WS

DRAM RAS to CAS Delay

DRAM Write Cycle

DRAM Write CAS Pulse

DRAM CAS Percharge Time

DRAM RAS to MA Delay

DRAM Speed

DRAM Slow Refresh

CPU Internal Cache

CPU Burst Write

L2 Cache Policy

L2 Cache Tag Bits

Onboard 496B IDE Port

Onboard VESA IDE-1 WS

Onboard VESA IDE-2 WS

Onboard 496B IDE Port

IDE 0 Master Mode

IDE 0 Slave Mode

IDE 1 Master Mode

IDE 1 Slave Mode

IDE Prefetch Read Buffer

IDE HDD Block Mode

Onboard FDD Controller

Onboard Serial Port 1

Onboard Serial Port 2

Onboard Parallel Port

Onboard Parallel Mode

Onboard Game Port

The setup items in Page 4 are

Power Management

PM Control by APM

Video Off Option

Video Off Method

Suspend Switch

Clock Down by SMOUT

HDD Off After

Doze Mode

Standby Mode

Suspend Mode

PCI Master Activity

COM Ports Activity

LPT Ports Activity

HDD Ports Activity

DMA Ports Activity

VGA Activity

IRQ 3 (COM 2)

IRQ 4 (COM 1)

IRQ 5 (LPT 2)

IRQ 6 (Floppy Disk)

IRQ 7 (LPT 1)

IRQ 8 (RTC Alarm)

IRQ 9 (IRQ 2 Redir)

IRQ10 (Reserved)

IRQ 11 (Reserved)

IRQ 12 (PS/2 Mouse)

IRQ 13 (Coprocessor)

IRQ 14 (Hard Disk)

IRQ 15 (Reserved)

The setup items in Page 5 are

PnP BIOS Auto-Config

Slot 1 Using INT#

Slot 2 Using INT#

Slot 3 Using INT#

Slot 4 Using INT#

1st Available IRQ

2nd Available IRQ

3rd Available IRQ

4th Available IRQ

PCI IRQ Actived By

PCI IDE 2nd Channel

PCI IDE IRQ Map To

Primary IDE INT#

Secondary IDE INT#

Master Arbitration Protocol

CPU->PCI Mem Post Write Buf

CPU->PCI Memory Burst Write

PCI Master Burst Read/Write

Auto Table (c)

If the BIOS you use is Version 4.5x, this option allows you to modify auto-programming chipset registers according to CPU speed, single bank of cache & double bank of cache.

Speed Pin Option

Selecting Speed Pin Option allows you to change keyboard controller status.

Speed Switching

Toggle the interpretation between speed pins being active high or

active low.

Polarity of output pins

Toggles the system speed.  When Speed Switching is set to Enable, you can select, based on the mainboard specification switch The speed output pin and power management output control pin of 8042 from the following pins,

Turbo					Green PC

Pin 23					  Pin 23

Pin 24					  Pin 24

Pin 27					  Pin 27

Pin 28					  Pin 28

Pin 29					  Pin 29

Pin 30					  Pin 30

Also, the pin 29 and 30 are set for PS/2 mode.

Keyboard controller type

The selections are Award, KB-200X ( KB-100, AMI, Regional JetKey-5), Phoenix, C&T, IBM standard ( JetKey), KB-200.

Program chipset for speed change

It allows you to program specific chipset if the speed has been changed.

ON/OFF cache for speed change

It allows you to change cache status if the speed has been changed.

Turbo switch input

Select turbo switch being input from keyboard.  If it is set to enable, turbo switch will be from keyboard controller input port.

Turbo switch 8042 pin

Select turbo switch being input from Pin 29 or Pin 30 once it is input from keyboard controller.

Turbo switch polarity

Select turbo switch hi active or low active being input once it is input from keyboard controller.

Reg. for Cyrix CPUs/Reg. for IBM CPUs

This option toggles the performance of CPU internal cache.  However, it depends on hardware design.

EISA Option

This option is available for EISA systems which allow you to set EISA manufacture ID and product ID.  These IDs can be detected and seen by EISA Configuration Utility.

EISA manufacture ID

The manufacture ID is limited to 3 characters from A to Z.

EISA product ID

The product ID is limited to 4 numbers from 0 to 9.

Standard CMOS Default

NOTE:  It does not function for BIOS Version 4.5x

BIOS Timing Table

This option shows the time table of peripheral input and output time out.  This option is designed for the convenience of manufacture test.

Refresh rate test factor

FDD motor spin up

FDD head settle

LPT initialize

Keyboard check

FDC IRQ6 completion

FDC status byte read

FDC command byte send

HDC busy check

HDD IRQ14 completion

HDC data request

COM port time out

LPT busy check

POST Display Select

NOTE:  It does not function for BIOS Version 4.5x

Additional Information

NOTE:  It does not function for BIOS Version 4.5x

Power management

Selecting this option will display the following options menu, which shows 			all of the options that the BIOS will check for.

Figure 4     Power Management

| 19 /09 /96 - i430HX - 2A59FE39C - 00 MODBIN (c) Award Software 1993-1996 V . 4.50 . 58 |  |
| --- | --- |
| Update File | Disable |
| Load File |  |
| Change BIOS Message | IDEX : E 1 E2 |
| Edit Drive Table | DATA : 00000000 00000000 |
| Load Drive Table |  |
| Save Drive Table |  |
| Change BIOS Options |  |
| Installed Option |  |
| Chipset Regs. Default |  |
| Chipset Setup Default |  |
| Auto Table (c) |  |
| Speed Pin Option |  |
| Reg. for Cyrix CPUs |  |
| Reg. for IBM CPUs |  |
| EISA option |  |
| Standard CMOS Default |  |
| BIOS timing table |  |
| POST Display Select |  |
| Additional Information |  |
| Power management |  |
| F2-Show setup ¯-Move cursor ENTER-Accept ESC-Abort & Exit TAB-Next group |  |

PCI Configuration

This option is available for PCI system which allow you to set PCI 			          configuration.

Figure 5    PCI configuration

| 19 /09 /95 - i430HX - 2A59FE39C - 00 MODBIN (c) Award Software 1993 V . 4.50 . 33 |  |
| --- | --- |
| Load File |  |
| Change BIOS Message | Device Number of PCI Slot |
| Edit Drive Table |  |
| Load Drive Table | PCI Slot 00 : 06 |
| Save Drive Table | PCI Slot 01 : 07 |
| Change BIOS Options | PCI Slot 02 : 08 |
| Installed Option | PCI Slot 03 : 09 |
| Chipset Regs. Default |  |
| Chipset Setup Default |  |
| Auto Table (c) |  |
| Speed Pin Option |  |
| Reg. for Cyrix CPUs |  |
| Reg. for IBM CPUs |  |
| EISA option |  |
| Standard CMOS Default |  |
| BIOS timing table |  |
| POST Display Select |  |
| Additional Information |  |
| Power management |  |
| PCI configuration |  |
| F2-Show setup ¯-Move cursor ENTER-Accept ESC-Abort & Exit |  |