A Linux IEEE 1394 Configuration ROM Decoder

Biography

Bill is a senior software engineer at LSI Logic.  His current responsibilities include developing 1394 device drivers, software, and utilities.  He can be contacted through http://www.geocities.com/billalexander.  The source to this utility is also available there.

Introduction

This is the third in a series of articles on decoding IEEE 1394 configuration ROMs.  The first article, IEEE 1394 Configuration ROM Decoder, discussed how to read and decode the configuration ROM of 1394 peripherals on Windows 95 using the TI TSBKPCI 1394 controller and the TI LynxSoft drivers.  The second article, A WDM IEEE 1394 Configuration ROM Decoder, was a port of the first into the Windows 98/2000 WDM architecture.  This article will discuss the implementation of the DumpRom program under the Linux operating system using an OHCI 1394 controller.

# IEEE 1394 and Linux

Currently Linux has no official 1394 device architecture.  However, there is an ongoing 1394 Linux project.  You can download the latest source at http://eclipt.uni-klu.ac.at/ieee1394.  The lack of an official interface creates a void that some programmers crave: a virtually untouched frontier in which to explore.  No driver stack to get in your way, no dogmatic architecture to limit possibilities and no pre-existing API to make life a little easier.  In short, roll your own driver and application.

# Quick Overview

The DumpRom utility for Linux consists of four files:

## File Name	Description

ohci.o	module driver that sets up the interface to the hardware

ohcitest	program that does most of the work

dodumprom	file to be redirected as standard input

dumprom	shell script to run the utility

The ohci.o file is a module driver that sets up the OHCI 1394 adapter resources, allocates system memory, and initializes the low-level interface.  The ohcitest file is a user-level program that loads ochi.o and then maps the OHCI register space and DMA space into process memory space in order to manipulate the hardware directly.  It also provides an interactive command interface for the user to perform certain operations on the hardware and the 1394 bus.  The dodumprom file is intended to be redirected as the standard input of ohcitest to perform the operations needed to decode the configuration ROM of device 0.  The dumprom file is a shell script that executes the program.

# The OHCI Module Driver

The ochi.o file is module driver that can be loaded and unloaded dynamically.  Under Linux, only the root user can load and unload modules using the insmod (install module) and rmmod (remove module) commands respectively.  Once ohci.o is loaded, Linux calls the init_module() routine which then calls device_init().  The main responsibilities of device_init() are as follows:

- Read the resources (i.e. vendor ID, device ID, IRQ number and memory base address) assigned to the card by PCI.

- Allocate DMA memory.

- Map DMA memory.

- Map the OHCI register memory.

- Setup the interrupt handler.

- Register the logical OHCI Register and OHCI DMA devices with the kernel as a character driver.

Because ohci.o is a “do nothing” driver, the most important service device_init() renders is the mapping the register and DMA memory spaces.  This allows an application to map the these spaces into it’s local memory and manipulate the OHCI registers and DMA structures directly.  The driver does not attempt to assert any control over the hardware.

Both of above mentioned memory spaces have a logical device wrapped around it.  When the ohci.o module registers with the kernel, it actually registers two logical devices.  The first logical device is the OHCI register device.  The second is the OHCI DMA device.  Through the use of the open() and mmap() functions, an application can call the driver to map these logical device memories into local memory space.

# The Ohcitest Program

The ohcitest application serves two functions.  First, it controls the OHCI adapter hardware.  It does this through the register and DMA memory spaces that are mapped from the driver to the local memory space.  To achieve this, ohcitest first loads the module ohci.o and then uses mknod to create “special files” \dev\ohci0 and \dev\ohci1.  Each of these files corresponds to the logical OHCI and DMA devices registered by the driver.  The special file \dev\ohci1 corresponds to the OHCI register driver and \dev\ohci1 corresponds to the OHCI DMA driver.  Ohcitest calls open() on each of these special files to open the interface to the logical devices.  Using the handles returned by open(), ochitest next calls mmap() to map the respective memory spaces into local space.

Ohcitest uses the register space to initiate 1394 packet transmissions and receptions.  The DMA memory is where the actual 1394 packets reside.  When a packet is to be transmitted, the program creates the packet in the DMA memory, points the transmit DMA register to it and then starts the transmit DMA.  When a packet is to be received, the program points the receive DMA register to DMA memory, starts the receive DMA and waits until a packet arrives.

After the memories have been mapped, ohcitest spawns a process to monitor the interrupt register for events to process.  EventProcess() is the main routine called by the child process.  EventProcess() is a loop that examines the OHCI IntEvent register.  If any bits are set, it processes the events one at a time.  After all events are processed, the loops starts all over.  After so many iterations through the loop without processing any events, the child process puts itself to sleep for a short time so that the main process can run.  This is important since the child process runs at a slightly higher priority than the main process.  Without putting itself to sleep, the child process would prevent the main process from running.

The second function that ohcitest provides is an interactive user interface.  After starting the child process, ohcitest displays a prompt for the user to manually input commands.  The commands that are needed to decode a configuration ROM are as follows:

- InitCard

- BusReset

- DumpRom <node number>

- Quit

The InitCard command causes the InitCard() routine to be called.  This routine resets the OHCI adapter and gets it ready for normal operation.  However, InitCard() does not enumerate the devices on the bus.  This is the job of the BusReset command.  When the BusReset command is issued, the BusReset() routine is executed.  BusReset() resets the 1394 bus and then waits for EventProcess() to receive the self ID packets generated by all of the nodes on the bus.  Once the “topology map” of all devices on the bus has been created, BusReset() returns.

Now that the OHCI adapter is ready, the DumpRom command can issued.  The node number of the target device is specified on the command line.  For example, the command “DumpRom 0” causes the configuration ROM of node 0 to be dumped and decoded.  The first thing the DumpRom command does is call ReadConfigRom().  This reads the entire configuration ROM from the specified device and places into a buffer.  Next, the DumpConfigRom() routine is called, which displays the contents of the buffer in raw hexadecimal quadlets.  Lastly, DumpRom calls DecodeConfigRom(), which decodes the buffer according to the 1394 specifications.

The Quit command exits the ohcitest program.  The OHCI board is reset, all structures are deallocated and the driver is unloaded.

# Running the Program

Before running the ohcitest program, you need to log in as root.  The reason for this is that ohcitest will first attempt to install the ohci.o module via the Linux insmod command.  Only the root user has privileges to run the insmod command.  Some long time veterans of UNIX and Linux will reel back in horror at the thought of a program calling root commands such as insmod, but this is a initial development tool intended for rapid changes in both the driver and the program.  If you start your 1394 development with this source, you will grow to appreciate the speed at which you can change and reload the driver using this method.

Once you are logged in as root, you can run the ohcitest program manually.  However, you must enter the InitCard, BusReset, DumpRom and Quit commands yourself.  To make things a little easier, I’ve provided a shell script which will automatically decode the configuration ROM of node 0.  The dumprom script is listed below:

./ohcitest < dodumprom

As you can see, dumprom redirects the standard input of ohcitest with the dodumprom file.  The dodumprom file is nothing more than the commands you need.  dodumprom is listed below:

initcard

busreset

dumprom 0

q

When the dumprom shell script is executed, the output is similar to the two previous versions of my utility.  If I plug in a Sony CCM-DS250 1394 camera and run the script, the results are as follows:

OHCI Register Memory Map = 40014800

OHCI DMA Memory Map = 40017000

OHCI DMA Memory Physical Address = 00E3A000

**************** EVENT OCCURRED *****************

Event = busReset

timeStamp = 000000A0

selfIDGeneration = 00000001

0 40017004 phy_ID 00, L 1, gap_cnt 3F, sp 1, del 0, c 0, pwr 6, p0 1, p1 1, p2 2

1 4001700C phy_ID 01, L 1, gap_cnt 00, sp 2, del 0, c 0, pwr 4, p0 3, p1 1, p2 1

**************** EVENT OCCURRED *****************

Event = phyRegRcvd

**************** EVENT OCCURRED *****************

Event = selfIDComplete

**************** EVENT OCCURRED *****************

Event = RSPkt

**************** EVENT OCCURRED *****************

Event = RSPkt

…

**************** EVENT OCCURRED *****************

Event = RSPkt

**************** EVENT OCCURRED *****************

Event = RSPkt

OHCI Register Memory Map = 40014800

OHCI DMA Memory Map = 40017000

OHCI DMA Memory Physical Address = 00E3A000

Raw Data Dump of the Configuration ROM

1394 Addr Off    Data

-------------    --------  --------  --------  --------

FFFF:F0000400    041F40AA  31333934  20FF4000  08004602

FFFF:F0000410    000003BD  0004C80A  03080046  0C0083C0

FFFF:F0000420    8D000002  D1000004  00029E8D  08004602

FFFF:F0000430    000003BD  0003D7FE  1200A02D  13000100

FFFF:F0000440    D4000001  00034FEA  403C0000  81000002

FFFF:F0000450    82000005  00033A64  00000000  00000000

FFFF:F0000460    534F4E59  0006010B  00000000  00000000

FFFF:F0000470    43434D2D  44533235  3020312E  30380000

Decode of the Configuration ROM

1394 Addr Off Quadlet  Meaning

------------- -------- --------------------------------------------------

Confiruation ROM Header

FFFF:F0000400 041F40AA info_length=04, crc_length=1F, rom_crc_value=40AA

Bus_Info_Block

FFFF:F0000404 31333934 bus_name=31333934 ("1394")

FFFF:F0000408 20FF4000 irmc=0, cmc=0, isc=1, bmc=0, cyc_clk_acc=FF, max_rec=4

FFFF:F000040C 08004602 node_vendor_id=080046, chip_id_hi=02

FFFF:F0000410 000003BD chip_id_lo=000003BD

Root_Directory

FFFF:F0000414 0004C80A length=0004, crc=C80A

FFFF:F0000418 03080046 Module_Vendor_Id 080046

FFFF:F000041C 0C0083C0 Node_Capabilities spt 64 fix lst drg

FFFF:F0000420 8D000002 Node_Unique_Id leaf ind_off=000002 (FFFF:F0000428)

FFFF:F0000424 D1000004 Unit_Directory directory ind_off=000004 (FFFF:F0000434)

Node_Unique_Id leaf referenced from FFFF:F0000420

FFFF:F0000428 00029E8D length=0002, crc=9E8D

FFFF:F000042C 08004602 ..F.

FFFF:F0000430 000003BD ....

Unit_Directory directory referenced from FFFF:F0000424

FFFF:F0000434 0003D7FE length=0003, crc=D7FE

FFFF:F0000438 1200A02D Unit_Spec_Id 00A02D

FFFF:F000043C 13000100 Unit_Sw_Version 000100

FFFF:F0000440 D4000001 Unit_Dependent_Info directory ind_off=000001 (FFFF:F0000444)

Unit_Dependent_Info directory referenced from FFFF:F0000440

FFFF:F0000444 00034FEA length=0003, crc=4FEA

FFFF:F0000448 403C0000 key=40 (UNKNOWN) crc_offset=3C0000 (FFFF:F0F00000)

FFFF:F000044C 81000002 Textual_Descriptor leaf ind_off=000002 (FFFF:F0000454)

FFFF:F0000450 82000005 Bus_Dependent_Info leaf ind_off=000005 (FFFF:F0000464)

Textual_Descriptor leaf referenced from FFFF:F000044C

FFFF:F0000454 00033A64 length=0003, crc=3A64

FFFF:F0000458 00000000 ....

FFFF:F000045C 00000000 ....

FFFF:F0000460 534F4E59 SONY

Bus_Dependent_Info leaf referenced from FFFF:F0000450

FFFF:F0000464 0006010B length=0006, crc=010B

FFFF:F0000468 00000000 ....

FFFF:F000046C 00000000 ....

FFFF:F0000470 43434D2D CCM-

FFFF:F0000474 44533235 DS25

FFFF:F0000478 3020312E 0 1.

FFFF:F000047C 30380000 08..

Listing 1.  Dumprom Shell Script Output

# Conclusion

The Linux version of DumpRom is an excellent place to start your 1394 development.  It provides the means to experiment.  Unlike Windows 2000, you have raw, direct access to the hardware, which has many advantages to developers like myself.  On the other hand, if you don’t want to wade through the details of programming the OHCI card directly, you might want to start with the latest code from the Linux IEEE 1394 Subsystem web page (see References below).

# References

Alexander, William F.  “IEEE 1394 Configuration ROM Decoder,” DDJ, August 1999.

Alexander, William F.  “A WDM IEEE 1394 Configuration ROM Decoder,” DDJ, December 1999.

Pirker, Emanuel, et al. “Linux IEEE 1394 Subsystem”,  http://eclipt.uni-klu.ac.at/ieee1394

ISO/IEC 13212: 1994(E) ANSI/IEEE Std 1212, 1994 Edition, pp. 79-100.

IEEE Standard for a High Performance Serial Bus, IEEE Std 1394-1995, August 30, 1996.