Subnet Addressing Worksheet

When examining a destination IP address, there are several possible interpretations that can be applied to the 32-bit address. These include the perspective of:

- The network designer

- The router

- The network administrator

Options 1) and 3) may be expected to answer questions from the Network Manager.

If the destination IP address of 36.110.33.11 /21 is subject to interpretation, it will be:

- A Class A address using a 13-bit extended mask

- A binary pattern whose first 21 bits must match an entry in a table

- A 32-bit dotted decimal number, just an expression

First, from the Network Designer’s perspective -

An IP address that starts with 36. x . x . x  is a Class A address and should have a default mask of 8 bits (this mask is often written as 255.0.0.0), but the /21 on the end indicates that a 21-bit mask is being used. The difference between the default mask (8 bits) and the actual mask (21 bits) is 13 bits. This address has been extended by 13 bits to create additional segment (subnet or “wire”) addresses within the company’s registered addressing space.

First question: If the network address has been extended by 13 bits, how many subnets have been created?

Since a binary (base2) numbering scheme is used to calculate all of the different combinations, the number of combinations is represented by the formula 
2n - 2, where n is the number of bits in the field. In this case, n is 13.

In order to visualize how many combination are possible, it is necessary to show the 13-bit field having binary weights (values) assigned to each position.

4     2     1

0     0     0     5     2     1

9     4     2     1     5     2     6     3     1

6     8     4     2     6     8     4     2     6     8     4     2     1

0     0     0     0     0     0     0     0     0     0     0     0     0  =  0

0     0     0     0     0     0     0     0     0     0     0     0     1  =  1

:      :      :     :      :      :      :      :      :     :      :      :      :

:      :      :     :      :      :      :      :      :     :      :      :      :

1     1     1     1    1     1     1     1     1     1    1      1     0  =  8190

1     1     1     1    1     1     1     1     1     1    1      1     1  =  8191

There are 8192 possible combinations with a 13-bit field. The subnetting RFC states that the all 0s and the all 1s pattern are unavailable, hence, there are 2n - 2,  in this case 8190, possible combinations.

Second question: If, in this case, there are 8190 combinations, which subnet is represented by 36. 110. 33. 11?

Again, the answer is indicated by the binary pattern in the 13-bit extension of the network number. If the left-most 13 bits of the value 110. 33. 11 were written in binary, the resulting field would be:

4     2     1                                      |

0     0     0     5     2     1                 |

9     4     2     1     5     2     6     3   |  1

6     8     4     2     6     8     4     2   |  6     8     4     2     1

0     1     1     0     1     1     1     0   |  0     0     1     0     0  =  3524

|___________________________| |_________________

|                                                        |

1 1 0                          .                       3 3  (Only first 5 bits shown)

Therefore, this destination address belongs to the 3524th out of 8190 subnets available to the company’s registered address space.

Secondly, from the routers perspective -

Routers only care about and track the network or network/subnet portion of any destination IP address.  The address 36. 110. 33. 11 contains a network part, a subnet part, and a host part. To see which part is which, write the address in 8-bit binary:

N N N N N N N N    S  S  S  S  S  S  S  S   S  S  S  S  S H H H    H H H H H H H H

0  0  1  0  0  1  0  0  |  0  1  1  0  1  1  1  0  |  0  0  1  0  0  0  0  1  |  0  0  0  0  1  0  1  1

|________________| |________________| |________________|  |________________|

|                                  |                                  |                                  |

3 6              .             1 1 0            .               3 3              .             1 1

The routers job is to match the first 21 binary bits of the destination address with an entry in the routing table. Routing table entries do not contain any bits in the host portion of the address. Before trying to get a match in the routing table, the router must perform a logical AND operation to remove the host bits from the destination address. With a 21-bit mask, the left-most 21 binary bits are the network and subnet portion and the right-most 11 bits are the host portion. When performing a logical AND operation, the bit positions covered by the subnet mask are represented as ones, but the remaining bits (the host field) are represented as zeros.

M M M M M M M   M M M M M M M   M M M M M 0  0  0     0  0  0  0  0  0  0  0

0  0  1  0  0  1  0  0  |  0  1  1  0  1  1  1  0  |  0  0  1  0  0  0  0  1  |  0  0  0  0  1  0  1  1

|________________| |________________| |________________|  |________________|

|                                  |                                  |                                  |

3 6              .             1 1 0            .               3 3              .              1 1

Once the logical AND function is performed, all of the host bits are set to zero and only the network and subnet portions remain as original values. The address without any host portion is referred to as the subnet ID. The result of the logical AND would indicate:

M M M M M M M   M M M M M M M   M M M M M 0  0  0     0  0  0  0  0  0  0  0

0  0  1  0  0  1  0  0  |  0  1  1  0  1  1  1  0  |  0  0  1  0  0  0  0  1  |  0  0  0  0  1  0  1  1

---------------------------------------------------------------------------------------------------------

0  0  1  0  0  1  0  0  |  0  1  1  0  1  1  1  0  |  0  0  1  0  0  0  0  0  |  0  0  0  0  0  0  0  0

|________________| |________________| |________________|  |________________|

|                                  |                                  |                                  |

3 6              .             1 1 0            .               3 2              .               0

First question:  What is the subnet ID (or “wire” address) of the destination address 
36. 110. 33. 11?

The subnet ID is the address that remains after the host bits have been set to a zero by the logical AND operation. In this case, the subnet ID for destination address 36. 110. 33. 11 is 36. 110. 32. 0.

Second question: When the router tries to look up the destination network/subnet in the routing table, what is the router looking for?

The router is trying to find an entry whose left-most 21 bits exactly match the binary pattern that resulted from the logical AND operation. If we were looking at the routing table entries, we would look for an entry listed as 36. 110. 32. 0.

Thirdly, from the perspective of the Network Administrator -

First question:  What does the destination address 36. 110. 33. 11 mean?

The destination address is a 32-bit dotted decimal IP address composed of a network portion and a host portion. There is no way to tell where the boundary between the two fields is located, unless the address is displayed in binary, just like the router does.

Second question: If you were given the subnet ID, could you list the range of valid host addresses?

Once the subnet ID is known, all of the remaining bits belong to the host portion.

|

N N N N N N N N    S  S  S  S  S  S  S  S   S  S  S  S  S | H H H    H H H H H H H H

0  0  1  0  0  1  0  0  |  0  1  1  0  1  1  1  0  |  0  0  1  0  0  | 0  0  0  |  0  0  0  0  0  0  0  0

|________________| |________________| |__________|______|  |_______________|

|                                  |                                  |    |                              |

3 6              .             1 1 0            .               3 2                .             0

In this case, there are 11 bits in the host field. The maximum number of hosts is calculated by the 2n - 2 formula. For hosts, the all 0s pattern is the “wire” address (the subnet ID) and the all 1s pattern is reserved to specify all hosts (the broadcast address). When n is 11, there are 2046 valid host addresses. In order to visualize all of the possible combinations, it is necessary to show the 11-bit field having binary weights (values) assigned to each position.

1

0     5     2     1

2     1     5     2     6     3     1

4     2     6     8     4     2     6     8     4     2     1

0     0     0     0     0     0     0     0     0     0     0  =  0		The “wire”

0     0     0     0     0     0     0     0     0     0     1  =  1		Host number 1

:     :      :      :      :      :      :     :      :      :      :

:      :     :      :      :      :      :      :     :      :      :

1     1    1     1     1     1     1     1    1      1     0  =  2046	Host number 2046

1     1    1     1     1     1     1     1    1      1     1  =  2047	All hosts-Broadcast

If the subnet ID (“wire” address) is 36. 110. 32. 0, adding the host bits to the subnet ID generates the range of valid host addresses. Shown in binary:

|

1                               |		          1

2     6     3     1          |                        2     6     3     1

8     4     2     6     8   |   4     2     1     8     4     2     6     8     4     2     1

0     0     1     0     0   |   0     0     0     0     0     0     0     0     0     0     1

|________________|__________|  |__________________________|

|           |           |                                       |

36.110.                   3 2        +          0        .                           1	Host number 1

Therefore, the first valid host address would be (36. 110. 32. 0 plus 0. 0. 0. 1)
36. 110. 32. 1; and the last valid host address is  (36. 110. 32. 0 plus 0. 0. 7. 254) 36. 110. 39. 254.

|

1                 	         |		         1

2     6     3     1          |                        2     6     3     1

8     4     2     6     8   |   4     2     1     8     4     2     6     8     4     2     1

0     0     1     0     0   |   1     1     1     1     1     1     1     1     1     1     0

|________________|__________|  |__________________________|

|          |           |                                       |

3 2       +          7         .                        2 5 4 	Host number 2046

Third question: If you were given the subnet ID, could you specify the broadcast address for this subnet?

Once the subnet ID is known, all of the remaining bits belong to the host portion. When all of the host bits are set to a one, a broadcast address is generated.

1                 	         |		         1

2     6     3     1          |                        2     6     3     1

8     4     2     6     8   |   4     2     1     8     4     2     6     8     4     2     1

0     0     1     0     0   |   1     1     1     1     1     1     1     1     1     1     1

|________________|__________|  |__________________________|

|           |           |                                       |

3 2        +          7         .                        2 5 5 	All hosts-Broadcast

Therefore, the address to reach all hosts on this subnet (a broadcast) would be (36. 110. 32. 0 plus 0. 0. 7. 255) 36. 110. 39. 255.

Another example of this procedure:

For a destination IP address of 217. 200. 3. 229 with a subnet mask of 255.255.255.224. The default mask for a Class C address is 255.255.255.0, so this mask has been extended by 3 bits. With a 3-bit extended mask, there are 2n - 2 (23 - 2) or 6 usable subnets. By writing the last octet of .229 in binary, the 3-bit subnet number is more visible:

4   2    1 |

S   S    S | H   H   H   H   H

1    1    1 | 0    0    1    0    1

|                                             |

|

2 1 7  .  2 0 0  .   3   .                 2 2 9

The subnet number contained in this octet is subnet number 7; this all 1s pattern is referred to as the broadcast subnet and is usually not assigned in a real network. After performing the logical AND operation, using the .2 2 4 mask, the subnet ID contains the network and subnet fields without any hosts applied:

1             |

2   6   3   |  1

8   4   2   |  6   8   4   2   1

M   M  M | H   H  H  H  H

1    1    1  |  0   0   0   0   0

|                  |                        |

|

2 1 7  .  2 0 0  .   3   .                  2 2 4

The subnet ID of 217. 200. 3. 224 with 0. 0. 0. 1 through 0. 0. 0. 30 added to it yields host addresses in the range of 217. 200. 3. 225 to 217. 200. 3. 254. The broadcast address has all 1s in the host portion and would be 217. 200. 3. 255.