DTC/2 jumpers and switches

-----------------------------------------------------------------------

A) DIP SWITCHES

S1	select PC memory address for QuadPort RAM

--------

|      |

| #  # |  ON

| #  # |

| #  # |

| #  # |

| |  | |

| |  | |

| |  | |

| |  | |

| |  | |  OFF

|      |

--------

0  1

memory address		1	0

-----------------------------------

no memory		OFF	OFF

C8000-C87FF		ON	ON	<default>

CC000-CC7FF		ON	OFF

D0000-D07FF		OFF	ON

-----------------------------------

-----------------------------------------------------------------------

B) STANDARD JUMPERS

JP1	select PC high interrupts

------------------

| o  o | o  o  o | 1

|      |         |

| o  o | o  o  o | 2

------------------

Interrupt 'H' from FPGA, pin 48 through U4C:

1-2	IRQ10

3-4	IRQ11

5-6	IRQ12

Interrupt from IDE port (direct):

7-8	IRQ11

9-10	IRQ15

Interrupts 2/9 (FPGA pin 50->U4A) and 5 (FPGA pin 49->U4B)

are permanently connected via shorts SH2 and SH4

====================================================================

JP2	select Xilinx boot mode

-----

| # | 3

| # |

| # | 2

|   |

| o | 1

-----

1-2	mode is U9-9 inverted

2-3	mode is U9-9 		<default>

Xilinx mode [M0-M1-M2] can be one of

000	Master serial, uses U3 PROM

101	Slave peripheral on PC bus

Output of U9 is 0 at reset - jumper on 2-3 sets 000 mode

State of the M0-M2 pins can be checked by reading port 0168, bit 0

====================================================================

JP3	C012 link switch

--------------

2	| ####  #### | 8

|            |

1	| ####  #### | 7

--------------

pin definition:

1	OUT	to TRAM1 link 0 in

2	IN	from C012 link out

3	IN	from TRAM2 link 0 out

4	OUT	to T425 link 0 in

5	IN	from TRAM1 link 0 out

6	OUT	to C012 link in

7	OUT	to TRAM2 link 0 in

8	IN	from T425 link 0 out

default connection:

1-3,5-7	TRAM1 link0 to TRAM2 link 0

2-4,6-8	C012 link to T425 link 0

alternate connection:

1-2,5-6	C012 link to TRAM1 link 0

3-4,7-8	T425 link 0 to TRAM2 link 0

====================================================================

JP4	Transputer memory wait

-----

| # | 1	GND

| # |

| # | 2 MemWait

|   |

| o | 3	MemS4

-----

1-2	no wait states added

2-3	wait states, depend on MemS4

====================================================================

JP8	IOF2O (connector pin 31) output select

-----------

3	| o  #### | 1

-----------

DPC8   SCO4

out

1-2	pin 31 is SCO4 (UART3 DTR)

2-3	pin 31 is DPC8 (DSP PC8/STD)

====================================================================

JP9	IOF0IB (connector pin 6) input select

-----

| # | 1	SCI0

| # |

| # | 2	in

|   |

| # | 3	DPC7

| # |

| # | 4 U52-5 (AD7869 DR)

-----

1-2	pin 6 is SCI0 (UART1 DSR)

3-4	DSP SRD connected to AD7869 DR

2-3	pin 6 is DPC7 (DSP PC7/SRD)

====================================================================

JP10	TIMIN (connector pin 7) input select

JP11	IO2O (connector pin 51) output select

JP10 JP11

--------

U34-13 1	| #  # | 1 SCO5

| #  # |

in    2 | #  # | 2 out

|      |

DPC6  3 | o  o | 3 DPC6

--------

JP10 1-2	pin 7 to U34-13 (TIMINB to PAL PLD6E)

2-3	pin 7 to DPC6 (DSP PC6/SCK)

JP11 1-2	pin 51 is SCO5 (UART3 RTS)

2-3	pin 51 is DPC6 (DSP PC6/SCK)

-----------------------------------------------------------------------

C) JP CONNECTORS

JP5	Codec aux connector

--------------------------

| o  o  o  o  o  o  o  o | 1

--------------------------

pin 1	M_OUT (U48-47)		AD1845 only

pin 2	AGND

pin 3	L_MIC (U48-29)

pin 4	AGND

pin 5	R_MIC (U48-28)

pin 6	AGND

pin 7	M_IN  (U48-46)		AD1845 only

pin 8	AGND

====================================================================

JP6	UART aux connector

-----------------

| o  o  o  o  o | 1

|               |

| o  o  o  o  o |

-----------------

pin 1	U35C-6	 IN+

pin 2	U35C-7	 IN-	diff in to SCI2 (UART2 DSR)

pin 3	U35D-10  IN+

pin 4	U35D-9   IN-	diff in to SCI4 (UART3 DSR)

pin 5	GND

pin 6	GND

pin 7	U50F-13		inverted in to SCI6 (UART4 DSR)

pin 8	U50C-5		inverted in to SCI3 (UART2 CTS)

pin 9	U50E-11		inverted in to SCI7 (UART4 CTS)

pin 10	U50D-9		inverted in to SCI5 (UART3 CTS)

====================================================================

JP7	high bits of DSP parallel port - see J3

====================================================================

FX5	power connector for piggyback

--------------

1	| o  o  o  o |

--------------

pin 1-2	GND

pin 3-4	VCC