DTC/2 62 pin connector by pin number

-----------------------------------------------------------------------

Pin	Signal	Dir	Desc

-----------------------------------------------------------------

1	Aux1R	IN	Audio input Aux1 Right to AD1848 (2V RMS)

2	OutR	OUT	Audio output Right from AD1848

3	LineR	IN	Audio input Line Right to AD1848 (2V RMS)

4	DPC3	I/O	PC3/SC0 pin from 56002

5	DP3O	OUT	inverted PC3/SCO pin from 56002

6	IOF01	IN	digital in

to U33C through 10k resistor -> JP9-2

JP9 1-2 connects to SCI0 (UART1 DSR)

JP9 2-3 connects to DPC7 (PC7/SRD on DSP)

7	TIMIN	IN	digital in

to U33A through 10k resistor -> JP10-2

JP10 1-2 connects to TIMINB (U34-13)

JP10 2-3 connects to DPC6 (PC6/SCK on DSP)

8	LKI1P	IN	input  RS422 Link 1 to T425 (+)

9	LKO1P	OUT	output RS422 Link 1 from T425 (+)

10	DRXDM	IN	input  RS422 DSP SSI (-)

11	DTXDP	OUT	output RS422 DSP SSI (+)

12	SIO22	IN	digital in to D5 diode/jumper -> U32C through 18k res,

75k pulldown, D3 rectifier, TP57

output of U32C goes to UART2 RI/DCD

13	SIO19	OUT	UART4 SOUT through U33H, 220 resistor

ch#4 MIDI out (p5)

14	VCC	PWR	+5V

15	SIO16	IN	input cathode opto#1B / anode D10 -> UART3 RI/DCD

16	SIO13	OUT	DAC output (U54B/TP67/R139/R140)

17	SIO10	INT	input cathode opto#2A / anode D8 -> UART2 SIN

ch#2 MIDI in (p5)

18	SIO8	PWR	VCC through 180ohm (25mA)

ch#2 MIDI out (p4)

19	SIO5	IN	input to U35A IN- -> UART3 SIN

20	SIO2	IN	input cathode opto#2B / anode D7 -> UART1 SIN

ch#1 MIDI in (p5)

21	SIO0	PWR	VCC through 180ohm (25mA)

ch#1 MIDI out (p4)

22	AGND	GND	Analog ground

23	AGND	GND	Analog ground

24	AGND	GND	Analog ground

25	AGND	GND	Analog ground

26	DPC4	I/O	PC4/SC1 pin from 56002

27	DP4O	OC	inverted PC4/SC1 pin from 56002 - Open collector

28	SUBXS	I/O	from XC4003 pin 24

input through jumper SH32

OC inverted output through jumper SH31 / TP70

29	LKI1M	IN	input  RS422 Link 1 to T425 (-)

30	LKO1M	OUT	output RS422 Link 1 from T425 (-)

31	IOF2OB	OUT	digital out from U33B - source selected by JP8

JP8 1-2 connects to SCO4 (UART3 DTR)

JP8 2-3 connects to DPC8 (PC8/STD from DSP)

32	DTXDM	OUT	output RS422 DSP SSI (-)

33	SIO23	IN	digital in to D6 diode/jumper -> U32D through 18k res,

75k pulldown, D4 rectifier, TP58

output of U32D goes to UART1 RI/DCD

34	SIO20	IN	input cathode opto#1A / anode D9 -> UART4 RI/DCD

35	GND	GND

36	SIO17	IN	input anode opto#1B thru 270ohm / cathode D10

37	SIO14	PWR	VCC through 180ohm (25mA)

ch#3 MIDI out (p4)

38	SIO11	IN	input anode opto#2A thru 270ohm / cathode D8

39	SIO9	OUT	UART2 SOUT through U33F, 220 resistor

ch#2 MIDI out (p5)

40	SIO6	IN	input to U35B IN+ / TP69 -> UART4 SIN

SH34 bypasses U35B, direct input to UART

41	SIO3	IN	input anode opto#2B thru 270ohm / cathode D7

42	SIO1	OUT	UART1 SOUT through U33E, 220 resistor

ch#1 MIDI out (p5)

43	Aux1L	IN	Audio input Aux1 Left to AD1848 (2V RMS)

44	OutL	OUT	Audio output Left from AD1848

45	LineL	IN	Audio input Line Left to AD1848 (2V RMS)

46	DPC5	I/O	PC5/SC2 pin from 56002

47	DP5O	OC	inverted PC5/SC2 pin from 56002 - Open collector

48	GND	GND

49	SIR160	OUT	inverted (active LO) Transputer Reset

50	GND	GND

51	IO2OB	OUT	digital out from U33D - source selected by JP11

JP11 1-2 connects to SCO5 (UART3 RTS)

JP11 2-3 connects to DPC6 (PC6/SCK from DSP)

52	DRXDP	IN	input RS422 DSP SSI (+)

53	GND	GND

54	SIO21	IN	input anode opto#1A thru 270ohm / cathode D9

55	SIO18	PWR	VCC through 180ohm (25mA)

ch#4 MIDI out (p4)

56	GND	GND

57	SIO15	OUT	UART3 SOUT through U33G, 220 resistor

ch#3 MIDI out (p5)

58	SIO12	IN	ADC input to U54A through 10k

59	GND	GND

60	SIO7	IN	input to U35B IN- -> UART4 SIN

61	SIO4	IN	input to U35A IN+ / TP68 -> UART3 SIN

SH33 bypasses U35A, direct input to UART

62	GND	GND

-----------------------------------------------------------------------

DTC/2 62 pin connector by function

-----------------------------------------------------------------------

Pin	Signal	Dir	Desc

-----------------------------------------------------------------

43	Aux1L	IN	Audio input Aux1 Left to AD1848 (2V RMS)

1	Aux1R	IN	Audio input Aux1 Right to AD1848 (2V RMS)

45	LineL	IN	Audio input Line Left to AD1848 (2V RMS)

3	LineR	IN	Audio input Line Right to AD1848 (2V RMS)

44	OutL	OUT	Audio output Left from AD1848

2	OutR	OUT	Audio output Right from AD1848

22	AGND	GND	Analog ground

23	AGND	GND	Analog ground

24	AGND	GND	Analog ground

25	AGND	GND	Analog ground

58	SIO12	IN	ADC input to U54A through 10k

16	SIO13	OUT	DAC output (U54B/TP67/R139/R140)

-----------------------------------------------------------------

8	LKI1P	IN	input  RS422 Link 1 to T425 (+)

29	LKI1M	IN	input  RS422 Link 1 to T425 (-)

9	LKO1P	OUT	output RS422 Link 1 from T425 (+)

30	LKO1M	OUT	output RS422 Link 1 from T425 (-)

52	DRXDP	IN	input  RS422 DSP SSI (+)

10	DRXDM	IN	input  RS422 DSP SSI (-)

11	DTXDP	OUT	output RS422 DSP SSI (+)

32	DTXDM	OUT	output RS422 DSP SSI (-)

-----------------------------------------------------------------

4	DPC3	I/O	PC3/SC0 pin from 56002

5	DP3O	OUT	inverted PC3/SCO pin from 56002

26	DPC4	I/O	PC4/SC1 pin from 56002

27	DP4O	OC	inverted PC4/SC1 pin from 56002 - Open collector

46	DPC5	I/O	PC5/SC2 pin from 56002

47	DP5O	OC	inverted PC5/SC2 pin from 56002 - Open collector

-----------------------------------------------------------------

6	IOF01	IN	digital in

to U33C through 10k resistor -> JP9-2

JP9 1-2 connects to SCI0 (UART1 DSR)

JP9 2-3 connects to DPC7 (PC7/SRD on DSP)

7	TIMIN	IN	digital in

to U33A through 10k resistor -> JP10-2

JP10 1-2 connects to TIMINB (U34-13)

JP10 2-3 connects to DPC6 (PC6/SCK on DSP)

31	IOF2OB	OUT	digital out from U33B - source selected by JP8

JP8 1-2 connects to SCO4 (UART3 DTR)

JP8 2-3 connects to DPC8 (PC8/STD from DSP)

51	IO2OB	OUT	digital out from U33D - source selected by JP11

JP11 1-2 connects to SCO5 (UART3 RTS)

JP11 2-3 connects to DPC6 (PC6/SCK from DSP)

12	SIO22	IN	digital in to D5 diode/jumper -> U32C through 18k res,

75k pulldown, D3 rectifier, TP57

output of U32C goes to UART2 RI/DCD

33	SIO23	IN	digital in to D6 diode/jumper -> U32D through 18k res,

75k pulldown, D4 rectifier, TP58

output of U32D goes to UART1 RI/DCD

28	SUBXS	I/O	from XC4003 pin 24

input through jumper SH32

OC inverted output through jumper SH31 / TP70

49	SIR160	OUT	inverted (active LO) Transputer Reset

-----------------------------------------------------------------

21	SIO0	PWR	VCC through 180ohm (25mA)

ch#1 MIDI out (p4)

42	SIO1	OUT	UART1 SOUT through U33E, 220 resistor

ch#1 MIDI out (p5)

20	SIO2	IN	input cathode opto#2B / anode D7 -> UART1 SIN

ch#1 MIDI in (p5)

41	SIO3	IN	input anode opto#2B thru 270ohm / cathode D7

18	SIO8	PWR	VCC through 180ohm (25mA)

ch#2 MIDI out (p4)

39	SIO9	OUT	UART2 SOUT through U33F, 220 resistor

ch#2 MIDI out (p5)

17	SIO10	INT	input cathode opto#2A / anode D8 -> UART2 SIN

ch#2 MIDI in (p5)

38	SIO11	IN	input anode opto#2A thru 270ohm / cathode D8

37	SIO14	PWR	VCC through 180ohm (25mA)

ch#3 MIDI out (p4)

57	SIO15	OUT	UART3 SOUT through U33G, 220 resistor

ch#3 MIDI out (p5)

15	SIO16	IN	input cathode opto#1B / anode D10 -> UART3 RI/DCD

36	SIO17	IN	input anode opto#1B thru 270ohm / cathode D10

55	SIO18	PWR	VCC through 180ohm (25mA)

ch#4 MIDI out (p4)

13	SIO19	OUT	UART4 SOUT through U33H, 220 resistor

ch#4 MIDI out (p5)

34	SIO20	IN	input cathode opto#1A / anode D9 -> UART4 RI/DCD

54	SIO21	IN	input anode opto#1A thru 270ohm / cathode D9

-----------------------------------------------------------------

61	SIO4	IN	input to U35A IN+ / TP68 -> UART3 SIN

SH33 bypasses U35A, direct input to UART

19	SIO5	IN	input to U35A IN- -> UART3 SIN

40	SIO6	IN	input to U35B IN+ / TP69 -> UART4 SIN

SH34 bypasses U35B, direct input to UART

60	SIO7	IN	input to U35B IN- -> UART4 SIN

-----------------------------------------------------------------

14	VCC	PWR	+5V

35	GND	GND

48	GND	GND

50	GND	GND

53	GND	GND

56	GND	GND

59	GND	GND

62	GND	GND

-----------------------------------------------------------------

DTC/2 62 pin connector pin location

-----------------------------------------------------------------------

(top)

22 o

1 o	 43 o

23 o

2 o	 44 o

24 o

3 o	 45 o

25 o

4 o	 46 o

26 o

5 o	 47 o

27 o

6 o	 48 o

28 o

7 o	 49 o

29 o

8 o	 50 o

30 o

9 o	 51 o

31 o

10 o	 52 o

32 o

11 o	 53 o

33 o

12 o	 54 o

34 o

13 o	 55 o

35 o

14 o	 56 o

36 o

15 o	 57 o

37 o

16 o	 58 o

38 o

17 o	 59 o

39 o

18 o	 60 o

40 o

19 o	 61 o

41 o

20 o	 62 o

42 o

21 o

-----------------------------------------------------------------------