DTC/2 MEMORY and I/O MAP

-----------------------------------------------------------------------

A) Base I/O registers

These are available also when the FPGA is not configured.

0160-016F	Card control registers - implemented by PAL U8 and

bit register U9

0160	W	Transputer Reset - active HI

0161	W	Transputer Analyze - active HI

0162	W	available on TP12 and U34-11

(IMSB008: DMA request register)

0163	W	available on TP13 and U34-4

(IMSB008: IRQ control register)

0164	W	Xilinx MODE

JP2 chooses between straight and inverted out

0165	W	DSP Reset - active LO

0166	W	DSP MODE to U34-2 - default LO (boot from Host Port)

0167	W	Xilinx PROG - active LO

0160	R	Xilinx INIT/ERR (FPGA not initialized)

Transputer Error - active LO (FPGA base version)

0164	R	Xilinx DONE - HI when configured

0168	R	Xilinx MODE readback

016C	R	LOCAL (FPGA pin 45)

All read/writes use bit 0

State at reset:	all outputs of U9 are LOW

Transputer reset is HI when PC reset=HI (U10B)

DSP is reset, mode 0

Xilinx PROG is HI

Xilinx MODE is HI if jumper JP2:1-2 (Slave parallel)

LO if jumper JP2:2-3 (Master serial)

Bits 0162-0163 are used by U34 to choose in/out mode for the DSP

timer pin - they can also be used by Inmos software - be careful

0170-0177	Secondary IDE port

0178-017B	FPGA control

0178	W	When FPGA is not configured and in Slave mode

(XMODE=1,DONE=0) this is the strobe for parallel

data going from the PC to the FPGA (pin 61).

FPGA pin 61 is defined in all other cases as

IORW [(!IOR # !IOW) & !AEN].

0178	R	FPGA readback when configuring in Slave mode

bit 7 is used as RDY/!BSY signal

In configuration mode addresses 0178..017B are equivalent

0374-0377	Secondary IDE port

NOTE - The IDE port is not fully independent from the FPGA

configuration; read and write strobes come from the U5 buffer,

which can be disabled by the PcADB signal (FPGA pin 72) in local

cycles. You should test LOCAL (read 016C) before accessing IDE

port.

-----------------------------------------------------------------------

B) QuadPort memory

Available for writes only when the FPGA is configured.

The decoding is done by PAL U7 and depends on the S1 switches:

S1=00	memory not selected

S1=01	D0000-D07FF

S1=10	CC000-CC7FF

S1=11	C8000-C87FF

Because the QRAM is a 1k part, only addresses 000..3FF are actually

significant.

The memory is accessed at 0 wait states (U1F-U10C-U53D)

-----------------------------------------------------------------------

C) FPGA-defined PC registers (current version A7:03)

0150-0157	IMSC012

0150	R	Data in

0151	W	Data out

0152	R/W	Control in

0153	R/W	Control out

0178-017B	FPGA control/capture registers

0178	W	FPGA control - reset at 0

bit 0=1	enable IRQ9 for capture

bit 1=1	enable IRQ5 for C012 interrupt

bit 2=1	enable EvReq

bit 3=1	reset M401 registers

bit 4=1 enable NMI for capture

bit 5=1 enable TFIRQ to DSP for capture

bit 6	(nd)

bit 7=1 enable capture mode

0178	R	read DATA capture register

after reset = VERSION

0179	R	read ADDRESS/FLAGS capture register

after reset = SUBVERSION

bit 0-2	address bus 0-2

bit 3-4	captured port

00	no port

01	03BC-03BF

10	0378-037B

11	0100-0103

bit 5	IRQ9 enable readback (for test)

bit 6	R/W cycle: 1=R 0=W

bit 7	1=double capture error

0330-0337	M401 (MIDI port) registers

The MPU401 standard defines port n+0 as DATA port, port n+1

as STATUS(R)/CONTROL(W) port. In the FPGA, accesses to the DATA

and CONTROL port are redirected to the corresponding locations

of the QuadPort RAM, while the STATUS port is emulated by internal

FFs. Only bits 6-7 of the STATUS port are defined:

bit 6	write flag (1=full 0=empty)

host can write when bit 6=0

bit 7	read flag (1=empty 0=full)

host can read when bit 7=0

Host writes to DATA/CONTROL registers when b6=0 set b6

Host reads from DATA register when b7=0 set b7

The DSP must explicitly clear these bits after reading/writing

in the QRAM.

0330-0331	MIDI port 1

0332-0333	MIDI port 2

0334-0335	MIDI port 3

0336-0337	MIDI port 4

0340-0347	DSP56002 Host port

0348-034A	OnCE port

0348	W	DR control

bit 0=1	set DR active(LO), clear ACK

bit 0=0	set DR inactive(HI), clear ACK

0348	R	OnCE status

bit 0	DSI/OS0 readback

bit 1	DSCK/OS1 readback

bit 2	ACK (latched)

bit 3	0=serial transfer in progress

1=serial transfer ready for next byte

bit 4	DSO readback

0349	R/W	OnCE data register

034A	W	start serial transfer

To start OnCE:

set DR (write 0348=1)

wait for ACK (read 0348, wait for bit 2=1)

reset DR and ACK (write 0348=0)

To write a byte/word:

a-write data into 0349 shift reg.

b-write 034A (start transfer)

c-wait for end transfer (read 0348, wait for bit 3=1)

repeat steps a-c for 24-bit word

if necessary check for ACK=1

To read a word:

a-write 0 into 0349 shift reg. (just to keep DSI=0, not

strictly necessary)

b-write 034A (start transfer)

c-wait for end transfer (read 0348, wait for bit 3=1)

d-read data from 0349 shift reg.

repeat steps (a)b-d for 24-bit word

if necessary check for ACK=1

The OnCE transfers data at a frequency equal to PCCLK/4, this

means 2MHz on the 8MHz standard AT bus (CKDIV=4 on 33MHz PC)

034B		Transputer Event control

034B	R	Event status

bit 0	EvAck	active HI

bit 1	EvReq	active HI (readback)

bit 2	IINT from C012	active HI

bit 3	OINT from C012	active HI

bit 4	TDIRQ - interrupt from DSP

034B	W	Set Event Request

bit 0	write 1 = set EvReq (if bit 2 of 0178 HI)

NOTES - all the EVEN registers in the ranges 0150-0157, 0178-017F, 0330-0337,

0278-027B, 0378-037B, 03BC-03BF generate 16-bit I/O cycles; this

should result in a faster access, up to 4x compared to normal 8-bit

cycles. It seems that it is not possible to access ODD registers

with 16-bit cycles.

the high address lines (A10-A15) are not decoded for I/O cycles, but

addresses A10-A12 are buffered and provided on test points TP11,

TP10,TP8.

-----------------------------------------------------------------------

D) FPGA-defined DSP registers (base version)

The DSP accesses the FPGA at addresses Y:$E000..$EFFF. Only address bits

A0-A3 and data bits D20-D23 are connected to the FPGA. Accessing the FPGA

requires at least 4 wait states at Fc=60 MHz.

E000-E00F	DSP FPGA registers (4 bits)

E000	R	M401 write flags

bit 20	-> PC status 0331 bit 6

bit 21	-> PC status 0333 bit 6

bit 22	-> PC status 0335 bit 6

bit 23	-> PC status 0337 bit 6

E001	R	M401 read flags

bit 20	-> PC status 0331 bit 7

bit 21	-> PC status 0333 bit 7

bit 22	-> PC status 0335 bit 7

bit 23	-> PC status 0337 bit 7

E002	R	Last register written from PC (address line A0)

bit 20-23	0 if PC wrote DATA reg.

1 if PC wrote CONTROL reg.

E003	R	Aux.information

bit 20		CAPTURE strobe (active LO)

bit 21-23	readback from register at Y:E008

E004	R	Capture data bus, low nibble

E005	R	Capture data bus, high nibble

E006	R	Capture address/flags, low nibble

bit 20-22	A0-A2

bit 23		capt.port bit 0

E007	R	Capture address/flags, high nibble

bit 20		capt.port bit 1

bit 21		IRQ9 enable

bit 22		R/W cycle: 1=R 0=W

bit 23		1=double capture error

E000	W	clear PC status 0331 bit 6

E001	W	clear PC status 0333 bit 6

E002	W	clear PC status 0335 bit 6

E003	W	clear PC status 0337 bit 6

E004	W	clear PC status 0331 bit 7

E005	W	clear PC status 0333 bit 7

E006	W	clear PC status 0335 bit 7

E007	W	clear PC status 0337 bit 7

E008	W	write bit 20=1 to enable PC IRQ9

write bit 20=0 to disable PC IRQ9

This bit is cleared also by setting bit 3 in register 0178

or by reading status registers 0331,0333,0335,0337

bits 21-23	scratch-echoed at Y:E003

E009	W	same as E008

E00A-E00F	currently undefined

-----------------------------------------------------------------------