OrCAD III/IV PCX/FACSIMILE DRIVERS      V3.00

These  printer drivers convert the OrCAD print output  into  PCX

format,  allowing schematic sheets to be imported and changed by

many graphics oriented programs, including Z-Soft's PC/Microsoft

Paintbrush, Alchemy Mindworks' Graphic Workshop,  and Power Up's

Express Publisher.  (There are dozens of others; I own copies of

these three, so that's why they are listed.)

They also allow sheets to be imported into the Quick Link II Fax

(QL2) program,  by Smith Micro Software,  which is bundled  with

many fax modems.  This program also accepts input in Epson MX-80

format, but I have found the MX-80's relatively low  resolution,

plus problems with data oversize prints  (data overruns are  not

always handled well by QL2; see more below),  leave a lot to  be

desired.  Also, PCX files can be cropped, merged, changed, etc.,

prior to transmission using a number of available tools.

This package contains two different drivers:

PCX.DRV         8" x 10" 216 x 200 DPI standard PCX

format output (black on white)

PCXI.DRV        same size output with data inverted

(white on black)

The  216 x 200 DPI was chosen to match the "standard"  facsimile

output.

If your uses are mainly importing sheets into DTP packages,  the

first driver is probably best.   If you fax documents with  QL2,

use PCXI.

General Tips

1.      These drivers will not send output to a printer or other

device.  If you forget to send the output to a file, the

driver will terminate with an "Access Denied" error.   I

know of no printer which can directly handle PCX data.

2.      PCX/PCXI  were designed for SINGLE PAGE documents  only!

If you attempt to print a second page,  the driver  will

shut down prematurely.   There is a good chance that the

first page will still be correctly written to the file.

3.      Do not attempt to print right up to (or over) the 8 inch

horizontal  and 10 inch vertical page limits;  this  may

cause file errors.   Typical symptoms include pages with

slanted borders or lines, data missing, etc.   No errors

have been found using a sheet size of 7.5 x 10 inches.

Tips on Sending Facsimile Documents

1.      Since the size of OrCAD sheets varies when using printer

drivers, use PLOTALL sent to the printer driver, scaling

the sheets to "A" size.   This will guarantee the sheets

take up the maximum usable area on the page.

2.      The facsimile standard requires white on black data.  If

you use files with non-inverted data, your documents will

take longer to send (your $$$) and use up a lot of ink or

toner on your recipient's fax machine (his/her $$$)!

Any questions or comments?  Drop me a line at:

Ashby Solutions

47 West State Road

Ashby, MA  01431-9701

(508) 386-7015

Compuserve [73520,1101]

Enjoy,

Bruce Wahler

IMPORTANT NOTICES

By adding his name to the copyright message in this driver,  the

CO-author makes no attempt to diminish the contribution of OrCAD

developers  to  the workings of the driver.   Nevertheless,  the

creation of this driver required a  good deal more than changing

a line or two of code and a couple of lookup parameters; in fact

40% of the source code was newly developed by the co-author.

All  additions and changes made to this driver by the  co-author

are donated to the public domain.

This driver could not have been developed without information in

the PC Paintbrush chapter of Bitmapped Graphics by Steve Rimmer.

FINALLY . . .

If  you are using an unregistered copy of OrCAD,  go out and BUY

one!   Unauthorized use of software hurts ALL users in the  long

run.