Casio Digital Diary User Manual

Version 0.2 © 2000 IMSL Software

This user manual provides a brief overview of the most common user operations for most Casio Digital Diary models. It is not intended to be a complete reference manual, or to provide keystroke-by-keystroke details. It tries to cover the variations of all common models in one description, with a few useful tips thrown in. If you need more information on a specific model, please request a replacement manual directly from Casio.

This manual is copyrighted by IMSL Software - please do not copy parts of it or re-distribute it without the copyright notice! If you find anyone else doing this, please notify IMSL Software at IMSL@imslsoft.com.

Contents

Casio Diary Models

Turning the diary on and off

Batteries

Functions

Settings

Setting Date and Time

Memory

Data Modes

Entering Data

Displaying Data

Searching for Data

Editing Data

Deleting Data

Telephone Items

Business Card Items

Memo Items

Schedule Items

Reminders

Calendar Mode

To Do Items

Expense Items

Calculator Mode

Secret Mode

Secret Mode - Standard SF/CSF/NX Family

Secret Mode - SF-5x80/5x90SY/6x00SY Family

Communications

How to Send and Receive Data

Serial Cables

Backup Software

Outdated Stuff

Plug-in Cards

Lucid 3D Spreadsheet

Infrared “Secret Sender”

Printing Data

Pen Printing Option

Floppy Disk Option

IP800 Pager

Troubleshooting

Help! I have an error message!

Help! It’s broken!

SF-5x80/5x90SY/6x00SY/7x00SY Family

FAQ and Tips

## Casio Diary Models

Casio has distributed well over 100 models of Digital Diary, so we aren’t going to list them all here. Here is a brief summary of the variations:

- There are 3 main families with different internal data formats and different communications protocols:

- The standard SF/CSF/NX models (e.g. SF-7500, SF-4600, SF-R20, SF-7900, SF-4900…, IP800, CSF-xxxx, NX-xxxx)

- The SF-A10/20/30 models (includes C-300)

- The SF-5x80/5x90SY/6x00SY/7x00SY models

Different models can communicate within a family, but not with other families.

- The suffix letters on the model numbers sometimes indicate a revision of the model (e.g. the SF-5500B is a later revision of the SF-5500), or sometimes a language-specific variation (e.g. the SF-4900RS is a Russian character set model), or sometimes a difference in style or packaging (e.g. the SF-4900L is a “Large-screen” model while the SF-4900C is a “Compact” model).

- Casio has used different names and model numbers in different countries for similar models. The “B.O.S.S.” product name was used only in the United States, and does not indicate any significant difference from the standard SF-model number.

- Some Casio models were sold under private labels, such as the Radio Shack “EC-3xx” series, equivalent to SF-4400/4600 models.

- The SF-M10/R10/R20 models were the pinnacle of the standard SF series, including such refinements as a large screen, a built-in spreadsheet, additional Reminder variations, and plug-in memory/program cards. They are no longer available, as the market moved to smaller and cheaper models. Current models are available in the standard SF series, such as the SF-4985ER, but there are no longer any models available in this series with large screens.

- In addition to the SF-M10/R10/R20, several other models in the SF-9000 series supported Casio plug-in memory and program cards. None of these models is available any longer, and the plug-in cards are now difficult to find. These cards were a proprietary design unique to the Casio Digital Diary. No current models support plug-in cards.

- The CSF color models belong the standard SF series. In addition to the limited color capability, they add a few features such as icons for Reminder items. Casio has not introduced any new CSF models for several years, and none are available in retail stores now.

- The NX pen graphics models belong to the standard SF series. They are internally similar to the CSF models, but have a larger graphics screen with pen input instead of a keyboard. They are the only models which support bitmap graphics. Casio did not sell many of them, and they have long disappeared from retail stores. Casio’s newer PV (Pocket Viewer) series are pen-input models, but with a monochrome screen and completely different design.

- The SF-A10/20/30 models are smaller and less expensive than the standard SF series. They were more widely sold in Europe than in North America. Although they have a communications port, Casio never introduced any software to support them, so the only backup option is the XLink/Win or CASIF/Win shareware software.

- The SF-5x80 “Illuminator” series introduced a new model line for Casio. They are incompatible with the earlier SF models, and have some operational differences (noted below). The SF-5x90SY, SF-6x00SY, and SF-7x00SY models are an extension to the SF-5x80 model line with the addition of a new “Synchronize” protocol. In North America the SY models are sold with a serial cable and backup software included, but these are sold separately in other countries. Note that Casio changed the serial port design on the SY models, so earlier serial cables are incompatible.

- This document does not cover any of the Casio Windows CE palmtop models, or the recent BN or PV models, or the earlier Casio Zoomer palmtop models.

## Turning the diary on and off

Press the On key to turn the diary on (usually at the upper left of the keyboard), press Off to turn it off again (usually right next to the On key). The diary turns off automatically if not used for a few minutes in order to conserve battery power.

If the display is dim, try adjusting the contrast. This can be done by a small adjustment wheel on the right side of  the diary on some older models, but is a menu option on all recent models. If the display cannot be adjusted to give enough contrast, then the main batteries may be low and need replacement. Another symptom of low batteries is if the display contrast dims and flickers when you press keys or while the diary is communicating. However, most models will tell you on the screen when their batteries are low and need replacing.

The startup state depends on which diary model you have. The standard SF/CSF/NX family always starts up on the index screen of the last mode you left it in. If it was left in Calendar mode, it’s normal for it to take a second or two to recompute the current calendar before the display appears. The SF-5x80/5x90/6x00 family always starts up exactly where you left it, even if it was in the middle of entering data. If the diary is set to Secret mode, you may be prompted to enter a password immediately before you can do anything else.

## Batteries

Batteries are normally replaced by opening the battery compartment on the back of the unit. Some older models require you to open the case with a screwdriver to replace batteries. All models have more than one battery in order to allow the batteries to be replaced one at a time without losing the power to sustain the memory.

Warning: the Casio diaries require battery power to sustain the memory contents - if you remove all batteries at the same time for more than a few seconds, or you allow the batteries to become too weak, the memory contents may be erased.

Most models use CR-2025 or CR-2032 button size batteries. Any brand or type which says it is compatible with these numbers should be OK. Both of these types actually have the same voltage and can be interchanged, but the CR-2032 batteries are thicker and have a longer life, and should be used if your model calls for them. If there are 2 sets of batteries marked Main and Backup, only replace one set at a time. You may have to move a slider to release the batteries - if so, make sure that you slide it back to the Operate position after replacing the batteries. If there are just 2 batteries, replace only one at a time (and be quick about it!).

Models which use AA or AAA-size main batteries usually have a secondary button size backup battery inside the unit which should be replaced separately.

The SF-5x80/5x90/6x00SY family require you to press the Start or Reset button on the back after changing the main batteries. In this case be sure to answer “No” to the “…New Unit… (Y/N)?” prompt.

Older models which use memory plug-in cards have a backup battery inside each memory card which must be replaced separately. You should do this while the card is inserted in the diary so that the diary batteries can act as a temporary backup.

Battery life varies by model and usage, but you should expect about 6 months of life with normal use. Communications and use of the backlight (if you have one) are the biggest power drains. Very short battery life (e.g. only a few days) usually indicates an electrical fault in the unit which should be repaired by Casio.

Warning: it is very common to encounter a “Data Error!” after changing batteries. This can render all of your data inaccessible, so always do a backup before you change batteries.

## Functions

The various Casio diary models have several ways of accessing the diary functions, not all of them obvious.

Some functions are on dedicated keys. You can usually tell what they are by the label or symbol on the key.

Some functions are Shifted functions of other keys. These functions are normally printed on the keyboard above the regular key label. Press Shift and then the key to access these functions (you don't have to hold down Shift). Caps Lock mode does not access these functions, you must use Shift.

Some functions are accessed via a Function menu. On most models this menu is displayed by pressing the Function or Func key. You then press the number of the function you want from the menu. On models with smaller displays, there may be more than one page to the menu (there is no visible indication that there is another page!) - in this case you must press the Function key again to access the next page. On the SF-5x80/5x90 models you can press Shift Function to display a second Function menu.

Some functions have further sub-menus which will appear when you select the main function number.

The function menu may change depending on which mode you are in, the operation you are performing (e.g. editing or viewing data), and whether the diary is in Input mode for models which have this feature. If you aren’t finding the function you want, or can’t find a function you saw before, try changing to the mode where it would be logical to find it.

The Escape key (Esc) found on most models will back out of the current menu or data entry record, and can also be used to cancel some (but not all) diary operations while they are in progress.

On models with a Set button or OK button, remember to press Set or OK when you are through making changes on a Function screen if you want to save the changes. Models which require you to press Set or OK to save changes will lose your changes if you leave that screen or turn the diary off before pressing Set or OK.

The only controls which are not on the keyboard are the Reset button (a tiny recessed button on the back, or inside the battery compartment on most models), and the screen contrast control wheel on the side of some old models.

## Settings

Some menu items related to diary settings appear by default in all modes, or always appears under a System menu item (Shift Func on the SF-5x80/5x90/6x00SY family). These include:

Sounds:

- turning the key beep sound on and off

- turning the daily alarm and schedule alarm beepers on and off (check this if your alarms don’t seem to be working!)

Display:

- adjusting the display contrast (except for old models with a contrast adjustment dial)

Language:

- selecting language on multi-language models

Secret Mode (SF-5x80/5x90/6x00SY family only)

## Setting Date and Time

The diary resets to its default date and time when you press Reset or insert batteries after leaving it with no power for more than a few minutes. The SF-5x8/5x90/6x00SY family will prompt you to set the time every time you turn the diary on until you do.

Warning: If you frequently see a prompt to “Set Time!”, it means that battery power is being interrupted and your diary is resetting. Check for weak or defective batteries, or dirty or corroded battery contacts.

To set the date and time, press the Home Time button to enter Home Time mode.

You can then edit the date and time shown by the usual data editing procedure using cursor keys and typing (remember to first select Input mode, then press Function and select Edit  for models which require this). Press Set or OK to save the new date and time on models which require this.

The Function menu in Home Time mode includes options for setting the Date and Time format, and for selecting Daylight Savings Time (moves the home time forward 1 hr without changing the World Time offsets).

The Function menu in Home Time mode also includes the option to set the home time zone This is necessary to show the correct time for various other time zones in World Time mode. On some models you select the time zone from a menu of standard zone names with principle cities. On others you select the zone graphically by stepping through the possible zones on a world map. On others you can select from a long list of city names. The SF-5x80/5x90/6x00SY family has a Zone Change function on the Function menu which lets you type a new city name to display for the selected time zone. Unfortunately the time difference is not calculated and updated automatically when you change your home time zone - you must update your home time after changing home time zone.

Press Home Time again while in Home Time mode to set the daily alarm time for the SF/CSF/NX family, or on the SF-5x80/5x90/6x00SY family select Daily Alarm Set on the Function menu. This is a separate alarm from any other schedule or reminder alarm. Remember that the alarm time sound must be enabled separately on the main System menu.

In World Time mode, the Function menu allows you to select a different time zone (by city name on higher-end models) and display the local time for that zone. The time difference from your current Home Time is calculated automatically.

Warning: alarms are relative to the Home Time. If you travel to a different time zone, and you have not updated your Home Time, your daily alarm and appointment alarms will not sound at the correct times.

You can set Daylight Savings Time individually for the current time zone on Function menu, but remember that DST should be opposite in the southern hemisphere (i.e. subtracting one hour instead of adding).

## Memory

Various diary models have memory capacities ranging from 32 Kbytes to 1024 Kbytes, and some of the older models could be extended with plug-in memory cards of 64 Kbytes to 256 Kbytes.

Not all of this memory is available for use - some is used up by the internal operating system. This unavailable memory may range from 3 Kbytes to 60 Kbytes, depending on the model. It has tended to increase over time on newer models, so beware if upgrading to a newer model of the same nominal memory capacity - it may not hold as many records.

How much memory does the item data take up? It’s pretty close to one byte per character. There is some overhead of record headers and field separators, but probably only about 10%.

On some models you can check the remaining memory capacity by pressing and holding down the Memory Capacity key (Capa), or by selecting this function from a menu. On some models Capa is a Shifted key function.

On the SF-5x80/5x90/6x00SY models, a memory usage bar graph (0 - 100%) is shown below the record count on the index screen for each data mode.

The memory is shared by all data types. When it is used up, you will get a Memory Full error message. You must then delete some data records to make room for new data.

Plug-in card memory is separate from the main memory on models which support plug-in cards. It is accessed by pressing the data mode button twice to switch between main memory and card memory (or multiple times if the card has multiple data segments).

## Data Modes

The Casio diary has several data types or modes, which you can select by pressing the button for the mode you want. These buttons are usually along the top of the keyboard, but on some low-end models you must press the Menu key and select by number from the menu instead. Depending on model, you will have some subset of the following data modes (no model has all of them):

Telephone	- all models

Memo		- all  models

Schedule	- all models

Calculator	- all models

Home/World Time	- all models

Calendar	- most models

Reminder	- all newer models, many variations

Business Card	- older models only

To Do		- some models, many variations

Expense	- some models, some variations

Free files	- SF-M10/R10/R20 models only

Sketch		- NX models only

Spreadsheet	- a few older models

Card		- older models which accept plug-in cards

Game		- a few models have simple games

When you press a mode button, normally the summary screen for that mode appears, listing the number of records in that mode. To enter data, just start typing and a blank record of the current type will appear. To view records in the current mode, either press the up/down arrow keys to start at the end/beginning of the list, or do a Search.

## Entering Data

Select the data mode and just start typing to enter a new record.

Most data types have multiple fields. Most models show a prompt at the start of an empty field to tell you the name of the field you are entering (e.g. “Name?”), but it disappears when you start to type in the field, and it is not shown when you look at the record later. It doesn’t really matter which field the data is in on the diary as long as it appears the way you want on the screen, but it can make a difference when you communicate with a PC program which depends on the data being in the correct field.

There is some variation between models, but here are the typical fields for each data type:

Telephone	- Name, Telephone No., Address, extras on some models

Memo		- Free-format memo, only one field on most models

Schedule	- Date, Start time, End time, Description on most models

Reminder	- Date, Description on most models

Business Card	- Company, Name, Position, Dept., Tel, Address, POBox, Fax, extras

To Do		- Date, Description on most models

Expense	- Date, Amount, Expense type, Payment type, Description on most models

Some models have a variable number (1-6) of extra “Free fields” at the end which can be used for user-defined notes on Telephone and Business Card items. Some models have a single “Notes” field as the last field of Telephone items. Try stepping through the fields on a new empty record by pressing the Next or OK key to see what the fields of the data type are for your model.

On older models, the diary must be in Input mode before you can edit or enter a record. Press the Data In/Out key and note that the In indicator appears on the display to put the diary in Input mode. On these models, you should press the Set key to save a new or edited record. If nothing happens when you press the Set key, the diary is probably not in Input mode. There is no error message or any other indication in this case.

On later models, you can just start typing to input new data, and the record is automatically saved when you press Next or OK enough times to step through all the fields. There is no special Input mode on these models.

You can enter any text in the text fields. Press Next or OK to move to the next field. You can press the Return key to insert a carriage return within the field (not the same as pressing  Next or OK to move to the next field). There is an overall limit on the number of characters in all fields allowed in a record: 384 characters for SF/CSF/NX models, 255 characters for SF-5x80/5x90SY models, 511 characters for the SF-6x00SY/7x00SY models. If you exceed the limit, the cursor will usually change shape, and/or you won’t be able to type any more characters.

You can move left/right/up/down within existing text by using the corresponding cursor arrow keys while entering or editing text. If you are moving left or right, the cursor will stop when you get to the end of the current field, but you can move up and down freely between fields using the up/down arrow keys.

Tip: to get to the beginning or end of the current field quickly, press Shift and then the left or right cursor arrow key.

Most models have a Shift and Caps Lock key (Caps) to allow upper and lower case characters to be entered. The Casio diary keyboard is different from a standard keyboard in that the Shift key does not have to be held down at the same time as the key you want to shift - when you press the Shift key the Shift indicator appears on the display, and the next key you type is shifted. This allows one-finger typing. The SF/CSF/NX models will stay in Caps Lock mode, but Caps Lock always resets to the off state on the SF-5x80/5x90/6x00SY models when you turn them off.

Many keys have special Shift symbols or functions marked above them on the keyboard. On some models there are additional unmarked shifted characters which you can find by experimentation.

Special symbols are handled differently on different models. Many special symbols can be accessed through the Shift key.  Some models also have a extra shift key called Code which can be combined with other keys to generate different symbols; these Code symbols are usually printed on the keyboard next to the regular shifted characters. Accented characters can be produced on the standard SF/CSF/NX family by typing a 2-character sequence: accent first, then the character to be accented. Other models have a special symbols menu which can be displayed by pressing the Symbol key (Smbl) or selecting Symbols from the function menu.

Inserting and deleting characters in a record is a little different on the Casio diary than most users are familiar with. The Delete key (or Del) deletes the character at the current cursor location like the Del key on a PC keyboard, rather than deleting the previous character and backing up one character like a Backspace-delete key). The BackSpace key (BS if you have one) or left arrow key backs up one character without deleting. On the SF-5x80/5x90/6x00SY family, there is an option on the System menu (Shift Func) to change the BackSpace key (BS) to the more usual destructive backspace which erases the character to the left of the cursor.

The Insert key (or Ins) inserts a blank character at the current cursor position and moves any following characters to the right. You can press it multiple times to insert as many blanks as necessary to allow you to insert the characters you want.

Tip: There is no way to put the diary in “insert mode”, where the characters shift to the right as you type. However, there is a shortcut for long inserts: insert one blank and then press Return to insert a line break; you can then back up one character and start typing at the end of the previous line without overtyping the next line. Just delete the Return character when you are through typing to rejoin the lines.

Date fields typically accept only numbers for the year/month/day. On some models you can select the default date order on the Function menu. On some models you must press the Time/Date key to enter the time and date fields (usually at the top-middle of the keyboard). On most models you can step forward or backward through the date field using the left-right arrow keys. On some models you can clear the date field and enter a new date by pressing Shift Clear. The Casio diary accepts 4-digit years, so it is “Year-2000 compliant” - however the shortcut of entering a 2-digit year won’t work beyond 1999, as 00 = 1980 on most models).

You can enter the time in either 12-hour (press P for PM) or 24-hour format. You can select on the Function menu in Home Time mode whether you want to display times in 12-hour (AM/PM) or 24-hour format.

When entering Schedule data on older models, you must first move to the date for which you want to enter data. The date initially shown when you select Schedule Mode is today’s date. To get to a desired date you can either step forward or backward with the up/down cursor keys, or press Shift Clear and type a new date. Shortcut tip: to get back to today’s date in the Schedule quickly, select another mode like Memo Mode and then switch back to Schedule Mode.

On newer models you enter the date as part of entering the Schedule item.

On some models the alarm time for Schedule items must be entered by pressing the dedicated Schedule Alarm key while entering the item.

Most fields which have special values (e.g. the Expense amount or Payment type) are actually text fields which will accept any values. However on some models the values for these special fields must be selected from a menu. Normally the menu will appear when you enter that field.

## Displaying Data

Press the data mode key for the data type you want to look at (e.g. Telephone, Memo, Schedule). This will display the summary screen showing the number of records of that type for most data types, or the current date time-bar for Schedule mode on some models.

Press the down arrow key to display the index list of records beginning with the first record.  The index list shows one record per line. Press the down arrow key again to step to the next record.

OR

Press the up arrow key to display the index list of records beginning with the last record. Press the up arrow key again to step to the previous record.

You can press the Display Change key (or Disp Chng) to switch back and forth between the index list of records and the full-screen view of the individual record the cursor is on in the list. In Schedule mode Display Change switches back and forth between the time-bar view and the item list view. Hidden function on some models: press Display Change in Calendar Mode to see a weekly overview of your Schedule.

While displaying a detailed record, the arrow keys move back and forth in the record - you must switch back to the index list view to move through the list of records by one record at a time. The arrow keys will move up and down one day at a time in the Schedule time bar display mode. Some models also have separate PageUp/PageDown keys (shown as bigger arrows or double arrows next to the cursor arrow keys) which move up and down by one screen-full in the index list view, or one record at a time in the detail view. On models with separate PageUp/PageDown keys, these will advance to the next scheduled item in Schedule mode, even if there are empty days in between.

Data is listed in sorted order, regardless of what order it was entered in. Telephone data is sorted in alphabetical order of Name. Business Card data is sorted in alphabetical order of Company, then Name. Schedule and Reminder data is sorted in order of Date, then Time. Expense data is sorted in order of date. ToDo data is sorted in order of priority (if any). Memo data is unsorted, except on the SF-5x80/5x90/6x00SY family where it is sorted in order of date.

Note that the Casio diary considers alphabetical order to be A-Z, then a-z (i.e. ASCII sort order).

On the SF-5x80/5x90 family you can select which data field you want to see listed in the index list for that data mode by pressing Func and selecting List type from the function menu.

## Searching for Data

Select the data mode you want to search (e.g. Telephone, Memo, Schedule).

Type the character string you want to search for.

Press the Search key. A list of matching records or the first matching record will appear,  depending on model number. A list may extend to more than one page, and there may not be any visible indicator that there are more items found than are shown on the first page.

This is a text search. It will find the text string embedded in any longer string. It is CASE SENSITIVE! (there is no way to do a case-insensitive search).

There is no way to search multiple data modes at once.

On the SF-5x80/5x90/6x00SY family, there is no separate Search key - instead you can perform this function by pressing Func and selecting Wild card search from the function menu.

There is a Combine feature on some models which allows you to create an “AND” condition. To do this, enter the first search string, press the Combine key (it inserts a special symbol on the line), then type the next search string. You can add more than one AND condition using Combine.

One more feature: you can press Shift Search instead of Search to match only the starting text of the record (e.g. the beginning of the name in the Telephone list). For example, entering “Jim” and pressing Shift Search will find “Jim Jones” faster then pressing Search, but won’t match “Jones, Jim”. On the SF-5x80/5x90 family, there is no separate Search key - instead you can perform this function by just entering the first few characters to search for, and then pressing the PageDown or PageUp key (looks like double arrows).

In data modes organized by date (such as Schedule), you can jump directly to a specific date: on the standard SF/CSF/NX models, press Shift Clear and type the date you want to go to; on the SF-5x80/5x90/6x00SY models press Func, select the function Date Search, then type the date you want to go to. You can then move back and forth through the items on that date and nearby dates using the movement keys as usual.

## Editing Data

Display the detailed view of the record you want to edit.

On older models, make sure that the diary is in Input mode (press the Data In/Out key if the In indicator is not shown on the display), then select Edit from the Function menu (press Function, then 1 for Edit).

On newer models, just cursor to the characters you want to change and start typing.

On models with a Set button or OK button, remember to press Set or OK when you are through if you want to save the changes to the record. Models which require you to press Set or OK to save changes will lose your changes if you leave that screen or turn the diary off before pressing Set/OK. The other models will generally save the changes you have typed once you have finished.

On the standard SF/CSF/NX models, the Function menu in Schedule mode provides an option to copy or move the current Schedule entry to another date. Unfortunately the SF-5x80/5x90/6x00SY models lack this useful function.

Some older models have a feature called Letter Memory which is intended to save any commonly-typed text string under a single-digit code 0-9. To set a Letter Memory, type any text string in Input mode, then press the Letter key on the keyboard, then Shift, then a number 0-9. You can then insert this string into data you are entering or editing by pressing Letter plus the number 0-9.

Tip: The Letter Memory feature can also be used as a copy and paste clipboard: while editing any item text, use the above procedure to assign the text to a Letter Memory; you can then paste a copy to the same item or to any other item.

The SF-M10/R10/R20 models are notorious for becoming slower and slower at accessing data as they become full and the data is frequently edited. This is normal, and can be fixed by clearing the memory and reloading the data in sequential order from backup.

## Deleting Data

You can delete an existing data item by selecting the Delete Item option from the Function menu while editing the item.

In most data modes there is an option to delete multiple items on the Function menu of the mode index display. For example in Schedule mode, you can delete all items from the current date to a specific future date, which allows you to delete all items in a date range.

On the SF-5x80/5x90/6x00SY models, for those data modes with 3 separate files there is a function to Delete all files - this function actually gives you the option to select any or all of the 3 files to delete for that data mode.

On the SF-5x80/5x90/6x00SY models, there is a function to delete only the listed schedule items, meaning those whose Schedule Type has been selected to list under the List type function.

Some models let you “mark” data items using the Mark key. On the older SF/CSF/NX models, marking items protects them against deletion. Don’t use this function if you back up your data regularly - it’s unnecessary and causes problems when you try to clear out old data or reload from backup.

On the SF-5x80/5x90/6x00SY models, things are opposite: marking an item designates it for deletion using the Delete marked records function!

Deleting all data in non-Secret mode does not delete the data in Secret mode, and vice-versa (except for Calendar highlights!).

To delete everything in memory at once, you can press the Reset button on the back. But note that this will also reset the date/time and any options you have set to the factory defaults.

Shortcut: on the SF-5x80/5x90/6x00SY models, you can press Shift Func  to get the System menu, and select Delete all - this gives you the option to select specifically which data modes you want to delete.

If you delete a large number of records, it is normal for it to take some time (up to a few minutes) to delete on any diary model.

## Telephone Items

Casio Telephone items include at least Name, Telephone Number, and Address fields. Most models support at least one additional Notes field. Some models support up to 6 additional Free Fields which can have user-assigned field names (a Function menu option).

The SF-5x80/5x90/6x00SY models have 5 additional fixed fields for 2 additional telephone numbers, Company, Email address, and Title/Notes. Any text can be entered in any of the fields up to the total limit of number of characters in the record (e.g. the Telephone number fields can contain letters and symbols in addition to digits). None of the fields are required, and any of them can left blank. Telephone data is sorted in order of Name.

In addition, the SF-5x80/5x90/6x00SY odels have 3 separate Tel files, initially named Tel-1, Tel-2, and Tel-3 which you can cycle between by pressing Tel repeatedly. You can change these default titles by pressing Func and selecting the function File Title from the Function menu. You can display a summary of the record counts in all 3 files by pressing Func and selecting the function Record Summary from the Function menu.

The SF-A10/20/30 models have separate data modes for Personal Telephone and Business Telephone.

## Business Card Items

Casio Business Card items (on older models which support this data type) are very similar to Telephone items except that the pre-defined text fields also include Company, Position, Department, P.O. Box, Fax, and Telex. Business Card data is sorted in order of Company first, then Name, and is shown listed by Company in the index list.

## Memo Items

Casio Memo items have a single free-format text field (except for the SF-5x80/5x90/6x00SY models, which add a date field). Memos are unsorted. You can add a memo ahead of an exiting one in the list order by scrolling to the location of the existing memo in the index list, and then starting to type a new memo.

The SF-5x80/5x90.6x00SY models have 3 separate Memo files, initially named Memo-1, Memo-2, and Memo-3 which you can cycle between by pressing Memo repeatedly.

## Schedule Items

Casio Schedule items include at least a Date, Time, and text Description. Most models support a separate Start Time and End Time, allowing schedule items to have a duration. You generally do not have to specify either Start Time or End Time, but you must specify a Start Time before you can specify and End Time.

Some models support multi-day or “Term” Schedule Items, which can extend over multiple days. This type of schedule item must be selected from the Function menu (you must press the Function key twice in Schedule mode to access this feature on some models).

Most models support a separate Alarm Time for Schedule items. On older models you must press the separate Schedule Alarm key while entering or editing the schedule item to set the alarm. The alarm can only be set for a time on the same day as the schedule item (except for the SF-5x80/5x90/6x00SY models, which allow a -24 hr alarm to be specified for the day before by pressing the up/down arrow keys while in the Alarm Time field). The audible alarm will sound for 30 seconds when the alarm time is reached, even if the diary is closed and turned off (unless you have disabled all schedule alarms on the System Sounds function menu). When you open the diary, the item which caused the alarm will be displayed. You cannot have two items set to the same alarm time.

Schedule Type - On the SF-5x80/5x90/6x00SY models Schedule items can have a Schedule Type. The default types are initially set to SCH TYPE 1-8, but the user can edit the name and icon of each of the 8 schedule types by pressing Func and selecting Define Type from the Function menu. The idea is to assign standard types like “Business appointment”, “Party”, “Medical appointment” etc., each with their own icon. The function menu in Schedule mode provides the option List type to list and check off which of the 8 schedule types should be displayed, on the theory that you might want to see only a subset of your appointments.

Restriction on moving Schedule items - The SF-5x80/5x90/6x00SY models have one significant operational restriction compared to the other Casio diaries: it is impossible to move Schedule items between dates - you must erase the item and re-enter it on the new date. On other models, the functions Copy to date and Move to date appear on the Function menu.

## Reminders

Casio Reminders are scheduled items which repeat on a regular basis. The older models do not support Reminders. Most later models support simple annual reminders, i.e. those which recur on a specific day of the year such as birthdays and fixed holidays. Some of these models also allow monthly and daily reminders. Only the SF-M10/R10/R20 models support the more flexible Reminder-2 data type which allows reminders to be set weekly, or on specific days of the week, or in a specific week of the month.

Some models allow an event time to be specified for each Reminder, while others do not have only a date for each Reminder. Some models allow an alarm time to be specified for each Reminder. The audible Reminder alarms can be globally enabled and disabled on the System function menu. The SF-5x80/5x90/6x00SY models uniquely allow an alarm to be specified as “previous day”, in which case it occurs 24 hours before the event time.

Some models allow a graphical icon to be specified for each reminder item, selected from a menu of icons when entering or editing the item. The icon is intended as a quick visual indicator of the Reminder event type. e.g. a birthday cake for birthdays.

Unfortunately the Casio diaries do not show a daily agenda view combining both Schedule and Reminder items - each data mode must be selected separately.

## Calendar Mode

The Calendar Mode displays a perpetual calendar for either one month at a time (small-screen models) or two months at a time (large screen models).  If there are two months displayed, the one on the left is always the “active” month. Days with Schedule items specified for them are shown with dots beside them in the calendar: a high dot for an item scheduled before noon, a low dot for an item scheduled after noon, or both for items scheduled both morning and afternoon or with no specified time.

You can “highlight” individual days on the Calendar by moving the cursor to that day and pressing Set or OK. Most models also allow you to highlight specific days of the week across a range of dates through a function on the Function menu.

On the CSF/NX color models you can also press the color keys to apply one of the 3 primary colors to a day on the calendar. Highlighted days are shown in inverted color.

You can move forward or backward in the calendar by one month at a time by pressing the cursor up/down keys. To move the cursor to a specific day within a month, press the left/right cursor keys first. To go to a specific month, press Shift Clear and enter the month (or select Date Search from the Function menu for SF-5x80/5x90/6x00SY models).

On the SF-5x80/5x90/6x00SY models you can also can also press + or -  and type a number of days to add to or subtract from the current date (answer N  to exclude highlighted holidays when prompted whether to include holidays).

Some models have a function to count the number of working days (not including highlighted holidays) between any two days on the calendar. On the standard SF/CSF/NX series, move the cursor to the start date, press the Time/Date key, and then move the cursor to the end date. On the SF-5x80/5x90/6x00SY models, move the cursor to the start date and then select Working Day Count from the Function menu, then move the cursor to the end date..

When you move from Calendar Mode to Schedule Mode, the day initially shown in Schedule mode will be the day the cursor is on in the Calendar (if any). When you move from Schedule mode to Calendar mode, the calendar month shown will correspond to the date currently shown in Schedule mode, and the cursor will be on the corresponding day in the calendar.

## To Do Items

Casio ToDo items are basically just a list of text items which can be marked “Completed” or otherwise checked off when done. The older models do not support ToDo items.

Some models record the completion date with the ToDo item when it is completed. Other models such as the CSF/NX series allow a separate Due Date, Due Time, and Completion Date to be specified.

A few models allow a priority to be specified. With the CSF/NX color models the priority is A, B, C and it is color-coded. With other models the priority is 1, 2, 3…. With some models it can be either.

To mark a ToDo item completed on the SF-5x80/5x90/6x00SY models, edit it and press OK.

The SF-5x80/5x90/6x00SY models have 3 separate ToDo files, initially named ToDo-1, ToDo-2, and ToDo-3 which you can cycle between by pressing ToDo repeatedly.

The SF-5x80/5x90/6x00SY models also allow you to select which ToDo priorities to display (function List type, similar to that for Schedule Type).

## Expense Items

Casio Expense items include at least the Date, Amount, and text Description on all models which support Expenses.

Some models include a Payment Type (e.g. “Cash”). Other models also include an Expense Type (e.g. “Food”). The Payment Type and Expense Type can be selected from a list when entering or editing Expense items. The list of pre-defined Payment Types and Expense Types can be edited by the user through the Function menu. The number of pre-defined types varies from 8-10 depending on the model. The SF-5x80/5x90/6x00SY models let you assign an icon to each Expense type using the Expense types function on the Function menu.

Some models include expense category check fields for “Business Expense?” and “Receipt?”.

Some models have a function to subtotal expenses by Expense Type or Date. On the SF-5x80/5x90/6x00SY models, press Func from the Expense index list display and select Report. There is also a function on this menu to set an exchange rate to multiply all the expense amounts by in the report (only useful if everything is in one currency, and you can use one exchange rate).

The SF-5x80/5x90/6x00SY models have 3 separate Expense files, initially named Exp-1, Exp-2, and Exp-3 which you can cycle between by pressing Exp repeatedly.

The SF-5x80/5x90/6x00SY models also allow you to select which Expense types to display (function List type, similar to that for Schedule Type).

## Calculator Mode

On most models this is a pretty standard 5-function calculator with memory. The memory value is retained when the Calculator mode is not in use or the unit is turned off. Unfortunately there is no way to paste the result of a calculation into another data mode.

Various models support different number of digits accuracy. Some also have a scrolling display which shows you the last few values entered or calculated.

Some models have a separate numeric keypad which makes it easier to use the calculator, but others just have calculator mode symbols printed in orange above the regular keys on the keyboard. This can be a little confusing, as these symbols are easy to miss unless you look closely, and they can be different from the equivalent normal keys (e.g. the divide / key in Calculator mode is not the same as the normal / key on the keyboard!).

Some models will allow you to do date calculations like subtracting two dates to find the number of days between them; use the Time/Date key to enter date values for this type of calculation.

Some models have a unit converter built in. On the SF-5x80/5x90/6x00SY models, you can access this converter by pressing the Conv/Game key (press Shift Conv/Game to access the games). Use the cursor keys to select or change the conversion units.

## Secret Mode

** Important **: If you are going to use the Digital Diary to store confidential information such as account numbers and passwords, it is absolutely essential that you use Secret mode! Think about how you’ll feel if your diary is ever lost or stolen with all your confidential information in it. On the other hand, if you use Secret mode, and you do frequent backups to your PC, you will be out only the price of a new diary if you ever lose it.

The Secret mode operates differently on different models.

### Secret Mode - Standard SF/CSF/NX Models

The standard SF/CSF/NX models allow you to have separate Secret and non-Secret areas. The records in the regular non-secret area can be seen and edited normally without entering a password. To switch to the Secret area, you must press the Secret key (it is labeled with a key symbol on most models) and enter the correct Password when prompted. The Secret mode symbol will appear on the display, and you will see only the data in the Secret area. To leave the Secret area, just press the Secret key again.

When the diary is not in Secret mode, you can’t see any of the records in the secret area, or even how many records there are. They do share the same memory though, so you could subtract the memory used by the non-secret records from the normal “empty” capacity to figure out how much memory is used by the secret records (taking into account the memory used by the operating system, of course).

If you have not entered a password before, you will be prompted for a new password the first time you press the Secret key. Write it down somewhere if you might not remember it, and note that the password is CASE SENSITIVE.

If you forget your password, there is no way to recover it or to access the Secret data. You can only Reset to clear the entire memory and start again.

To change an existing password, enter Secret mode and then press Clear and then the Secret key. The password will appear. Press Function then select 1 - Edit to edit the password, then press the Secret key again when finished editing to save the change.

The data item editing menu has an option to move individual items in or out of Secret mode.

### Secret Mode - SF-5x80/5x90/6x00SY models

The SF-5x80/5x90/6x00SY models have a Secret mode which protects all data areas - there is no separate secret and non-secret memory area. When Secret mode is turned off, all the records in can be seen and edited normally without entering a password. To switch on the Secret mode, press Shift Func, select Password, then enter a new password up to 6 characters long and press OK. Once the password is set, you will be prompted to enter the password each time you turn the diary on. If you do not enter the correct password and press OK, you will not be allowed to proceed past the Password? prompt.

To turn Secret mode off, you must first enter the correct password to allow access to the diary, then press Shift Func and select Delete Password. To edit the password, press Shift Func and select Edit Password.

If you forget your password, there is no way to recover it or to access the Secret data. You can only Reset to clear the entire memory and start again.

## Communications

### How to Send and Receive Data

Most Casio Digital Diary models have a small serial port on the side which allows them to communicate with other similar models, or with a PC. However, the least expensive models do not have this serial port, and therefore cannot communicate externally at all.

The following steps are necessary to send data to another Casio diary or PC backup program:

- Connect a compatible cable (see below), pushing the small plug end of the cable firmly into the port on the side of the diary until it clicks into place. Note: most models have a small rubber plug protecting the port which you must remove - be careful not to lose it.

- Connect the other end of the cable to the other Casio diary or to the PC serial port.

- On models which allow the communications parameters to be changed, verify or set these parameters (the SF-5x80/5x90/6x00SY models use fixed settings which cannot be displayed or changed). On the standard SF/CSF/NX models, you can only access communications from the index screen of any data mode. To access the communications parameters, press the Function key (twice on some models) and select Data Communications, then Set Hardware Parameters (your menus may differ slightly). Suggestion: use the following settings by default: 9600 bps, bit length 8, parity None. However, any settings are OK as long as the other end is set to match. A few older diary models have a maximum rate of 4800 bps and cannot be set to 9600 bps.

- Make sure that the communications parameters are set to the same values on the other Casio diary or PC serial port. If you are using a backup program on the PC, you would normally have to set these parameters within the program rather than within the operating system (e.g. Windows).

- Put the receiving Casio diary in Receive mode: make sure that the diary is in Input mode if your model requires this (press the Data In/Out key if the In indicator is not shown on the display), then press Function (twice on some models) and select Data Communications, then Receive.

- The diary will show “Receive OK”. On the SF-5x80/5x90/6x00SY models, you must first answer Y to the question “Ready to Receive?”, which will put the diary in “Receiving” mode.

- If you are sending to a PC backup program instead, the actions required to put the backup program in receive mode will depend on your particular program.

- Start sending data from the sending end (** Important **: the receiver must be ready and waiting in Receive mode BEFORE you start sending).

- On the standard SF/CSF/NX models, you can only access communications from the index screen of any data mode. From the index screen, press the Function key (twice on some models) and select Communications, then Transmit or Send (depending on your model), then select one of the options:

- One Data Item (a single data item which you must select by searching)

- Mode Data Items (all data items in the current mode)

- All Data Items (all data items in all modes)

- and press the Set key to start sending data when prompted

- If you select One Data Item, you must use the Search function to find the first record you want to send, or step through your records with the cursor arrow keys to find it. The diary will stay in this Send One Item mode after you send the first record, so you can continue to find other records and send them one at a time while the receiver stays in Receive mode. Press Esc to exit Send One Item mode when finished.

- On the SF-5x80/5x90/6x00SY models, the communications options you see on the Function menu will depend on whether you were displaying a single record, an index list, or the index screen. They may include:

- Send record (the single data item you were displaying)

- Send all files (select which files in the current mode you want to send)

- Send all (select which of all data modes you want to send)

- Select which data to send, then press the OK key to start sending data when prompted.

If you are sending from a PC backup program instead, the actions required to start the program sending data will depend on your particular program.

If you have done everything correctly, you will see “Transmitting…” or “Sending…” on the sending unit. In some cases an icon may blink or rotate to show activity, but in fact this does not necessarily mean that any data is actively being communicated.

If the sender fails to establish contact with the receiver within a few seconds according the Casio communications protocol, it will show “Transmit Error!” or “Send Error!”. This message basically says nothing more than that the diary failed to establish communications. There are many possible reasons for this - see the Communications Troubleshooting guide in CASIF/Win for suggestions.

If all goes well, the receiving diary will show “Receiving..”. Depending on the model, it may show individual records as they are received, or it may show a blinking or rotating icon to indicate activity.

If you need to cancel the operation, you can usually do this by pressing Escape (Esc) on the diary. If that doesn’t work, try pressing a data mode key, or just turn the unit off and on again.

When the transmission is complete, both diaries will return to their standard data display. (The SF-5x80/5x90/6x00SY models do not automatically come out of Receive mode - you must press Esc on the receiver or wait 30 sec. for it to time out).

If the transmission is interrupted for any reason, the records transferred so far will be saved correctly by the receiver, but the transmission must be restarted from the beginning.

The Casio diary uses software flow control (Xon/Xoff) to control the data rate, so you will notice that data does not flow continuously at the maximum rate. When receiving data, most diary models will pause for a moment as each complete record is received in order to index it properly in their memory. The Casio diary uses checksums to verify each data field as it is received, and the receiver will automatically ask the sender for a “retry” if any communications error is detected.

Some Casio diary models will automatically recognize and discard duplicate records when receiving data, such as the SF-M10/R10/R20 models and the SF-5x80/5x90/6x00SY models. All the other models do not recognize duplicate records, and will create a second copy of any duplicate record.

The more recent SF-5x90SY/6x00SY/7x00SY models are the only models which allow true data synchronization. For all other models you must manually delete any outdated or replaced records in the diary. The Synch procotol is different from the normal Send and Receive protocol, and must be used with special Synch software on the PC. You can still use Send and Receive to exchange data with older diaries and PC backup software.

### Serial Cables

The low-level communications protocol of the Casio serial port is a standard RS-232 serial protocol compatible with PCs and Macs, but the electrical interface is not (see the important notes below).

For most models the plug at the diary end is a 2.5mm stereo plug with 3 contacts (serial pins Send, Receive, and Ground). Note that this size is smaller than the standard 3.5mm stereo plug, and it can be hard to find.

The SF-5x90/6x00/7x00SY models use a different cable with a 3.5mm plug.

Warning: the 3.5mm port operates at a different voltage than the 2.5mm port. Do not try to use a size adaptor with to connect a 2.5mm cable to a 3.5mm port or vice-versa!

The Casio SB-60/62 cable is just a 2.5mm-to-2.5mm connector which crosses over Send and Receive to allow two Digital Diary units to communicate with each other.

Serial adaptors are available from Casio and other sources to connect the Casio diary to a standard 9-pin or 25-pin PC serial port, or a Mac DIN serial port. A special voltage adaptor is needed, usually embedded in the serial cable, as the Casio diary operates on 5 volts DC, while standard RS-232 serial ports require 7.5-12 volts DC. The Casio serial cables include this voltage adaptor, powered either from batteries or from the PC serial port itself. Warning: Do not attempt to connect a Casio Digital Diary directly to a standard RS-232 serial port without the correct voltage adaptor cable, or you will undoubtedly destroy it!

Cables which are powered from the PC serial port usually draw their +/-12 volt power either from the RTS pin, or the DTR pin, or both. However different cable designs vary in how they use these pins, so the software you use should match the cable or it may not receive the electrical power it needs to function properly. If you bought the cable and software as a kit, the software will use the correct settings automatically. If you are using the cable with software acquired separately, make sure they are compatible. If you are using CASIF/Win, you can change the serial pin settings to match your cable under Settings/Communications.

The same type of serial cable is used for all Casio diary models (with the exception of the SF-5x90SY/6x00SY models), all Casio programmable calculators, and all Casio QV digital cameras. Cables can be interchanged between these units as long as the software you use sets the RTS and DTR pins correctly. Cables are available from Casio and from many other sources, with and without software. Simple plans for building your own cable are available on the Internet, and mail-order cables are available for those not handy with a soldering iron.

The SF-5x90SY/6x00SY/7x00SY models are the only models to use a different cable. These models operate at a higher voltage, and they therefore have a 3.5mm plug on their cable in order to avoid accidental use with other models.

### Backup Software

Casio uses its own unique communications protocol to back up data or exchange data with another unit. There is no general-purpose communications program built into any Casio Digital Diary model, and it is not possible to use the Casio diary as a text terminal or email device (just in case you were wondering).

Software to support the Casio serial communications protocol is available for DOS, Windows, MacOS, and Unix. Commercial software for DOS, Windows, and Mac is available from Casio, and DOS and Windows software is available from other vendors, usually sold with a kit including the serial cable. Casio kits include the FA-100 (DOS software), FA-126 (Windows software), and FA-127/128 (Win9x software). Shareware software such as XLink/Win or CASIF/Win (http://www.imslsoft.com) is available on the Internet. Software is available for DOS, Windows, MacOS, and Unix. Unfortunately for non-Windows users, only Windows software is currently up-to-date and available to support all models. XLink/Win can be used on Mac and Unix systems with a Windows emulator though.

As noted above, there are 3 major diary families with different communications protocols. Most software packages support only one or two families, so check that the software you select is compatible with your model. There are also many model variations within the standard SF/CSF/NX series which are not supported by all software packages. Older software packages may not fully support the features or data types of more recent models. Also you may find that older DOS communications software packages may not work on newer and faster PCs due to timing problems. If you find yourself with this problem, try a more recent shareware package like CASIF/Win.

## Outdated Stuff

### Plug-in Cards

Some older Casio models such as the SF-R10/R20 and a few models in the SF-9000 series can use proprietary Casio plug-in memory and program cards. These models have a row of function keys F1-F5 and Key Shift key across the top of the keyboard for use by card programs. These keys are not used for anything else.

Cards slide into a slot in the side of the diary. There is a Lock/Unlock switch beside the card slot which must be in the Unlock position to insert or remove a card. There is a slider on the front of the diary to help pop the card out of the unit when removing it. These diary models have a placeholder plastic card which should be inserted in the card slot to keep dust out if no actual card is inserted.

The Casio cards are almost unavailable now, so if you have one of these older models and you don’t have any cards, forget about finding any cards now.

The memory on the memory cards is separate and distinct from the main memory in the diary - it must be displayed, searched, and backed up separately. There is an option on the Data Communications to move data back and forth between card memory and main memory.

Each card has its own memory backup battery in a small slot at the end. Memory cards must be initialized before first use, or if they lose power or otherwise become corrupted; a Function menu option is available to do this.

The card programs are unique, so no general advice on using them can be given. Note that some card programs (like the Lucid 3D spreadsheet card) do not follow Casio’s standard keyboard conventions, and may operate differently than the general instructions given above.

( IMSL Software has the ES-100 64 Kbyte memory card, the ES-630 Lucid 3D spreadsheet card, and the ES-610 Dictionary/Thesaurus card if you need advice on using any of these items).

### Lucid 3D Spreadsheet

This spreadsheet was a built-in option on some older models, and is also available as a card program for those models which support plug-in cards. An excellent spreadsheet in its own right, and well-adapted to the small screen, but:

- don’t bother trying to translate to and from other spreadsheet formats like Lotus or Excel - the software claims to be able to do this, but it doesn’t work very well even in later updates from Lucid

- Serious Bug 1:  do not try to read a Lucid 3D version 2 spreadsheet (from a diary model with built-in spreadsheet) on a model with the ES-630 plug-in card version - it will crash the diary and require a hard reset which wipes out all memory

- Serious Bug 2:  the search function in the spreadsheet can corrupt main memory in the diary - don’t use it!

### Infrared “Secret Sender”

A few low-end Casio diary models aimed at children have been equipped with a “Secret Sender” infrared communications port capable of wireless communication with other similar units. This is not a standard PC IrDA port, and the protocol is not compatible with any other device.

### Printing Data

Some older models support a Print function on the Data Communications menu which will print formatted data directly to a serial printer. Little use for this option these days, so ignore it. Newer models don’t have it.

### Pen Printing Option

This option on the Data Communications menu of a few models supports a unique Casio I/O device: a data pen with a dot-matrix head which could be swept across a sheet of paper to print data from the diary - very few produced, no longer available.

### Floppy Disk Option

Believe it or not, some older models supported a direct floppy disk serial interface. Casio probably produced only a handful of these specialized floppy disk units in Japan, but just in case you wondered why this option was on the Data Communications menu…

### IP800 Pager

This unique model was a pager wedded to a Casio diary of the standard SF series. No instructions available for the pager portion, but the diary portion is standard.

## Troubleshooting

### Help! I have an error message!

Here is a list of common error messages with their causes and what you can do about them:

Memory full - the memory is full, dummy! Erase something to make room.

Data Error! - uh, oh! Your memory has developed a checksum error, probably due to a random electrostatic discharge or power fluctuation. This is common after changing batteries with some models. Unfortunately the only way to clear this error is to Reset the unit and erase the memory. If you are very lucky, you may still be able to access part of your data and even back it up in pieces over the communications link before doing this. If you encounter the error location partway through your data list and can’t access the records after the point of error, try backing up from the end of the data (i.e. start from the index screen and press the up arrow). You probably won’t be able to do a complete backup in one operation, because it will halt when it encounters the error location.

Batteries weak - what it says. Change them. If it recurs immediately, check that you inserted new batteries, check that you inserted them the right way round, check that the batteries are OK (or try a second set), then call Casio if the problem persists.

Transmit Error! (or Send Error!) - (while transmitting) Communications was not successfully established with the receiver, or something went wrong while transmitting. This message doesn’t give any details. Serial communications setup can be complex the first time you try it - check the detailed serial communications troubleshooting list. If this appears during a transmission after it started successfully, look for a loose cable or low batteries.

### Help! It’s broken!

Won’t turn on - Hopefully it’s just that the batteries are dead. Change them. If you already changed them, double-check that you put them in the right way round. Next step would be to try pressing the On key repeatedly, and if it seems intermittent, then try opening up the keyboard and cleaning it. If it’s something else, you are probably out of luck.

Funny lines on the display - If you have a model with the display on the hinged lid and you feel ambitious, open it up and check that the ribbon cable through the hinge is not pinched anywhere. Otherwise, back up the data, then throw it out and buy a new model.

Display broken - buy a new one and don’t sit on it next time. Actually, if you have a very expensive model which you really like, a Casio service center can replace a broken display, but it’s expensive. You may be able to back up your data by “feel” if you are lucky enough to have a backup program and you remember the exact communications key sequence.

Dust under the screen - annoying, but not serious. If it really bothers you, you can VERY CAREFULLY peel back the plastic cover over the display, clean off the dust, and then replace it. People report having done this successfully - personally, I wouldn’t.

Sticky keys or keys not working - the keypad membrane used by Casio can be prone to these problems if you are not careful to keep it clean. If you are ambitious, you can open the case, VERY CAREFULLY remove the circuit board, peel the keyboard membrane off, clean the membrane and the circuit board surface with distilled water, and put it back exactly as you found it.

I forgot my password - There is no way to recover your data. Press Reset and write it down next time. (If you entered the cure for cancer or the number of your multi-million $ Swiss bank account in Secret mode, Casio has been known to retrieve vital data on special request…)

A game or a Test Menu appeared when I turned it on - Congratulations! You have accidentally triggered one of the hidden modes in the firmware. No-one knows how this happens - check it out for a while, then try turning the unit off and on again. If that doesn’t cure it, try Reset (so long, data!). If that doesn’t work, call Casio.

I get a lot of communications errors (SF-5500B) - Unfortunately the SF-5500B model has a design defect in its electrical interface which makes it especially prone to random communications errors with many common Casio serial cables. Some backup software packages will detect and correct these errors on-the-fly by “retrying” while communicating, while others will just pass the bad data along. If you are getting too many errors to correct, or the communications operation terminates unsuccessfully, you may need a different cable. The Casio FA-100 interface (the battery-powered box) has no problem with the SF-5500B. The standard PCLink cable encounters more errors, but will usually work adequately.

## SF-5x80/5x90/6x00/7x00SY Models

Most of the differences and unique features of the SF-5x80/5x90/6x00SY are described in context throughout this document. This section is a quick summary of the main differences between these models and the standard SF/CSF/NX series.

Display - The SF-5x80/5x90SY models have a fairly large 26-character by 10-line display, complete with an electro-luminescent backlight. The backlight consumes a lot of power, so it shuts off automatically after a few minutes. They use AAA batteries as their main power source, with a separate memory backup battery.

The SF-6x00SY and SF-7x00SY models have a wider display, and use AA size batteries. The only other significant change is that they allow entries to be up to 511 characters instead of 255 characters.

Reset (or Start) button - pressing the recessed Reset/Start button on the back of these models causes a prompt to be displayed: “…New Unit ..(Y/N)?”. If you answer Yes, the memory is erased just like the other Casio models. If you answer No, the processor is reset (which may suffice to clear a temporary error), but the memory is not cleared. If you are trying to fix a problem, always try answering No first to see if you can fix the problem without erasing the memory. After a reset the date and time are set to the default, and you will be prompted to set the time each time you turn the diary on until you do.

Extended Telephone data type - these models have an extended Telephone data type with some additional pre-defined fields, including some new ones (e.g. Email) and some which were found in the Business Card data type on the older diaries (e.g. Fax).

Multiple Files -  these models have 3 separate files for each of the following data types: Telephone, Memo, Expense, ToDo. These files are intended for storing different categories of data separately, e.g. personal vs. business.

Restriction on moving Schedule items -  these models have one significant operational restriction compared to the other Casio diaries: it is impossible to move Schedule items between dates - you must erase the item and re-enter it on the new date.

Record size restriction - maximum record size on the SF-5x80/5x90SY models is 255 characters instead of 384 characters on the older SF models. The SF-6x00/7x00SY models allow 511 characters.

Games - you can press Shift Conv/Game to access a built-in Poker game. From the Poker game you can press M to access the Blackjack mode.

Sync function - the SF-5x90SY/6x00SY/7x00SY models have a new Sync function which synchronizes data automatically between two SY models or with the FA-128 PC backup program. This function does not work between the SY models and other Casio diary models, or with other backup software.

## FAQ and Tips

Which model to buy? What features to look for? - This is a matter of individual preference, and the models available change rapidly anyway. Casio has made over 100 models of the Digital Diary precisely because individual tastes differ and technology changes quickly.

The only important rule is: Do NOT buy any model without a communications port! Without the ability to back up the data you put into it, a digital diary is nothing more than a time-wasting toy.

These are some of the other factors to consider:

- Do you want something small enough to slip into a pocket? If so, only the smallest models (like the current SF-4985ER) will do.

- Do you want a big screen which shows lots of characters so you can see all the data in a record at once instead of in pieces? If so, only the SF-6x00SY/7x00SY models currently offer this (26 x 10 character screen). The SF-M10/R10/R20 models were the champs with a 40 x 10 character screen, but they are no longer available.

- Do you want a screen which shows big characters which are easier to read? If so, the SF-5x80/5x90/6x00SY series and the new SF-4900L models are pretty good. Avoid models like the SF-7900/8900 which squeeze more characters into a small screen by using small characters.

- Do you want a bigger keyboard which is easier to type on? If so, the SF-5x80/5x90/6x00SY series and the new SF-4900L models have larger keyboards than most. Avoid the old models with flat membrane keyboards which were harder to type on, like the SF-7500.

- Do you want a backlight for easier viewing in low light conditions? Many of the newer models have this feature, but most older ones don’t. Also, it eats up the batteries very quickly.

- How much memory do you need? Most people find 64 Kbytes perfectly adequate for storing their personal phone numbers and appointments, but you may need more if you store long lists of business phone numbers and large memos.

- Do you need it to be compatible with another diary you already own? If so, read the notes in the Digital Diary Models section about compatibility between diary families.

- Any features or data types especially valuable? Depends on your personal usage - all models store basic telephone, memo, and schedule data just fine. All have a built-in Calendar and Calculator. Most people find Reminders handy, while ToDo and Expenses can easily be duplicated by Memos.

Do I need to buy a backup kit? - Absolutely essential! Don’t buy the diary if you can’t afford a backup kit to go with it. This doesn’t have to be as expensive as the Casio kits: you can mail-order the cable (or build your own) and obtain shareware software for about $40.

How often should I back up? - Depends on how often you update the data in the diary. In any case, Murphy’s Law says that you will forget to back up just before your diary gets broken or lost. Unfortunately there is no easy way to do incremental backups of the Casio diary (except for the Synch models), so the easiest option is to do a complete backup and let the PC backup software sort out the changes. However, a complete backup does take time (typically about 5 minutes) and uses up the batteries faster than normal use, so you might not want to do it every day if you have only made minor changes.

Should I enter data in ALL CAPS or Upper/Lower case? - A matter of personal preference. Some people find mixed case more readable, others prefer the larger size capital letters. If you are transferring your data to and from a PC database, you will probably prefer to use mixed case. The SF/CSF/NX models retain the CAPS LOCK setting when you turn them off, but the SF-5x80/5x90/6x00SY models do not.

Should I use First Name + Last Name order or Last Name, First Name order for Telephone entries? - A matter of personal preference. If you are transferring your data to and from a PC database, it may impose a preference.

How should the Free Fields in Telephone and Business Card items be used? - The only real reason to use these fields is to keep the information separate for use in a PC database program. Otherwise you might just as well add additional lines to a single general Notes field (which is all that some models support). If you do use the Free Fields, try to be consistent in which information you put in which fields - it may help you if you need to transfer your data to a different model later.

My data disappeared after I entered it - Probably you are failing to complete the operation by pressing Set to store the data on models which have a Set key, or by pressing Next or OK enough times to complete the record on models which require this. Also see the suggestions below about using Search.

Search doesn’t seem to locate data correctly - Probably you have one of the following problems: 1) You are pressing Search to find the character string at the start of the record when you should be pressing Shift Search to find the character string anywhere in the record; 2) You are entering the UPPER/lower case wrong, since the search is case-sensitive; 3) You are searching in the wrong data mode (e.g. you are searching TEL when you entered the data in MEMO); 4) You have used a mixture of UPPER and lower case in your entries, causing them to appear in a different order than you expected (remember, on the Casio diary upper-case comes before lower case).

Should I Mark items to protect them against accidental deletion? - On models which allow this, you should only mark items which are truly permanent. Otherwise it just makes it more difficult to restore a complete backup from the PC without generating duplicates. The PC can restore your permanent items along with all the others.

My alarms don’t sound - You may have alarms disabled on the System Sounds function menu. Note that there are separate enable/disable settings for the main alarm, schedule alarms, and reminder alarms. Alternatively you may just not be hearing the alarms if you keep the diary in a case or pocket - the beeper isn’t very loud. If the diary is displaying the alarm item when you open it, then the alarm has been triggered.

How can I find out which field the text is in when displaying a record? - Text is displayed without field names, and with no marks distinguishing a line break within a field from the end of a field, but you can see more in data entry/editing mode. Edit the record and you will notice that a Return arrow is displayed for a line break, while a triangular field-end mark is displayed at the end of fields. On most models the field name prompt will appear when the cursor enters an empty field, or if you delete all the existing characters in a field,. You can delete all the characters in a field just to see the field name, then cancel the edit without saving.

I can’t find the symbol I want to type on the keyboard - The Casio diary has an extensive 256-symbol character set (not the same as the Windows ANSI character set though), but not all symbols appear on the keyboard. You may have to experiment to find out how to type the symbol you want on your model, e.g. try the Shift key with every regular key, same with the Code key, try the Smbl key if you have one, look for Symbols on the function menu.

Some of the special characters don’t match on the PC or another diary - The Casio diary has its own character set, which must be translated to the PC character set by the backup program. Each diary family has its own different character set, and the language-specific models for some languages (such as Russian or Chinese) have their own other character sets. You may have the wrong character set translation selected in your software, or your software may not handle this correctly. On some language-specific diary models, you can select whether you are sending data to a language-compatible unit or not, which causes the character set to be translated differently.

Can I prevent duplicates when restoring data to the diary from a backup? - Unfortunately most models do not detect or prevent duplication of data. For these models, it is easier to erase all contents of the diary and restore it completely from backup.

Is there any way to back up the diary settings along with the data? - No, only the data items can be sent and received. There is no way to back up settings like date and time, custom field names, key beep etc.

Should I keep the key beep sound turned on? - It does give feedback that you have pressed the key correctly, which is not always certain on these small membrane keyboards - but it also takes extra power and annoys people nearby in quiet settings. You can turn it off on the System/Sounds function menu.

How do I know when the batteries need replacing? - Most models tell you on the screen when you turn them on. Most models will give some early warning by doing this sporadically for a few days while still allowing you to clear the message and continue working  Then they start shutting off automatically. The message will identify whether it is the main or backup battery (or batteries) which need changing. You will usually notice that the display contrast starts to fade later in the battery life, and needs adjustment a few times. A common symptom of low batteries is that the display flickers and dims when you type or while you are communicating. Since communications is a heavy drain on power for several minutes at a time, it is possible to start encountering communications errors caused by low battery power during the operation without having received any warning at the start - you may even still be able to turn the unit off and on again without getting the low-battery warning at that point. If you start getting unusual communications errors, and the batteries are old, and you also notice symptoms like low display contrast or flickering display, try changing batteries.

Can I program the diary? - No. There is no programming capability of any kind on any Casio Digital Diary model. The closest you can get is the ability to enter spreadsheet formulas on the few models which support Lucid 3D spreadsheets. (Actually one company produced a programmable BASIC card for the old models which accept plug-in cards, but they are extremely rare).