Serial Data Stream Information

The PGLinkbox is designed to communicate with a Nintendo Power Glove.  It converts the proprietary Nintendo serial data format to a standard PC serial data stream.

The data sent out of the PGLinkbox box is at a baud rate of 4800 in a 8,N,1 format.

The PGLinkbox continuously streams 8 bytes of data communicating Power Glove events and location.  The byte output of the data stream is shown below.

8 Bytes

Byte 1 – This is a header byte (Set to hex A0).

Byte 2 – X axis positional information in relation to the sensor array.

Byte 3 – Y axis positional information in relation to the sensor array.

Byte 4 – Z axis positional information in relation to the sensor array.

Byte 5 – Rotation information.

Byte 6 – Finger status.

Byte 7 – Button status.

Byte 8 – End data stream (Set to hex 00).