VFX1LcdBrd.zip Changes

Here are some updates to the schematics and parts list found in the VFX1LcdBrd.zip file.

Additional Parts:

- There is a cap to ground on pin 9 of the IR3Y15 IC that is not shown on the schematics.  Its value is 0.1uF and mouser part# 81-GRM39X104K25.  This part is labeled C18 on the VFX1 LCD board.

- R3 is not shown as a missing part on the schematics.  It is in fact missing and its value is 1M ohm.  The part number for this component is already on the list.

- R5 is a missing component and its value should be 1M ohm.

- DPDT Switches.  These are for the video/RBG switch and the backlights.  Mouser part# 108-1MD3T1B1M1QE

- Pot Knobs for the 10k pots. Mouser part# 450-4760

- 5Volt 2A supply.  This is to power the VFX1. Mouser part#552-PSC-10A-050S

- Female end connector for the supply. Mouser part# 163-0302

Part # Changes:

- 1uF cap Mouser part# has been changed to 80-C1206C105M4V.  The original part was too small.  This part is labeled C7 and C2 on the VFX1 LCD board.

- 3.3K ohm resistor Mouser part# has been changed to 263-3.3K.  The original part was too small.  This part is labeled R36 on the VFX1 LCD board.

- 0.47uF cap Mouser part# has been changed to 77-VJ12Y25V474K.  The original part was too small.  This part is labeled C24 on the VFX1 LCD board.

- 39uH Inductor Mouser part# has been changed to 807-1210-393J.  The original part was too small.  This part is part of the T1 trap on the VFX1 LCD board.

- 47pF cap Mouser part# has been changed to 77-VJ12A50V470J.  The original part was too small.  This is part of the T1 trap on the VFX1 LCD board.

Missing Mouser Part #’s

- 10k Pots. 313-1250-10k

Additional Notes:

- R33 and R11 are 20k Pots in the viewfinder schematic but 10K ohm should work.

- R33(color level), R11(brightness), R26(tint), and R21(contrast) can all be 10K ohm pots.

- All resistors are of 5% tolerance.  1% tolerance would be better.

- One LCD board will also have the microphone signal lines going to the LCD as well.  Just wanted to make a note here.