Colour DOS

A guidline for the Disk Operating System of the EACA EG2000 Colour Genie Computer

Table of Contents.

1. Introduction

2. Floppy disk composition

3. File formats

4. The File Control Block (FCB)

5. DOS routines

6. Interrupt routine format

7. Compatibility with TRS 80 / Video Genie Operating Systems

8. Floppy disk formats

9. Reading alien floppy disks

Appendix A: Disk BASIC Memory usage.

Appendix B: Memory map

Appendix C: Error codes

Appendix D: System Entry Table

Appendix E: Description of the Restart commands

Appendix F: Vector tables for DOS / Disk BASIC

Appendix G: Program examples

# 1. Introduction

This document contains helpful information about the functioning and a more extensive use of DOS on the EG2000 Colour Genie computer. It gives an overview of the details of DOS, and how to access DOS from assembly language programs. Some program examples are included.

# 2. Floppy disk composition

This chapter gives an overview of the way that floppy disks are formatted and how the data on the floppy disk is distributed and managened.

The smallest data unit on a floppy disk is one sector. One sector always contains 256 bytes. Sectors are numbered. Machine programs can access sectors directly for either read or write operations.

Every 5 consecutive sectors form a granule (1280 bytes). In a normal floppy disk composition in files, that are addressed with their filename, the granule is the smallest addressable unit. This means  that every file occupies a certain number of granules. This can lead to the situation that DOS reports a floppy disk that contains a lot of small files as full, although the majority of sectors is unused.

Another unit that is important in the use of DOS, is the lump. The size of a lump depends on the floppy disk format:

| Floppy disk format | Granules per lump |
| --- | --- |
| A,E,I (SS,SD) | 2 |
| B,F,J (DS,SD) | 4 |
| C,G,K (SS,DD) | 3 |
| D,H,L (DS,DD) | 6 |

During formatting, 2 system files a written onto the floppy disk. These system files are hidden, and are not listed using the CMD"I" command:

DIR/SYS

This file contains the directory of the floppy disk. This is the only file that contains protected sectors. This means, that when reading this file, an error 6 is reported. This serves as an extra protection and recognition of this file. DIR/SYS differs in length depending on the floppy disk format.

NCW1983/JHL

This file always occupies the first granule (sector 0 to 4) on the floppy disk. Its main task is to make the floppy disks compatible with the operating systems for the TRS 80 and Video Genie computers (there it is called BOOT/SYS and it contains the load routine for DOS). On the Colour Genie, only the third byte of the first sector is important; it indicates, in which lump on the floppy disk, the directory starts.

For studying the floppy disk composition, a program like Colour-Zap is strongly recommended. Such a program allows you to look at the sectors on the floppy disk.

The first sector of  DIR/SYS contains information about the floppy disk.  The bytes 00H to 05H indicate the free granules on the floppy disk. Every byte corresponds with one lump. Starting with bit 0, only that number of bits is used, that corresponds with the number of granules per lump. So, bit 0 of the first byte indicates whether the first granule on the floppy disk is occupied (this granule is always occupied by NCW1983/SYS). If a granule is occupied, the corresponding bit is set to 1. Unused bits are always set to 1. A completely used lump has its corresponding byte set to FFH.

The bytes 60H to BFH are not used by the Colour Genie. They are used in some TRS80/Video Genie operating systems to indicate defective granules.

The bytes D0H to D7H contain name, the bytes D8H to DFH contain the date of the floppy disk.

The remaining bytes of the first sector have no meaning on the Colour Genie.

The second sector of DIR/SYS contains the hash code table for several files. Here, for every file, a hash code from the name is stored to speed up the search of a file. The position of a hash code in the second sector indicates, where the real entry in DIR/SYS is located. When one would imagine this sector as a table with 8 rows and 32 columns, then every column contains the hash codes for one of the next sectors. Column 0 for the third sector, column 1 for the fourth sector etc.. When a byte is 0, it means that there is no file entry at the corresponding location.

Byte 1FH of the second sector has a special meaning. It indicated the length of DIR/SYS-10. So, when is contains 5 (as with SS/DD floppy disks), DIR/SYS is occupies 15 sectors.

The next sectors contain the file entries for the files present.  Every file uses a block of 32 bytes, one sector can hold up to 8 files. There are 2 types of entries:

The first (and in most cases only) entry for a file and its continuation (necessary for long files, when the file is stored on different locations on the floppy disk). The bytes of the first entry contain following information:

| Byte 0: | Bit | Meaning when set |
| --- | --- | --- |
|  | 7 | Always 0 |
|  | 6 | System file |
|  | 5 | No meaning |
|  | 4 | Entry in use, contains a file |
|  | 3 | Hidden file, not listed using CMD"I" |
|  | 2-0 | Always 0 on the Colour Genie |

| Byte 1: | Bit | Meaning when set |
| --- | --- | --- |
|  | 7 | Always 0 |
|  | 6 | System file |
|  | 5-0 | No meaning |

| Byte 2: | No meaning |
| --- | --- |
| Byte 3: | Indicates which byte in the last sector of the file does not belong to the file anymore. Here the rule is 0=256. |
| Byte 4: | Not used by DOS |
| Bytes 5-12: | Contain the file name, padded with spaces. |
| Bytes 13-15 | Contain the file extension. |
| Bytes 16-19: | Always 0 on the Colour Genie |
| Bytes 13-15 | Indicate how many sectors are used by this file. |

The remaining bytes indicate the location of the data. It is possible that the file is divided into several blocks. For this reason, 4 pairs of bytes are reserved. If they are not sufficient, the file will obtain a second entry.

When the first byte of a pair equals FFH, the file has no more following blocks. The remaining byte pairs are meaningless.

When the first byte of a pair equals FEH, the file has an additional entry in the directory. The second byte indicates the location of the entry; the bits 0-4 show in which sector, and the bits 5-7 indicate which entry within the sector.

Otherwise, the first byte shows in which lump the data block belonging to the file starts. Bits 5-7 in the second byte then show, in which granule of this lump the data block starts, and the bits 0-4 indicate the length of the data block (in granules).

An additional entry of a file can be recognised by bit 7 of byte 0. Whenever this bit is set to 1, then byte 1 indicates the location of the previous entry (see above). Bytes 2-12 stay unused, and the bytes 23-31 have the same meaning as described above.

The separation of files into several data blocks may look somewhat complicated, but it allows to use of every free granule on the floppy disk.

Deleting a file is done by setting the first byte of the file entry and the hash code to zero. The disk space occupied by this file is then released. The data and all file entries remain unchanged.

# 3. File formats

This chapter gives an overview of the way that programs are stored on the floppy disk.

BASIC programs starts with a byte set to FFH. After that follows the program as it is stored in memory.

BASIC programs saved with the SAVE" ",A option, contain the ASCII characters, that are also shown by the LIST command.

Pascal source files created with the Colour Pascal 2.0 compiler are also stored as a text file containing the ASCII characters of the source file.

Machine language programs have a more complex format:

They are divided into blocks of 256 bytes maximum. Every block contains 4 extra bytes of load information:

| Byte | Meaning |
| --- | --- |
| 1 | 1 = Load data block. 2 = Last data block. 3 = Ignore data block. |
| 2 | Length+2: 2 means 256 bytes, 1 means 255 bytes, etc.. |
| 3-4 | Load address of the data block |

The last block of a file has only 4 bytes, 2 times 2 followed by the entry address of the program.

# 4. The File Control Block (FCB)

This chapter gives information about the FCB structure and the usage of an FCB.

To every opened file, a File Control Block is assigned. This FCB is the only connection to the file on the floppy disk. The computer does not store if and how many files are opened, but installs a FCB. All operations use and go through the FCB.

When using DOS routines, the address of the FCB used must be stored in register DE. Also, a buffer of 256 bytes is needed for every file, to read data into. The address of this buffer must be indicated when the file is opened.

The programmer indicates, when opening a file, where the FCB is located in memory. This requires a 32 byte memory area, that must be reserved for this purpose only.

Normally, DOS routines manage the FCB themselves; the programmer has just to call the right routines. It is however possible, to change the FCB for own purposes. The meaning of the bytes inside the FCB are as follows:

| Byte 0: | Bit | Meaning when set |
| --- | --- | --- |
|  | 7 | File opened |
|  | 6-2 | No meaning |
|  | 1 | File uses the complete floppy disk |
|  | 0 | Sectors are written protected (only DIR/SYS) |

| Byte 1: | Bit | Meaning when set |
| --- | --- | --- |
|  | 7 | Always 0 |
|  | 6 | System file |
|  | 5 | Buffer contains data of the next sector |
|  | 4 | Buffer contains data that still has to be written |
|  | 3 | Always set on the Colour Genie |
|  | 2-0 | Always 0 on the Colour Genie |

| Byte 2: | Identical to byte 1 of the directory entry |
| --- | --- |
| Byte 3-4: | Contain the address of the buffer |
| Byte 5: | Indicates which byte of the current sector will be read/written next |
| Byte 6: | Contains the used drive number |
| Byte 7: | Contains the corresponding directory entry position |
| Byte 8: | Identical to byte 3 of the directory entry |
| Byte 9: | Contains the record length (0=256 bytes) |
| Bytes 10-11: | Indicate the current sector that is processed |
| Bytes 12-21: | Correspond with the bytes 20-29 of the directory entry |
| Bytes 22-23 | FFH, if the file has only one directory entry. Else, they indicate the location of the second directory entry. |
| Bytes 24-31: | Correspond with the bytes 22-29 of the second directory entry (if present) |

For the use of most DOS routines, the address of the used FCB must be loaded into register pair DE. After returning, the zero flag is cleared in case no error occurred. If an error occurred, register A contains the error number. The error code is identical to the DISK-Error messages.

# 5. DOS routines

A list of the DOS routines, with a short description will follow. First, the register values that are expected are indicated. After that, the return values are given. If nothing else is mentioned, the routine will not change any registers except AF.

File Open : CE24H

Entry:

DE	:	FCB address

HL	:	Buffer address

B	:	Record length

in FCB	:	Filename

Return:

AF	: 	Error code

in FCB	:	see FCB

This routine opens an existing file on the floppy disk. The filename in the FCB must follow the usual conventions and must end with a 03 or 0DH. HL points to a buffer with a minimum size of 256 bytes.

Register B indicates the record length. This means how many bytes are processed at a read or write operation. With B = 0, data are processed sector wise, so with 256 bytes at a time. All values from 0 to 255 are valid.

File Initialise : CE20H

Entry:

DE	:	FCB address

HL	:	Buffer address

B	:	Record length

in FCB	:	Filename

Return:

AF	: 	Error code

in FCB	:	see FCB

This routine is similar to the file open, which is called first. When the file however does not exist, the file is created on the floppy disk. Therefor, this routine should only be used for write operations.

Read and write of single bytes.

For reading / writing single bytes, special easy to use routines are available. When opening the file, the record length must be put into register B.

Read one byte : 0013H

Entry:

DE	:	FCB address

Return:

AF	: 	Byte read, or error code when zero flag is clear.

Write one byte : 001BH

Entry:

DE	:	FCB address

A	:	Byte to be written

Return:

AF	: 	Error code

Read one record : CE36H

Entry:

DE	:	FCB address

HL	:	When record length = 0 : buffer address

Return:

AF	: 	Error code

in buffer:	Data read.

This routine reads one record. The record length is determined when the file is opened. If one works sector wise, the sector that is read is stored in the buffer that is indicated when the file is opened. If the record length = 0, the record is stored into a second buffer; register pair HL contains the address of this buffer.

Write on record : CE39H or CE3CH

Entry:

DE	:	FCB address

HL	:	When record length = 0 : buffer address

Return:

AF	: 	Error code

This routine writes one record. The record length is determined when the file is opened. If one works sector wise, the data must be present in the buffer that is indicated when the file is opened. Otherwise, the data must be present in a second buffer; register pair HL contains the address of this buffer. Routine CE3CH also verifies the data written.

File Close : CE28H

Entry:

DE	:	FCB address

Return:

AF	:	Error code

This routine closes a file a writes all necessary information onto the floppy disk. This routine must only be called, when the file has changed due to write operations.

Random Access Routines.

The following routines enable you to change the NEXT pointer that determines which record has to be read or written next.

Set NEXT to BC : CE42H

Entry:

DE	:	FCB address

BC	:	Record number

Return:

AF	:	Error code

This routine puts the NEXT pointer on the record indicated by the record number stored in register pair BC.

Set NEXT to 0 : CE3FH

Entry:

DE	:	FCB address

Return:

AF	:	Error code

This routine puts the NEXT pointer to the start of the file.

Set NEXT to EOF : CE48H

Entry:

DE	:	FCB address

Return:

AF	:	Error code

This routine puts the NEXT pointer to the end of file (only useful for writing).

Set NEXT to byte address : CE4EH

Entry:

DE	:	FCB address

HLC	:	Byte address

Return:

AF	:	Error code

This routine puts the NEXT pointer onto a byte, addressed by the 3 byte address stored into the registers HLC. This routine is useful when working with the byte routines 0013H and 001BH.

Decrement NEXT : CE45H

Entry:

DE	:	FCB address

Return:

AF	:	Error code

This routine decrement the NEXT pointer; the record last processed is used again.

Check Filename and copy into FCB : CE1CH

Entry:

DE	:	FCB address

HL	:	Address of filename

Return:

AF	:	Error code

This routine checks the indicated filename. If it complies to the rules, it will copy it into the FCB so that the file can be opened.

File Delete : CE2CH

Entry:

DE	:	FCB address

Return:

AF	:	Error code

This routine deleted the directory entry that belongs to the opened file.

Load Machine code program : CE30H

Entry:

DE	:	FCB address

in FCB	:	Filename

Return:

AF	:	Error code

This routine opens the indicated file and loads the machine code program into memory.

Execute Machine code program : CE33H

Entry:

DE	:	FCB address

in FCB	:	Filename

(does not return)

This routine opens the indicated file, loads the machine code program into memory and jumps to the entry address when no error occurred. If an error occurred, a DISK-Error message is given.

Enter EOF into Directory : CE51H

Entry:

DE	:	FCB address

Return:

AF	:	Error code

This routine enters an end of file into the directory of the floppy disk.

The routines mentioned before make full usage of files from machine code programs possible. There are however more routines available, that allow access to the floppy disk without using files. They should be used with care, since a wrong usage can destroy data on the floppy disk.

Drive Select : CE5BH

Entry:

A	:	Drive number

Return:

AF	:	Error code

This routine selects the drive for the all following read/write operations.

Drive Test : CE5EH

Entry:

A	:	Drive number

Return:

AF	:	Error code

This routine selects the drive A and tests, whether the drive is ready and if a floppy disk is inserted.

Read Sector : CF6FH

Entry:

DE	:	Sector number

HL	:	Buffer address

Return:

AF	:	Error code

in buffer:	Data read

This routine reads one sector and stores the data in a buffer. The sector number is present in register pair DE; the buffer address in register pair HL.

Test Sector : CF73H

Entry:

DE	:	Sector number

Return:

AF	:	Error code

This routine test if a sector can be read.

Write Sector : CF7FH

Entry:

DE	:	Sector number

HL	:	Buffer address

in buffer:	Data to be written

Return:

AF	:	Error code

This routine writes the data from a buffer to a floppy disk sector. The sector number is present in register pair DE; the buffer address in register pair HL.

Write Protected Sector : CF7BH

Entry:

DE	:	Sector number

HL	:	Buffer address

in buffer:	Data to be written

Return:

AF	:	Error code

This routine writes the data from a buffer to a floppy disk sector. The sector number is present in register pair DE; the buffer address in register pair HL. The sector is written with a mark. This normally only happens with directory sectors. When such a sector is read, an error 6 occurs.

Re-select Drive : CE16H

Return:

AF	:	corrupted

This routine again selects the last selected drive.

Read Directory sector : D25FH

Entry:

A	:	Directory sector number

Return:

AF	:	Error code

HL	:	Buffer address (5900H)

This routine reads a directory sector (A contains the sector number) and stores it into the system buffer at 5900H.

Write Directory sector : D274H

Entry:

in system buffer : Data to be written

Return:

AF	:	Error code

HL	:	corrupted

This routine writes the directory sector last read back to the floppy disk.

One should take care using the write routines. A floppy disk with a damaged directory becomes unreadable in most cases.

DOS also contains additional routines, that are not related to the usage of floppy disks, but can also be useful.

Issue Error Message : CE90H

Entry:

A	:	Error code

This routine generates a DISK-Error message and returns to BASIC.

Multiply : CE76H

Entry:

HL	:	Multiplicand

A	:	Multiplier

Return:

AHL	:	Result

This routine performs a multiplication. The result has a length of 3 bytes maximum.

Division : CE79H

Entry:

HL	:	Dividend

A	:	Divisor

Return:

HL	:	Result

A	:	Fraction

BC	:	corrupted

Give Time : CE6DH

Entry:

HL	:	Buffer address

Return:

in buffer:	The time as an 8 byte string

BC	:	corrupted

DE	:	corrupted

HL	:	corrupted

The time string uses the format hh:mm:ss.

Give Date : CE70H

Entry:

HL	:	Buffer address

Return:

in buffer:	The date as an 8 byte string

BC	:	corrupted

DE	:	corrupted

HL	:	corrupted

The date stored in the memory locations 4044H - 4046H is put in the buffer using the format dd.mm.yy

Insert Interrupt routine : CE10H

Entry:

DE	:	Address of the routine to be inserted

Return:

DE	:	corrupted

HL	:	corrupted

This routine inserts a program present in memory into the interrupt chain. This call has the same effect as the CMD "Y address" statement in BASIC. The program must not already be a part of the interrupt chain! It may not corrupt any Z80 register. This DOS call also executes a EI instruction.

Remove Interrupt routine : CE13H

Entry:

DE	:	Address of the routine to be removed

Return:

BC	:	Corrupted

DE	:	Corrupted

HL	:	Corrupted

This routine removes a program from the interrupt chain. This call has the same effect as the CMD"V address" statement in BASIC. This DOS call also executes a EI instruction.

# 6. Interrupt routine format

On the Colour Genie, it is possible to insert routines in a so called interrupt chain. Every 25 milliseconds, the computer walks through this chain and looks if routines should be executed.

A routine in this chain must have the following header:

| Address | Description |
| --- | --- |
| xxxx+0,xxxx+1 | Vector to next interrupt routine (set by system) |
| xxxx+2 | Initial timer value |
| xxxx+3 | timer counter value |
| xxxx+4 | Entry address of the interrupt routine. |

Inserting the routine is done from BASIC by using the CMD"Y xxxx" statement or by executing DOS call CE10H with the address xxxx in register DE. The system then inserts it into the chain and stores the vector of the next interrupt routine in the header. The address xxxx+2 contains a initial timer value.

The timer counter value at xxxx+3 is decremented every timer tick of 25 ms. When it reaches zero, the interrupt routine is executed and the initial timer value on address xxxx+2 is copied to the timer counter value of xxxx+3.

The address xxxx+4 is the entry address of the interrupt routine, here is where the executable code starts. The routine must not corrupt any registers or the stack!

Note that the Z80 CPU must be in interrupt mode 1 (IM 1) to perform the executing of the interrupt chain.

# 7. Compatibility with TRS 80 / Video Genie Operating Systems

The most important routines in DOS are compatible with the corresponding routines of the most TRS80 / Video Genie Operating Systems. The following routines are present in all these operating systems:

| Colour Genie | TRS 80 / Video Genie |
| --- | --- |
| 0013H | 0013H |
| 001BH | 001BH |
| CE20H | 4420H |
| CE24H | 4424H |
| CE36H | 4436H |
| CE39H | 4439H |
| CE3CH | 443CH |
| CE42H | 4442H |
| CE28H | 4428H |
| CE2CH | 442CH |

Other routines are not present in all operating systems. A routine that starts at CEXXH on the Colour Genie, starts at 44XXH on the TRS 80 / Video Genie

The floppy disks are fully exchangeable. The formats for BASIC and machine code programs is identical.

# 8. Floppy disk formats

The Colour Genie DOS can handle the following floppy disks formats:

SS = Single Sided

DS = Double Sided

SD = Single Density

DD = Double Density

| Sided | Density | Tracks | Sector/Track | Total Sectors | Capacity |
| --- | --- | --- | --- | --- | --- |
| SS | SD | 40 | 10 | 400 | 102 |
| SS | DD | 40 | 18 | 720 | 184 |
| SS | SD | 80 | 10 | 800 | 204 |
| SS | DD | 80 | 18 | 1440 | 368 |
| DS | SD | 40 | 20 | 800 | 204 |
| DS | DD | 40 | 36 | 1440 | 368 |
| DS | SD | 80 | 20 | 1600 | 408 |
| DS | DD | 80 | 36 | 2880 | 736 |

In order to achieve the right format, the floppy disk drive has to be set to the proper drive type. This can be done with the CMD"< drive# = Type" statement in BASIC. The drive# indicates for which of he 4 drives (0..3) the new type applies. Default at start-up is drive type C.

| Type | Floppy (Tracks) | Drive (Tracks) | Density | Sided |
| --- | --- | --- | --- | --- |
| A | 40 | 40 | SD | SS |
| B | 40 | 40 | SD | DS |
| C | 40 | 40 | DD | SS |
| D | 40 | 40 | DD | DS |
| E | 40 | 80 | SD | SS |
| F | 40 | 80 | SD | DS |
| G | 40 | 80 | DD | SS |
| H | 40 | 80 | DD | DS |
| I | 80 | 80 | SD | SS |
| J | 80 | 80 | SD | DS |
| K | 80 | 80 | DD | SS |
| L | 80 | 80 | DD | DS |

The types E, F, G and H are necessary for reading a 40 track floppy disk on a 80 track disk drive.

# 9. Reading alien floppy disks

It is possible for the Colour Genie to read floppy disks, that are made on the Genie I/II/III computers. Important are the Pdrive (see G-DOS manual) correspond to the settings of the Colour Disk BASIC, since no adaptation is possible from within the Colour Disk BASIC.

Following settings are required by the Colour Genie, so that alien floppy disks can be read:

| Type | Type Interface | Type Drive | Units In Block | Number Dir. Units | Start Block Directory |
| --- | --- | --- | --- | --- | --- |
| A | A | A | 2 | 2 | 20 |
| B | A | C | 4 | 4 | 20 |
| C | CK | E | 3 | 3 | 24 |
| D | CK | G | 6 | 6 | 24 |
| E | AL | A | 2 | 2 | 20 |
| F | AL | C | 4 | 4 | 20 |
| G | CKL | E | 3 | 3 | 24 |
| H | CKL | G | 6 | 6 | 24 |
| I | A | A | 2 | 2 | 40 |
| J | A | C | 4 | 4 | 40 |
| K | CK | E | 3 | 3 | 48 |
| L | CHK | G | 6 | 6 | 48 |

Floppy disks, made on the Colour Genie, can be read on the Genie I/II/III using the Pdrive settings above. The value for Start Block Directory however does not have to match.

Note that the Pdrive settings are not put on the floppy disks by the Colour Genie and a call of this table on a Genie I/II/III causes an error.

# Appendix A: Disk BASIC Memory usage.

Disk BASIC needs parts of the BASIC system memory (4000H - 43FFH) and parts of the normal memory from 5800H to 5C9AH, followed by the buffers (0-9) needed by files that are used by BASIC programs. Then the memory for BASIC programs follows.

The following memory locations are important under DOS :

| 4040H | 25 milliseconds counter for the time |
| --- | --- |
| 4041H | Seconds counter |
| 4042H | Minutes counter |
| 4043H | Hours counter |
| 4044H - 4046H | Date |
| 4050H - 4056H | Interrupt routine for the time |
| 4057H - 405DH | Interrupt routine for displaying the time |
| 405EH | Number of last read directory sector (for write routine at D274H) |
| 4076H | Hash code of the last opened file |
| 407CH - 407DH | Entry address for a program loaded with CMD"L" |
| 408EH | Number of files in Disk BASIC |
| 43CEH - 43E1H | Disk BASIC FCB addresses (2 byte pairs) |
| 43E3H | Error code of last reported Disk Error |
| 43ECH - 43FFH | USR functions entry addresses (2 bytes pairs) |
| 5800H - 58FFH | Buffer for Close and Kill |
| 5900H - 59FFH | System buffer for reading of directory |
| 5A08H | Actual drive number |
| 5A0AH - 5A13H | Actual drive data |
| 5A71H - 5A98H | Data for drives 0 - 3 |
| CEA0H - CF17H | Data for drive types A - L |

Drive data are loaded from EPROM to 5A71H - 5A98H when a CMD"<" statement is executed. When a drive is selected, the drive data of the selected drive is copied to 5A0AH - 5A13H. This data block has to following contents :

| 5A0AH | Number of first lump belonging to directory |
| --- | --- |
| 5A0BH | Number of lumps on floppy disk |
| 5A0CH | Stepper motor speed: 7 for SD, 53H for DD |
| 5A0DH | Number of tracks (40/80) |
| 5A0EH | Number of sectors per track |
| 5A0FH | Number of granules per lump |
| 5A10H | Always 0 |
| 5A11H | Flags used for control of floppy disk controller |
| 5A12H | Number of sectors per granule (always 5) |
| 5A13H | Directory length in granules |

# Appendix B: Memory map

|  | Hex | Decimal | Peek/Poke |
| --- | --- | --- | --- |
| ROM Cartridge (1 KB) | FFFF FC00 | 65535 64512 | -1 -1024 |
| Keyboard Matrix (1 KB) | FBFF F800 | 64511 63488 | -1025 -2048 |
| User-definable Characters (1 KB) | F7FF F400 | 63487 62464 | -2049 -3072 |
| Character Colour Memory (1 KB) | F3FF F000 | 62463 61440 | -3073 -4096 |
| DOS Utilities (4 KB) | EFFF E000 | 61439 57344 | -4097 -8192 |
| DOS Routines (4¼ KB) | DFFF CF00 | 57343 52992 | -8193 -12544 |
| DOS Vectors (¼ KB) | CEFF CE00 | 52991 52736 | -12545 -12800 |
| BASIC Vectors (3½ KB) | CDFF C000 | 52735 49152 | -12801 -16384 |
| User RAM (EG 2011) (16 KB) | BFFF 8000 | 49151 32768 | -16385 -32768 |
| User RAM (8 KB) | 7FFF 6000 | 32767 24576 | 32767 24576 |
| Program Buffers 1 to 3 (1 at lowest address) (1½ KB) | 5FFF 5A00 | 24575 23040 | 24575 23040 |
| I/O Buffer (DOS) (¼ KB) | 59FF 5900 | 23039 22784 | 23039 22784 |
| Floppy Disk Buffer (¼ KB) | 58FF 5800 | 22783 22528 | 22783 22528 |
| Graphic Screen Memory (4 KB) | 57FF 4800 | 22527 18432 | 22527 18432 |
| Text Screen Memory (1 KB) | 47FF 4400 | 18431 17408 | 18431 17408 |
| Vectors, Marks and System Variables (DOS and BASIC) (1 KB) | 43FF 4000 | 17407 16384 | 17407 16384 |
| BASIC Interpreter (EPROM) (16 KB) | 3FFF 0000 | 16383 0 | 16383 0 |

# Appendix C: Error codes

The following table contains the error codes and their meaning. This table is also valid for the G-DOS operating system (Genie I/II/III) and contains therefor some error codes that can not occur under the Colour Disk BASIC.

| Code |  |  | Error description |
| --- | --- | --- | --- |
| Decimal | Hex |  |  |
| 0 | 00 |  | No error |
| 1 | 01 |  | Bad file data |
| 2 | 02 |  | Read error: search error |
| 3 | 03 |  | Read error: data lost |
| 4 | 04 |  | Read error: checksum error |
| 5 | 05 |  | Read error: record not found |
| 6 | 06 |  | Read error: trying to read protected sectors |
| 7 | 07 |  | Read error: trying to read system sectors |
| 8 | 08 |  | Device unreachable |
| 9 | 09 |  | Undefined error code |
| 10 | 0A |  | Write error: search error |
| 11 | 0B |  | Write error: data lost |
| 12 | 0C |  | Write error: checksum error |
| 13 | 0D |  | Write error: record not found |
| 14 | 0E |  | Write error on disk drive |
| 15 | 0F |  | Floppy disk is write protected |
| 16 | 10 |  | Peripheral device unreachable |
| 17 | 11 |  | Directory read error |
| 18 | 12 |  | Directory write error |
| 19 | 13 |  | Illegal filename |
| 20 | 14 |  | Track number too high |
| 21 | 15 |  | Illegal DOS call function |
| 22 | 16 |  | Undefined error code |
| 23 | 17 |  | Undefined error code |
| 24 | 18 |  | File not in directory |
| 25 | 19 |  | Access denied by file |
| 26 | 1A |  | Directory full |
| 27 | 1B |  | Floppy disk full |
| 28 | 1C |  | End of file reached |
| 29 | 1D |  | Beyond end of file |
| 30 | 1E |  | Directory full: cannot extend file |
| 31 | 1F |  | File not found |
| 32 | 20 |  | Illegal or missing disk drive |
| 33 | 21 |  | No device reachable |
| 34 | 22 |  | Load error: bad format |
| 35 | 23 |  | Memory error |
| 36 | 24 |  | Try to load in ROM |
| 37 | 25 |  | Load error: access denied |
| 38 | 26 |  | File not open |
| 39 | 27 |  | Illegal initialisation data on floppy disk |
| 40 | 28 |  | Illegal track number |
| 41 | 29 |  | Illegal logical filenumber |
| 42 | 2A |  | Illegal DOS function |
| 43 | 2B |  | Illegal function under chaining |
| 44 | 2C |  | Directory incorrect |
| 45 | 2D |  | Bad FCB data |
| 46 | 2E |  | System program not found |
| 47 | 2F |  | Bad parameter |
| 48 | 30 |  | No filename |
| 49 | 31 |  | Bad floppy disk type |
| 50 | 32 |  | Read error BOOT |
| 51 | 33 |  | Fatal DOS error |
| 52 | 34 |  | Illegal abbreviation, separator or end marker |
| 53 | 35 |  | File already exists |
| 54 | 36 |  | Command too long |
| 55 | 37 |  | Access denied by floppy disk |
| 56 | 38 |  | Not a Mini-DOS function |
| 57 | 39 |  | Forced termination of function |
| 58 | 3A |  | Difference at verify |
| 59 | 3B |  | Insufficient memory |
| 60 | 3C |  | Incompatible drive or floppy disk |
| 61 | 3D |  | ADE=N attribute, cannot extend file |
| 62 | 3E |  | Cannot extend file at read |

# Appendix D: System Entry Table

| Address | Name | Description |
| --- | --- | --- |
| CE00H | $DOS | Return to command mode |
| CE05H | $DOSCMD | Calls machine code program or DOS command |
| CE08H | $NERROR | RET Z, else |
| CE09H | $DERROR | Report error message |
| CE0DH | $DEBUG | Call system monitor |
| CE10H | $ENQUE | Insert interrupt routine |
| CE13H | $DEQUE | Remove interrupt routine |
| CE16H | $RESEL | Reselect current disk drive |
| CE19H | $DOSCAL | as $DOSCMD, but returns |
| CE1CH | $EXFIL | Examine and transfer filename into FCB |
| CE20H | $INIT | Initialises file and opens FCB |
| CE24H | $OPEN | Opens FCB for current file |
| CE28H | $CLOSE | Closes FCB |
| CE2CH | $KILL | Deletes file entry |
| CE30H | $LOAD | Loads machine code program |
| CE33H | $RUN | Runs machine code program |
| CE36H | $RDSEC | Reads sector |
| CE39H | $WRSEC | Writes sector |
| CE3CH | $WRSECV | Writes and verifies sector |
| CE3FH | $POS0 | Set FCB sector number to zero |
| CE42H | $POSBC | Set FCB sector number to (BC) |
| CE45H | $POSDEC | Decrements FCB sector number |
| CE48H | $POSEOF | Sets FCB sector number at end of file |
| CE4BH | $ALLOC | Reserves unit on floppy disk for file |
| CE4EH | $POSBRA | Positions FCB at relative byte address |
| CE51H | $WREOF | Write end of file into directory |
| CE54H | $DELIM | Tests delimiters at parameters |
| CE5BH | $DRVSEL | Selects disk drive |
| CE5EH | $DSKMNT | Tests if floppy disk in drive |
| CE67H | $PRINT | Prints text on screen |
| CE6AH | $LPRINT | Prints text on printer |
| CE6DH | $CONTIM | Gives time as 8 bytes string |
| CE70H | $CONDAT | Gives date as 8 bytes string |
| CE76H | $MULT | Multiply A*HL=AHL |
| CE79H | $DIV | Divides HL div A=HL mod A |
| CE79H | $HEXDE | Gives hexadecimal representation of DE |
| CE80H | DOSFCB | System internal FCB of 32 byte length |

# Appendix E: Description of the Restart commands

The Restart commands (RST) are subroutines in Z80 machine language (like CALL routines). The advantage of RST commands is that they use little memory. A CALL command uses 3 bytes; a RST command uses only 1 byte. The following routines are used as subroutines in Colour Disk BASIC:

RST 00		Jumps to memory location 0 and corresponds with a cold boot

RST 08		Syntax check

The memory address that is addressed by HL is compared to the byte following the RST 08. If they are equal, a RST 10 is executed, else a syntax error is reported.

RST 10		Loads the memory location addressed by HL+1 into the accumulator A.

Blanks and Linefeeds are skipped. Digits cause the setting of the Carry-flag, a ':' or a 00 cause the setting of the Zero-flag

RST 18		Compares HL with DE register pair

HL > DE:	Zero-flag=0,	Carry-flag=0

HL = DE:	Zero-flag=1,	Carry-flag=0

HL < DE:	Zero-flag=0,	Carry-flag=1

RST 20		Tests the type of the contents of the X-register stored at address 40AFH.

Integer:		Zero-flag=0,	Carry-flag=1, 	Parity-Flag=1,	Sign-Flag=1

Single:		Zero-flag=0,	Carry-flag=1, 	Parity-Flag=0,	Sign-Flag=0

Double:		Zero-flag=0,	Carry-flag=0, 	Parity-Flag=1,	Sign-Flag=0

String:		Zero-flag=1,	Carry-flag=1, 	Parity-Flag=1,	Sign-Flag=0

RST 28		Used to load a system module. The Accumulator A contains the loader

code. RST 28 is also called when pressing the BREAK key.

RST 30		Generally used to call a DEBUG program. It returns after termination of the

DEBUG program.

RST 38		This routine is executed every 25 milliseconds if the Z80 CPU is set to

interrupt mode 1 (IM 1)

# Appendix F: Vector tables for DOS / Disk BASIC

The system variable space between 4000H and 43FFH contains the vectors for the Disk BASIC statements and the vectors into DOS. Every vectors requires 3 bytes (space necessary for a absolute jump instruction).

The first table occupies the area from 4152H to 41A5H. It contains the vectors for the Disk BASIC statements. Under Level II BASIC, these vectors all contain a jump to 013BH in the BASIC interpreter ROM area. Under Disk BASIC, the vectors contain a jump to the following addresses:

| Address (Dec, Hex) |  | Jump to: | Statement |
| --- | --- | --- | --- |
| 16722 | 4152H | C565H | CVI |
| 16725 | 4155H | C3AEH | FN |
| 16728 | 4158H | C562H | CVS |
| 16731 | 415BH | C353H | DEF |
| 16734 | 415EH | C55FH | CVD |
| 16737 | 4161H | CA25H | EOF |
| 16740 | 4164H | CA04H | LOC |
| 16743 | 4167H | C9FFH | LOF |
| 16746 | 416AH | C54EH | MKI$ |
| 16749 | 416DH | C54BH | MKS$ |
| 16752 | 4170H | C548H | MKD$ |
| 16755 | 4173H | C2FBH | CMD |
| 16758 | 4176H | C24DH | TIME$ |
| 16761 | 4179H | CB5CH | OPEN |
| 16764 | 417CH | C5D0H | FIELD |
| 16767 | 417FH | CAA1H | GET |
| 16770 | 4182H | CAA0H | PUT |
| 16773 | 4185H | C9C4H | CLOSE |
| 16776 | 4188H | C8D6H | LOAD |
| 16779 | 418BH | C962H | MERGE |
| 16782 | 418EH | CC0EH | NAME |
| 16785 | 4191H | CBEBH | KILL |
| 16788 | 4194H | 1E4AH | none (Previous &) |
| 16791 | 4197H | C57EH | LSET |
| 16794 | 417AH | C57FH | RSET |
| 16797 | 419DH | C456H | INSTR |
| 16800 | 41A0H | C99CH | SAVE |
| 16803 | 41A3H | C65BH | LINE |

The second table contains the BASIC vectors into DOS. Under Level II BASIC, these vectors contain a return instruction followed by 2 nops to fill the remaining 2 bytes memory space reserved for each vector. Under Disk BASIC, the vectors contain a jump to the following addresses:

| Address (Dec, Hex) |  | Jump to: | Description |
| --- | --- | --- | --- |
| 16806 | 41A6H | C1D3H | Error routine |
| 16809 | 41A9H | C390H | USR |
| 16812 | 41ACH | C959H | Return to active command mode |
| 16815 | 41AFH | C6BBH | Input line |
| 16818 | 41B2H | C98BH | After tokenizing |
| 16821 | 41B5H | C75FH | After accepting a new program line |
| 16824 | 41B8H | C76EH | After accepting a new program line |
| 16827 | 41BBH | C9F5H | Clear after cleaning all variables |
| 16830 | 41BEH | C685H | After ending printer output |
| 16833 | 41C1HHH | C6F2H | Character output |
| 16836 | 41C4H | C701H | Keyboard read at program execution |
| 16839 | 41C7H | C8D3H | RUN |
| 16842 | 41CAH | C62AH | PRINT |
| 16845 | 41CDH | C77CH | PRINT (numeric value) |
| 16848 | 41D0H | C77BH | Start of a new program line |
| 16851 | 41D3H | C747H | PRINT or PRINTTAB |
| 16854 | 41D6H | C68DH | INPUT |
| 16857 | 41D9H | C4DAH | MID$ left of = sign |
| 16860 | 41DCH | C7C2H | Data processing after READ / INPUT |
| 16863 | 41DFH | C6A5H | Termination of INPUT |
| 16866 | 41E2H | CC16H | SYSTEM |

# Appendix G: Program examples

Error report with return to program:

ERROR		PUSH	HL

PUSH	AF

CALL	2169H		;Output to screen

CALL	20F9H		;Cursor to start of line

LD	HL,TEXT1

CALL	2B75H		;Output text

POP	AF

LD	L,A

LD	H,0		;Error code to HL

CALL	0FAFH		;Output HL

CALL	20FEH		;Output CR

POP	HL

OR	FFH		;Clear Z flag

RET

TEXT1		DEFM	'Disk-Error '

DEFB	0

Open file with request for filename (ROPEN for read, WOPEN for write):

TEXT2		DEFM	'Filename: '

DEFB	0

OPEN		LD	HL,TEXT2

CALL	2B75H		;Output text

PUSH	BC		;Save record length

LD	HL,5B08H	;Buffer address

LD	B,18H		;Maximum length of input

CALL	5D9H		;Input filename

LD	C,B

LD	B,0

PUSH	HL

ADD	HL,BC

LD	(HL),0	;End marker

POP	HL

LD	DE,FCB	;FCB address

CALL	0CE1CH	;Copy filename

CALL	NZ,ERROR	;Call if error

POP	BC		;Restore record length

JR	NZ,OPEN	;Input new filename

LD	HL,BUFFER	;256 bytes buffer

CALL	0CE24H	;Open file

RET	Z		;If no error return

CP	18H		;Test error code

CALL	NZ,ERROR	;Report error

JR	NZ,OPEN	;Enter new filename

OR	A		;File not found

RET

ROPEN		CALL	OPEN		;Open file

RET	Z		;If no error return

CALL	ERROR		;Report error

JR	ROPEN		;Try again

WOPEN		CALL	OPEN		;Open file

JR	Z,EXI		;If file already exists jump

CALL	0D694H	;Create file

RET	Z		;If no error return

CALL	ERROR		;Report error

JR	WOPEN		;Try again

EXI		LD	HL,TEXT3

CALL	2B75H		;Ouput text

CALL	384H		;Wait for key pressed

CP	'Y'

RET	Z		;If file may be used return

JR	WOPEN		;Try again

TEXT3		DEFM	'File already exists.'

DEFB	0DH

DEFM	'Use anyway?'

DEFW	0DH

Close file:

CLOSE		LD	DE,FCB	;FCB address

CALL	0CE28H	;Close file

CALL	NZ,ERROR	;If failure report error

JP	NZ,PROG	;And jump to start of program

RET

Write byte:

WBYTE		PUSH	DE		;Save registers

PUSH	AF

LD	DE,FCB	;FCB address

CALL	1BH		;Write byte

CALL	NZ,ERROR

JR	NZ,PROG

POP	AF

POP	DE

RET

Read byte (Carry flag is set at end of file):

RBYTE		PUSH	DE		;Save register

LD	DE,FCB

CALL	13H		;Read byte

JR	Z,NOERR	;If no error jump

CP	1CH		;EOF code

CALL	NZ,ERROR	;If not EOF report error

JP	NZ,PROG

SCF

JR	EOF

NOERR		OR	A		;Clear carry flag

EOF		POP	DE

RET

Write a memory block from BSTART to BEND:

SAVE		LD	B,0		;Sector operation

CALL	WOPEN		;Open file

LD	HL,BSTART-1

LD	DE,BEND

LOOP		INC	HL

LD	A,(HL)	;Byte into A

CALL	WBYTE		;Write byte

RST	18H		;Compare HL and DE

JR	NZ,LOOP	;Loop if not equal

JP	CLOSE		;Close file

Load a memory block:

LOAD		LD	B,0		;Sector operation

CALL	ROPEN		;Open file

LD	HL,BSTART

LD	DE,BEND

LOOP1		CALL	RBYTE		;Read byte

RET	C		;If EOF then return

LD	(HL),A	;Store byte

INC	HL

JR	LOOP1		;Next byte

The next two routines make it possible to use an opened file as a memory of 64 KByte at maximum. READ corresponds with LD A,(HL) and WRITE corresponds with LD (HL),A .

SETPOS	PUSH	BC

PUSH	HL

LD	C,L		;Address to HLC

LD	L,H

LD	H,0

CALL	0CE4EH	;Set position

POP	HL

POP	BC

RET

READ		CALL	SETPOS	;Set position

JR	RBYTE		;Read byte to A

WRITE		CALL	SETPOS	;Set position

JR	WBYTE		;Write byte

Write records that contain a maximum of 20 characters, read from the keyboard:

WRITE1	LD	B,20		;Record length

CALL	WOPEN		;Open file

LOOP2		LD	HL,BUFFE1	;20 byte buffer

LD	B,20		;Maximum length

PUSH	HL

CALL	5D9H		;Input from keyboard into buffer

POP	HL

JP	C,CLOSE	;If BREAK key pressed jump

LD	DE,FCB	;FCB address

CALL	0CE39H	;Write record

CALL	NZ,ERROR

JP	NZ,PROG

JR	LOOP2		;Next record

Read the fifth record of the file created above:

READ1		LD	B,20		;Record length

CALL	ROPEN		;Open file

LD	DE,FCB	;FCB address

LD	BC,4		;Number of fifth record

CALL	0CE42H	;Set position

CALL	NZ,ERROR

JP	NZ,PROG

LD	HL,BUFFE1	;20 byte buffer

CALL	0CE36H	;Read record

CALL	NZ,ERROR

JP	NZ,PROG

RET

Calculate the sector number of the first directory sector of the floppy disk in drive 0:

CALC		LD	A,0		;Drive number

CALL	0CE5EH	;Test drive

CALL	NZ,ERROR

JP	NZ,PROG

LD	HL,0		;Sector number

LD	DE;BUFFE2	;256 byte buffer

CALL	0CF6FH	;Read sector 0

CALL	NZ,ERROR

JP	NZ,PROG

LD	A,(BUFFE2+2);Third byte into A

LD	L,A

LD	H,0

LD	A,(5A0FH)	;Granules per lump

CALL	0CE76H	;Multiplication

LD	A,(5A12H)	;Sectors per granule

CALL	0CE76H	;Multiplication

RET			;Result in HL

Calculate the number of free granules of the floppy disk in drive 0:

CALC1		LD	A,0		;Drive number

CALL	0CE5EH	;Test drive

CALL	NZ,ERROR

JP	NZ,PROG

XOR	A		;A=0

CALL	0D25FH	;Read directory sector 0

CALL	NZ,ERROR

JP	NZ,PROG

LD	IX,0		;Set counter on 0

LD	A,(5A0FH)	;Granules per lump

LD	D,A

LD	A,(5A0BH)	;Lumps on the floppy disk

LD	E,A

LD	HL,5900H	;Address of system buffer

LOOP3		LD	A,(HL)	;Read byte into A

LD	B,D

LOOP4		RRA			;Bit 0 into carry

JR	C,NTFREE	;If granule not free jump

INC	IX		;Increment counter

NTFREE	DJNZ	LOOP4

INC	HL		;Next byte

DEC	E

JR	NZ,LOOP3	;Loop for every lump

RET			;Result in IX

Change the name of the floppy disk:

NEWNAM	LD	A,0		;Drive number

CALL	0CE5EH	;Test drive

CALL	NZ,ERROR

JP	NZ,PROG

XOR	A

CALL	0D25FH	;Read directory sector 0

CALL	NZ,ERROR

JP	NZ,PROG

LD	DE,59D0H	;Address of floppy disk name in

;buffer

LD	HL,NAME	;Address of the new name

LD	BC,8		;Length of the name

LDIR			;Replace old with new name

CALL	0D274H	;Write directory sector 0 back to

;the floppy disk

CALL	NZ,ERROR

JP	NZ,PROG

RET

Insertion of an interrupt routine (the routine beeps every 2 seconds):

PING		LD	DE,INT	;Address of interrupt routine

PUSH	DE

CALL	0CE13H	;Just to be safe the routine is

;first removed from

;the interrupt chain

POP	DE

CALL	0CE10H	;Insert interrupt routine

RET

INT		DEFW	0		;Storage space for interrupt

;vector

DEFB	80		;Starting value for counter

;80*25ms

DEFB	80		;Counter

PUSH	AF		;Save registers

PUSH	BC

PUSH	DE

PUSH	HL

CALL	357CH		;Beep

POP	HL		;Restore registers

POP	DE

POP	BC

POP	AF

RET