- IFR CHECKLIST

| - 1) AIRCRAFT LOGS: Progressive Inspection COMPLIED WITH Static System W/IN 24 CAL. MONTHS Altimeter W/IN 24 CAL. MONTHS Transponder W/IN 24 CAL. MONTHS ELT COMPLIED WITH 91.207 - 2) PREFLIGHT INSPECTION: Approved Checklist COMPLETE Fuel ADEQUATE + RESERVE Oxygen REQUIRED Pitot Heat CHECK OPERATIVE Anti-Ice/De-Ice QUANTITY/CONDITION Aircraft Surfaces FROST/ICE REMOVED VOR Check CURRENT W/IN 30 DAYS - 3) BEFORE STARTING ENGINE: Approved Checklist COMPLETE Charts, Plates, Computer AVAILABLE Magnetic Compass FLUID/DEV. W/IN 10 Turn Coordinator FLAG CHECKED Inclinometer FLUID/BALL CENTERED Navigation Instruments FLAGS CHECKED Alternate Static Source CHECK OPERATIVE ATIS & IFR Clearance COPY - 4) STARTING ENGINE: Approved Checklist COMPLETE VOR Receivers CHECK OPERATIVE Marker Beacons TEST ADF Receiver CHECK OPERATIVE Turn Coordinator NO RED FLAG Pitot Heat ON/CHECK COMPASS/OFF - 5) TAXI: Magnetic Compass CHECK OPERATION Heading Indicator CHECK OPERATION Turn Coordinator BALL OUTSIDE TURN Attitude Indicator MAX BANK 5 DEGRES - 6) DEPARTURE BRIEFING: Take-Off REVIEW MINIMUMS Departing Freq. COM1 Approach OR Departure Freq COM2 2-Way Communication Failure REVIEW Approach Plates READY | - 7) PRE-TAKEOFF: Approved Checklist COMPLETE Charts & Plates IN SEQUENCE COMM / NAV Radios SET IFR Departure Procedure/SID BRIEF BEFORE T/O Transponder CODE SET - ON ALT Pitot Heat AS REQUIRED CLOCK SET - NOTE T/O TIME - 8) CLIMB, ENROUTE & DESCENT: Approved Checklist COMPLETE Heading Indicator CHECK EVERY 15 MINUTES - 9) BEFORE DESCENT TO INITIAL APPROACH FIX: ATIS COPY Gyros SET Altimeter SET COMM / NAV Radios SET MDA / DH NOTE Time to MAP NOTE Initial MAP Heading / Altitude NOTE MAP : (Options) CONSIDER - 10) BEFORE INITIAL APPROACH FIX: Approach Clearance COPY Approach Briefing COMPLETE Before Landing Checklist COMPLETE - 11) BEFORE FINAL APPROACH FIX (MENTAL REVIEW) GUMPS COMPLETE Minimums REVIEW Missed Approach Procedures REVIEW - 12) AFTER LANDING: Approved Checklist COMPLETE IFR Flight Plan CLOSE |
| --- | --- |